# Supplementary information

# Organic matter in drinking water or wastewater vary in its influence on micropollutants adsorption to activated carbon

Table S1 Tucker’s congruency coefficient (TCC) obtained in split-half analysis of PARAFAC model

Table S2 Ratio of mciropolllutants concentration after 7 and 14 days in relation to day 0 in                    biodegradability tests

Table S3 Freundlich coefficients fitted to adsorption isotherms

Fig. S1 Excitation and emission loadings at different wavelengths of models tested in the split-half analysis

Fig. S2 Micropollutants breakthrough curves in lab-scale GAC filter treating wastewater effluent spiked with micropollutants

Fig. S3 Micropollutants load on GAC over time in influent of GAC filters in drinking water plants (infGAC) and wastewater effluent (WWef) at three DOC levels: low ~ 3 mg/L, medium ~ 6 mg/L, high ~ 10 mg/L

Fig. S4 Fluorescence spectra of 4 components identified in the PARAFAC analysis

Fig. S5 Relative loading (effluent:influent) of PARAFAC components in effluent of lab-scale GAC filter treating wastewater treatment plant effluent spiked with micropollutants

Table S1 Tucker’s congruency coefficient (TCC) obtained in split-half analysis of PARAFAC model

| Component | Combination | TCC emission | TCC excitation |
| --- | --- | --- | --- |
| C1 | ABvsCD | 0.9997660 | 0.9820451 |
| C1 | ACvsBD | 0.9986763 | 0.9945762 |
| C1 | ADvsBC | 0.9999741 | 0.9997069 |
| C2 | ABvsCD | 0.9837282 | 0.9998252 |
| C2 | ACvsBD | 0.9959500 | 0.9997660 |
| C2 | ADvsBC | 0.9994648 | 0.9999225 |
| C3 | ABvsCD | 0.9992624 | 0.9960166 |
| C3 | ACvsBD | 0.9997029 | 0.9959541 |
| C3 | ADvsBC | 0.9994291 | 0.9987357 |
| C4 | ABvsCD | 0.9987918 | 0.9983389 |
| C4 | ACvsBD | 0.9940932 | 0.9997683 |
| C4 | ADvsBC | 0.9996818 | 0.9997756 |

Table S2 Ratio of mciropolllutants concentration after 7 and 14 days in relation to day 0 in                    biodegradability tests

| Micropollutant | Condition | Ratio concentration day 7:day 0 | Ratio concentration day 14:day 0 |
| --- | --- | --- | --- |
| Benzotriazole | abiotic | 1.06 | 1.14 |
| Benzotriazole | biotic | 0.97 | 0.96 |
| Carbamazepine | abiotic | 1.04 | 1.11 |
| Carbamazepine | biotic | 0.98 | 0.96 |
| DEET | abiotic | 1.04 | 1.16 |
| DEET | biotic | 0.99 | 0.98 |
| Furosemide | abiotic | 1.15 | 1.20 |
| Furosemide | biotic | 1.03 | 1.00 |
| Mecoprop | abiotic | 1.10 | 1.17 |
| Mecoprop | biotic | 1.00 | 1.00 |
| Pyrimethanil | abiotic | 1.05 | 1.16 |
| Pyrimethanil | biotic | 1.02 | 0.97 |
| Sulfamethoxazole | abiotic | 1.06 | 1.15 |
| Sulfamethoxazole | biotic | 0.99 | 0.96 |
| Trimethoprim | abiotic | 1.08 | 1.17 |
| Trimethoprim | biotic | 0.99 | 0.91 |

Table S3 Freundlich coefficients fitted to adsorption isotherms

| Matrix | DOC level | Micropollutant | Kf | n | R2 |
| --- | --- | --- | --- | --- | --- |
| demi | 0 | Benzotriazole | 4231 | 1.11 | 0.72 |
| WWef | low |  | 3464 | 2.63 | 0.99 |
|  | medium |  | 3450 | 3.75 | 0.91 |
|  | high |  | 1878 | 2.57 | 0.80 |
| infGAC | low |  | 4426 | 3.89 | 0.97 |
|  | medium |  | 4230 | 3.16 | 0.90 |
|  | high |  | NA | NA | NA |
| demi | 0 | Carbamazepine | NA | NA | NA |
| WWef | low |  | 4315 | 1.43 | 0.87 |
|  | medium |  | 2894 | 2.61 | 0.77 |
|  | high |  | 192 | 1.75 | 0.83 |
| infGAC | low |  | 12707 | 0.80 | 0.88 |
|  | medium |  | NA | NA | NA |
|  | high |  | 137 | 1.76 | 0.80 |
| demi | 0 | DEET | 11864 | 0.77 | 0.88 |
| WWef | low |  | 3106 | 1.85 | 0.84 |
|  | medium |  | 2168 | 2.92 | 0.81 |
|  | high |  | 386 | 2.49 | 0.70 |
| infGAC | low |  | 5103 | 1.47 | 0.75 |
|  | medium |  | NA | NA | NA |
|  | high |  | 110 | 1.64 | 0.87 |
| demi | 0 | Furosemide | NA | NA | NA |
| WWef | low |  | 1865 | 3.25 | 0.80 |
|  | medium |  | 1402 | 3.70 | 0.83 |
|  | high |  | 1 | 0.89 | 28.75 |
| infGAC | low |  | NA | NA | NA |
|  | medium |  | NA | NA | NA |
|  | high |  | 1 | 0.90 | 31.04 |
| demi | 0 | Mecoprop | NA | NA | NA |
| WWef | low |  | 868 | 2.50 | 0.78 |
|  | medium |  | 749 | 3.02 | 0.82 |
|  | high |  | 8 | 1.01 | 0.87 |
| infGAC | low |  | NA | NA | NA |
|  | medium |  | NA | NA | NA |
|  | high |  | 27 | 1.42 | 0.83 |
| demi | 0 | Pyrimethanil | NA | NA | NA |
| WWef | low |  | 8612 | 0.86 | 0.81 |
|  | medium |  | 4244 | 1.05 | 0.93 |
|  | high |  | NA | NA | NA |
| infGAC | low |  | 14840 | 0.90 | 0.70 |
|  | medium |  | 5530 | 0.60 | 0.77 |
|  | high |  | 309 | 1.99 | 0.74 |
| demi | 0 | Sulfamethoxazole | 3967 | 1.41 | 0.84 |
| WWef | low |  | 2767 | 2.16 | 0.92 |
|  | medium |  | 1690 | 3.16 | 0.83 |
|  | high |  | 62 | 1.37 | 0.71 |
| infGAC | low |  | 6125 | 1.99 | 0.84 |
|  | medium |  | NA | NA | NA |
|  | high |  | 219 | 2.08 | 0.82 |
| demi | 0 | Trimethoprim | NA | NA | NA |
| WWef | low |  | 6242 | 1.05 | 0.92 |
|  | medium |  | 4266 | 1.67 | 0.86 |
|  | high |  | NA | NA | NA |
| infGAC | low |  | 6557 | 1.27 | 0.84 |
|  | medium |  | 6390 | 0.72 | 0.72 |
|  | high |  | 140 | 1.73 | 0.77 |

Fig. S1 Excitation and emission loadings at different wavelengths of models tested in the split-half analysis

Fig. S2 Micropollutants breakthrough curves in lab-scale GAC filter treating wastewater effluent spiked with micropollutants

Fig. S3 Micropollutants load on GAC over time in influent of GAC filters in drinking water plants (infGAC) and wastewater effluent (WWef) at three DOC levels: low ~ 3 mg/L, medium ~ 6 mg/L, high ~ 10 mg/L

Fig. S4 Fluorescence spectra of 4 components identified in the PARAFAC analysis

Fig. S5 Relative loading (effluent:influent) of PARAFAC components in effluent of lab-scale GAC filter treating wastewater effluent spiked with micropollutants