SUPPLEMENTARY INFORMATION

Feasibility of Fenton-Based Oxidation for Reducing Organic Content, Toxicity, and Carbon Footprint of Automotive Industry Wastewater

Table S1 Selected levels of independent variables for Fenton oxidation experiments

| Variable | Independent variable | Low level | High level |
| --- | --- | --- | --- |
| A | H2O2/COD molar ratio (mol mol-1) | 0.58 | 2.62 |
| B | Fe concentration (g L-1) | 0.58 | 2.92 |
| C | Reaction time (minute) | 30 | 90 |

Table S2 Details of the UV fluorescent lamp

| Model | UVC T8 30W G13 |
| --- | --- |
| W | 30 |
| Vac – Hz - I | 220 V – 56/60 Hz – 370 mA |
| nm | 253.7 |
| L70 (h) | 10800 |
| W [UV] | 12.6 |
| T [Celcius] (oC) | 15-50 |
| I × d (mm) | 893.4 × 25.5 |

Fig. S1 Actual COD concentrations (%) predicted by the model and measured in Fenton Oxidation Experiments

Table S3 ANOVA test results for the COD model developed for Fenton oxidation

| Source | Sum of squares | Mean squares | F value | p-value Prob>F |
| --- | --- | --- | --- | --- |
| Model | 1344.80 | 112.07 | 29.81 | < 0.0001 |
| A-H2O2/COD | 324.80 | 324.80 | 86.41 | < 0.0001 |
| B-Fe2+ concentration | 343.51 | 343.51 | 91.39 | < 0.0001 |
| C-Time | 2.09 | 2.09 | 0.56 | 0.4767 |
| AB | 34.40 | 34.40 | 9.15 | 0.0164 |
| AC | 1.66 | 1.66 | 0.44 | 0.5250 |
| BC | 105.79 | 105.79 | 28.14 | 0.0007 |
| A2 | 1.36 | 1.36 | 0.36 | 0.5647 |
| B2 | 18.41 | 18.41 | 4.90 | 0.0578 |
| C2 | 23.76 | 23.76 | 6.32 | 0.0361 |
| ABC | 27.72 | 27.72 | 7.37 | 0.0264 |
| A2B | 96.18 | 96.18 | 25.59 | 0.0010 |
| B2C | 19.36 | 19.36 | 5.15 | 0.0529 |
| Model discrepancy | 29.11 | 4.16 | 4.35 | 0.3538 |

Fig. S2 Actual COD concentrations (%) predicted by the model and measured in Fenton Oxidation Experiments

Table S4 ANOVA test results for the COD model developed for Fenton-based oxidation

| Source | Sum of squares | Mean squares | F value | p-value Prob>F |
| --- | --- | --- | --- | --- |
| Model | 682.50 | 62.05 | 23.65 | < 0.0001 |
| A-H2O2/COD | 99.17 | 99.17 | 37.80 | 0.0003 |
| B-Fe3+ concentration | 206.61 | 206.61 | 78.75 | < 0.0001 |
| C-Time | 102.10 | 102.10 | 38.92 | 0.0002 |
| AB | 82.55 | 82.55 | 31.47 | 0.0005 |
| AC | 3.74 | 3.74 | 1.42 | 0.2669 |
| BC | 16.09 | 16.09 | 6.13 | 0.0383 |
| A2 | 81.85 | 81.85 | 31.20 | 0.0005 |
| B2 | 7.45 | 7.45 | 2.84 | 0.1304 |
| C2 | 35.01 | 35.01 | 13.35 | 0.0065 |
| ABC | 91.08 | 91.08 | 34.72 | 0.0004 |
| A2B | 58.33 | 58.33 | 22.23 | 0.0015 |
| B2C | 17.67 | 2.52 | 0.76 | 0.7107 |
| Model discrepancy | 682.50 | 62.05 | 23.65 | < 0.0001 |