SUPPLEMENTARY MATERIAL

Removal of Antibiotic Resistant Bacteria and Genes by Conventional and  Nature-based Municipal Wastewater Treatment Systems

Maria Kalli1*, Dimitrios Kontogiannatos2, Angeliki Maria Koi1, Marina Angeli1, Constantinos Noutsopoulos1, Dimitrios G. Georgakopoulos2, Simos Malamis1, Daniel Mamais1

1Sanitary Engineering Laboratory, Department of Water Resources and Environmental Engineering, School of Civil Engineering, National Technical University of Athens, 15780 Athens, Greece

2Laboratory of General and Agricultural Microbiology, Department of Crop Science, Agricultural University of Athens, 11855 Athens, Greece

*kalli_maria@mail.ntua.gr

Table SM1. Concentration of total coliforms in WWTP1.

|  | Total coliforms (CFU/100 mL) |
| --- | --- |
| In | 4.5×107 ± 6.1×106 |
| Primary treated effluent | 9×106 ± 1.8×106 |
| Secondary treated effluent | 4.5×104 ± 6×103 |

Table SM2. Concentration of total coliforms in WWTP2.

|  | Total coliforms (CFU/100 mL) |
| --- | --- |
| In | 3.9×107 ± 5.8×105 |
| Effluent of UASBs | 1.8×107 ± 3.3×106 |
| Effluent of CWs | 3.9×106 ± 1.2×106 |
| Effluent of UV | 8.9×104 ± 1.2×104 |

Table SM3. Standard curves for the quantification of studied genes.

| Target gene | Primers | Primers sequence | Annealing Temperature (°C) | Product (bp) | References |
| --- | --- | --- | --- | --- | --- |
| 16S rRNA | F1048 | GTGSTGCAYGGYTGTCGTCA | 60 | 146 | (Maeda et al. 2003) |
|  | R1194 | ACGTCRTCCMCACCTTCCTC |  |  |  |
| blaTEM | blaTEM-FW | GCKGCCAACTTACTTCTGACAACG | 60 | 247 | (Xi et al. 2009) |
|  | blaTEM-RV | CTTTATCCGCCTCCATCCAGTCTA |  |  |  |
| qnrS | qnrS-FW | GACGTGCTAACTTGCGTGAT | 62 | 240 | (Marti and Balcázar, 2013) |
|  | qnrS-RV | TGGCATTGTTGGAAACTTG |  |  |  |
| ermB | ermB-FW | GATACCGTTTACGAAATTGG | 58 | 364 | (Chen et al. 2007) |
|  | ermB-RV | GAATCGAGACTTGAGTGTGC |  |  |  |
| sulI | sul1-FW | CGCACCGGAAACATCGCTGCAC | 55.9 | 163 | (Pei et al. 2006) |
|  | sul1-RV | TGAAGTTCCGCCGCAAGGCTCG |  |  |  |
| sulII | sul2-FW | TCCGGTGGAGGCCGGTATCTGG | 55.9 | 191 | (Pei et al. 2006) |
|  | sul2-RV | CGGGAATGCCATCTGCCTTGAG |  |  |  |
| tetA | tetA-FW | GCTACATCCTGCTTGCCTTC | 64 | 210 | (Fonseca et al. 2006) |
|  | tetA-RV | CATAGATCGCCGTGAAGAGG |  |  |  |
| intI1 | intI1-FW | GCCTTGATGTTACCCGAGAG | 60 | 196 | (Barraud et al. 2010) |
|  | intI1-RV | GATCGGTCGAATGCGTGT |  |  |  |
| uidA | uidA-FW | TGGTAATTACCGACGAAAACGGC | 55 | 162 | (Bej et al. 1990) |
|  | uidA-RV | ACGCGTGGTTACAGTCTTGCG |  |  |  |

Table SM4. List of primers and conditions used to detect the target genes.

Table SM5. Main qualitative characteristics of the secondary treated effluent of WWTP1 and constructed wetlands-treated effluent of WWTP2.

| Parameter | Wastewater type |  |
| --- | --- | --- |
|  | Secondary treated effluent from WWTP1 | Constructed wetlands-treated effluent from WWTP2 |
| pH | 7.8 ± 0.1 | 6.8 ± 0.1 |
| Turbidity | 2.0 ± 0.1 NTU | 2.0 ± 0.2 NTU |
| Chemical Oxygen Demand (COD) | 32.0 ± 5.4 mg L−1 | 35.8 ± 3.1 mg L−1 |
| Total Suspended Solids (TSS) | 16.0 ± 3.5 mg L−1 | 0.4 ± 0.1 mg L−1 |
| Nitrates (NO3-N) | 7.17 ± 2.7 mg L−1 | 50.5 ± 3.6 mg L−1 |
| Nitrite (NO2-N) | 0.08 ± 0.02 mg L−1 | 0.10 ± 0.02 mg L−1 |
| Ammonium (NH4-N) | 1.4 ± 1.2 mg L−1 | 0.7± 0.6 mg L−1 |

Table SM6. Average WWTP1 and WWTP2 operational parameters

|  | WWTP1 | WWTP2 |
| --- | --- | --- |
| Qin (m3 d-1) | 730,000 | 60 |
| Temperature (°C) | 19.1 | 19.5 |
| UASB Hydraulic Retention Time, HRTUASB (h) | n/a | 16.4 |
| UASB Organic Loding Rate, OLRUASB (kgCOD m-3 d-1) | n/a | 0.9 |
| CW Hydraulic Retention Time, HRTVF,SAT (d) | n/a | 7 |
| CW Hydraulic Loading Rate, HLR (L m-2 d-1) | n/a | 71 |
| CW Organic Loding Rate, OLR (gCOD m-2 d-1) | n/a | 28 |
| CW Solids Loading Rate, SLRVF,SAT (gTSS m-2 d-1) | n/a | 14 |
| HRT of primary treatment (h) | 1.2 | n/a |
| HRT of secondary activated sludge system (h) | 16.6 | n/a |
| Activated Sludge Solids Retention Time (d) | 8 | n/a |
| Activated Sludge OLR (kgCOD m-3 d-1) | 1.0 | n/a |

References

Barraud O, Baclet MC, Denis F, Ploy MC (2010) Quantitative multiplex real-time PCR for detecting class 1, 2 and 3 integrons. Journal of Antimicrobial Chemotherapy 65:1642–1645. https://doi.org/10.1093/JAC/DKQ167

Bej AK, Steffan RJ, DiCesare J, Haff L, Atlas RM (1990) Detection of coliform bacteria in water by polymerase chain reaction and gene probes. Appl Environ Microbiol 56:307. https://doi.org/10.1128/AEM.56.2.307-314.1990

Chen J, Yu Z, Michel FC, Wittum T, Morrison M (2007) Development and application of real-time PCR assays for quantification of erm genes conferring resistance to macrolides-lincosamides-streptogramin B in livestock manure and manure management systems. Appl Environ Microbiol 73:4407–4416. https://doi.org/10.1128/AEM.02799-06

Fonseca EL, Mykytczuk OL, Asensi MD, Reis EMF, Ferraz LR, Paula FL, Ng LK, Rodrigues DP (2006) Clonality and antimicrobial resistance gene profiles of multidrug-resistant Salmonella enterica serovar infantis isolates from four public hospitals in Rio de Janeiro, Brazil. J Clin Microbiol 44:2767–2772. https://doi.org/10.1128/jcm.01916-05

Maeda H, Fujimoto C, Haruki Y, Maeda T, Kokeguchi S, Petelin M, Arai H, Tanimoto I, Nishimura F, Takashiba S (2003) Quantitative real-time PCR using TaqMan and SYBR Green for Actinobacillus actinomycetemcomitans, Porphyromonas gingivalis, Prevotella intermedia, tetQ gene and total bacteria. FEMS Immunol Med Microbiol 39:81–86. https://doi.org/10.1016/S0928-8244(03)00224-4

Marti E, Balcázar JL (2013) Real-time PCR assays for quantification of qnr genes in environmental water samples and chicken feces. Appl Environ Microbiol 79:1743–1745. https://doi.org/10.1128/AEM.03409-12

Pei R, Kim SC, Carlson KH, Pruden A (2006) Effect of River Landscape on the sediment concentrations of antibiotics and corresponding antibiotic resistance genes (ARG). Water Res 40:2427–2435. https://doi.org/10.1016/J.WATRES.2006.04.017

Xi C, Zhang Y, Marrs CF, Ye W, Simon C, Foxman B, Nriagu J (2009) Prevalence of antibiotic resistance in drinking water treatment and distribution systems. Appl Environ Microbiol 75:5714–5718. https://doi.org/10.1128/AEM.00382-09