IMPACT OF CLIMATE CHANGE ON THE HYDROLOGICAL REGIME IN A SMALL WATERSHED OF THE TROPICAL ANDEAN PLATEAU

Darwin Mena-Rentería1 , Carlos Peña-Guzmán2, Ronal Sierra1, Manuel Rodriguez3

1Facultad de Ingeniería Ambiental, Universidad Santo Tomas, Cra 9 # 51-11, Bogotá, Colombia

4Programa de Ingeniería Ambiental y Sanitaria, Universidad de La Salle, Cra. 2 #No 10-70, Bogotá, Colombia

3École Supérieure D'aménagement Du Territoire et de Développement Régional, Université Laval, Québec, Canada

Abstract

Water availability has become a major worldwide concern due to climate change. This study evaluated the impact of climate change on the hydrological regime of a small watershed in the Tropical Andean Plateau in Colombia. We used the modeling tool Hydro-BID, a specialized software with hydrological and climatic modules that can evaluate hydrological processes to generate models. This tool has the potential to inform water resource management and planning in Latin America and the Caribbean. To simulate future hydrologic flows, we selected the MOI-ESM-MR-CORDEX model for precipitation and the MOI-ESM-MR model for temperature. Projections for precipitation and temperature on a reduced scale were calculated using a method based on chaos theory for three representative concentration pathway (RCP) scenarios designated 2.6, 4.5, and 8.5 based on global climate models (GCMs) and regional climate models (RCMs). The results of the calibration process showed that the evaluated hydrological parameters are better represented by the model at a monthly scale. In addition, the climate change model showed an estimated increase in the average streamflow of 2.1% to 6% compared to a base scenario that represented current conditions (1.22 m3 s-1), with flow rates of 1.28 m3 s-1, 1.24 m3 s-1, and 1.29 m3 s-1 for RCPs 2.6, 4.5, and 8.5, respectively. These values, associated with a significant increase in precipitation and a decrease in temperature, are within the expected ranges?/are expected? for the Tropical Andean Plateau watersheds of Colombia. The results obtained in this study are useful for water resource planners and policy makers, especially for the management of water supply infrastructure, flood risk, and loss of agricultural productivity in the area.

Keywords: climate change, Hydro-BID, hydrological modeling, representative concentration pathway (RCP)

1. Introduction

There is a consensus that climate change will cause changes in variables such as global temperature and precipitation. Projecting the trends of these variables are helpful for assessing the impact of climate change on future hydrological regimes, both globally and regionally (Jain and Kumar 2012). Since water crises are expected to increase due to climate change, estimates of water availability under future climatic conditions must become an integral part of water resource planning, land development, and environmental management (Bhatt and Mall 2015).

Consequences of climate variability for watersheds include changes in water availability and hydrological processes, as well as loss of watershed productivity due to floods or droughts, which in turn have social and cultural impacts on communities. Due to their topographic characteristics, size, and ecological, economic, and social importance for South America, climate change impacts on the Andean basins have been studied in recent years. For example, Ragettli et al. (2016) found that runoff in a watershed in Chile decreased significantly under different representative concentration pathway (RCP) scenarios. Similarly, Vargas et al. (2013) identified the most critical conditions for water availability under an RCP 8.5 scenario in the Chilean Andean basins. Lavado et al. (2011) simulated the potential effects of climate change in two watersheds in Peru. They found increased levels of precipitation in the winter season and a significant decrease in precipitation, ranging from 0 to 20%, in other seasons of the year. In Ecuador, Mora et al. (2014) found an increase in the maximum peak flows and a significant decrease in the low flows in a watershed essential for agriculture in that region. Montenegro et al. (2022) concluded that in the tropical Andean basin of Ecuador, climate change will cause an increase in precipitation in all seasons. In Colombia, Munar et al. (2023) found that there will be an increase in precipitation under different climate scenarios. Similarly, Mena et al. (2020) concluded that the greatest reduction in stream flow, between 2.6% and 9.6%, will occur between 2013 and 2040.

In Colombia, water resource management is a high-priority issue due to the large proportion of the? population living with a high index of water scarcity (Dominguez et al. 2008). The effects of the El Niño and La Niña phenomena (referred to jointly as ENSO) on the flow rates in Colombia are of great importance. The El Niño phenomenon causes large reductions in flow rates in river basins, sometimes generating water shortages of more than 30%, while the La Niña phenomenon generates increases in flow rates of over 60% (Poveda and Mesa 1996; Hoyos et al. 2013; Córdoba-Machado et al. 2015). The La Niña phenomenon caused an increase in precipitation of more than 20% during the 2020–2023 period, reaching historic highs in multiple cities and municipalities in Colombia (Manciu et al. 2023; Vega et al. 2024). Meanwhile, the El Niño phenomenon is currently causing droughts and high temperatures that will lead to a water deficit in Colombia’s energy production and water consumption by 2024.

To assess the impact of climate change on the hydrological regime at the watershed level, water runoff projections had been made from hydrological models driven by General Circulation Models (GCMs) and/or Regional Climate Models (RCMs) (Tan et al. 2017). GCMs are poorly suited as inputs for water resource impact assessment models because of bias in the representation of the current climate and uncertainties in future projections (Gutmann et al. 2011; Meenu et al. 2013; Dixon et al. 2016; Lutz et al. 2016; Gebrechorkos et al. 2019). More authors are starting to use/use different techniques to obtain accurate precipitation data, applying different and more holistic rainfall-runoff models (Mohammadi et al. 2023, 2024).

Researchers have obtained more detailed information by using RCMs to make climate projections on a regional scale. According to the Intergovernmental Panel on Climate Change (IPCC), the number of regional climate modeling studies worldwide has increased over the last decade (Cabré et al. 2016). In recent years, a large new set of dynamically-reduced-scale climate model simulations were generated worldwide under the global Coordinated Regional Climate Downscaling Experiment (CORDEX) initiative (Stanzel et al. 2018). The CORDEX project has been used in South America, leading to enormous progress in the quantification of uncertainty in projections of regional climate (Solman 2013). However, to our knowledge, it has not been applied specifically to Colombia.

Hydrological models are a robust way to estimate hydrological conditions under different climate change scenarios, with studies using them to assess the impact of climate change on water availability (Reshmidevi et al. 2018; Shrestha et al. 2018; Gebrechorkos et al. 2019; Bhatta et al. 2019; Puno et al. 2019; Zhang et al. 2019). Hydro-BID, a modeling tool for Latin America and the Caribbean, has been used to estimate water resources for a base period and then to simulate future hydrological changes due to climate change (Mena et al. 2019, 2020). This software has been applied in different countries in the region. Hydro-BID was used in Argentina to assess climate change impacts and hydrological and water demand analysis  (Moreda et al. 2014b; Wyatt et al. 2014), Peru to assess climate change impacts on Andean watersheds and for hydrological and sediment analysis in rivers (Moreda and Daussa 2016; Mamani Cosi 2018), Ecuador to assess climate change impacts and for hydrological water supply analysis (Moreda and Daussa 2017; Bravo Ortiz 2020; León Olmedo 2020), and Colombia to determine how climate change alters flows and for hydrological and water supply/demand analysis (Mena et al. 2019, 2020; Leveri et al. 2020).

In Colombia, the Balsillas River watershed is particularly important for agriculture, ecology, and drinking water. As for many watersheds in Colombia, it’s essential to determine the current and future impacts of climate change, which may cause drastic social, environmental, or economic changes (Sauchyn et al. 2016; Villamizar et al. 2019; Mena et al. 2020). Thus, there is an urgent need to propose decision-making tools suitable for developing countries for environmental protection and land use planning.

This article combines Hydro-BID software and the CORDEX dataset to assess the expected impacts of climate change on the hydrological regime of the Balsillas River watershed in Colombia in order to support the development of sustainable adaptation and mitigation measures.

2. Case studies

2.1. Study Area

The Balsillas River watershed is located in the Department of Cundinamarca in Colombia and is part of the Bogotá River watershed (Fig. 1). It occupies an area of 624 km2, with an average elevation of 2730 m.a.s.l (2414 to 3696 m.a.s.l), as shown in Fig. 2, and has very steep slopes, with gradients between 20–45%. The watershed is important from an environmental and economic perspective because the river and its tributaries supply water to the Herrera Lagoon, a Special Management District used for crop irrigation and water supply for several municipalities.

Figure 1. Balsillas River watershed and hydrological stations

Historical annual precipitation within the watershed varies between 769.2 and 1031.0 mm, with maximum values ranging between 953.4 and 1031.0 mm in the northwestern part of the watershed. Precipitation decreases towards the middle and lower parts of the watershed, where values range from 769.2 to 886.4 mm per year. The annual distribution of the precipitation is bimodal, with low values of 650 mm and 710 mm occurring in January and February, respectively, and again in the months of August and September, with 705.96 mm and 730 mm, respectively. The month with the highest precipitation is October, with a record high of approximately 1000 mm. The average temperature is 13 °C, with maximum values in the south of the watershed ranging from 13.46 to 13.7 °C, and decreasing to 12.96 to 13.45 °C towards the northwest. At elevations between 2600 and 2800 m.a.s.l., the temperature ranges from 13.6 °C to 14 °C, whereas the higher sectors of the basin/watershed? (3000 m.a.s.l. to 3300 m.a.s.l.) have temperatures below 13 °C. The average temperature of the watershed is seasonally bimodal, with the lowest values in July, August, and September and the highest values in April and May. Similarly, the average temperature of the watershed is bimodal, with the lowest values in July, August and September and the highest in April and May. In general, temperature variations over the year do not exceed 3 °C. For the hydrological regime, the average output flow rate of the river is 1.22 m3 s-1. Water from the watershed is used for economic activities related to agriculture (potato and corn crops, greenhouses) and the industrial sector (mining and milk production).

Figure 2. Elevation map? of the Balsillas basin

The main vegetation coverage in the area is forest, high Andean Forest, shrubland, and paramo vegetation, as shown in Fig. 3. Soil types are predominantly characterized by fine textures, and are susceptible to water erosion.

Figure 3. Soil types in the Balsillas basin

2.2. Data source

The observed meteorological data for precipitation and daily temperatures for the 1989–2015 period, as well as daily flow data, were obtained from the Institute of Hydrology, Meteorology and Environmental Studies (IDEAM). The historical and future simulated meteorological datasets for the GCMs were obtained from the Centre for Environmental Data Analysis (CEDA; http://data.ceda.ac.uk/badc/cmip5/data/cmip5/output1/). This dataset has a spatial coverage of 207.6 km by 208.6 km. For the RCMs, data on South America was sourced from CORDEX (https://esg-dn1.nsc.liu.se/search/cordex/), with a spatial coverage of 55.6 km by 55.6 km. The climate models were used at daily time steps, and the information from 1990 to 2100 included historical data and scenarios for RCP 2.6, 4.5, and 8.5, which have been established as a priority for Colombian government? policy.

2.3. Hydro-BID modeling tool

Hydro-BID is a simulation tool consisting of an integrated, quantitative system that simulates water resource management in the Latin America and the Caribbean (LAC) region. This tool allows for the assessment of water quantity and quality, infrastructure needs, strategic design, and adaptation projects in response to the changes in climate, land use, or demographics under different scenarios (Mena et al. 2019).

Hydro-BID contains an Analytical Hydrology Database (AHD) that includes more than 230,000 delimited watersheds and rivers throughout LAC. The AHD incorporates data on land use, soil types, precipitation, and temperature within the study area over the period of interest. Hydro-BID uses a rainfall-runoff model based on a Generalized Watershed Load Factor (GWLF) formulation enhanced by a delay-routing methodology. Using the GWLF model, we were able to generate daily and monthly runoff volumes for each watershed, including smaller-scale simulations such as sub-watershed levels (Moreda et al. 2014).

The total flow (Ft) generated by a watershed is the sum of runoff (ROt) and groundwater (Gt), calculated by Eq. (1).

(1)

(2)

Dt= 2540CN-25.4                                                                                                                  (3)

Rt: Rainfall and snowmelt

Dt: Detention parameter      CN: Curve number assigned according to land use

The model parameters were curve number (CN), available water content in the soil (AWC), recession coefficient (R), permeability coefficient (seepage), evapotranspiration (ET) factor for the growing season, ET factor for the dormant season, and percentage of permeable cover.

The model uses an approach proposed by Haith (1985) to calculate the curve number, whereby the curve number for a given soil use is modified based on the antecedent soil moisture condition. The model requires the average humidity (CN2), which is assigned by the researcher and then used to estimate CN1 and CN3, which correspond to the dry and wet conditions, respectively? (Haith and Andre 2000). These calculations are shown in Eq. (4) and (5).

CN1=CN22.334-0.01334CN2                                                                                                     (4)

CN3=CN20.4036-0.0059CN2                                                                                                     (5)

The AHD is parameterized and pre-calibrated by using values from different regional databases. Therefore, the calibration and validation processes are necessary to ensure that the model represents real conditions as accurately as possible. The Hydro-BID model system and its components are shown in Fig. 4.

Figure 4. Hydro-BID model system

The calibration process and validation were conducted through trial and error, by adjusting the simulated flows to the observed field flows. The calibration process was carried out progressively from the most upstream watershed to the most downstream, with each watershed’s parameters adjusted according to? those of the watershed immediately upstream from it. Different metrics, including general volume error, Pearson’s correlation, modified correlation, and the Nash-Sutcliffe efficiency coefficient, were used to evaluate model performance.

For calibration, the period from 1-January-1992 to 31-December-2004 was used because it included dry and wet seasons, ensuring that the range of conditions in which the model was expected to operate was reflected. The validation was performed for a later period, from 1-January-2005 to 31-December-2009. All of the data were obtained from the El Bosque station.

2.4. Climate model selection and downscaling

The uncertainty in selecting information sources to use in hydrological studies is a an underlying? problem when evaluating the impacts of climate change. This work therefore aims to reduce uncertainty by comparing different sources of information on precipitation and temperature for the 19892005 period, including local data and data obtained from the various CORDEX and GCM models. This allowed us to select the best combination of global and regional data sources to represent the climate conditions at a watershed scale through a downscaling process. CORDEX was used to provide accessible, small-scale climate information to make maximum use of the domains covering all of Africa, Australia, South America, North America, and Europe, thus encompassing most areas of the world. As such, CORDEX manages to solve the regionalization problems that arise in other models by providing more accurate information at local and regional levels to better understand the behavior and formulate better measures of climate change (Trzaska and Schnarr 2014; Duarte et al. 2018).

Four global models and two CORDEX experiments were considered when selecting the climate model. The models evaluated were CanESM2, CNMR-MR5, MPI-ESM-MR, and CCSM4. The CORDEX experiments were CORDEX CanESM2 and CORDEX MPI-ESM-MR. The models are described in Table 1.

Table 1. Evaluated climate change models

| Model | Description | Source |
| --- | --- | --- |
| CanESM2 | The Canadian Earth System Model CanESM2 combines the Fourth Generation Coupled Global Climate Model and the terrestrial carbon cycle based on the Canadian Terrestrial Ecosystem Model (CTEM), which simulates land–atmosphere carbon exchange. The CTEM models all primary terrestrial ecosystem processes, including land-use change based on areal changes. The greenhouse gas concentrations and solar variability are based on CMIP5 recommendations (Chylek et al. 2011). | Canadian Centre for Climate Modeling and Analysis |
| CNMR-CM5 | CNRM-CM5 is an Earth system model designed to perform climate simulations. It is composed of five models (ARPEGE–Climate for the atmosphere, NEMO for the ocean, GELATO for sea ice, SURFEX for land, and TRIP for simulating river routing and water discharge from rivers to the ocean) that are designed separately and coupled using the OASIS software (Voldoire et al. 2013). | Centre National de Recherches Meteorologiques Earth |
| MPI-ESM-MR | The Max Planck Institute for Meteorology Earth System Model consists of a coupled general circulation model for the atmosphere and ocean (ECHAM6), and subsystem models for land and vegetation (JSBACH) and for marine biogeochemistry (HAMOCC5). The mixed resolution (MR) version of MPI-ESM (MPI-ESM-MR) has a higher vertical resolution in the atmosphere and a higher horizontal resolution for the ocean (Giorgetta et al. 2013; Stevens et al. 2013) | Max Planck Institute for Meteorology |
| CCSM4 | The Community Climate System Model (CCSM) is a adjusted to climate model for simulating the Earth’s climate system. It consists of four separate models that simultaneously simulate the atmosphere (CAM4 and DATM), ocean (CCSM POP2 and DOCN), land surface (CLM4 and DLND), and sea ice (CICE4 and DICE) (Lawrence et al. 2012). | National Science Foundation and U.S. Department of Energy |

Climate models were evaluated and prioritized based on results from retrospective simulations at multiannual, annual, and monthly levels. For both precipitation and temperature, the individual ability of climate models to reproduce mean, maximum, minimum, and number of wet/cold and hot/dry days was evaluated. Standard deviations and correlation coefficients were used as comparison metrics.

We applied a novel statistical downscaling method based on deterministic chaos and synchronization of dynamical systems. Deterministic chaos is a type of behavior discovered by Lorenz (1963) that involves the loss of information of the attractor of the dynamic system over time, while ensuring stability for the prediction of the attractor’s behavior. Systems characterized by deterministic chaos have the property of limited or no predictability of the system’s future evolution, despite the deterministic character of the system. This property also provides a prominent explanation for irregular behavior and instabilities in dynamical systems, (e.g., a climate system and its related non-linear processes).

The downscaling method is based on three steps: phase space reconstruction for different time steps, identification of deterministic chaos, and application of a general synchronization model. This model evaluates the presence of deterministic chaos over different intervals of rain accumulation for the time series using local station data and climate model results for both rainfall and temperature.

The behavior of multiple related dynamical systems has been analyzed in recent decades (Vrac et al. 2007; Chen et al. 2011; Ekström et al. 2015). The concept of synchronization refers to changes in the defined trajectories of two or more related attractors as a result of coupling, allowing for coherent and shared?/consistent? behavior between the coupled systems. This concept has made it possible to understand the mutual influence of such attractors (unidirectional or bidirectional) on each other’s? behavior (Rulkov et al. 1995). Multiple applications of synchronization can be found in physics and engineering (i.e., electronics and telecommunications). A synchronization model has been used to find the Time Delay, the Embedding Dimension, and the Lyapunov exponents of a system?.

Among the different types of synchronization, this novel downscaling technique aims to find a general synchronization between two climatic systems, the first related to the GCM results (Driver System) and the second defined by local measured climate data (Response System), using the mutual false nearest neighbor (MFNN) methodology (Rulkov et al., 1995; Duarte et al. 2018). MFNN is a statistical technique based on the calculation of a parameter  that evaluates the local neighborhoods between two time series, such that  takes values of the order of 1 when there is complete general synchronization. Otherwise,  must be a number whose magnitude is comparable to the product of the size of the attractor divided by the product of the distance between the nearest time series (𝑡𝑘) 𝑦 𝑦(𝑡𝑘).

μ=1Nn=1Nyn-ynNNDxn-xnNNRxn-ynNNDyn-ynNNR

Where

xn and yn are points within the Driver and Response Systems at the time n,

xnNND and ynNNR are the closest neighbors of xn and  yn in the Driver and Response Systems, respectively, and

xnNNR and ynNND are the points of the opposing system at time n for xnNND and ynNNR.

This technique has been used successfully in the Bogotá River watershed, of which the Balsillas River is a sub-watershed (Duarte et al. 2018). Using this method, future climate data for precipitation and temperature were generated at the local level using information at a global scale to evaluate the behavior of the flow until the year 2100 under different climate change scenarios.

The precipitation and temperature projections obtained from downscaling each of the three RCPs were used as inputs for Hydro-BID to obtain the future flow series for the 2014–2100 period.

3. Results and discussion

3.1. Hydro-BID calibration and validation

The model was calibrated and validated by comparing the observed flows at the outlet of the watershed with the values predicted by the model. During the calibration process, the parameters NC, AWC, R, seepage, cultivation station ET factor, latency station ET factor, and percentage of permeable cover were varied until an optimum result was obtained according to the performance metrics used by the Hydro-BID tool. Calibration was carried out through trial and error by evaluating one parameter at a time. The obtained parameter values were considered suitable for the model if results fell within established ranges according to performance metrics (Tables 2 and 3).

Table 2. Metrics resulting from calibration

| Metrics | Daily value | Monthly value |
| --- | --- | --- |
| Overall volume error (%) | 11.85 | 4.66 |
| Correlation, r (-) | 0.56 | 0.69 |
| Modified Correlation, Rmod | 0.35 | 0.42 |
| Nash-Sutcliffe Efficiency, NSE | 0.31 | 0.47 |

As shown in Table 2, the model performed better at the monthly level than the daily level, which may be explained by the fact that calibrating at the daily level is more complex, and more stations are needed when there are several sub-watersheds.

Validation was performed on data from a different time period as described in Section 2.3. The metrics obtained by validation, shown in Table 3, are within the ranges stipulated by the Inter-American Development Bank (Moreda et al. 2014).

Table 3. Metrics resulting from validation

| Metrics | Daily value | Monthly value |
| --- | --- | --- |
| Overall volume error (%) | -6.84 | -17.55 |
| Correlation, r (-) | 0.54 | 0.64 |
| Modified Correlation, Rmod | 0.46 | 0.41 |
| Nash-Sutcliffe Efficiency, NSE | 0.2 | 0.38 |

The low Nash–Sutcliffe efficiency coefficient values are due to the metric’s sensitivity to extreme, high values. The optimal value for this metric is 1, with values between 0.0 and 1.0 generally seen as acceptable. Values < 0.0 indicate that the observed mean is a better predictor than the simulated value, making model performance unacceptable (Moriasi et al. 2007).

The simulated flows were adequate for the calibration and validation periods, with low and medium flows being well captured. The performance metrics were within the acceptable ranges established by authors such as Dawson et al. (2007), reflecting the good performance of the Hydro-BID tool in simulating the hydrology in the watershed.

Although the model was accurate in simulating minimum and average flows, maximum flows were not well represented. (Fig. 5). However, this is not a serious problem because Hydro-BID is a model for management rather than data prediction. In general, the most important element is the flow volume, and not the frequency at which these flows occur.

Figure 5. Precipitation data and measured versus simulated flow rates

3.2. Climate model selection and downscaling

To select the model that best fits local climate conditions, we evaluated annual and multiannual monthly graphs that compared data from the model with historical temperature and precipitation data. The best-performing climate models were the MPI-ESM-MR CORDEX model for precipitation and the MPI-ESM-MR GCM? model for temperature. The GCM was closer to the observed rainfall and temperature data, had the best correlation of all the models considered, and had the lowest percentage error for the dry- and wet-day criteria (Table 4). This model was selected due to similarities between the behavior of the observed? data curve and the model data for twelve months of the year, starting in January. In addition, the values ​​produced by the GCM were closest to the recorded precipitation data. On the other hand, the CORDEX? model presents the best correlation of all the models evaluated. The percentage of error for the criteria for dry and wet days is also minimal, which indicates that the observed and modeled data were similar?. Table 4 compares GCM and CORDEX model results for different statistics at research stations within the watershed. Fig. 6 and Fig. 7 compare observed and modeled? information for precipitation and temperature.

Table 4. Comparison of modeled values of flow rate? statistics at climate? stations within the watershed

| Model | Station | Average | Max | Min | Desv | Corr | Wet / cold days (% error) | Wet / warm days (% error) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MPI-ESM-MR | 301413300 | 37.82 | 40.22 | 35.43 | 101.86 | 0.03 | 7.17 | 0.16 |
|  | 301388900 | 51.56 | 55.13 | 48 | 101.52 | 0.05 | 3.72 | 0.16 |
|  | 301388800 | 40.99 | 42.93 | 39.49 | 105.19 | 0.04 | 1.74 | 0.64 |
| MPI-ESM-MR CORDEX | 301413300 | 392.15 | 404.13 | 380.17 | 423.76 | 0.04 | 0.22 | 3.42 |
|  | 301388900 | 271.05 | 275.89 | 266.21 | 363.5 | 0.03 | 0.48 | 0.32 |
|  | 301388800 | 258.65 | 263.99 | 253.31 | 349.31 | 0.03 | 0.16 | 0.96 |

Figure 6. Multiannual comparison of observed? monthly precipitation in all stations with the values obtained from the CORDEX MPI-ESM-MR model

Figure 7. Multiannual comparison of observed? monthly temperature in all stations with the values obtained from the MPI-ESM-MR model

For the calibration process of the downscaling model, we used the period from 1-January-1989 to 31-December-2005. For the validation process, we used the period from 1-January-2006 to 30-December-2014. We tested the results of validation using the RMSE (Root Mean Squared Error). Tables 5 shows the results of evaluation for precipitation and temperature at stations within the watershed.

Table 5. Validation of the downscaling technique for precipitation and temperature

| Station | RMSE (monthly) |  |
| --- | --- | --- |
|  | Precipitation | Temperature |
| 301388800 | 60.85 | 0.39 |
| 301388900 | 58.23 | 0.44 |
| 301413300 | 51.12 | 0.37 |

Figs. 8 and 9 compare the results of the downscaling technique and the historical observed? data for the validation period. As expected, the results for temperature show better accuracy?/behavior than precipitation, which is difficult to predict in tropical areas.

Figure 8. Validation of the downscaling technique for precipitation against historical data at Station 301388800

Figure 9. Validation of the downscaling technique against historical data for temperature at Station 301388800

Once the climate models were selected, we evaluated the downscaling performance using the RMSE to estimate the correlation between the climate model data before and after downscaling and the historical data from the climate stations, as shown in Table 6 and Figs. 10 and 11. The correlation coefficient between the data was closer to the optimum value of zero after downscaling was conducted. This showed that the technique minimizes the error levels in the climate models in the study area.

Table 6. The effects of downscaling

| RMSE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Before downscaling |  |  |  |  |  | After downscaling |  |  |  |  |  |
| Precipitation |  |  | Temperature |  |  | Precipitation |  |  | Temperature |  |  |
| RCP 2.6 | RCP 4.5 | RCP 8.5 | RCP 2.6 | RCP 4.5 | RCP 8.5 | RCP 2.6 | RCP 4.5 | RCP 8.5 | RCP 2.6 | RCP 4.5 | RCP 8.5 |
| 6.64 | 7.25 | 7.12 | 8.36 | 8.32 | 8.69 | 1 | 1.31 | 1.25 | 3.0 | 2.87 | 2.94 |

Figure 10. Comparison of historical data from climate stations and future data from the CORDEX MPI-ESM-MR model (RCP 2.6, 4.5, 8.5) for precipitation.

Figure 11. Comparison of historical data from climate stations and future data from the GCM MPI-ESM-MR model (RCP 2.6, 4.5, 8.5) for temperature.

For each scenario, a comparison analysis was conducted for future climate conditions by increasing or decreasing precipitation and temperature, both at mid-century (MOC) and at the end of the century (EOC). All scenarios showed an increase in precipitation events both at MOC and at EOC. In an RCP 2.6 scenario, the average increase was 1.83% at MOC and 4% at EOC; for RCP 4.5, 6.53% at MOC and 6.9% at EOC; and for RCP 8.5, 7.43% at MOC and 7.97% at EOC. Temperatures in the RCP 2.6 scenario increased by 0.17% at MOC and 0.23% at EOC (Table 3). For RCP 4.5, temperatures increased by 0.57% at MOC and 0.47% at EOC, and decreased by 0.43% at MOC and 0.60% at EOC for RCP 8.5.

The lowest temperature projected for RCP 2.6 was 12.73 °C at Station 301388900. Meanwhile, the temperature at Station 301388800 was close to 13 °C, and close to 13.73 °C at Station 301413300. Temperatures in RCP 4.5, the stabilization scenario, were significantly higher than in RCP 2.6. Station 301388900 showed ​​increased temperatures that fluctuated between 13.26 °C and 13.53 °C, while Station 301388800 oscillated between 12.98 °C and 13.25 °C, and the sub-watershed area? at Station 301413300 was between 13.54 °C and 13.81 °C. In RCP 8.5, the final scenario, temperatures ​​were lower, with behavior resembling that of RCP 4.5. At Station 301388900 in the upper part of the watershed, the temperature oscillated between 12.56 °C and 12.7 °C, at Station 301388800 the temperature was 13 °C, and at Station 301413300 the temperature was as high as 13.56 °C.

3.3. Hydrological modeling under three climate change scenarios

The projected climate data series obtained using the downscaling method were used as inputs for the calibrated and validated Hydro-BID model to obtain the daily flow series for the period 2014 to 2100. These results were used to obtain the expected future variations in the hydrological regime of the watershed (see Fig. 12). The hydrologic flows gradually increased for each RCP throughout the 2014 to 2100 period. The RCP 8.5 scenario recorded flows of higher magnitude, with peaks reaching approximately 7 m3 s-1, suggesting an increase in/greater runoff processes. On the other hand, flows for RCP 4.5 fluctuated between 1 and 3 m3 s-1, with peaks close to 5 m3 day-1 during the period from 2030 to 2034. RCP 2.6 presented the lowest flows, only reaching 2 m3 s-1.

Figure 12. Simulated flows (m3 s-1) from 2014 to 2100 under three? climate change scenarios

Climatic factors can alter the water balance in a watershed, leading to changes in the hydrological regime. We analyzed the variation in the average daily hydrologic flow of the Balsillas River at the outlet of the watershed for three different climate change scenarios: RCP 2.6, 4.5, and 8.5. We expected the future average daily flow of the Balsillas River watershed to increase for all three scenarios.

The average simulated flow was approximately 1.28 m3 s-1 for RCP 2.6, which represents an increase of 5.2% with respect to the reference period. The average simulated flow for RCP 4.5 was 1.24 m3 s-1, representing an increase of 2.1%, and the flow was 1.29 m3 s-1 for RCP 8.5, representing an increase of approximately 6%. The impact of climate change is?/on the hydrologic flow was? highest for scenario RCP 8.5, which could be attributed to a greater increase in precipitation and decrease in temperature.

For the different climate change scenarios, the results show an increase in the average hydrologic flow compared to the base period. This suggests that in the future, an increase in water yield leads to greater water availability in the watershed, such that water resources could be sufficient to sustain the population's socioeconomic activities and ecological conditions. However, an effective and appropriate water resource management system must be implemented in the watershed to avoid a decline in the quantity and quality of water resources associated with land use pressures, especially during the dry season. The predicted increased flows are directly related to the increase in precipitation, which can cause floods in the watershed. It is important to note that the objective of our study was to evaluate the impact of climate change on average streamflow conditions. Future studies could analyze the impact of extreme events.

4. Discussion

According to Zhang et al. (2019), hydrological models have difficulty reproducing high runoff values because their underlying? theories and model calibration criteria use an incorrect/inadequate? theory of runoff production. For example, the GWLF model assumes that runoff does not occur until the soil moisture content of the aeration zone reaches field capacity. This assumption may not be valid during intense rainfall, as these events can produce runoff when soils are not completely saturated due to insufficient infiltration, which is not simulated by the model. Therefore, modifying the model to include unsaturated runoff during heavy rainfall events may improve hydrological simulations during these periods.

The most sensitive model parameters are the curve number and AWC, which together control the amount of runoff generated within the watershed. This is because the curve number represents the soil? permeability in the study area, and thus whether infiltration is favored, while AWC represents the soil water content that has been absorbed prior to soil saturation.

Although the RCP 8.5 scenario is related to an increase in temperature on a planetary scale, this increase does not occur uniformly across the globe, meaning that there may be regional temperatures that are higher or lower for the different RCP scenarios. The presence of regional variations? is an important point, given that the models linked to different RCP scenarios are independent of each other, and share the initial and boundary conditions as well as the physical model. This is why it is important to evaluate various climate change scenarios on a regional scale?.

It is also noteworthy that the Balsillas River watershed is located in a tropical region that is highly affected by regional climate variability phenomena (e.g., ENSO) that are more accurately? represented by in-situ data than data from GCM or RCM models. Different RCP scenarios affect  the intensity and frequency of such phenomena to varying degrees, resulting in varying levels of agreement? between the climate projections and in-situ information.

The advantage of the hydrological model used in the present study is that the modeling tool contains the AHD, which contains parameterized information for watersheds in the Latin American and Caribbean region. This reduces the problem of scarce information in developing countries such as Colombia, which lacks information on its watersheds, including the Balsillas River Watershed. Despite the advantages of AHD, using a single hydrological model to simulate water resource availability may lead to some uncertainty due to the choice of model parameter and structure (Zhang et al. 2019). For example, some studies that developed six monthly water balance models found large differences in the resulting runoff values. Therefore, it is recommended to use a set of hydrological models to provide a more complete range of future hydrological simulations.

Our results project an increase in average hydrologic flows. This is consistent with other studies on watersheds with different climate and topographic characteristics that projected flow increases by the end of the century for different RCPs. For instance, Meenu et al. (2013) concluded that in the Tunga-Bhadra River Watershed, scenarios A2 and B2 are forecasted to have flow increases of 18.5% and 43.9%, respectively. In the Himalayan River watershed, CNRM projected that compared to the baseline flow, the average annual streamflow will increase by 10.7% and 0.8% during the 2030s and 2060s, respectively? (Bhatta et al. 2019). In the Manipur River watershed, India, researchers projected an increase of 40.9 m3 s-1 in the discharge at the outlet point for the period 2081–2100 for scenario RCP 8.5 (Anand and Oinam 2019).

The results of our study are consistent with the information presented to the United Nations Framework Convention on Climate Change in the Colombia National Communication Number 3. That document? states that for the department of Cundinamarca in the area of the Central Savana Province, where the Balsillas River Watershed is located, precipitation and temperature increases are forecasted for the different climate change scenarios.

4. Conclusions

In this study, we investigated the future projections of the impact of climate change on hydrology, specifically the water flow, of the Balsillas River Watershed for the 2014–2100 period. Maintaining a sufficient water supply is essential for crop production. The main economic activity in the Balsillas River basin is agriculture, which provides livelihoods for most of the population. It is also important to meet domestic water needs for daily household use and ensure the quality of life for the communities in the study area. This project assessed the risk of water supply failure in order to anticipate the impact that water shortages may have on the communities, and to identify measures to reduce such risks.

The hydrological model included in the Hydro-BID tool accurately represented the average and minimum flows of the watershed, as shown by comparing the runoff simulation with historical flow records of the “El Bosque” hydrometric station. According to the performance metrics, the model performed better using a monthly rather than daily timescale.

Using downscaling, precipitation was projected to increase compared to historical data from 1.83% to 8%. Average temperatures were also projected to increase slightly in most scenarios, by 0.17% to 0.57%, except for RCP 8.5, where an average decrease of 0.5% was projected.

With respect to historical flows recorded in the watershed, this study projected an increase of 5.2% for RCP 2.6, corresponding to an average of 1.28 m3 s-1. For RCP 4.5, an increase of 2.1% was projected, corresponding to an average of 1.24 m3 s-1, while an increase of approximately 6% was projected for RCP 8.5. Based on the results obtained from the modeling, we determined that the water availability in the watershed will increase between 2014 and 2100 under all of the climate change scenarios evaluated. This may have a positive impact on the future development of economic activities (e.g., agriculture and industrial activities) in the region, including increasing water availability for urban growth. However, such potential economic growth should be accompanied by water protection and conservation strategies.

The results of this study make it possible to formulate and implement an effective water management plan to reduce future water-related problems in the Balsillas River watershed. In addition to the effects of climate change, future studies must consider other factors, such as land use change and increased carbon dioxide concentrations. This study also contributes to the research on climate change in the Andean Plateau region, and opens avenues for research on the application of new hydrological modeling and water resource management tools. It highlights the need for new methodologies that will become critical for decision making in the planning and management of water resource systems.

Acknowledgements: The authors wish to thank the Research Fund (FODEIN) of the University of Santo Tomás, the CAR, the IDEAM, all of the entities that provided information, and the people who contributed in some way to this work.

Declarations

Ethical Approval and Competing interests not applicable

Authors’ contributions: Darwin Mena developed the models and simulation. Darwin Mena, Carlos Peña-Guzmán, Ronal Sierra, and Manuel Rodriguez analyzed the data and results, and developed the discussion and conclusions.

Funding: This research project was financed by the FODEIN research call from Santo Tomas University.

Availability of data and materials not applicable

References

Anand V, Oinam B (2019) Future climate change impact on hydrological regime of river basin using SWAT model. Glob J Environ Sci Manag 5:471–484. https://doi.org/10.22034/GJESM.2019.04.07

Bhatt D, Mall RK (2015) Surface Water Resources, Climate Change and Simulation Modeling. Aquat Procedia 4:730–738. https://doi.org/10.1016/j.aqpro.2015.02.094

Bhatta B, Shrestha S, Shrestha PK, Talchabhadel R (2019) Evaluation and application of a SWAT model to assess the climate change impact on the hydrology of the Himalayan River Basin. CATENA 181:104082. https://doi.org/10.1016/j.catena.2019.104082

Bravo Ortiz VP (2020) Evaluación de la herramienta de simulación del recurso hídrico Hydro-BID para la cuantificación de la disponibilidad de agua en la cuenca alta del río Guayllabamba y unidades hídricas aportantes de agua para el Distrito Metropolitano de Quito. bachelorThesis, Quito, 2020.

Cabré MF, Solman S, Núñez M (2016) Regional climate change scenarios over southern South America for future climate (2080-2099) using the MM5 Model. Mean, interannual variability and uncertainties. Atmósfera 29:35–60. https://doi.org/10.20937/ATM.2016.29.01.04

Chen J, Brissette FP, Leconte R (2011) Uncertainty of downscaling method in quantifying the impact of climate change on hydrology. J Hydrol 401:190–202. https://doi.org/10.1016/j.jhydrol.2011.02.020

Chylek P, Li J, Dubey MK, et al (2011) Observed and model simulated 20th century Arctic temperature variability: Canadian Earth System Model CanESM2. Atmospheric Chem Phys Discuss 11:22893–22907. https://doi.org/10.5194/acpd-11-22893-2011

Córdoba-Machado S, Palomino-Lemus R, Gámiz-Fortis SR, et al (2015) Assessing the impact of El Niño Modoki on seasonal precipitation in Colombia. Glob Planet Change 124:41–61. https://doi.org/10.1016/j.gloplacha.2014.11.003

Dawson CW, Abrahart RJ, See LM (2007) HydroTest: A web-based toolbox of evaluation metrics for the standardised assessment of hydrological forecasts. Environ Model Softw 22:1034–1052. https://doi.org/10.1016/j.envsoft.2006.06.008

Dixon KW, Lanzante JR, Nath MJ, et al (2016) Evaluating the stationarity assumption in statistically downscaled climate projections: is past performance an indicator of future results? Clim Change 135:395–408. https://doi.org/10.1007/s10584-016-1598-0

Duarte F, Corzo G, Santos G, Hernández O (2018) Chaotic Statistical Downscaling (CSD): Application and Comparison in the Bogotá River Basin. In: EPiC Series in Engineering. EasyChair, pp 626–634

Efra&iacute, Dom&iacute  n A, Calle N, et al (2008) Relaciones demanda-oferta de agua y el indice de escasez de agua como herramientas de evaluacion del recurso hidrico colombiano. Rev Acad Colomb Cienc Exactas Fis Nat 32:195–213

Ekström M, Grose MR, Whetton PH (2015) An appraisal of downscaling methods used in climate change research. WIREs Clim Change 6:301–319. https://doi.org/10.1002/wcc.339

Gebrechorkos SH, Bernhofer C, Hülsmann S (2019) Impacts of projected change in climate on water balance in basins of East Africa. Sci Total Environ 682:160–170. https://doi.org/10.1016/j.scitotenv.2019.05.053

Giorgetta MA, Jungclaus J, Reick CH, et al (2013) Climate and carbon cycle changes from 1850 to 2100 in MPI-ESM simulations for the Coupled Model Intercomparison Project phase 5. J Adv Model Earth Syst 5:572–597. https://doi.org/10.1002/jame.20038

Gutmann ED, Rasmussen RM, Liu C, et al (2011) A Comparison of Statistical and Dynamical Downscaling of Winter Precipitation over Complex Terrain. J Clim 25:262–281. https://doi.org/10.1175/2011JCLI4109.1

Haith DA, Andre B (2000) Curve Number Approach for Estimating Runoff from Turf. J Environ Qual 29:1548–1554. https://doi.org/10.2134/jeq2000.00472425002900050022x

Hoyos N, Escobar J, Restrepo JC, et al (2013) Impact of the 2010–2011 La Niña phenomenon in Colombia, South America: The human toll of an extreme weather event. Appl Geogr 39:16–25. https://doi.org/10.1016/j.apgeog.2012.11.018

Jain SK, Kumar V (2012) Trend analysis of rainfall and temperature data for India. Curr Sci 102:37–49

Lavado Casimiro WS, Labat D, Guyot JL, Ardoin-Bardin S (2011) Assessment of climate change impacts on the hydrology of the Peruvian Amazon–Andes basin. Hydrol Process 25:3721–3734. https://doi.org/10.1002/hyp.8097

Lawrence DM, Oleson KW, Flanner MG, et al (2012) The CCSM4 Land Simulation, 1850–2005: Assessment of Surface Climate and New Capabilities. J Clim 25:2240–2260. https://doi.org/10.1175/JCLI-D-11-00103.1

León Olmedo V (2020) Modelo de gestión del recurso hídrico en la cuenca del río Tarqui, analizando escenarios de cambio climático mediante la herramienta de simulación Hydro-BID. bachelorThesis

Leveri A, Pascual JA, Sadres M, Rincón P (2020) Escenarios de Cambio Climático en Colombia y Uruguay. Explorando el modelo hidrológico HYDROBID Climate Change Scenarios in Colombia and Uruguay. Exploring HYDROBID hydrological model. IMDEA AGUA, Centro para el Conocimiento del Paisaje, Alcalá de Henares

Lutz AF, Maat HW ter, Biemans H, et al (2016) Selecting representative climate models for climate change impact studies: an advanced envelope-based selection approach. Int J Climatol 36:3988–4005. https://doi.org/10.1002/joc.4608

Mamani Cosi S (2018) Modelamiento hidrológico semidistribuido con aplicación de Hydro-BID: caso río Verde Puno - Perú. Autoridad Nacional del Agua

Manciu A, Rammig A, Krause A, Quesada BR (2023) Impacts of land cover changes and global warming on climate in Colombia during ENSO events. Clim Dyn 61:111–129. https://doi.org/10.1007/s00382-022-06545-1

Meenu R, Rehana S, Mujumdar PP (2013) Assessment of hydrologic impacts of climate change in Tunga–Bhadra river basin, India with HEC-HMS and SDSM. Hydrol Process 27:1572–1589. https://doi.org/10.1002/hyp.9220

Mena D, Ramos MAC, Rodríguez AMS (2020) Modelación de los impactos de los escenarios de cambio climático en la cuenca del Río Pamplonita en Norte de Santander. Rev EIA 17:33015 pp. 1–12. https://doi.org/10.24050/reia.v17i33.1265

Mena D, Solera A, Restrepo L, et al (2019) An analysis of unmet water demand under climate change scenarios in the Gualí River Basin, Colombia, through the implementation of Hydro-BID and WEAP hydrological modeling tools. J Water Clim Change 12:185–200. https://doi.org/10.2166/wcc.2019.118

Mohammadi B, Gao H, Feng Z, et al (2023) Simulating glacier mass balance and its contribution to runoff in Northern Sweden. J Hydrol 620:129404. https://doi.org/10.1016/j.jhydrol.2023.129404

Mohammadi B, Vazifehkhah S, Duan Z (2024) A conceptual metaheuristic-based framework for improving runoff time series simulation in glacierized catchments. Eng Appl Artif Intell 127:107302. https://doi.org/10.1016/j.engappai.2023.107302

Montenegro M, Mendoza D, Mora D, et al (2022) Extreme Rainfall Variations Under Climate Change Scenarios. Case of Study in an Andean Tropical River Basin. Water Resour Manag 36:5931–5944. https://doi.org/10.1007/s11269-022-03332-9

Mora DE, Campozano L, Cisneros F, et al (2014) Climate changes of hydrometeorological and hydrological extremes in the Paute basin, Ecuadorean Andes. Hydrol Earth Syst Sci 18:631–648. https://doi.org/10.5194/hess-18-631-2014

Moreda F, Daussa PCV (2016) Hydro-BID Case Study No. 3: Impact of El Niño Events on Sediment Loading in the Chancay-Lambayeque Basin, Peru. Inter-American Development Bank

Moreda F, Daussa PCV (2017) Hydro-BID Case Study No. 5: Impact of Climate Change on Proposed Water Investments in Chalpi Basin, Ecuador. Inter-American Development Bank

Moreda F, Millares F, Castillo RM (2014a) Hydro-BID: Un Sistemas Integrado para la Simulación de Impactos del Cambio Climático sobre los Recursos Hídricos. Parte

Moreda F, Miralles-Wilhelm F, Castillo RM (2014b) Hydro-BID: An integrated system for modeling impacts of climate change on water resources. Part

Moreda F, Miralles-Wilhelm F, Castillo RM (2014c) Hydro-BID: Un Sistema Integrado para la Simulación de Impactos del Cambio Climático sobre los Recursos Hídricos. Parte 2

Moriasi DN, Arnold W, Van Liew MW, et al (2007) Model evaluation guidelines for systematic quantification of accuracy in watershed simulations. Model Eval Guidel Syst Quantif Accuracy Watershed Simul 50:885–900

Munar AM, Mendez N, Narvaez G, et al (2023) Modelling the climate change impacts on river discharge and inundation extent in the Magdalena River basin – Colombia. Hydrol Sci J 68:1286–1300. https://doi.org/10.1080/02626667.2023.2215932

Poveda G, Mesa ÓJ (1996) Las fases extremas del fenómeno ENSO (El Niño y La Niña) y su influencia sobre la hidrología de Colombia. Tecnol Cienc Agua 11:21–37

Puno RCC, Puno GR, Talisay B a. M (2019) Hydrologic responses of watershed assessment to land cover and climate change using soil and water assessment tool model. Glob J Environ Sci Manag 5:71–82. https://doi.org/10.22034/gjesm.2019.01.06

Ragettli S, Immerzeel WW, Pellicciotti F (2016) Contrasting climate change impact on river flows from high-altitude catchments in the Himalayan and Andes Mountains. Proc Natl Acad Sci 113:9222–9227. https://doi.org/10.1073/pnas.1606526113

Reshmidevi TV, Nagesh Kumar D, Mehrotra R, Sharma A (2018) Estimation of the climate change impact on a catchment water balance using an ensemble of GCMs. J Hydrol 556:1192–1204. https://doi.org/10.1016/j.jhydrol.2017.02.016

Rulkov NF, Sushchik MM, Tsimring LS, Abarbanel HDI (1995) Generalized synchronization of chaos in directionally coupled chaotic systems. Phys Rev E 51:980–994. https://doi.org/10.1103/PhysRevE.51.980

Sauchyn DJ, St-Jacques J-M, Barrow E, et al (2016) Adaptive Water Resource Planning in the South Saskatchewan River Basin: Use of Scenarios of Hydroclimatic Variability and Extremes. JAWRA J Am Water Resour Assoc 52:222–240. https://doi.org/10.1111/1752-1688.12378

Shrestha S, Bhatta B, Shrestha M, Shrestha PK (2018) Integrated assessment of the climate and landuse change impact on hydrology and water quality in the Songkhram River Basin, Thailand. Sci Total Environ 643:1610–1622. https://doi.org/10.1016/j.scitotenv.2018.06.306

Solman SA (2013) Regional Climate Modeling over South America: A Review. In: Adv. Meteorol. https://www.hindawi.com/journals/amete/2013/504357/. Accessed 20 Apr 2020

Stanzel P, Kling H, Bauer H (2018) Climate change impact on West African rivers under an ensemble of CORDEX climate projections. Clim Serv 11:36–48. https://doi.org/10.1016/j.cliser.2018.05.003

Stevens B, Giorgetta M, Esch M, et al (2013) Atmospheric component of the MPI-M Earth System Model: ECHAM6. J Adv Model Earth Syst 5:146–172. https://doi.org/10.1002/jame.20015

Tan ML, Ibrahim AL, Yusop Z, et al (2017) Climate change impacts under CMIP5 RCP scenarios on water resources of the Kelantan River Basin, Malaysia. Atmospheric Res 189:1–10. https://doi.org/10.1016/j.atmosres.2017.01.008

Trzaska S, Schnarr E (2014) A review of downscaling methods for climate change projections

Vargas X, Gómez T, Ahumada F, Rubio E (2013) Water availability in a mountainous Andean watershed under CMIP5 climate change scenarios. In: Cold and Mountain Region Hydrological Systems Under Climate Change: Towards Improved Projections, Iahs Publication. pp 33–38

Vega J, Barco J, Hidalgo C (2024) Space-time analysis of the relationship between landslides occurrence, rainfall variability and ENSO in the Tropical Andean Mountain region in Colombia. Landslides. https://doi.org/10.1007/s10346-024-02225-9

Villamizar SR, Pineda SM, Carrillo GA (2019) The Effects of Land Use and Climate Change on the Water Yield of a Watershed in Colombia. Water 11:285. https://doi.org/10.3390/w11020285

Voldoire A, Sanchez-Gomez E, Salas y Mélia D, et al (2013) The CNRM-CM5.1 global climate model: description and basic evaluation. Clim Dyn 40:2091–2121. https://doi.org/10.1007/s00382-011-1259-y

Vrac M, Stein ML, Hayhoe K, Liang X-Z (2007) A general method for validating statistical downscaling methods under future climate change. Geophys Res Lett 34:. https://doi.org/10.1029/2007GL030295

Wyatt A, Moreda F, Miralles-Wilhelm F, Castillo RM (2014) Caso de estudio de Hydro-BID No 1: Modelo de gestión del recurso hídrico en la cuenca del Río Grande en Argentina. Inter-American Development Bank

Zhang H, Wang B, Liu DL, et al (2019) Impacts of future climate change on water resource availability of eastern Australia: A case study of the Manning River basin. J Hydrol 573:49–59. https://doi.org/10.1016/j.jhydrol.2019.03.067