Table S1. Proposed triangular fuzzy numbers for effective factors on the likelihood of failure (LoF). In all of the presented fuzzy classifications, the lower the class, the better the factor status.

| Corresponding triangular fuzzy numbers | Quantitative/qualitative/nominal classification of each effective factor | Effective factors | Main effective groups |
| --- | --- | --- | --- |
| (0,0,3) (2,5,8) (7,10,10) | 2-5 >5 <2 in meter | Pipe depth | Construction factors |
| (0,0,1.5) (1,2.5,4) 3.5,5,6.5(6,7.5,9)(8.5,10,10) | ABCDE grade of the contractor executing the project | Installation condition |  |
| (0,0,1.5) (1,2.5,4) 3.5,5,6.5(6,7.5,9)(8.5,10,10) | PlasticConcreteAsbestos cementVitrified ClayBrick | Pipe material |  |
| (0,0,1.5) (1,2.5,4) 3.5,5,6.5(6,7.5,9)(8.5,10,10) | <25 25<LD<300300<LD<600600<LD<900>900 L: Length of the pipe between 2 consecutive manholes (ranges from 20 to 180 meters) D: Pipe diameter (ranges from 200 to 900 mm) | Pipe length to diameter ratio |  |
| (0,0,1.5) (1,2.5,4) 3.5,5,6.5(6,7.5,9)(8.5,10,10) | ABCDE grade of the contractor operating the network | Operating condition |  |
| (0,0,3) (2,5,8) (7,10,10) | Sidewalk, green spaces, and empty groundsSide road Main road | Road type | Local external factors |
| (0,0,3) (2,5,8) (7,10,10) | Groundwater level is far below the pipeGroundwater level is in the vicinity of the pipeGroundwater level is above the pipe | Groundwater level |  |
| (0,0,3) (2,5,8) (7,10,10) | Gravel, coarse sand Fine sand and siltClay | Soil type |  |
| (0,0,3) (2,5,8) (7,10,10) | >63<d<6<3 d: Distance between surrounding trees and the pipe | Root penetration |  |
| (0,0,3) (2,5,8) (7,10,10) | <50005000<Z<10000>10000 Z: Parameter obtained from an empirical formula presented in Pomeroy (1990)* | Released H2S |  |
| (0,0,1.5) (1,2.5,4) 3.5,5,6.5(6,7.5,9)(8.5,10,10) | 0-20% Service life20-40% Service life40-60% Service life60-80% Service life>80% Service life | Pipe age | Other factors |
| (0,0,3) (2,5,8) (7,10,10) | Storm sewers Combined sewersIndustrial Sewers | Wastewater type |  |

* Pomeroy, R. (1990). The Problem of Hydrogen Sulphide in Sewers, 2nd ed.; Boon, A.G., Ed.; Clay Pipe Development Association: London, UK.

Table S2. Proposed triangular fuzzy numbers for different groups of consequences of failure (CoF). In all of the presented fuzzy classifications, the lower the class, the better the factor status.

| Corresponding triangular fuzzy numbers | Quantitative/qualitative/nominal classification of each effective factor | Effective factors | Main consequence groups |
| --- | --- | --- | --- |
| (0,0,3) (2,5,8) (7,10,10) | Clay Fine sand, silt, calcareous soilGravel, coarse sand | Soil contamination (considering soil permeability) | Environmental impact |
| (0,0,3) (2,5,8) (7,10,10) | Groundwater level is far below the pipeGroundwater level is in the vicinity of the pipeGroundwater level is above the pipe | Groundwater contamination |  |
| (0,0,3) (2,5,8) (7,10,10) | Villa houses are predominant in the areaThe area has a significant number of residential apartmentsThere are offices, institutions, and residential apartments in the area | Parking loss | Traffic & road |
| (0,0,3) (2,5,8) (7,10,10) | Rural roadsMain roadsFreeway and highway | Decreasing the lifespan of road asphalt |  |
| (0,0,3) (2,5,8) (7,10,10) | The pipe is not located under any road or it is installed under a local road The pipe is located under main roadsThe pipe is located under freeway or highway | Traffic density |  |
| (0,0,3) (2,5,8) (7,10,10) | >x0.5x<d<x<0.5x d: Distance between a critical building and the pipe x: Safe distance for various critical buildings recommended by Baah et al. (2015)** | Service interruption of critical buildings |  |
| (0,0,3) (2,5,8) (7,10,10) | Agricultural lands, gardens, and empty groundsResidential areas Areas with institutions, offices, companies, educational and commercial uses | Drop in people’s work efficiency & comfort | Life quality and public/private property |
| (0,0,3) (2,5,8) (7,10,10) | Residential areasComplex landuses (e.g. residential, offices, small shops, etc.)Large commercial centers such as shopping malls | Damage to nearby buildings |  |

** Baah, K., Dubey, B., Harvey, R., & McBean, E. (2015). A risk-based approach to sanitary sewer pipe asset management. Science of the Total Environment, 505, 1011-1017.