# Supplementary Materials

Table S1: Conversion data table for the fish waste conversion facilities

| Facility/Technology | Input | Output | Conversion Ratio | References |
| --- | --- | --- | --- | --- |
| Fish tank (Aquaponic) | Fish feed | Fish harvested | 0.77 t/t FF | (Quagrainie et al., 2018) |
| Vegetable tank (Hydroponic) | Vege seed | Vegetables | 0.71 t/t FW |  |
| Fish processing plant | Fish harvested | Fish waste | 0.54 t/t fish |  |
| BSF | Fish waste | Fish feed | 0.35 t/t FW | (Soetemans et al., 2020) |
|  |  | Chitosan | 0.56 t/t FW | (Gougbedji et al., 2021) |
| Biogas plant (Anaerobic digestion) |  | Biogas | 0.09 t/t FW | (Bücker et al., 2020) |
| Pyrolysis |  | Biochar | 0.1 t/t FW | (Kraiem et al., 2015) |
|  |  | Bio-oil | 0.4 t/t FW |  |
| Gasification |  | Syngas | 0.9 t/t FW | (Rowland et al., 2009) |
| CHP a | Biogas | Electricity | 1680 kWh/t | - |
|  |  | Heat | 2400 kWh/t | - |
|  | Biochar | Electricity | 3136 kWh/t | - |
|  |  | Heat | 4480 kWh/t | - |
|  | Bio-oil | Electricity | 1568 kWh/t | - |
|  |  | Heat | 2240 kWh/t | - |
|  | Syngas | Electricity | 194 kWh/t | - |
|  |  | Heat | 277.2 kWh/t | - |
| FT process | Syngas | Biofuel | 0.148 t/t | (Saeidi et al., 2014) |
| Hydrotreating | Syngas | Biofuel | 0.4 t/t | (Alonso-Pippo et al., 2009) |

a The conversion ratio of CHP is approximated by taking the calorific values of the input with a consideration of 35% and 50% efficiency for electricity and heat production, respectively.

Table S2: Scale factors of CAPEX and OPEX

| Facility/Technology | Capital costs, $/t a | Operating costs, $/t a | References |
| --- | --- | --- | --- |
| Pyrolysis | 138.40 | 123.07 | (Wright et al., 2010) |
| Gasification | 79.18 | 67.60 | (Mustafa et al., 2017) |
| BSF facility | 3,500.00 | 1,840.00 | (Gougbedji et al., 2021) |
| Biogas plant | 2,015.00 | 100.75 | (Al-Wahaibi et al., 2020) |
| WWTP | 540.00 | 320.00 | (Singh et al., 2016) |
| CHP | 266.21 | 121.28 | (Camporeale et al., 2015) |
| FT process | 37.20 | 43.00 | (Mawhood et al., 2014) |
| Hydrotreating | 281.90 | 41.90 | - |
| Aquaponics | 31.00 | 27.30 | (Quagrainie et al., 2018) |
| Hydroponics | 38.21 | 19.00 | (Quagrainie et al., 2018) |

a Pricing is shown in US Dollar.

Table S3: Unit cost of imported resources and output products

| Facility | Price a | References |
| --- | --- | --- |
| Imported |  |  |
| Steam | 9.68 $/t HPS | (Saeidi et al., 2014) |
| Electricity | 0.16 $/kWh | (Ward and Løes, 2011) |
| Water | 0.22$/t | (KWB, 2023) |
| Chitosan | 400 $/t | - |
| Fish feed | 430 $/t | - |
| Vege seed | 179 $/t | - |
| Exported |  |  |
| Biofuel | 1072$/t | - |
| Biochar | 350 $/t | (Amherst & Kulyk, 2012) |
| Bio-oil | 80 $/t | (EUBIA, 2012) |
| Chitosan | 400 $/t | - |
| Biogas | 0.22 $/m3; 176 $/t | (Rowland et al., 2009) |
| Syngas | 0.10 $/m3 ; 105 $/t | - |
| Vegetables | 838 $/t | - |
| Fish feed | 430 S/t | - |
| Fish product | 2479 $/t | - |
| Electricity | 0.16 $/kWh | (Ward and Løes, 2011) |
| Steam | 9.68 $/t HPS | (Saeidi et al., 2014) |
| Water | 1,200 $/t | (Quagrainie et al., 2018) |

a Pricing is shown in US Dollar.

Table S4: CO2 emissions from FWEIP facilities

| Facility | CO2 emission (kg-CO2e/unit) | References |
| --- | --- | --- |
| Emission from electricity consumption | 0.57 | (World Benchmarking Alliance, 2020) |
| Gasification | 45.00 | (Saeidi et al., 2014) |
| Combined Heat and Power (CHP) | 0.43 | (Saeidi et al., 2014) |
| Fischer-Tropsch Process | 1.70 | (Saeidi et al., 2014) |
| Hydrotreating a | 0 | - |
| Anaerobic Digestion | 21.50 | (Bücker et al., 2020) |
| BSF facility | 182.00 | (Gougbedji et al., 2021) |
| Pyrolysis | 93.40 | (Wright et al., 2010) |

a Assume no carbon dioxide emission, process itself does not produce CO2.

Table S5: Scale factor of steam and electricity demand.

| Site/Technology | Steam, t MPS/t input | Electricity, kWh/t input | Ref. |
| --- | --- | --- | --- |
| Fish tank | - | 122.00 | (Love et al., 2015) |
| Vege tank | - | 112.00 |  |
| Fish processing plant | - | 87.00 | (Murali et al., 2021) |
| BSF Facility | - | 14.50 | (Soeters et al., 2021) |
| WWTP | - | 0.80 | - |
| Biogas plant | - | 46.52 | (Jin et al., 2015) |
| Pyrolysis | 0.50 | 36.80 | (Kantarelis, 2014; Yang et al., n.d.) |
| Gasification | 0.45 | 280.00 | - |
| FT Process | - | 50.00 | - |
| Hydrotreating | - | 60.00 | - |

Table S6: Frequency of technology as pivotal player for the 3 cases.

| Cases | Gasification (Rank 1) | BSF (Rank 3) | Pyrolysis & Gasification (Rank 11) |
| --- | --- | --- | --- |
| WWTP | 0 | 0 | 0 |
| Hydroponic | 10 | 3 | 696 |
| Gasification | 7 | - | 6 |
| FT Process | 7 | - | 6 |
| BSF facility | - | 3 | - |
| Pyrolysis | - | - | 6 |
| CHP | - | - | 6 |
| Total Premutation (N!) | 24 | 6 | 720 |

Table S7: Shapley-Shubik Power Index (αP) of respective technology for the 3 cases.

| Cases | Gasification (Rank 1) | BSF (Rank 3) | Pyrolysis & Gasification (Rank 11) |
| --- | --- | --- | --- |
| WWTP | 0.00% | 0.00% | 0.00% |
| Hydroponic | 41.67% | 50.00% | 96.67% |
| Gasification | 29.17% | - | 0.83% |
| FT Process | 29.17% | - | 0.83% |
| BSF facility | - | 50.00% | - |
| Pyrolysis | - | - | 0.83% |
| CHP | - | - | 0.83% |