Appendix for Techno-economic Optimisation of Green and Clean Hydrogen Production

Yong Ying Loh1, Denny K S Ng2,1, Viknesh Andiappan3*

1School of Engineering and Physical Sciences, Heriot-Watt University Malaysia, 62200, Putrajaya, Wilayah Persekutuan Putrajaya, Malaysia

2School of Engineering and Technology, Sunway University, No. 5, Jalan Universiti, Bandar Sunway, 47500 Petaling Jaya, Selangor, Malaysia

3Biomass Waste-to-Wealth Special Interest Group, Research Centre for Sustainable Technologies, Faculty of Engineering, Computing and Science, Swinburne University of Technology Sarawak Campus, Jalan Simpang Tiga, 93350, Kuching, Sarawak, Malaysia

*corresponding author: vmurugappan@swinburne.edu.my

Figure A1: Summary of Overall Methodology (S is referring to Stage)

Table A1: Efficiency/Yield/Conversion Factor

| Electricity Generation |  | H2 Production |  | H2 Storage |  |
| --- | --- | --- | --- | --- | --- |
| Solar Energy |  | Green H2 (Electrolysis) |  | Physical Method |  |
| CS | 22% (Benda, 2020) | PEM | 0.0554 kg H2/kg H2O (Mehmeti, 2018) | Compressed H2 | 95.0% (Norouzi, 2021) |
| TF | 16% (Benda, 2020) | AWE | 0.0989 kg H2/kg H2O (Kuckshinrichs, 2017) | LH2 | 75.0% (Norouzi, 2021) |
| Parabolic Trough | 16% (Gielen, 2012) | SOEC | 0.110kg H2/kg H2O (Mehmeti, 2018) | Chemical Method |  |
| Power Tower | 20% (Gielen, 2012) | AEM | 1250 kg H2/kg H2O (Schmidt, 2021) | Solid Hydride |  |
| Biomass Energy (EFB) |  | Green H2 (POME) |  | Metal Hydride | 88.0% (Vincent, 2018) |
| Combustion | 1.037 MW/kg HPS | DF | 0.621 kg H2/kg POME | Chemical Storage |  |
| Pyrolysis | 3.556 MW/kg CO | MEC | 1.023 kg H2/kg POME | LOH | 88.7% (Wang, 2016) |
| Gasification | 3.048 MW/kg CO | Turquoise H2 |  | Circular Carrier |  |
|  |  | Methane Pyrolysis | 0.9 kg H2/kg CH4 (Daliah, 2021) | NH3 | 93.0% (Norouzi, 2021) |
| Biomass Energy (POME) |  |  |  |  |  |
| Anaerobic Digestion | 13.680 MW/kg BG |  |  |  |  |

Table A2: CAPEX Factor

| Electricity Generation |  | H2 Production |  | H2 Storage |  |
| --- | --- | --- | --- | --- | --- |
| Solar Energy |  | Green H2 (Electrolysis) |  | Physical Method |  |
| CS | 1,130.00 $/kW (Perry, 2016) | PEM | 2,034.00 $/kW (Roode, 2019) | Compressed H2 | 23.40 $/(kg/hr) (Patonia, 2020) |
| TF | 92.10.00 $/m2 (Águas, 2011) | AWE | 1,582.00 $/kW (Roode, 2019) | LH2 | 199.00 $/(kg/hr) (Patonia, 2020) |
| Parabolic Trough | 4,600.00 $/kW (Gielen, 2012) | SOEC | 2,599.00 $/kW (Roode, 2019) | Chemical Method |  |
| Power Tower | 7,500.00 $/kW (Gielen, 2012) | AEM | 4,237.50 $/kW (Schmidt, 2021) | Solid Hydride |  |
| Biomass Energy (EFB) |  | Green H2 (POME) |  | Metal Hydride | 82,000.00 $/(kg/hr) (Sharma, 2017) |
| Combustion | 3,860.00 $/kW (Kennedy, 2013) | DF | 0.017 $/(kg POME/hr) (James, 2016) | Chemical Storage |  |
| Pyrolysis | 949.00 $/kW (Ganesh, 2001) | MEC | 105.29 $/(kg POME/hr) (Aiken, 2019) | LOH | 14,200.00 $/(kg/hr) (Sharma, 2017) |
| Gasification | 562.74 $/kW (Susanto, 2018) | Turquoise H2 |  | Circular Carrier |  |
| Biomass Energy (POME) |  | Methane Pyrolysis | 37,022.07 $/(kg H2/hr) (Labanca, 2006) | NH3 | 37,200.00 $/(kg/hr) (Sharma, 2017) |
| Anaerobic Digestion | 4,292.40 $/(kg POME/hr) (Sharvelle, 2016) |  |  |  |  |

Table A3: OPEX Factor

| Electricity Generation |  | H2 Production |  | H2 Storage |  |
| --- | --- | --- | --- | --- | --- |
| Solar Energy |  | Green H2 (Electrolysis) |  | Physical Method |  |
| CS | 0.042 $/kW (Bellini, 2021) | PEM | 5.14 $/kg H2 (Zore, 2021) | Compressed H2 | 0.40 $/kg H2 (Dias, 2020) |
| TF | 0.038 $/kW (Bellini, 2021) | AWE | 5.92 $/kg H2 (Pinsky, 2020) | LH2 | 2.17 $/kg H2 (Dias, 2020) |
| Parabolic Trough | 0.36 $/kW (Gielen, 2012) | SOEC | 4.96 $/kg H2 (Zore, 2021) | Chemical Method |  |
| Power Tower | 0.29 $/kW (Gielen, 2012) | AEM | 2.00 $/kg H2 (Li, 2021) | Solid Hydride |  |
| Biomass Energy (EFB) |  | Green H2 (POME) |  | Metal Hydride | 1,430.00 $/kg H2 (EERE, 2013) |
| Combustion | 0.15 $/kW (FEMP, 2016) | DF | 2.57 $/kg H2 (Kadier, 2016) | Chemical Storage |  |
| Pyrolysis | 0.21 $/kW (Montoya, 2020) | MEC (Kadier et al., 2016) | 2.60 $/kg H2 (Kadier, 2016) | LOH | 1.54 $/kg H2 (Daniel, 2020) |
| Gasification | 0.27 $/kW (Dimpl, 2011) | Turquoise H2 |  | Circular Carrier |  |
| Biomass Energy (POME) |  | Methane Pyrolysis | 2.19 $/kg H2 (Zore, 2021) | NH3 | 0.18 $/kW (Brown, 2020) |
| Anaerobic Digestion | 0.23 $/kW (Juliana, 2018) |  |  |  |  |

Table A4: Energy Factor

| H2 Production |  | H2 Storage |  |
| --- | --- | --- | --- |
| Green H2 (Electrolysis) |  | Physical Method |  |
| PEM | 54.60 kW/kg H2 (Mehmeti, 2018) | Compressed H2 | 6.00 kW/kg H2 (Sheffield, 2014) |
| AWE | 53.90 kW/kg H2 (Kuckshinrichs, 2017) | LH2 | 10.00 kW/kg H2 (Gardiner, 2009) |
| SOEC | 36.14 kW/kg H2 (Mehmeti, 2018) | Chemical Method |  |
| AEM | 60.09 kW/kg H2 (Wang, 2012) | Solid Hydride |  |
| Green H2 (POME) |  | Metal Hydride | 3.00 kW/kg H2 (Sheffield, 2014) |
| DF | 21.60 kW/kg H2 (Mehmeti, 2018) | Chemical Storage |  |
| MEC |  | LOH | 0.70 kW/kg H2 (Andersson, 2019) |
| Turquoise H2 |  | Circular Carrier |  |
| Methane Pyrolysis | 5.23 kW/kg H2 (Machhammer, 2016) | NH3 | 2.00 kW/kg H2 (Andersson, 2019) |

Table A5: Cost of Raw Material

| Raw Material | Cost ($/kg) |
| --- | --- |
| EFB | 0.012 (Reeb, 2014) |
| POME | 0.74 (Parmar, 2020) |
| Water | 0.00055 (AirSelangor, 2022) |
| Methane | 2.37 (Valev, 2022) |

Table A6: Solar Energy Can be Collected

| Technology | Solar Energy Can be Collected (kW/h m2) |
| --- | --- |
| CS | 0.125 |
| TF | 0.0667 |
| Parabolic Trough | 0.253 |
| Power Tower | 0.138 |

Table A7: Minimum Area of Solar Panels

| Technology | Minimum Area Required (m2) |
| --- | --- |
| CS | 1.635 (Swanson, 2021) |
| TF | 1.635 (Swanson, 2021) |
| Parabolic Trough | 76.440 (Günther, 2015) |
| Power Tower | 100.000 (Breeze, 2016) |

Although solar panels does not consist upper limit on the implemented area, a maximum area of 1 x 107 m2 is applied to all of the solar energy technologies.

Table A8: Detailed Information for Combustion of EFB

| Parameter | Value | Unit |
| --- | --- | --- |
| Calorific Value of EFB (Nyakuma, 2014) | 17.02 | MJ/kg EFB |
| Boiler Efficiency (CDM, 2001) | 75 | % |
| Heat Content of EFB (Protásio, 2013) | 12.765 | MJ heat/kg EFB |
| Pressure of High Pressure Steam (HPS) (CDM, 2001) | 25 | barg |
| Temperature of HPS (CDM, 2001) | 226 | oC |
| Enthalpy of HPS (CDM, 2001) | 2.802 | MJ heat/kg HPS |
| Conversion of EFB to HPS | 4.555 | kg HPS/kg EFB |
| Steam Turbine Efficiency (Jane, 1982) | 37 | % |
| Electricity Produced from HPS | 1.037 | MW/kg HPS |

Table A9: Detailed Information for Pyrolysis and Gasification of EFB

| Parameter | Pyrolysis | Gasification | Unit |
| --- | --- | --- | --- |
| Calorific Value of EFB (Nyakuma, 2014) | 17.02 |  | MJ/kg EFB |
| Yield of Syngas (SG) (Bogale, 2017) | 35 | 85 | % |
| Temperature | 500 (Zajec, 2009) | 700 (Mohammed, 2011) | oC |
| Composition of H2 | 0.429 (Zajec, 2009) | 0.485 (Mohammed, 2011) | wt% |
| Composition of Carbon Monoxide (CO) | 47.912 (Zajec, 2009) | 27.720 (Mohammed, 2011) | wt% |
| Heating Value of CO | 5.478 (Zajec, 2009) | 10.160 (Mohammed, 2011) | MJ heat/kg CO |
| Gas Engine Efficiency | 35 (MacKay, 2011) | 30 (Ralph, 2007) | % |
| Electricity Produced from CO | 3.556 | 3.048 | MW/kg CO |
| Recovery of H2 (Logan, 2015) | 95 |  | % |

Table A10: Detailed Information for Anaerobic Digestion of POME

| Parameter | Value | Unit |
| --- | --- | --- |
| COD of POME (Aziz, 2018) | 40,667 | mg/L |
| Conversion from POME to Biogas (BG) (Aziz, 2018) | 0.0314 | kg BG/kg POME |
| Heat Content of POME (Protásio, 2013) | 1,275.374 | MJ heat/kg POME |
| Heating Value of Methane (CH4) (Shakib, 2019) | 55.384 | MJ heat/kg CH4 |
| Composition of CH4 (Shakib, 2019) | 65 | % |
| Heating Value of BG (Shakib, 2019) | 36.000 | MJ heat/kg BG |
| Gas Engine Efficiency (Beegle, 2018) | 38 | % |
| Electricity Produced from BG | 13.680 | MW/kg BG |

Table A11: Detailed Information for DFMEC

| Parameter | DF | MEC | Unit |
| --- | --- | --- | --- |
| Density of POME (Ali, 2013) | 876 |  | g/litre |
| COD (Ali, 2013) | 82.911 | 61.183 | g COD/litre POME |
| Density of H2 (Portal, 2021) | 0.0899 |  | g/litre |
| Yield of H2 from COD | 0.007 | 0.015 | g H2/g COD |
| Yield of H2 from POME | 0.621 | 1.023 | kg H2/kg POME |

Table A12: Production Capacity

| Technology | Equipment | Minimum | Maximum | Unit |
| --- | --- | --- | --- | --- |
| Combustion | Water Tube Boiler (Hong, 2022) | 5,000 | 120,000 | kg HPS/hr |
|  | Steam Turbine (Jane, 1982) | 1,000 | 1,000,000 | kW |
| Pyrolysis | Pyrolyser (Monnet, 2020) | 188 | 1,500 | kg EFB/hr |
|  | Gas Engine (Haga, 2011) | 1,000 | 1,000,000 | kW |
| Gasification | Gasifier (Frank, 2015) | 83.333 | 2,000 | kg EFB/hr |
|  | Gas Engine (Haga, 2011) | 1,000 | 1,000,000 | kW |
| Anaerobic Digestion | Gas Engine (Haga, 2011) | 1,000 | 23,000 | kW |
| Green H2 Production | PEM (Roode, 2019) | 0.0899 | 35.953 | kg H2/hr |
|  | AWE (Roode, 2019) | 0.0899 | 107.856 | kg H2/hr |
|  | SOEC (Roode, 2019) | 0.0899 | 5.662 | kg H2/hr |
|  | AEM (Wang, 2012) | 0.0449 | 0.899 | kg H2/hr |
| Turquoise H2 Production | Pyrolyser (Mesrobian, 2021) | 1 | 400 | kg H2/hr |

Table A13: Unit Cost for Maximum Capacity Electrolyser

| Technology | Value | Unit |
| --- | --- | --- |
| PEM | 3.99 | M$ |
| AWE | 9.20 | M$ |
| SOEC | 0.53 | M$ |
| AEM | 0.23 | M$ |

Table A14: Storage Condition

| Technology | Value | Unit |
| --- | --- | --- |
| Compressed H2 | 70 (Sheffield, 2014) | MPa |
| LH2 | 20 (Gardiner, 2009) | K |
| Metal Hydride | 1 (Sheffield, 2014) | MPa |
| LOH | 3 (Andersson, 2019) | MPa |
| NH3 | 1 (Andersson, 2019) | MPa |

Table A15: Water Consumption

| Technology | Value | Unit |
| --- | --- | --- |
| Combustion | 20.986 (Wirth, 2014) | kg H2O/kWh |
| Gasification | 0.5 (Rajabi, 2018) | kg H2O/kg EFB |

References

Águas, H., Ram, S.K., Araújo, A., Gaspar, D., Vicente, A., Filonovich, S.A., Fortunato, E., Martins, R., Ferreira, I., 2011. Silicon thin film solar cells on commercial tiles. Energy Environ. Sci. 4, 4620–4632. https://doi.org/10.1039/c1ee02303a

Aiken, D.C., Curtis, T.P., Heidrich, E.S., 2019. Avenues to the financial viability of microbial electrolysis cells [MEC] for domestic wastewater treatment and hydrogen production. Int. J. Hydrogen Energy 44, 2426–2434. https://doi.org/10.1016/j.ijhydene.2018.12.029

AirSelangor, 2022. WATER TARIFF [WWW Document].

Ali, E.N., Tay, C.I., 2013. Characterization of Biodiesel Produced from Palm Oil via Base Catalyzed Transesterification. Procedia Eng. 53, 7–12. https://doi.org/10.1016/j.proeng.2013.02.002

Andersson, J., Grönkvist, S., 2019. Large-scale storage of hydrogen. Int. J. Hydrogen Energy 44, 11901–11919. https://doi.org/10.1016/j.ijhydene.2019.03.063

Aziz, N.I.H.A., Hanafiah, M.M., 2018. Anaerobic digestion of palm oil mill effluent (POME) using bio-methane potential (BMP) test. p. 020026. https://doi.org/10.1063/1.5027941

Beegle, J.R., Borole, A.P., 2018. Energy production from waste: Evaluation of anaerobic digestion and bioelectrochemical systems based on energy efficiency and economic factors. Renew. Sustain. Energy Rev. 96, 343–351. https://doi.org/10.1016/j.rser.2018.07.057

Bellini, E., 2021. Solar retains it LCOE edge in latest Lazard analysis. pv Mag.

Benda, V., Černá, L., 2020. PV cells and modules – State of the art, limits and trends. Heliyon 6, e05666. https://doi.org/10.1016/j.heliyon.2020.e05666

Benny Wang, 2012. AEM Water Electrolysis Equipment [WWW Document]. Weiqing Ind. Park.

Bogale, W., 2017. Preparation of Charcoal Using Flower Waste. J. Power Energy Eng. 05, 1–10. https://doi.org/10.4236/jpee.2017.52001

Breeze, P., 2016. Solar Towers. Sol. Power Gener. 35–40. https://doi.org/10.1016/B978-0-12-804004-1.00005-1

Brown, T., 2020. Engie, Siemens, Ecuity, and STFC publish Feasibility of Ammonia-to-Hydrogen [WWW Document]. Ammon. Energy Assoc.

CDM, 2001. Energy efficiency improvement projects: boiler rehabilitation or replacement in industrial and district heating sectors.

Daliah, R., 2021. TECHNOLOGY LANDSCAPE: KEY PLAYERS IN METHANE PYROLYSIS [WWW Document]. Lux Res.

Daniel, 2020. Unlocking the Global Hydrogen Opportunity, by Charlie Clark, AP Ventures [WWW Document]. Hydrog. LOHC Technol. GmbH.

Dias, V., Pochet, M., Contino, F., Jeanmart, H., 2020. Energy and Economic Costs of Chemical Storage. Front. Mech. Eng. 6. https://doi.org/10.3389/fmech.2020.00021

Dimpl, E., 2011. Small-scale Electricity Generation from Biomass. Eschborn.

EERE, 2013. Materials-Based Hydrogen Storage [WWW Document]. Off. Energy Effic. Renew. Energy.

FEMP, 2016. Biomass For Electricity Generation [WWW Document]. U.S. Dep. Energy.

Frank, 2015. Gasification of Biomass/Waste [WWW Document]. Beltran Technol.

Ganesh, A., Banerjee, R., 2001. Biomass pyrolysis for power generation - A potential technology. Renew. Energy 22, 9–14. https://doi.org/10.1016/S0960-1481(00)00026-4

Gardiner, M., 2009. Energy requirements for hydrogen gas compression and liquefaction as related to vehicle storage needs.

Gielen, D., 2012. Concentrating Solar Power. Renew. ENERGY Technol. COST Anal. Ser. 1.

Günther, M., 2015. No Title. Parabol. Trough Technol. 1–106.

Haga, N., 2011. Combustion engine power plants. Finland.

Hong, 2022. The next generation steamboilers [WWW Document]. HKB Boil. Solut.

James, B.D., DeSantis, D.A., Saur, G., 2016. Final Report: Hydrogen Production Pathways Cost Analysis (2013 – 2016). Golden, CO (United States). https://doi.org/10.2172/1346418

Jane, 1982. Production of electrical power, in: The Efficient Use of Energy. Elsevier, pp. 286–308. https://doi.org/10.1016/B978-0-408-01250-8.50022-2

Juliana, 2018. Economic Implications of Anaerobic Digestion for Bioenergy Production and Waste Management. Agric. Nat. Resour.

Kadier, A., Simayi, Y., Abdeshahian, P., Azman, N.F., Chandrasekhar, K., Kalil, M.S., 2016. A comprehensive review of microbial electrolysis cells (MEC) reactor designs and configurations for sustainable hydrogen gas production. Alexandria Eng. J. 55, 427–443. https://doi.org/10.1016/j.aej.2015.10.008

Kennedy Smith, R., Hobbs, B.F., 2013. Biomass combustion for electric power: Allocation and plant siting using non-linear modeling and mixed integer optimization. J. Renew. Sustain. Energy 5. https://doi.org/10.1063/1.4819493

Kuckshinrichs, W., Ketelaer, T., Koj, J.C., 2017. Economic Analysis of Improved Alkaline Water Electrolysis. Front. Energy Res. 5. https://doi.org/10.3389/fenrg.2017.00001

Labanca, 2006. ECONOMIC ANALYSIS OF CO2 – FREE HYDROGENPRODUCTION FROM NATURAL GAS BY PLASMA PYROLYSIS. Rio Oil Gas Expo Conf. 1–8.

Li, C., Baek, J.-B., 2021. The promise of hydrogen production from alkaline anion exchange membrane electrolyzers. Nano Energy 87, 106162. https://doi.org/10.1016/j.nanoen.2021.106162

Logan, C., 2015. Membrane gas separation: Fundamentals and applications Giuseppe BARBIERI Institute on Membrane Technology (ITM-CNR), National Research Council [WWW Document]. Natl. Res. Counc.

Machhammer, O., Bode, A., Hormuth, W., 2016. Financial and Ecological Evaluation of Hydrogen Production Processes on Large Scale. Chem. Eng. Technol. 39, 1185–1193. https://doi.org/10.1002/ceat.201600023

MacKay, D., 2011. Biomass Pyrolysis. UK.

Mehmeti, A., Angelis-Dimakis, A., Arampatzis, G., McPhail, S.J., Ulgiati, S., 2018. Life cycle assessment and water footprint of hydrogen production methods: From conventional to emerging technologies. Environ. - MDPI 5, 1–19. https://doi.org/10.3390/environments5020024

Mesrobian, C., 2021. Taking Methane Pyrolysis from Concept to Industrial Plant.

Mohammed, M.A.A., Salmiaton, A., Wan Azlina, W.A.K.G., Mohammad Amran, M.S., Fakhru’L-Razi, A., 2011. Air gasification of empty fruit bunch for hydrogen-rich gas production in a fluidized-bed reactor. Energy Convers. Manag. 52, 1555–1561. https://doi.org/10.1016/j.enconman.2010.10.023

Monnet, J., 2020. Find the right size for industrial pyrolysis unit [WWW Document]. Biogreen.

Montoya, J., Valdés, C., Chaquea, H., Pecha, M.B., Chejne, F., 2020. Surplus electricity production and LCOE estimation in Colombian palm oil mills using empty fresh bunches (EFB) as fuel. Energy 202, 117713. https://doi.org/10.1016/j.energy.2020.117713

Norouzi, N., 2021. An overview on the renewable hydrogen generation market. Int. J. ENERGY Stud. 6, 67–94.

Nyakuma, B.B., Johari, A., Ahmad, A., Tuan Abdullah, T.A., 2014. Thermogravimetric Analysis of the Fuel Properties of Empty Fruit Bunch Briquettes. J. Teknol. 67. https://doi.org/10.11113/jt.v67.2768

Parmar, A., 2020. Asian biodiesel resurgence limited by costs [WWW Document]. Arg. Media.

Patonia, A., 2020. Ammonia as a storage solution for future decarbonized energy systems. Oxford Inst. Energy Stud. 1–35.

Perry, N., 2016. Best Practice Guidelines for PV Cost Calculation. UK.

Pinsky, R., Sabharwall, P., Hartvigsen, J., O’Brien, J., 2020. Comparative review of hydrogen production technologies for nuclear hybrid energy systems. Prog. Nucl. Energy 123, 103317. https://doi.org/10.1016/j.pnucene.2020.103317

Portal, 2021. Basic Hydrogen Properties [WWW Document]. Pacific Northwest Natl. Lab.

Protásio, T.D.P., Bufalino, L., Tonoli, G.H.D., Guimarães Junior, M., Trugilho, P.F., Mendes, L.M., 2013. Brazilian Lignocellulosic Wastes for Bioenergy Production: Characterization and Comparison with Fossil Fuels. BioResources 8. https://doi.org/10.15376/biores.8.1.1166-1185

Rajabi Hamedani, S., Villarini, M., Colantoni, A., Moretti, M., Bocci, E., 2018. Life Cycle Performance of Hydrogen Production via Agro-Industrial Residue Gasification—A Small Scale Power Plant Study. Energies 11, 675. https://doi.org/10.3390/en11030675

Ralph, 2007. Bioenergy Project Development & Biomass Supply. France.

Reeb, C.W., Hays, T., Venditti, R.A., Gonzalez, R., Kelley, S., 2014. Supply Chain Analysis, Delivered Cost, and Life Cycle Assessment of Oil Palm Empty Fruit Bunch Biomass for Green Chemical Production in Malaysia. BioResources 9, 5385–5416. https://doi.org/10.15376/biores.9.3.5385-5416

Roode‐Gutzmer, Q.I., Kaiser, D., Bertau, M., 2019. Renewable Methanol Synthesis. ChemBioEng Rev. 6, 209–236. https://doi.org/10.1002/cben.201900012

Schmidt, 2021. Enapter Makes Headway on AEM Electrolyzer Mass Production [WWW Document]. Enapter.

Shakib, N., Rashid, M., 2019. Biogas Production Optimization from POME by Using Anaerobic Digestion Process. J. Appl. Sci. Process Eng. 6, 369–377. https://doi.org/10.33736/jaspe.1711.2019

Sharma, M., Kaushik, A., 2017. Biohydrogen Economy: Challenges and Prospects for Commercialization, in: Biohydrogen Production: Sustainability of Current Technology and Future Perspective. Springer India, New Delhi, pp. 253–267. https://doi.org/10.1007/978-81-322-3577-4_12

Sharvelle, 2016. Anaerobic Digesters [WWW Document]. Explor. Energy Effic. Altern.

Sheffield, J.W., Martin, K.B., Folkson, R., 2014. Electricity and hydrogen as energy vectors for transportation vehicles, in: Alternative Fuels and Advanced Vehicle Technologies for Improved Environmental Performance. Elsevier, pp. 117–137. https://doi.org/10.1533/9780857097422.1.117

Susanto, H., Suria, T., Pranolo, S.H., 2018. Economic analysis of biomass gasification for generating electricity in rural areas in Indonesia. IOP Conf. Ser. Mater. Sci. Eng. 334, 012012. https://doi.org/10.1088/1757-899X/334/1/012012

Swanson, 2021. How Many Solar Panels Do You Need: Panel Size and Output Factors [WWW Document]. Sunpower.

Valev, N., 2022. Methane prices [WWW Document]. Glob. Pet. Price.

Vincent, I., Bessarabov, D., 2018. Low cost hydrogen production by anion exchange membrane electrolysis: A review. Renew. Sustain. Energy Rev. 81, 1690–1704. https://doi.org/10.1016/j.rser.2017.05.258

Wang, H., Zhou, X., Ouyang, M., 2016. Efficiency analysis of novel Liquid Organic Hydrogen Carrier technology and comparison with high pressure storage pathway. Int. J. Hydrogen Energy 41, 18062–18071. https://doi.org/10.1016/j.ijhydene.2016.08.003

Wirth, M., 2014. Steam Boilers – Accuracy Matters [WWW Document]. WCP.

Zajec, L., 2009. SLOW PYROLYSIS IN A ROTARY KILN REACTOR: OPTIMIZATION AND EXPERIMENTS. Iceland.

Zore, U.K., Yedire, S.G., Pandi, N., Manickam, S., Sonawane, S.H., 2021. A review on recent advances in hydrogen energy, fuel cell, biofuel and fuel refining via ultrasound process intensification. Ultrason. Sonochem. 73. https://doi.org/10.1016/j.ultsonch.2021.105536