Interface-Optimized Hollow Metal–Phenolic PDA@ZIF-8 Nanocarriers for Stimuli-Responsive Zn2+/Salicylate Co-Delivery and Efficient Control of Tobacco Bacterial Wilt

Ze Lv1, 2, Yu Fang1, 2, Yang Gao1, 2, Hongwei Li1, 2, Pengkun Yan1, 2, Zheng Zhang1, 2, Lei Wang1, 2, Fengyu Li1, 2, Yingjian Ma1, 2, Rui Zhao1, 2, Xinyu Guo1, 2, Xuemin Wu*1, 2, and Yong Xu*1, 2

1 Innovation Center of Pesticide Research

Department of Applied Chemistry

College of Science

China Agricultural University

Beijing 100193, P. R. China

E-mail: cauxy@cau.edu.cn; wuxuemin@cau.edu.cn

2 Key Laboratory of National Forestry and Grassland Administration on Pest Chemical Control

China Agricultural University

Beijing 100193, P. R. China.

Material and Method

ROS Detection

Intracellular reactive oxygen species (ROS) levels in R. solanacearum were evaluated using the fluorescent probe 2′,7′-dichlorodihydrofluorescein diacetate (DCFH-DA). Briefly, bacterial cells were cultured in nutrient broth at 28 °C to the logarithmic growth phase, collected by centrifugation (5000 rpm, 5 min), washed twice with sterile phosphate-buffered saline (PBS, pH 7.4), and resuspended to an optical density of OD600 ≈ 0.5. The bacterial suspensions were treated with ZIF-8 or NaSA@PDA@ZIF-8 at final concentrations of 100, 200, and 500 mg L⁻¹, respectively, and incubated at 30 °C for 4 h under gentle shaking. Untreated bacterial suspensions served as the control group. After treatment, cells were collected by centrifugation, washed twice with PBS to remove residual materials, and incubated with DCFH-DA (final concentration: 10 μM) in the dark at 30 °C for 30 min. Perform a semi-quantitative analysis of the laser confocal imaging results to obtain the average fluorescence signal intensity under different treatments.

Following incubation, the cells were washed again with PBS to eliminate excess probe and resuspended in PBS for fluorescence analysis. Intracellular ROS-associated fluorescence was observed using a confocal laser scanning microscope with an excitation wavelength of 488 nm and emission collected at 500–550 nm.  All experiments were performed in triplicate.

Table S1 Seventeen sets of experiments conducted with different experimental factorial designs for response surface model.

| Run | Response time (h) | Reaction temperature (℃) | Dopamine addition（g） | Loading contact（%） |
| --- | --- | --- | --- | --- |
| 1 | 6 | 55 | 1 | 21.5 |
| 2 | 6 | 35 | 1.5 | 6.6 |
| 3 | 5 | 55 | 0.5 | 4.5 |
| 4 | 6 | 35 | 0.5 | 5.8 |
| 5 | 7 | 75 | 1 | 13.8 |
| 6 | 7 | 55 | 0.5 | 17.2 |
| 7 | 7 | 35 | 1 | 14.2 |
| 8 | 5 | 35 | 1 | 7.4 |
| 9 | 6 | 75 | 1.5 | 6.1 |
| 10 | 6 | 55 | 1 | 22.4 |
| 11 | 6 | 55 | 1 | 23.1 |
| 12 | 6 | 75 | 0.5 | 6.1 |
| 13 | 5 | 75 | 1 | 5.4 |
| 14 | 6 | 55 | 1 | 20.7 |
| 15 | 6 | 55 | 1 | 23.3 |
| 16 | 7 | 55 | 1.5 | 20.7 |
| 17 | 5 | 55 | 1.5 | 7.8 |

Table S2 ANOVA for response surface quadratic regression equation.

| Source | df | Mean Square | F-value | p-value |  |
| --- | --- | --- | --- | --- | --- |
| Model | 9 | 95.35 | 28.49 | 0.0001 | significant |
| A-Response time | 1 | 208.08 | 62.18 | < 0.0001 |  |
| B-Reaction temperature | 1 | 0.8450 | 0.2525 | 0.6307 |  |
| C-Dopamine addition | 1 | 7.22 | 2.16 | 0.1853 |  |
| AB | 1 | 0.6400 | 0.1912 | 0.6751 |  |
| AC | 1 | 0.0100 | 0.0030 | 0.9579 |  |
| BC | 1 | 0.1600 | 0.0478 | 0.8332 |  |
| A² | 1 | 33.01 | 9.86 | 0.0164 |  |
| B² | 1 | 356.38 | 106.50 | < 0.0001 |  |
| C² | 1 | 197.57 | 59.04 | 0.0001 |  |
| Residual | 7 | 3.35 |  |  |  |
| Lack of Fit | 3 | 6.21 | 5.17 | 0.0731 | not significant |
| Pure Error | 4 | 1.20 |  |  |  |
| Cor Total | 16 |  |  |  |  |

Figure S1 TEM images of PDA@ZIF-8 at different temperatures and reaction times.

Figure S2 Normal Plot of Residuals (A) and Predicted vs Actual (B) values.

Table S3 Surface Element Composition in XPS Analysis

| element | Atomic (%) |  |
| --- | --- | --- |
|  | ZIF-8 | PDA@ZIF-8 |
| Zn | 3.96 | 5.68 |
| C | 69.29 | 60.58 |
| N | 14.27 | 9.23 |
| O | 7379 | 18.36 |

Figure S3 Normalized diameter profiles of droplets in different suspensions over 20 ms.

Figure S4 Dynamic contact angle images (A), data (B) and adhesion work (C) of ZIF-8, PDA@ZIF-8, and NaSA@PDA@ZIF-8 on tobacco seedlings.

Figure S5 Changes in fluorescence of FITC-labeled water, ZIF-8, PDA@ZIF-8-treated leaves before and after rainfall washout.

Table S4 Release fitting parameters for NaSA's Zero order, First order, Higuchi and Ritger-peppas models.

| Response condition | Fitting model | Kinetic equation | a | b | R2 |
| --- | --- | --- | --- | --- | --- |
| 35 ℃ | Zero-order | y=a*x+b | 0.6515 | 46.8791 | 0.9050 |
|  | First-order | y=a*(1−e^(−b*x)) | 72.3599 | 2.9582 | 0.7894 |
|  | Higuchi | y=a*(x^(1/2))+b | 5.4166 | 42.8002 | 0.9811 |
|  | Ritger-peppas | y=a*(x^b) | 51.1378 | 0.1154 | 0.9938 |
| 25 ℃ | Zero-order | y=a*x+b | 0.4232 | 38.1992 | 0.6224 |
|  | First-order | y=a*(1−e^(−b*x)) | 60.7215 | 0.5475 | 0.5758 |
|  | Higuchi | y=a*(x^(1/2))+b | 4.9768 | 30.3945 | 0.8707 |
|  | Ritger-peppas | y=a*(x^b) | 34.5299 | 0.1649 | 0.9532 |
| 15 ℃ | Zero-order | y=a*x+b | 0.1366 | 44.5877 | 0.4460 |
|  | First-order | y=a*(1−e^(−b*x)) | 48.5965 | 15.4047 | 0.3306 |
|  | Higuchi | y=a*(x^(1/2))+b | 1.8998 | 41.4574 | 0.7034 |
|  | Ritger-peppas | y=a*(x^b) | 43.2994 | 0.0667 | 0.9555 |
| pH=5 | Zero-order | y=a*x+b | 0.1349 | 70.1988 | 0.3567 |
|  | First-order | y=a*(1−e^(−b*x)) | 73.9753 | 1.1505 | 0.4538 |
|  | Higuchi | y=a*(x^(1/2))+b | 2.4458 | 61.3655 | 0.6815 |
|  | Ritger-peppas | y=a*(x^b) | 56.8687 | 0.0809 | 0.9609 |
| pH=9 | Zero-order | y=a*x+b | 0.1411 | 30.7464 | 0.4918 |
|  | First-order | y=a*(1−e^(−b*x)) | 35.2822 | 4.6523 | 0.3292 |
|  | Higuchi | y=a*(x^(1/2))+b | 1.6587 | 27.4723 | 0.6925 |
|  | Ritger-peppas | y=a*(x^b) | 29.8638 | 0.0790 | 0.8898 |

| Response condition | Fitting model | Kinetic equation | a | b | R2 |
| --- | --- | --- | --- | --- | --- |
| 35 ℃ | Zero-order | y=a*x+b | 0.0993 | 7.7257 | 0.8440 |
|  | First-order | y=a*(1−e^(−b*x)) | 16.9937 | 0.1163 | 0.9319 |
|  | Higuchi | y=a*(x^(1/2))+b | 1.3309 | 5.0511 | 0.9453 |
|  | Ritger-peppas | y=a*(x^b) | 5.1230 | 0.2757 | 0.9837 |
| 25 ℃ | Zero-order | y=a*x+b | 0.1677 | 1.6572 | 0.6692 |
|  | First-order | y=a*(1−e^(−b*x)) | 11.8165 | 0.1547 | 0.8839 |
|  | Higuchi | y=a*(x^(1/2))+b | 1.7962 | 0.3874 | 0.9506 |
|  | Ritger-peppas | y=a*(x^b) | 2.4094 | 0.4265 | 0.9704 |
| 15 ℃ | Zero-order | y=a*x+b | 0.1730 | 4.0916 | 0.7559 |
|  | First-order | y=a*(1−e^(−b*x)) | 11.8785 | 0.1164 | 0.9025 |
|  | Higuchi | y=a*(x^(1/2))+b | 1.6749 | 1.4454 | 0.9516 |
|  | Ritger-peppas | y=a*(x^b) | 3.0622 | 0.3671 | 0.9803 |
| pH=5 | Zero-order | y=a*x+b | 0.2376 | 0.6945 | 0.8842 |
|  | First-order | y=a*(1−e^(−b*x)) | 19.9361 | 0.0537 | 0.9822 |
|  | Higuchi | y=a*(x^(1/2))+b | 2.3378 | -0.3745 | 0.9849 |
|  | Ritger-peppas | y=a*(x^b) | 2.4319 | 0.4845 | 0.9823 |
| pH=9 | Zero-order | y=a*x+b | 0.0463 | 5.6488 | 0.6692 |
|  | First-order | y=a*(1−e^(−b*x)) | 9.8238 | 0.1020 | 0.9809 |
|  | Higuchi | y=a*(x^(1/2))+b | 0.6827 | 3.6769 | 0.9031 |
|  | Ritger-peppas | y=a*(x^b) | 3.4257 | 0.2431 | 0.9667 |

Table S5 Release fitting parameters for Zn2+'s Zero order, First order, Higuchi and Ritger-peppas models.

Figure S6 Cumulative release of NaSA and Zn2+ at different pH and temperatures were fitted with zero-order, first-order, Higuchi, and Ritger-peppas models.

The release profiles were fitted using the empirical power-law equation:

y=a*(x^b)

which is mathematically consistent with the Ritger–Peppas model1,2. In this equation, y is the cumulative release, x is time, a is the apparent release rate constant, and b is the release exponent associated with the transport mechanism. For approximately spherical carriers, b≤0.43 generally indicates Fickian diffusion, whereas 0.43<b<0.85 suggests anomalous (non-Fickian) transport involving both diffusion and structural relaxation/reorganization3,4.

To further clarify the release mechanism, the release profiles of NaSA and Zn2+ were analyzed using the Ritger–Peppas model. During the release of NaSA, b = 0.0809 at pH 5 and b = 0.0790 at pH 9, both of which are significantly lower than 0.43, indicating that the process is primarily governed by Fickian diffusion under both conditions. This result suggests that, as an encapsulated small-molecule active ingredient, NaSA is primarily released from the hollow carrier via diffusion driven by a concentration gradient5. The a value at pH 5 (56.8687) is higher than that at pH 9 (29.8638), indicating that acidic conditions accelerate the release of NaSA. This acceleration can reasonably be attributed to the weakening of the coordination bond between Zn2+ and phenolic groups under acidic conditions, as well as the increased permeability of the hollow PDA-Zn framework6,7. These changes reduce diffusion resistance but do not fundamentally alter the primary transport mechanism. Furthermore, this hollow structure shortens the diffusion path, which is consistent with the strongly diffusion-controlled behavior reflected by the low b-value.

The fitted b values for NaSA release at 35°C, 25°C, and 15°C were 0.1154, 0.1649, and 0.0667, respectively. All these values are significantly lower than 0.43, indicating that the NaSA release process is consistently governed by Fickian diffusion, a behavior consistent with the structural characteristics of the carrier8. Specifically, NaSA acts as a guest molecule within the hollow PDA-Zn framework, and the hollow structure shortens the diffusion path, thereby facilitating concentration-gradient-driven outward diffusion9,10. The temperature-dependent variation in parameter a reflects changes in the apparent release kinetics, but the low b values indicate that temperature does not fundamentally alter the primary release mechanism.

Regarding the release of Zn2+, the fitted value of b was 0.2431 at pH 9, indicating Fickian diffusion; whereas at pH 5, the value of b increased to 0.4845, corresponding to non-Fickian transport. Unlike NaSA, Zn2+ is structurally associated with the reconstructed PDA-Zn coordination framework. Consequently, its release process is more sensitive to changes in network stability induced by pH variations. Under basic conditions, the release of Zn2+ is primarily governed by diffusion processes. However, under acidic conditions, the release process likely involves not only the outward diffusion of Zn2+ but also the weakening/reorganization of the metal-phenolic coordination network, which contributes to the shift toward non-Fickian behavior. Since a and b are interrelated parameters in the Ritger-Peppas model, the a values for Zn2+ under different pH conditions should not be interpreted in isolation but should be considered in conjunction with the corresponding changes in b. Overall, the results indicate that the release of NaSA is primarily diffusion-controlled, whereas the release of Zn2+ is more closely linked to the pH-responsive structural dynamics of the hollow PDA-Zn framework11,12.

The b values obtained from the Zn2+ fit were 0.2757, 0.4265, and 0.3671, corresponding to temperatures of 35°C, 25°C, and 15°C, respectively. These values fall within or are very close to the Fickian diffusion range, indicating that the release behavior of Zn2+ remains primarily diffusion-controlled across the tested temperature range. However, the b-value at 25°C is close to the empirical threshold between Fickian and anomalous transport, suggesting that, compared to NaSA, the release of Zn2+ may be more sensitive to subtle structural dynamics within the PDA-Zn coordination network. This difference can be reasonably explained by the fact that Zn2+ is structurally closely associated with the reconstructed metal-phenolic framework, whereas NaSA acts primarily as a active ingredient stored within the hollow carrier. Overall, the results of the temperature-dependent fitting indicate that both the release of NaSA and Zn2+ are largely dominated by diffusion processes; at the same time, the release of Zn2+ is more significantly influenced by the dynamic coordination properties of the hollow PDA-Zn framework.

Figure S7Differences in ROS levels (A) and mean signal intensity (B) in R. solanacearum after treatment with ZIF-8 at different concentrations and NaSA@PDA@ZIF-8.

Figure S8 Effect of protective applications on plants and protection against tobacco green blight (A). Effect of treatments of different materials on the growth of tobacco plants and the protective therapeutic effect on tobacco (B).

Figure S9Heatmap representation of bacterial community composition across different subgroups.

Figure S10 Alpha diversity indices (A–G) of soil microorganisms after different material treatments, Wayne plots of ASV/OTU (H), graphs of taxonomic unit presentation of differences between groups (I), species loading plots of PCA analyses (J), and two-dimensional ordination plots of samples (K).

Reference

1.	Ritger, P. L. & Peppas, N. A. A simple equation for description of solute release I. Fickian and non-fickian release from non-swellable devices in the form of slabs, spheres, cylinders or discs. J. Controlled Release 5, 23–36 (1987).

2.	Ritger, P. L. & Peppas, N. A. A simple equation for description of solute release II. Fickian and anomalous release from swellable devices. J. Controlled Release 5, 37–42 (1987).

3.	Siepmann, J. & Siepmann, F. Mathematical modeling of drug dissolution. Int. J. Pharm. 453, 12–24 (2013).

4.	Siepmann, J. & Peppas, N. A. Modeling of drug release from delivery systems based on hydroxypropyl methylcellulose (HPMC). Adv. Drug Delivery Rev. 64, 163–174 (2012).

5.	Li, Y. et al. Hollow mesoporous silica nanoparticles with tunable structures for controlled drug delivery. ACS Appl. Mater. Interfaces 9, 2123–2129 (2017).

6.	Lynge, M. E., Westen, R. van der, Postma, A. & Städler, B. Polydopamine—a nature-inspired polymer coating for biomedical science. Nanoscale 3, 4916–4928 (2011).

7.	Liu, Y., Ai, K. & Lu, L. Polydopamine and its derivative materials: synthesis and promising applications in energy, environmental, and biomedical fields. Chem. Rev. 114, 5057–5115 (2014).

8.	Cauda, V. & Canavese, G. Mesoporous materials for drug delivery and theranostics. Pharmaceutics 12, 1108 (2020).

9.	Ejima, H. et al. One-step assembly of coordination complexes for versatile film and particle engineering. Science 341, 154–157 (2013).

10.	Han, Y. et al. Polyphenol-mediated assembly of proteins for engineering functional materials. Angew. Chem., Int. Ed. 59, 15618–15625 (2020).

11.	Faure, E. et al. Catechols as versatile platforms in polymer chemistry. Prog. Polym. Sci. 38, 236–270 (2013).

12.	Rahim, Md. A., Kristufek, S. L., Pan, S., Richardson, J. J. & Caruso, F. Phenolic building blocks for the assembly of functional materials. Angew. Chem. Int. Ed. 58, 1904–1927 (2019).