Supporting Information

Adaptive Process Control for Achieving Consistent Particles' States in Atmospheric Plasma Spray Process

B. Guduria*, M. Cybulskyb, G. R. Pickrellc, and R. C. Batraa

a)  b)

Fig. s1 a) Histogram of the injected particle size distribution and b) schematic of the observation window of width 1 cm to measure mean particles’ states located 9.5 cm from the nozzle exit.  For a mass flow rate of 20 g/min, at any time nearly 2,767 (142) particles are in the plasma (observation window)

Theory for the adaptive controller design

Here we follow [1] to design the MRAC for the multi input- multi output (MIMO) system and include a scaling parameter in the adaptive law. Consider the following MIMO plant with m inputs and n outputs (for the problem at hand, m = 3, n = 2):

|  | yt=Ayt+But,y(0)=y0,t≥0 | 1 |
| --- | --- | --- |

where for t≥0,  yt∈Rn is an output vector and ut∈Rm is an input vector. The constant matrices A∈Rn×n and B∈Rn×m are unknown. The plant dynamics in Eq. 1 is devoid of process uncertainties and external disturbances. 	We start by choosing the following reference model that meets the design criteria of an ideal plant model

|  | ymt=Amymt+Bmrt,y0=ym0,t≥0 | 2 |
| --- | --- | --- |

where ymt∈Rn is a reference output vector, and rt∈Rm is the piecewise bounded desired output vector. The matrices Am∈Rn×n and Bm∈Rn×m are chosen to achieve the desired trajectory of the reference model’s output. The exponential convergence of y→ym is achieved for any bounded rt for the following feedback control law

|  | ut=-K*yt+L*rt | 3 |
| --- | --- | --- |

where K*∈Rm×n and L*∈Rn×n are the ideal gains obtained from the following matching conditions

|  | K*=B-1A-Am;L*=B-1Bm | 4 |
| --- | --- | --- |

Since matrices A and B are unknown, the ideal gain matrices K* and L* are also unknown. Instead of using the ideal control law given by Eq. 3, we consider the following adaptive control law:

|  | ut=-Ktyt+Ltrt | 5 |
| --- | --- | --- |

where K∈Rm×n and K∈Rm×n are estimates of the ideal gain. From Eqs. 1, 2 and 5, the error, e, in the desired output is given by

|  | et=Ame+B-Ky+Lr,e0=e0 | 6 |
| --- | --- | --- |

where Kt≜Kt-K* and Lt≜Lt-L*	 are estimated gain errors that depend on the unknown matrix B. We develop the MRAC scheme that is independent of plant parameters A and B as follows [1]. For either a positive or a negative definite L* choose a constant matrix Γ∈Rm×m such that Γ-1=L*sgnl where l=1 (-1) if L* is positive (negative) definite. Thus, the matrix Γ is positive definite. Using B=BmL*-1, Eq. 6 becomes

|  | et=Ame+BmL*-1-Ky+Lr,e0=e0,t≥0 | 7 |
| --- | --- | --- |

We adaptively find gain matrices, Kt, L(t), as follows:

|  | Kt=ΛBmTPe(t)y(t)Tsgnl,K0=K0,t≥0Lt=-ΛBmTPe(t)r(t)Tsgnl,L0=L0,t≥0 | 8 |
| --- | --- | --- |

where Λ=ΛT>0∈Rm×m is a gain matrix that also serves as a scaling parameter, and P=PT∈Rm×m is a positive definite matrix derived from the following Lyapunov equation.

For every Hurwitz matrix Am and a given positive definite matrix R∈Rn×n there exists a unique positive-definite matrix P∈Rn×n such that

|  | AmTP+PAm+R=0 | 9 |
| --- | --- | --- |

The following theorem proved in [1] guarantees that the error, e, is bounded from above.

Theorem: Consider the plant dynamics given by Eq. 1, the reference model by Eq. 2, the control law by Eq. 5. Then, the solution et,Kt,Lt of the dynamical system given by Eqs. 7 and 8 is Lyapunov stable and et→0 as t→∞ for all et,Kt,Lt∈Rn×Rm×n×Rm×m and , and

|  | etL22≤1λminPλmaxPe022+ΓΛ-1FK0F2+L0F2 | 10 |
| --- | --- | --- |

The following Lyapunov function is consider in this theorem
Ve,K,L=eTPe+trKTΓΛ-1K+trLTΓΛ-1L

For the tracking error to be asymptotically stable, there should exist a positive definite and symmetric matrix R such that Ve,K,L=-eTRe≤0 [1]. A different choice of R affects the transient response but not the boundedness and the asymptotic convergence of the error. This theorem implies that all signals in the closed-loop dynamics are bounded (i.e., et,Kt,Lt,ut,yt∈L∞) and the tracking error asymptotically converges. The bound on et can be estimated for known plant matrices A and B.

Table s1 Predicted mean particles’ state from the response function for given process parameters

| Process parameter |  |  |  | The solution from response functions |  | Values found using LAVA-P |  | Difference from LAVA-P |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P (slm) | Q (slm) | I (A) | dy (cm) | v (m/s) | T (K) | v (m/s) | T (K) | v (%) | T (%) |
| 40 | 10 | 500 | 0.6 | 107.1 | 2866 | 107.0 | 2833 | 0.1 | 1.2 |
| 55 | 4 | 450 | 0.4 | 125.4 | 2468 | 126.1 | 2478 | -0.6 | -0.4 |
| 35 | 6 | 450 | 0.6 | 92.8 | 2867 | 92.4 | 2811 | 0.4 | 2.0 |
| 48 | 2 | 490 | 0.7 | 108.1 | 2484 | 107.3 | 2485 | 0.7 | 0.0 |
| 52 | 8 | 550 | 0.8 | 119.9 | 2689 | 119.6 | 2702 | 0.3 | -0.5 |
| 45 | 7 | 475 | 0.9 | 102.0 | 2640 | 102.8 | 2688 | -0.8 | -1.8 |
| 37 | 9 | 525 | 0.5 | 107.2 | 2930 | 107.2 | 2941 | 0.0 | -0.4 |
| 41 | 3 | 425 | 0.6 | 97.3 | 2602 | 96.5 | 2586 | 0.8 | 0.6 |

Table s2 For desired values of mean particles’ states values of the three process parameters found from the response functions using GA using the second set of seed numbers, and comparison of desired particles’ states with those computed using LAVA-P with the GA provided values of the process parameters

| Reference values |  | Solution from GA |  |  | Simulated values – LAVA-P |  | Difference between desired and simulated values |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| v (m/s) | T (K) | P (slm) | Q (slm) | I (A) | v (m/s) | T (K) | v (%) | T (%) |
| 90 | 2600 | 37.69 | 2.08 | 410.60 | 89.3 | 2578 | 0.8 | 0.8 |
| 110 | 2800 | 43.93 | 8.49 | 478.76 | 109.5 | 2768 | 0.4 | 1.1 |
| 90 | 2850 | 35.00 | 8.93 | 407.98 | 89.9 | 2803 | 0.1 | 1.6 |
| 100 | 2950 | 35.00 | 6.94 | 537.62 | 99.5 | 2948 | 0.5 | 0.1 |
| 95 | 2800 | 37.70 | 6.63 | 424.52 | 94.2 | 2761 | 0.8 | 1.4 |
| 100 | 3100 | 35.00 | 7.01 | 550.00 | 99.7 | 2961 | 0.3 | 4.5 |
| 105 | 3000 | 36.38 | 8.19 | 550.00 | 103.6 | 2919 | 1.3 | 2.7 |
| 90 | 2750 | 37.51 | 8.07 | 377.07 | 91.6 | 2731 | -1.8 | 0.7 |

Table s3 Values of variables used for exciting the H2 flow rate for validation of multi-inputs and single-output model of MSPs

| Parameters | Qb | Qa1 | Qa2 | Qa3 | Qa4 | Qa5 | ω1Q | ω2Q | ω3Q | ω4Q | ω5Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean, μ | 7 | 0 | 0 | 0 | 0 | 0 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| S.D., σ | 1 | 1 | 1 | 1 | 1 | 1 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Sample 1 | 6.99 | -1.47 | 1.36 | 0.52 | 0.42 | 0.08 | 0.43 | 0.40 | 0.55 | 0.34 | 0.54 |
| Sample 2 | 7.50 | 2.22 | -1.75 | -1.72 | -0.04 | -0.78 | 0.21 | 0.19 | 0.35 | 0.55 | 0.63 |
| Sample 3 | 6.28 | 0.08 | 0.43 | 0.24 | 0.21 | -1.82 | 0.55 | 0.35 | 0.49 | 0.41 | 0.70 |
| Sample 4 | 7.59 | -0.58 | -0.07 | -0.11 | 1.81 | -0.25 | 0.78 | 0.46 | 0.60 | 0.60 | 0.38 |
| Sample 5 | 3.97 | -1.21 | -0.37 | 1.05 | -1.50 | 0.53 | 0.53 | 0.52 | 0.63 | 0.46 | 0.31 |
| Sample 6 | 8.21 | -0.45 | 0.55 | 0.83 | 0.92 | 1.96 | 0.35 | 0.59 | 0.52 | 0.65 | 0.60 |
| Sample 7 | 8.79 | 0.41 | 0.02 | -0.54 | -0.67 | -1.09 | 0.42 | 0.47 | 0.74 | 0.11 | 0.35 |
| Sample 8 | 7.08 | 1.28 | 1.03 | -1.24 | -0.38 | -0.45 | 0.62 | 0.56 | 0.28 | 0.51 | 0.49 |
| Sample 9 | 6.65 | -0.04 | -1.09 | -0.39 | 0.80 | 1.04 | 0.50 | 0.73 | 0.40 | 0.47 | 0.46 |
| Sample 10 | 5.79 | 0.75 | -0.75 | 1.79 | -1.22 | 0.29 | 0.65 | 0.63 | 0.44 | 0.70 | 0.50 |

Table s4 Values of the variables used for exciting the current for validation of multi-inputs and single-output model of MSPs

| Parameters | Ib | Ia1 | Ia2 | Ia3 | Ia4 | Ia5 | ω1I | ω2I | ω3I | ω4I | ω5I |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean, μ | 450 | 0 | 0 | 0 | 0 | 0 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| S.D., σ | 200 | 200 | 200 | 200 | 200 | 200 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Sample 1 | 423.35 | 4.01 | 7.94 | -5.62 | -5.49 | -9.59 | 0.58 | 0.35 | 0.60 | 0.15 | 0.62 |
| Sample 2 | 442.55 | 13.68 | 17.69 | -19.67 | -2.42 | 17.27 | 0.39 | 0.65 | 0.55 | 0.43 | 0.46 |
| Sample 3 | 459.05 | -3.03 | 5.45 | -15.50 | 14.26 | 1.62 | 0.37 | 0.57 | 0.72 | 0.66 | 0.39 |
| Sample 4 | 455.80 | 20.23 | 2.02 | 0.13 | 5.53 | 4.74 | 0.55 | 0.49 | 0.31 | 0.37 | 0.33 |
| Sample 5 | 449.12 | 8.21 | 18.92 | 5.32 | -21.09 | 30.79 | 0.48 | 0.74 | 0.45 | 0.70 | 0.93 |
| Sample 6 | 443.96 | -25.32 | -5.95 | 9.63 | 21.97 | 8.34 | 0.31 | 0.44 | 0.41 | 0.48 | 0.52 |
| Sample 7 | 465.79 | -16.00 | -11.82 | 19.93 | 7.77 | -14.06 | 0.51 | 0.29 | 0.48 | 0.55 | 0.24 |
| Sample 8 | 453.20 | 1.10 | -15.18 | -7.80 | 1.13 | -7.01 | 0.43 | 0.39 | 0.62 | 0.40 | 0.60 |
| Sample 9 | 435.13 | -4.77 | -30.35 | 12.52 | -17.70 | -1.09 | 0.67 | 0.58 | 0.36 | 0.61 | 0.55 |
| Sample 10 | 480.16 | -9.68 | -1.67 | -0.01 | -7.82 | -18.63 | 0.69 | 0.51 | 0.53 | 0.50 | 0.47 |

Fig. s2 Variations of the H2 flow rate and current for samples 1, 2 and 3 used for validation of multi-inputs and single-output model of MSPs

References

[1]	P. A. Ioannou and J. Sun, Robust adaptive control. Courier Corporation, 2012.

Type equation here.

# *Corresponding author, Email: gbalu@vt.edu

2Clifton C. Garvin Professor, Tel (540) 231-6051, Fax (540) 231-4574, email rbatra@vt.edu

# a Department of Biomedical Engineering and Mechanics, M/C 0219, Virginia Polytechnic Institute and State University, Blacksburg, VA 24061, USA

b Rolls-Royce Corporation, P.O. Box 420, Speed Code W-08, Indianapolis, IN 46206-0420, USA

# c Department of Material Science and Engineering, M/C 0237, Virginia Polytechnic Institute and State University, Blacksburg, VA 24061, USA