Supporting Information for

Life Cycle Assessment of the manufacturing and operation of distillation column for eliminating volatile and organic halogen compounds from process wastewater

Huyen Trang Do Thi1, Andras Jozsef Toth1,*

1 Environmental and Process Engineering Research Group, Department of Chemical and Environmental Process Engineering, Budapest University of Technology and Economics, H-1111, Hungary, Budapest, Műegyetem rkp. 3.

* Corresponding author. E-mail address: andrasjozseftoth@edu.bme.hu, Tel: +36 1 463 1490; Fax: +36 1 463 3197, ORCID: 0000-0002-5787-8557

Table S1 Life cycle inventory data

| Phase | Materials | Input | Output | Unit |
| --- | --- | --- | --- | --- |
| Manufacturinga | Aluminum oxide | 3.08E-06 | - | kg |
|  | Glass coated | 1.67E-06 |  | kg |
|  | Iron and steel | 5.42E-05 |  | kg |
|  | Iron/Nickel/Chromium | 2.92E-06 |  | kg |
|  | Stainless steel (AISI 304) | 2.38E-04 |  | kg |
|  | Synthetic Rubber | 2.92E-06 |  | kg |
|  | Water | 1.25E-04 |  | kg |
|  | Electricity | 5.96E-04 |  | kWh |
| Operationb | 1,1,2-Trichloroethane | 9.04E-09 | 1.63E-21 | kg |
|  | 1,2-Dichloroethane | 3.90E-07 | 1.54E-21 | kg |
|  | 1,4-Dioxane | 2.02E-07 | 2.54E-09 | kg |
|  | 2-Hexanone | 1.00E-09 | 1.01E-09 | kg |
|  | Acetone | 2.90E-05 | 5.36E-13 | kg |
|  | Benzene | 2.34E-06 | 1.79E-23 | kg |
|  | Carbon tetrachloride | 4.00E-09 | 1.06E-21 | kg |
|  | Chloroform | 2.67E-07 | 3.76E-22 | kg |
|  | Dichloromethane | 1.02E-03 | 5.36E-13 | kg |
|  | Diethyl ether | 6.32E-08 | 6.88E-24 | kg |
|  | Ethanol | 1.11E-05 | 1.95E-08 | kg |
|  | Ethyl acetate | 1.59E-06 | 7.69E-19 | kg |
|  | Ethyl acetone | 1.13E-07 | 4.90E-17 | kg |
|  | Ethylbenzene | 4.93E-07 | 2.44E-23 | kg |
|  | Isobutanol | 6.63E-05 | 2.01E-09 | kg |
|  | Isopropanol | 2.18E-05 | 1.53E-10 | kg |
|  | Methanol | 4.43E-03 | 1.00E-03 | kg |
|  | Methyl Ethyl ketone | 3.13E-07 | 5.36E-17 | kg |
|  | Methyl Isobutyl Ketone | 5.83E-08 | 2.47E-20 | kg |
|  | m-Xylene | 1.90E-06 | 2.33E-21 | kg |
|  | n-Butanol | 1.70E-08 | 5.47E-17 | kg |
|  | n-Propanol | 2.01E-08 | 1.84E-14 | kg |
|  | o-Xylene | 3.64E-07 | 3.80E-20 | kg |
|  | p-Xylene | 4.97E-07 | 2.44E-23 | kg |
|  | t-Butanol | 3.01E-08 | 5.41E-15 | kg |
|  | Tetrachloroethylene | 3.50E-08 | 1.91E-22 | kg |
|  | Toluene | 1.70E-05 | 0.00E+00 | kg |
|  | Trichloroethylene | 7.17E-06 | 3.70E-21 | kg |
|  | Water | 9.94E-01 | 9.99E-01 | kg |
|  | Electricity | 4.08E-03 | - | kWh |

a Adapted from [1]

b Adapted from [2]

References

1. 	Brondani M, de Oliveira JS, Mayer FD, Hoffmann R (2020) Life cycle assessment of distillation columns manufacturing. Environ. Dev. Sustain. 22(6):5925−5945. https://doi.org/10.1007/s10668-019-00459-5

2. 	Do Thi HT, Fozer D, Toth AJ (2023) Extensive comparison of methods for removal of organic halogen compounds from pharmaceutical process wastewaters with life cycle, PESTLE, and multi-criteria decision analyses. J. Environ. Manage. 345, 118593. https://doi.org/10.1016/j.jenvman.2023.118593