Supportive Materials for the environmental impact assessment of steel reinforcing bar manufacturing process: a case study in the Ethiopian Industries.

Table S1: The inventory data to produce 1.1 tons of molten steel production

| Type of material | Unit | Quantity per functional unit | Source of data |
| --- | --- | --- | --- |
| Inputs from nature |  |  |  |
| - Water | m3 | 90 | Factory |
| Inputs from the Techno sphere |  |  |  |
| - Steel scrap | Kg | 1250 | Factory |
| - Ferromanganese | Kg | 10 |  |
| - Ferrosilicon | Kg | 3 |  |
| - Refractory material | Kg | 12 |  |
| - Pure oxygen | Kg | 10.26 |  |
| - Lime | Kg | 30 |  |
| Outputs |  |  |  |
| - Molten metal | Kg | 1100 | Considering there is about more than 12 % loss in the form of slag, sludge, and emission of gases, etc |
| From Techno sphere Electricity/Fuel |  |  |  |
| - High-voltage electricity generated from hydropower | KWH | 605 | From the factory |
| Emissions to air |  |  |  |
| - Carbon monoxide | G | 568.623 | Z.G. Alp Ozdemir (2018) |
| - Nitrogen dioxide | G | 206.525 |  |
| - Sulfur dioxide | G | 7.7 | D. Burchart Korol (2013) |
| Heavy Metals |  |  |  |
| - Pb | G | 15.4 | Neil R Passant (2002) |
| - Cr | G | 1.1 |  |
| - Cd | G | 0.275 |  |
| - Cu | G | 0.88 |  |
| - Zn | G | 55 |  |
| - Ni | G | 0.275 |  |
| - Ar | G | 0.11 |  |
| - Hg | G | 0.165 |  |
| - V | G | 1.056 |  |
| - Dust | G | 80 | Z.G. Alp OZdemir (2018) |
| - HF | G | 0.99 |  |
| - PM | G | 55 |  |
| Waste |  |  |  |
| - Wastewater | m3 | 90 | IPCC (2010) |
| - Refractory waste (dust) | Kg | 8 |  |
| - Sludge | Kg | 77 |  |
| - Slag | Kg | 110 |  |

Table S2: Inventory data to produce 1.05 tons of billets production

| Sr. No. | Type of material | Unit | Quantity per functional unit | Source |
| --- | --- | --- | --- | --- |
|  | Inputs from nature |  |  |  |
|  | - Water | m3 | 10.32 | Factory |
|  | Inputs from the techno sphere |  |  |  |
|  | - Molten Steel | kg | 1100 |  |
|  | - Lubricating Oil | kg | 2.46 |  |
|  | - Refractory | kg | 12 |  |
|  | Electricity |  |  |  |
|  | - Hydropower of Electricity high voltage from the grid | kWh | 83.33 | D.P. Lynn Price (2001) |
|  | Output to the techno sphere |  |  |  |
|  | Billets | kg | 1050 | Factory data considering the waste assumption |
|  | Emissions to air |  |  |  |
|  | - Iron dust | kg | 4.43 | M.A.D.S Bedarkar (2016) |
|  | - SO2 | G | 170 |  |
|  | - NO2 | G | 170 |  |
|  | - CO | G | 260 |  |
|  | - Particulates | kg | 1.84 |  |
|  | Waste |  |  |  |
|  | - Wastewater | m3 | 10.32 | From Factory supported by Expert Judgement |
|  | - Refractory waste | kg | 12.00 |  |
|  | - Iron scraps | kg | 25.75 |  |
|  | - Iron particles –Scale | kg | 2.34 |  |

Table S3: Inventory data to produce 1.00-ton reinforcing bar

| Type of material | Unit | Quantity per functional unit | Source |
| --- | --- | --- | --- |
| Inputs from nature |  |  |  |
| - Water | m3 | 14.42 | Factory |
| Inputs from the techno sphere |  |  |  |
| - Billets | Kg | 1050 | Factory |
| - Residual fuel oil | L | 48.3 |  |
| - Lubricating oil | kg | 33.85 |  |
| Electricity/Fuel |  |  |  |
| - High voltage hydropower electricity from the grid | kWh | 1069.445 | D.P. Lynn Price (2001) |
| Outputs to Techno sphere |  |  |  |
| Rebar | kg | 1000 | FU production volume is taken into consideration |
| Waste |  |  |  |
| - Wastewater | m3 | 1.42 | From factory supported by expert judgment |
| - Scale | kg | 10 |  |
| - Iron scrap | kg | 33 |  |
| Emissions |  |  |  |
| - CO2 | kg | 167.64 | IPCC (2006) and Jan Berdowski (2021) |
| - CH4 | g | 8.0664 |  |
| - N2O | g | 16.1328 |  |
| - CO | g | 30.249 | EMEP/ EEA (2019) |
| - NMVOC | g | 8.0664 |  |
| - NOX | g | 342.822 |  |
| - SO2 | g | 2667.96 |  |
| Waste |  |  |  |
| - Iron | kg | 0.05 | From factory supported by expert estimation |
| - Dust | kg | 0.092 |  |
| - Scale | kg | 5 |  |