Supplementary material

Table S1. Model quality metrics for the four linear mixed models. AIC = Akaike Information Criterion; BIC = Bayesian Information Criterion; Sensor SD = standard deviation of the random intercept (between-sensor variability); Residual SD = within-cell residual standard deviation; ACF lag-1 = autocorrelation of Pearson residuals at lag 1. All models fitted by REML: lmer(Value ~ Light × Fresh-air supply × Height + (1|Sensor)), n = 36 per model (9 sensors × 4 blocks).

| Experiment | Response variable | n | Sensors | Blocks | AIC | BIC | Sensor SD | Residual SD | ACF lag-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E1 | Temperature | 36 | 9 | 4 | 74.74 | 96.91 | 0.846 | 0.32 | 0.23 |
| E1 | Relative humidity | 36 | 9 | 4 | 140.83 | 163 | 5.294 | 1.092 | -0.23 |
| E2 | Temperature | 36 | 9 | 4 | 82.53 | 104.7 | 0.624 | 0.433 | -0.59 |
| E2 | Relative humidity | 36 | 9 | 4 | 167.81 | 189.98 | 2.909 | 2.733 | -0.46 |

Figure S1. Diagnostic plots for the four linear mixed models (rows a–d: E1 Temperature, E1 Relative humidity, E2 Temperature, E2 Relative humidity). Left: Pearson residuals vs. fitted values with LOESS smooth (blue). Right: Normal Q-Q plots. Models: lmer(Value ~ Light × Fresh-air supply × Height + (1|Sensor)), n = 36 (9 sensors × 4 blocks).

Table S2. Measured maximum difference across all nine sensors at the same time for three different temperatures (-20 °C, 0 °C and 25 °C) and three different levels of humidity (10 %, 50 % and 95 %). Humidity measurements were conducted at 25 °C. Setpoint = Chamber setpoint, Min = lowest value measured, Max = highest values measured, Difference = Biggest difference across all nine at the same time.

| Setpoint | Min. | Max. | Difference |
| --- | --- | --- | --- |
| -20 °C | -20.3 | -19.0 | 1.3 |
| 0 °C | -0.3 | 0.73 | 1.0 |
| 25 °C | 24.8 | 25.5 | 0.7 |
| 10 % | 8.8 | 11.6 | 2.8 |
| 50 % | 50.4 | 53.8 | 3.4 |
| 95 % | 92.9 | 100 | 7.1 |

Table S3. Temperature and relative humidity at 33, 110, and 240 cm and at three chamber positions (C center, S side, K corner). Means are averaged across positions (by height) or across heights (by position). RH reported at 25 °C; Setpoint = target; SD = within-group SD; Diff = mean deviation.

| Setpoint | Height | Mean | SD | Diff |  | Position | Mean | SD | Diff |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -20 °C | 33 | -19.60 | 0.04 | 0.15 |  | C | -19.89 | 0.03 | 0.13 |
|  | 110 | -19.59 | 0.04 | 0.18 |  | S | -19.32 | 0.02 | 0.09 |
|  | 240 | -19.56 | 0.02 | 0.09 |  | K | -19.53 | 0.05 | 0.22 |
| 0 °C | 33 | 0.52 | 0.01 | 0.07 |  | C | 0.46 | 0.01 | 0.08 |
|  | 110 | 0.22 | 0.03 | 0.12 |  | S | 0.19 | 0.02 | 0.12 |
|  | 240 | 0.29 | 0.01 | 0.07 |  | K | 0.38 | 0.03 | 0.11 |
| 25 °C | 33 | 25.16 | 0.03 | 0.16 |  | C | 25.12 | 0.04 | 0.20 |
|  | 110 | 25.27 | 0.05 | 0.31 |  | S | 25.23 | 0.04 | 0.29 |
|  | 240 | 25.21 | 0.02 | 0.10 |  | K | 25.29 | 0.05 | 0.19 |
| 10 % | 33 | 9.91 | 0.37 | 1.70 |  | C | 10.01 | 0.36 | 1.64 |
|  | 110 | 9.32 | 0.34 | 1.48 |  | S | 9.08 | 0.38 | 1.64 |
|  | 240 | 9.34 | 0.40 | 1.69 |  | K | 9.47 | 0.39 | 1.77 |
| 50 % | 33 | 52.39 | 0.40 | 1.19 |  | C | 52.22 | 0.37 | 1.09 |
|  | 110 | 51.50 | 0.30 | 0.90 |  | S | 50.97 | 0.25 | 0.80 |
|  | 240 | 50.68 | 0.26 | 0.81 |  | K | 51.38 | 0.34 | 1.01 |
| 95 % | 33 | 98.76 | 0.14 | 0.57 |  | C | 96.62 | 0.23 | 1.20 |
|  | 110 | 97.65 | 0.16 | 0.70 |  | S | 96.10 | 0.25 | 1.07 |
|  | 240 | 94.36 | 0.29 | 1.50 |  | K | 98.05 | 0.10 | 0.57 |

Table S4. Type III ANOVA (Satterthwaite’s method) for the fixed effects and interactions in the linear mixed-effects models fitted to temperature (°C) and relative humidity (%) in experiments E1 and E2. Reported are numerator and denominator degrees of freedom (df_num, df_den), F-statistics and p-values for Light (L), Fresh-air supply (W), Height (H), and their two- and three-way interactions (L × W, L × H, W × H, L × W × H). Significance codes: ns, not significant; * p < 0.05; ** p < 0.01; *** p < 0.001.

| Source of variation | df num | df den | F | p | Sig. |
| --- | --- | --- | --- | --- | --- |
| E1 — Temperature (°C) |  |  |  |  |  |
| Light (L) | 1 | 18 | 66.51 | < 0.001 | *** |
| Fresh-air supply (W) | 1 | 18 | 3.58 | 0.075 | ns |
| Height (H) | 2 | 6 | 0.01 | 0.992 | ns |
| L × W | 1 | 18 | 1.68 | 0.211 | ns |
| L × H | 2 | 18 | 0.53 | 0.597 | ns |
| W × H | 2 | 18 | 0.13 | 0.878 | ns |
| L × W × H | 2 | 18 | 0.001 | 0.999 | ns |
| E1 — Relative humidity (%) |  |  |  |  |  |
| Light (L) | 1 | 18 | 559.30 | < 0.001 | *** |
| Fresh-air supply (W) | 1 | 18 | 66.49 | < 0.001 | *** |
| Height (H) | 2 | 6 | 0.31 | 0.742 | ns |
| L × W | 1 | 18 | 102.94 | < 0.001 | *** |
| L × H | 2 | 18 | 2.46 | 0.114 | ns |
| W × H | 2 | 18 | 0.31 | 0.739 | ns |
| L × W × H | 2 | 18 | 0.70 | 0.507 | ns |
| E2 — Temperature (°C) |  |  |  |  |  |
| Light (L) | 1 | 18 | 19.20 | < 0.001 | *** |
| Fresh-air supply (W) | 1 | 18 | 2.13 | 0.162 | ns |
| Height (H) | 2 | 6 | 0.69 | 0.539 | ns |
| L × W | 1 | 18 | 0.48 | 0.499 | ns |
| L × H | 2 | 18 | 0.09 | 0.917 | ns |
| W × H | 2 | 18 | 3.13 | 0.068 | ns |
| L × W × H | 2 | 18 | 0.05 | 0.953 | ns |
| E2 — Relative humidity (%) |  |  |  |  |  |
| Light (L) | 1 | 18 | 3.99 | 0.061 | ns |
| Fresh-air supply (W) | 1 | 18 | 8.87 | 0.008 | ** |
| Height (H) | 2 | 6 | 0.09 | 0.917 | ns |
| L × W | 1 | 18 | 1.54 | 0.231 | ns |
| L × H | 2 | 18 | 0.08 | 0.927 | ns |
| W × H | 2 | 18 | 3.71 | 0.045 | * |
| L × W × H | 2 | 18 | 0.01 | 0.987 | ns |

Table S5. Pairwise contrasts for Light (L+ vs L−) and Fresh-air supply (W+ vs W−) from the estimated marginal means, conditioned on all combinations of the respective other factor and measurement height. SE = standard error. p-values and degrees of freedom based on Satterthwaite approximation. Significance codes: * p < 0.05, ** p < 0.01, *** p < 0.001, ns = not significant.

| Effect | Fixed at | Height (cm) | Estimate | SE | p | Sig. |
| --- | --- | --- | --- | --- | --- | --- |
| E1 — Temperature (°C) |  |  |  |  |  |  |
| Light (L+ vs L−) | W− | 33 | +1.00 | 0.26 | < 0.001 | ** |
|  | W− | 110 | +0.88 | 0.26 | 0.004 | ** |
|  | W− | 240 | +1.15 | 0.26 | < 0.001 | *** |
|  | W+ | 33 | +0.74 | 0.26 | 0.011 | * |
|  | W+ | 110 | +0.59 | 0.26 | 0.035 | * |
|  | W+ | 240 | +0.86 | 0.26 | 0.004 | ** |
| Fresh-air supply (W+ vs W−) | L− | 33 | +0.38 | 0.26 | 0.166 | ns |
|  | L− | 110 | +0.27 | 0.26 | 0.320 | ns |
|  | L− | 240 | +0.38 | 0.26 | 0.167 | ns |
|  | L+ | 33 | +0.11 | 0.26 | 0.668 | ns |
|  | L+ | 110 | −0.02 | 0.26 | 0.948 | ns |
|  | L+ | 240 | +0.09 | 0.26 | 0.723 | ns |
| E1 — Relative humidity (%-points) |  |  |  |  |  |  |
| Light (L+ vs L−) | W− | 33 | −13.33 | 0.89 | < 0.001 | *** |
|  | W− | 110 | −11.04 | 0.89 | < 0.001 | *** |
|  | W− | 240 | −12.52 | 0.89 | < 0.001 | *** |
|  | W+ | 33 | −4.79 | 0.89 | < 0.001 | *** |
|  | W+ | 110 | −3.90 | 0.89 | < 0.001 | *** |
|  | W+ | 240 | −6.05 | 0.89 | < 0.001 | *** |
| Fresh-air supply (W+ vs W−) | L− | 33 | −7.29 | 0.89 | < 0.001 | *** |
|  | L− | 110 | −6.16 | 0.89 | < 0.001 | *** |
|  | L− | 240 | −6.52 | 0.89 | < 0.001 | *** |
|  | L+ | 33 | +1.25 | 0.89 | 0.178 | ns |
|  | L+ | 110 | +0.98 | 0.89 | 0.288 | ns |
|  | L+ | 240 | −0.05 | 0.89 | 0.955 | ns |
| E2 — Temperature (°C) |  |  |  |  |  |  |
| Light (L+ vs L−) | W− | 33 | +0.43 | 0.35 | 0.240 | ns |
|  | W− | 110 | +0.50 | 0.35 | 0.172 | ns |
|  | W− | 240 | +0.67 | 0.35 | 0.076 | ns |
|  | W+ | 33 | +0.75 | 0.35 | 0.048 | * |
|  | W+ | 110 | +0.68 | 0.35 | 0.071 | ns |
|  | W+ | 240 | +0.77 | 0.35 | 0.043 | * |
| Fresh-air supply (W+ vs W−) | L− | 33 | −0.87 | 0.35 | 0.025 | * |
|  | L− | 110 | −0.15 | 0.35 | 0.676 | ns |
|  | L− | 240 | +0.09 | 0.35 | 0.808 | ns |
|  | L+ | 33 | −0.55 | 0.35 | 0.139 | ns |
|  | L+ | 110 | +0.03 | 0.35 | 0.943 | ns |
|  | L+ | 240 | +0.19 | 0.35 | 0.598 | ns |
| E2 — Relative humidity (%-points) |  |  |  |  |  |  |
| Light (L+ vs L−) | W− | 33 | −3.07 | 2.23 | 0.186 | ns |
|  | W− | 110 | −2.59 | 2.23 | 0.261 | ns |
|  | W− | 240 | −3.19 | 2.23 | 0.170 | ns |
|  | W+ | 33 | −0.52 | 2.23 | 0.817 | ns |
|  | W+ | 110 | −0.21 | 2.23 | 0.928 | ns |
|  | W+ | 240 | −1.34 | 2.23 | 0.557 | ns |
| Fresh-air supply (W+ vs W−) | L− | 33 | −7.17 | 2.23 | 0.005 | ** |
|  | L− | 110 | −3.59 | 2.23 | 0.125 | ns |
|  | L− | 240 | −0.77 | 2.23 | 0.734 | ns |
|  | L+ | 33 | −4.63 | 2.23 | 0.053 | ns |
|  | L+ | 110 | −1.21 | 2.23 | 0.595 | ns |
|  | L+ | 240 | +1.09 | 2.23 | 0.633 | ns |

Table S6. Estimated marginal means (EMMs) for all treatment combinations across both experiments and response variables. EMMs were derived from models of the form lmer(Value ~ Light × Fresh-air supply × Height + (1|Sensor)). SE = standard error; 95% CI = confidence interval based on Satterthwaite degrees of freedom. L− = 50% light; L+ = 100% light; W− = 10 m³ h⁻¹ fresh-air supply; W+ = 50 m³ h⁻¹ fresh-air supply.

| Height (cm) | Light | Fresh-air supply | EMM | SE | Lower 95% CI | Upper 95% CI |
| --- | --- | --- | --- | --- | --- | --- |
| E1 — Temperature (°C) |  |  |  |  |  |  |
| 33 | L− | W− | 26.08 | 0.52 | 24.86 | 27.31 |
| 33 | L− | W+ | 26.46 | 0.52 | 25.23 | 27.68 |
| 33 | L+ | W− | 27.08 | 0.52 | 25.85 | 28.30 |
| 33 | L+ | W+ | 27.19 | 0.52 | 25.97 | 28.42 |
| 110 | L− | W− | 26.19 | 0.52 | 24.97 | 27.42 |
| 110 | L− | W+ | 26.46 | 0.52 | 25.24 | 27.68 |
| 110 | L+ | W− | 27.07 | 0.52 | 25.85 | 28.29 |
| 110 | L+ | W+ | 27.05 | 0.52 | 25.83 | 28.28 |
| 240 | L− | W− | 26.08 | 0.52 | 24.86 | 27.31 |
| 240 | L− | W+ | 26.46 | 0.52 | 25.24 | 27.68 |
| 240 | L+ | W− | 27.23 | 0.52 | 26.00 | 28.45 |
| 240 | L+ | W+ | 27.32 | 0.52 | 26.10 | 28.55 |
| E1 — Relative humidity (%) |  |  |  |  |  |  |
| 33 | L− | W− | 83.87 | 3.12 | 76.35 | 91.40 |
| 33 | L− | W+ | 76.58 | 3.12 | 69.05 | 84.11 |
| 33 | L+ | W− | 70.54 | 3.12 | 63.01 | 78.07 |
| 33 | L+ | W+ | 71.79 | 3.12 | 64.26 | 79.32 |
| 110 | L− | W− | 83.97 | 3.12 | 76.44 | 91.50 |
| 110 | L− | W+ | 77.81 | 3.12 | 70.28 | 85.34 |
| 110 | L+ | W− | 72.93 | 3.12 | 65.40 | 80.46 |
| 110 | L+ | W+ | 73.91 | 3.12 | 66.38 | 81.43 |
| 240 | L− | W− | 81.63 | 3.12 | 74.10 | 89.15 |
| 240 | L− | W+ | 75.11 | 3.12 | 67.58 | 82.63 |
| 240 | L+ | W− | 69.11 | 3.12 | 61.58 | 76.63 |
| 240 | L+ | W+ | 69.06 | 3.12 | 61.53 | 76.58 |
| E2 — Temperature (°C) |  |  |  |  |  |  |
| 33 | L− | W− | 7.02 | 0.44 | 6.04 | 7.99 |
| 33 | L− | W+ | 6.15 | 0.44 | 5.17 | 7.12 |
| 33 | L+ | W− | 7.45 | 0.44 | 6.47 | 8.42 |
| 33 | L+ | W+ | 6.90 | 0.44 | 5.92 | 7.87 |
| 110 | L− | W− | 6.18 | 0.44 | 5.21 | 7.16 |
| 110 | L− | W+ | 6.03 | 0.44 | 5.06 | 7.01 |
| 110 | L+ | W− | 6.69 | 0.44 | 5.71 | 7.66 |
| 110 | L+ | W+ | 6.71 | 0.44 | 5.74 | 7.69 |
| 240 | L− | W− | 5.87 | 0.44 | 4.90 | 6.85 |
| 240 | L− | W+ | 5.96 | 0.44 | 4.99 | 6.94 |
| 240 | L+ | W− | 6.54 | 0.44 | 5.57 | 7.52 |
| 240 | L+ | W+ | 6.73 | 0.44 | 5.75 | 7.71 |
| E2 — Relative humidity (%) |  |  |  |  |  |  |
| 33 | L− | W− | 53.91 | 2.30 | 48.93 | 58.88 |
| 33 | L− | W+ | 46.74 | 2.30 | 41.76 | 51.72 |
| 33 | L+ | W− | 50.84 | 2.30 | 45.86 | 55.82 |
| 33 | L+ | W+ | 46.21 | 2.30 | 41.24 | 51.19 |
| 110 | L− | W− | 52.04 | 2.30 | 47.06 | 57.02 |
| 110 | L− | W+ | 48.45 | 2.30 | 43.47 | 53.43 |
| 110 | L+ | W− | 49.45 | 2.30 | 44.48 | 54.43 |
| 110 | L+ | W+ | 48.24 | 2.30 | 43.27 | 53.22 |
| 240 | L− | W− | 51.95 | 2.30 | 46.97 | 56.93 |
| 240 | L− | W+ | 51.18 | 2.30 | 46.20 | 56.16 |
| 240 | L+ | W− | 48.76 | 2.30 | 43.78 | 53.74 |
| 240 | L+ | W+ | 49.85 | 2.30 | 44.87 | 54.82 |