# Supplementary Information

for

# Sorption and transport of Escherichia coli CN-13 in saturated sand columns amended with biochar derived from malt spent rootlets

Christos P. Giannopoulos a, Christos A. Kolotouros a, Ioannis D. Manariotis a

a Department of Civil Engineering, Environmental Engineering Laboratory, University of Patras, Patras 26504, Greece

Table S1. Parameter values and radj2 of inactivation models fitted to observed data.

| Inactivation model | Ionic strength, [KCl] | Parameters* |  |  | radj2 |
| --- | --- | --- | --- | --- | --- |
| First-order |  |  |  | kd |  |
|  | 1 mM |  |  | 9.166·10-4 | 0.72 |
|  | 150 mM |  |  | 7.272·10-3 | 0.75 |
| Log-logistic |  | A | σ | τ |  |
|  | 1 mM | 1000 | 303.913 | 9.461 | 0.80 |
|  | 150 mM | 1000 | 164.663 | 13.948 | 0.86 |
| Modified Gompertz |  | A | B | T |  |
|  | 1 mM | 1.165 | 1.771·10-3 | 0 | 0.73 |
|  | 150 mM | 1.166 | 1.002·10-2 | 0 | 0.85 |
| *Inactivation model formulations and parameter units are given in the manuscript |  |  |  |  |  |

Table S2. Key fixed parameter values for 1D-ADE and 1D-BTE model simulations.

| Column | C1 | C2 | C3 |
| --- | --- | --- | --- |
| Biochar content (% w/w MSRB) | 0 | 5 | 10 |
| Biochar content (% v/v MSRB) | 0 | 28.8 | 38.0 |
| Column diameter (cm) | 1 | 1 | 1 |
| Column height (cm) | 30 | 30 | 30 |
| Flow rate (mL min-1) | 1 | 1 | 1 |
| Mixture bulk density (g cm-3) | 1.651 | 1.238 | 1.137 |
| Mixture median particle size (μm)* | 505.0 | 381.2 | 341.6 |
| Porosity (-) | 0.335 | 0.399 | 0.407 |
| Straining function shape parameter, β | 0.432 | 0.432 | 0.432 |
| Pulse duration (min) | 41.080 | 48.932 | 49.868 |
| Initial cell concentration (CFU mL-1) | 1.05·105 | 1.45·105 | 1.01·105 |
| Weibull inactivation constant, kd (min-α) | 0.02329 | 0.02329 | 0.02329 |
| Weibull shape factor, α | 0.52778 | 0.52778 | 0.52778 |
| Mean pH value | 6.6 | 7.5 | 7.6 |
| *Considering the volumetrically weighted average of the median particle sizes of sand (d50 = 505 μm) and biochar (d50 = 75 μm) for each mixture. |  |  |  |

Figure S1. Schematic flow diagram of stepwise bacterial stock preparation, including isolation, cultivation, centrifugation, storage, and dilution, with additional operational parameters.

Figure S2. Measured Cl-, PO43-, and SO42- anion concentration time series (a) throughout a batch experiment ([KCl] = 1 mM, Solution volume = 10 mL, MSRB mass = 10 mg) and (b) during washing prior to C2 column tracer experiment (MSRB content = 5% w/w, Q = 1 mL/min).