Table 1S. Heat balance check: available vs. required energy

| Item | Value | Units | Notes |
| --- | --- | --- | --- |
| CH₄ flow (simulated) | 7 | kg·h⁻¹ | Design-Spec converged |
| CH₄ chemical energy | 97.22 | kW | LHV = 50 MJ·kg⁻¹ |
| CH₄ useful (η = 0.85) | 82.64 | kW | 97.22 × 0.85 |
| Syngas combustion enthalpy (from Aspen) | −1,225.74 cal·s⁻¹ = −5.13 | kW | Aspen Plus reported enthalpy flow for syngas flue gas |
| Total available heat | 87.77 | kW | 82.64 + 5.13 |
| Sensible heating (25 → 550 °C) | 33.49 | kW | cp(HDPE)=2.3 kJ·kg⁻¹K⁻¹ |
| Reaction enthalpy | 46.75 | kW | ΔH = 1,683 kJ·kg⁻¹ |
| Total required | 80.24 | kW | 33.49 + 46.75 |
| Margin (available − required) | 7.53 | kW | positive margin ≈ 9.4% |

Table 2S. Purchase cost of Pyrolysis pilot plant

| Name | Type | Capacity | Unit | ($/Unit) |
| --- | --- | --- | --- | --- |
| Pyrolysis reactor system |  |  |  | 90000 |
| DECOMP | RYield | 100 | kg/h |  |
| HEATER | HeatX |  |  |  |
| REACTOR | Rplug |  |  |  |
| Pyrolysis Fraction separation system |  |  |  |  |
| CYCLON | SSplit | 15 | kg/h |  |
| CONDEN-1 | HeatX | 1.24 | m2 |  |
| SEPARATI | Flash | 0.01 | m3 |  |
| Gas burning system |  |  |  |  |
| CHAMBER1 and 2 | Rstoic | 0.09 | m3 | 30000 |
| Exhaust gas conditioning |  |  |  |  |
| HEATEX-1 | HeatX | 1.31 | m2 | 20,000 |
| Cooling water treatment |  |  |  |  |
| CLTower | RadFrac | 777.26 | L | 15,000 |
| Total Pyrolysis plant |  |  |  | 155000 |

Table 3S. Mass fraction of heavy hydrocarbons of pyrolytic oil stream (OIL) at 550 °C and 20°C/min

| heavy hydrocarbons |  |  |
| --- | --- | --- |
| Compound | Formula | Mass Fraction (%) |
| N-Undecane | C11H24 | 2.438 |
| 1-Undecene | C11H22 | 2.03 |
| 1-Dodecene | C12H24 | 1.807 |
| N-Dodecene | C12H24 | 1.807 |
| 1-Tridecene | C13H26 | 0.981 |
| N-Tridecane | C13H28 | 2.736 |
| 1-Tetradecene | C14H28 | 2.087 |
| N-Tetradecane | C14H30 | 3.246 |
| 1-Pentadecene | C15H30 | 2.24 |
| N-Pentadecane | C15H32 | 3.637 |
| N-Hexadecane | C16H34 | 5.239 |
| 1-Heptadecene | C17H34 | 6.944 |
| 1-Octadecene | C18H36 | 2.923 |
| N-Octadecane | C18H38 | 4.224 |
| 1-Nonadecene | C19H38 | 4.763 |
| 1-Eicosene | C20H40 | 4.55 |
| N-Heneicosane | C21H44 | 9.255 |
| N-Docosane | C22H46 | 12.121 |
| N-Tricosane | C23H48 | 8.808 |
| N-Tetracosane | C24H50 | 7.632 |

Table 4S. Fixed estimate cost (2023 prices in USD)

| Total Plant Direct Cost (TPDC) (physical cost) |  |
| --- | --- |
| 1. Equipment Purchase Cost | 155,000 |
| 2. Installation | 78,000 |
| 3. Process Piping | 50,000 |
| 4. Instrumentation | 57,000 |
| 5. Insulation | 5,000 |
| 6. Electrical | 15,000 |
| 7. Buildings | 65,000 |
| 8. Yard Improvement | 22,000 |
| 9. Auxiliary Facilities | 57,000 |
| TPDC | 504,000.00 |
| 3B. Total Plant Indirect Cost (TPIC) |  |
| 10. Engineering | 121,000 |
| 11. Construction | 168,000 |
| TPIC | 289,000 |
| 3C. Total Plant Cost (TPC = TPDC+TPIC) |  |
| TPC | 793,000.00 |
| 3D. Contractor's Fee & Contingency (CFC) |  |
| 12. Contractor's Fee | 38,000 |
| 13. Contingency | 74,000 |
| CFC = 12+13 | 114000 |
| 3E. Direct Fixed Capital Cost (DFC = TPC+CFC) |  |
| DFC | 905,000.00 |