Supplementary Material A: Categorization of Circularity, Environmental Sustainability & Social Sustainability Indicators

Table A1: Categorization of Circularity Indicators: Types and Definitions

| Circularity Indicator | Indicator Type | Definition/ Description | Measurement Unit | Manufacturing Sector(s) from reference(s) | Reference(s) |
| --- | --- | --- | --- | --- | --- |
| Indicator group C0: System-Level Circularity |  |  |  |  |  |
| Circularity (CIRC) | Composite | It quantifies the number of times a resource passes through different product cycles within a system. | Dimensionless | Outdoor Power Equipment | [1–3] |
| Circular Economy Index (CEI) | Quantitative | Material Value Recycled from EOL Product(s)/Material Value Needed for (Re-) Producing EOL Product(s) | Dimensionless | Automotive, Outdoor Power Equipment, Packaging, Healthcare Equipment | [3–8] |
| Material Circularity Indicator (MCI) | Quantitative | It measures how circular a product or process is by evaluating the use of recycled materials, product lifespan, material recovery at EOL, and waste during production. MCI = (1 - LFI)*F(x), where: LFI (Linear Flow Index) is calculated using the formula: LFI=(1−CR−CU)/W0.​ Here, CR is the recycling rate, CU is the fraction of the product reused, W0 is the irrecoverable waste and F(X) is a utility factor representing the functional performance of the product. | Dimensionless (scores from 0 [fully linear] to 1 [fully circular]) | Healthcare Equipment, Agricultural Manufacturing, Energy & Plastics, Construction, Plastics, Packaging, Automotive, Textile, Bio-based products, Outdoor Power Equipment Sector, HVAC (Heating, Ventilation, and Air Conditioning) Equipment, Agri-food, Chemical, Electronics | [2–5,7,9–30] |
| Circular Economy Performance Indicator (CEPI) | Composite | It measures the ratio of the actual obtained environmental benefit from a waste treatment process to the ideal environmental benefit based on the quality and recoverability of the material or waste stream. | Dimensionless | Automotive, Bio-based products, Packaging, Healthcare Equipment | [5,9,19,29,31] |
| Circular Input Rate (CIR) | Quantitative | CIR=(R1.1 +R1.2 + R1.3 + G1.1 + G1.2) / (B1.1 + B1.2 +C1.3 + D1.3 + C1.4 + G1.1 + G1.2 + R1.1 + R1.2 + R1.3), where; R1.1: Materials embedded in products/components subject to direct reuse, R1.2​: Materials embedded in products/components subject to remanufacturing and recirculated in in-stock products/components, R1.3​: Materials embedded in products/components subject to remanufacturing and recirculated in manufacturing new products/components, G1.1​: Production of secondary material from post-consumer functional recycling sent for processing, G1.2​: Production of secondary material from post-consumer functional recycling sent for manufacturing, B1.1​: Production of primary material as the main product sent for processing, B1.2​: Production of primary material as a by-product sent for manufacturing, C1.3​: Imports of primary material, D1.3​: Imports of processed material, C1.4​: Imports of secondary materials. | Dimensionless | Automotive Sector | [32] |
| Circular Material Use Rate (CMU) | Quantitative | CMU Rate = [Amount of secondary (recycled) material used/ Total Material Used (primary + secondary)]​×100 | Percentage (%) | Energy & Plastics | [22] |
| End of Life Index (EOLI) | Composite | EoLI=α⋅DAIP+β⋅ RIP +(1/n)*Σ(γ⋅ MIT)+(1/m)*Σ(λ⋅ CIT) where; DAIP is the disassembly sub-index for the product, RIP is the recovery sub-index for the product, MIT is the module overall EoL index of modules in the product, CIT is the components EoL index of the loose components attached to the module directly, n is the total number of modules in the product, m is the total number of the loose components attached to the module directly, and α, β, γ, λ are weighting factors for the indices | Percentage (%) or Unitless Score OR Monetary Units | General, Automotive | [17,18,33] |
| Material Reutilization Score (MRS) | Quantitative | MRS = [% Recycled or rapidly renewable product content +2*(% of product recyclable or biodegradable/compostable]/3 *100 OR MRS = [%Intrinsic Recyclability(IR)*2]+[%Recycled Content (RC)]/3*100 | Percentage (%) | Plastics, General, Packaging, Automotive, Textile | [7,17,18,21,23,27,30] |
| Percentage of collected or taken-back end-of-life products prepared for reuse, refurbishment, remanufacture, and recycling compared to sales of new products | Quantitative | Collected or Processed EOL Products (in units or weight)/Sales of new Products (in units or weight) | Percentage (%) | Electrical & Electronics | [34] |
| Indicator group C1: Product Design & Adaptability |  |  |  |  |  |
| Material Durability Indicator (MDI) | Composite | MDI = Dch*Dm*E, where; Dch: quartic square root of (a1*a2*a3*a4) (a1: flammability resistance, a2: resistance to UV radiation, a3: resistance to water, a4: resistance to organic solvents), Dm: square root of (b1*b2) (b1: fatigue strength, b2: yield strength), E=I/Imax (I: environmental impact calculated from energy consumption and carbon footprint, Imax: maximum environmental impact) | Dimensionless Score | Healthcare Equipment | [19] |
| Recycling Desirability Index (RDI) | Quantitative | DDesirability = (DSimplicity + DMSI +DTR) .The RDI indicator results from the sum of the three factors: the Simplicity Index of separation of materials, the Material Safety Index (MSI), and the assessment of technological maturity through the Technology Readiness Level (TRL). The closer the result of this indicator is to 3, the more desirable it is to recycle the product under study. | Dimensionless | Packaging, Automotive, General | [7,17,18] |
| Index of Reusability/ Recyclability/Recoverability (RRR) | Composite | It is a weighted sum of four components: technical (RRRT​), environmental (RRREn​), economic (RRREc), and societal (RRRSb​). Mathematically, it is expressed as: RRR = aT⋅RRRT + aEn⋅RRREn + aEc⋅RRREc + aSb⋅RRRSb, where aT​, aEn​, aEc​, and aSb​ are the respective weights, and ST, SEn​, SEc​, SSb​ are the expert-assigned scores for each component. | Dimensionless | Energy, Chemical | [35,36] |
| Reconfiguration Index (RI) | Composite | RI = R*P/n , where; R is the number of possible reconfigurations in the product family, P is the number of product variants within the product family, n is the number of pieces in the whole product family. | Dimensionless | Healthcare Equipment | [37] |
| Indicator group C2: Disassembly & Repair |  |  |  |  |  |
| Cost of non-destructive disassembly | Quantitative | The costs associated with the disassembly of products or components in a manner that does not cause damage to them, allowing for their reuse, remanufacture, or refurbishment. | Monetary Units (e.g. $USD/unit) | General | [38] |
| Disassembly Effort Index (DEI) | Composite | Work and processes required to disassemble a product to provide a score for the product. | Dimensionless | Chemical, General | [4,17,18,39] |
| Effective Disassembly Time (EDT) | Quantitative | It calculates the total time required to disassemble a product and isolate target components, using a simpler approach that focuses solely on disassembly time without breaking down specific tasks. | Time (seconds) | Automotive, Chemical, General | [4,17,18] |
| Degree of repairability of product | Semi-Quantitative | It is defined by the product’s technical ease of repair—such as access to spare parts, repair instructions, and disassembly—and the availability of repair services. | Index/Score from the evaluation of factors such as ease of repair, availability of parts, etc. | Electrical & Electronics | [34] |
| Energy consumption for disassembly | Quantitative | It quantifies the energy required to perform disassembly tasks | kilowatt-hours (kWh) or joules (J) | Outdoor Power Equipment | [3] |
| Indicator group C3: Material Input & Composition |  |  |  |  |  |
| Fraction of renewable raw materials | Quantitative | (Quantity of Renewable Raw Materials Used/Total Quantity of Raw Materials Used)*100 | Percentage (%) | General | [38] |
| Reduction of Toxic Substances | Quantitative | It aims to quantify the reduction of the use of toxic substances considering RoHS (Restriction of Certain Hazardous Substances). | Percentage (%), concentration (ppm, mg/L) or weight (kg, t) | Electro-electronic, Plastics, Textiles & Fashion | [40] |
| Proportion of recycled content in a product | Quantitative | ENV CI1 = (Mass of Recycled Material Used/ Total Mass of the Product)*100 | Percentage (%) | Electrical & Electronics | [34] |
| Proportion of recyclable materials in a product | Quantitative | (Mass of Recyclable Materials/Total Product Mass)*100 | Percentage (%) | Electrical & Electronics | [34] |
| Indicator group C4: Reuse, Remanufacturing & Refurbishment |  |  |  |  |  |
| Reuse Potential Indicator (RPI) | Quantitative | RPI = Amount of Material Economically Recoverable through available technologies/ Total amount of material generated | Percentage (%) | Coal-fired electricity generation, General | [41,42] |
| Reuse Rate (ReR) | Quantitative | RER = (Mass of Reused Product/ Total Mass of the Product)*100 | Percentage (%) | Packaging | [21] |
| Proportion of reused components in manufacturing process | Quantitative | ENV CI7 =(Total number of reused components/Total number of components used in manufacturing) *100 | Percentage (%) | Electrical & Electronics | [34] |
| Market share of repair and reuse services compared to sales of new products | Quantitative | (Total Value of Repair and Reuse Services/Total Value of New Product Sales)*100 | Percentage (%) | Electrical & Electronics | [34] |
| Percentage of products remanufactured compared to total manufactured products | Quantitative | ECO C12 = (Number of remanufactured products/Total Number of Manufactured Products)*100 | Percentage (%) | Electrical and Electronics | [34] |
| Quantity of refurbished products | Quantitative | It is the quantity of the total recovery or parts (components) of the product, without necessarily going through all stages of remanufacturing. | Number/ Percentage/ Weight/ Volume of total products /components refurbished | Electro-electronics, Plastics, Textiles & Fashion | [40] |
| Indicator group C5: Efficiency & Value Performance |  |  |  |  |  |
| Resource Efficiency Account (REA) | Quantitative | REA=f/(Rv+De+Dp), where; f = the function or service provided by the system (this represents the societal benefit or utility provided by the product or service over its lifecycle), Rv: the total value of materials mined from the eco-sphere, De: the dissipation of resource value at the EoL stage, Dp: the dissipation of resource value during the manufacturing and processing stages. | Functional or Service Unit/(Monetary Value/ kg) | Automotive | [5] |
| Conversion efficiency ratio | Quantitative | C = Useful Output/Input Quantity required to meet the functional unit | Dimensionless | Furniture | [43] |
| Eco-cost/Value Creation (EVR) | Quantitative | EVR=Eco-costs/Value Creation where; Eco-costs represent the environmental burden associated with the production, use, and EOL stages of a product, Value creation refers to the economic value generated by the product for the stakeholders. | Dimensionless | Chemical, Healthcare Equipment, General | [4,17,19] |
| Value-Based Resource Efficiency Indicator (VRE) | Quantitative | VRE=Y/∑pi​*Xi​, where; Y = output value (which could be the value added by a product, process, or industry), Xi​ = volume of the resource i used in production, pi​ = market price of resource i, used as a weight to reflect both the scarcity and quality of the resource. | Dimensionless | General | [17,44] |
| Material Loss during Production | Quantitative | Material Loss (%) = (Total Material Wasted/Total Material Input)*100 | Percentage (%) | Household Products | [45] |
| Indicator group C6: Waste Generation & Management |  |  |  |  |  |
| Total Waste Generated (W) | Quantitative | W: Total waste generated | Mass (kg) | Construction | [26] |
| Waste from Recycled Content (WF) | Quantitative | WF: Waste generated to produce any recycled content used as raw material. It is calculated as (1−EF)×(FR−EF), where EF is the efficiency of the recycling process, and FR is the mass fraction of product raw material from recycled resources | Mass (kg) | Construction | [26] |
| Irrecoverable Waste (WO) | Quantitative | WO: Irrecoverable waste, which is calculated as 1−CR−CU, where CR is the mass fraction of the product being collected for recycling, and CU is the mass fraction of the product taken for component reuse | Mass (kg) | Construction | [26] |
| Waste Generated in Recycling (WC) | Quantitative | WC: Waste generated in the recycling process, calculated as (1−EC)×CR, where EC is the efficiency of the recycling process for the part of the product collected for recycling. | Mass (kg) | Construction | [26] |
| Environmental Factor | Quantitative | EF = Non-product mass/Mass of product, where: Non-product mass includes all mass generated in the process that does not contribute to the final product, such as waste or by-products, Mass of product is the total mass of the desired product output. | Dimensionless unit | Chemical and Petrochemical Sector | [46] |
| By-Product Recovery Rate | Quantitative | It is the proportion of by-products generated during production that are successfully recovered and repurposed for secondary uses. | Percentage (%) | Agri-food | [14] |
| Energy Recovery | Quantitative | It measures the extent to which energy is recovered from waste instead of landfilling. It is considered a lower-tier circular practice but preferable to disposal without recovery. | Megajoules (MJ) or Kilowatt-hours (kWh) | Construction, Electrical & Electronics | [12,34] |
| Indicator group C7: Recycling Performance |  |  |  |  |  |
| Potential Recycle Index | Quantitative | PRI = Σ(mi*Fi*Ei)/Mf, where: mi​: the mass of the i-th component, Fi​: the fraction of recyclable mass for the i-th component, Ei​: the efficiency of the recycling process for the same component, Mf​: the total mass of the product family. | Percentage (%) | Healthcare Equipment | [37] |
| Resource Footprint of the Recycling Process (R) | Quantitative | R = CEENE value of the recycling process (Cumulative Exergy Extraction from the Natural Environment) | Megajoules of exergy (MJex) | Plastics | [31] |
| Overall Processing Rate (Overall PR) | Quantitative | OPR = (i+k)/(g+j), where; i: the amount of product contained in separated old scrap sent to recovery, k: the amount of product contained in separated new scrap sent to recovery, g: the amount of product contained in collected old scrap, j: the amount of product contained in new scrap available for recycling | Dimensionless | Metal | [47,48] |
| End of Life Recycled Content (EoL RC) | Composite | EOL RC = (i+k)/(a+i+k), where i: Flow of collected and separated old scrap (EOL scrap), k: Flow of collected and separated new scrap (manufacturing scrap), a: Flow of primary product into the anthropogenic cycle | Percentage (%) | Metal | [47] |
| End of Life Recycling Input Rate (EoL RIR) | Quantitative | EOL RIR = i/(a+i+k), where: a: the flow of primary product into the anthropogenic cycle, i: the amount of EoL scrap collected and separated for recycling, k: the amount of new scrap (generated during manufacturing) collected and separated for recycling | Percentage (%) | Metal, Energy & Plastics | [22,47,48] |
| End of Life Process Efficiency Rate (EoL ER) | Quantitative | EOL ER = g/e2, where; g represents the amount of secondary material recovered (after processing), e2 represents the amount of material that enters the EOL recycling system (after collection) | Percentage (%) | Automotive | [49] |
| End of Life Processing Rate (EoL PR) | Quantitative | EoL PR=i/g where; i represents the amount of secondary product from post-consumer flows, g represents the total EoL product flows that are collected for recycling | Dimensionless | Metal | [48] |
| Indicator group C8: Resource Retention & Product Lifespan |  |  |  |  |  |
| Acircularity Indicator | Composite | Acircularity=(Rv+De+Dp)/M, where; Rv= Residual value loss, De= Downcycling effect cost, Dp= Dissipative loss, M = value of the total constituent material in the product. It shows the fraction of material value lost due to waste, downcycling, and dissipation, compared to the total material value of the product. | Dimensionless | Automotive, Electronics | [5,20] |
| Final Retention in Society (FRS) | Composite | FRS = (mth/ms,1)*100%, where; mth: the mass of the primary raw material still available at the end of the time horizon, ms,1​: the mass of the primary raw material initially used in the first product cycle. | Percentage (%) | Electronics | [20] |
| In-Use Occupation Ratio (UOR) | Composite | UOR=(ΣOccUj/OccUmax)*100%, where; OccUj = (mu,j-lu,j/2)*Δtu,j, where; mu,j​: the mass of material entering the in-use phase in product cycle j,lu,j​: the material loss during the in-use phase in product cycle j, Δtu,j​: the duration of the in-use phase for product cycle j, OccUmax = ms,1*TH, where; mS,1​: the mass of the primary raw material entering the supply phase of the first product cycle, TH: the given time horizon, typically measured in years. | Percentage (%) | Electronics | [20] |
| Resource Duration Indicator (RDI) | Composite | It measures the total time a resource is retained within a product system to maximize its exploitation through use, reuse, and recycling. It is expressed as the sum of three components: the product's initial lifetime (A), the additional lifetime gained through refurbishment (B), and the lifetime extended through material recycling (C). | Time (e.g. years) | Electronics, Automotive, Packaging | [7,17,18,24,50] |

Table A2: Categorization of Environmental Sustainability Indicators: Types and Definitions

| Environmental Sustainability Indicator | Indicator Type | Definition/ Description | Measurement Unit | Manufacturing Sector(s) from reference(s) | Reference(s) |
| --- | --- | --- | --- | --- | --- |
| Indicator Group E0: Air Pollutant Emissions |  |  |  |  |  |
| NOx Emissions | Quantitative | NOx Emissions = Efuel​×EFNOx​, where: Efuel​ is the total fuel consumption (e.g., in metric tons) over a given period or operational cycle, EFNOx​ is the NOx emission factor specific to the type of fuel used. | g/kWh | Maritime | [51] |
| Particulate Matter (PM) Concentration | Quantitative | PM Concentration = Mass of Collected Particles/Volume of Air Sampled | μg/m3 | Electrical Equipment & Components, Food Packaging | [52,53] |
| SOx Emissions | Quantitative | SOx Emissions=Fuel Consumption×Sulfur Content in Fuel×Emission Factor, where: Fuel Consumption is measured in tons of fuel burned over a specific period or distance, Sulfur Content in Fuel is a percentage representing the sulfur concentration in the fuel, Emission Factor (EF) is based on the chemical assumption that most sulfur in the fuel converts to SOx during combustion | Mass (g,t) or g/kWh or g/km | Maritime | [51] |
| Indicator Group E1: Atmospheric Impact |  |  |  |  |  |
| Ozone Depletion Potential (ODP) | Quantitative | ODP = Σ(mi x ODPi), where: mi​ is the mass of substance I emitted, and ODPi is the ozone depletion potential factor of substance i ODP refers to the impact of substances on the stratospheric ozone layer, which protects life on Earth from harmful ultraviolet radiation. It measures the potential of a substance to contribute to ozone layer depletion in the upper atmosphere (stratosphere). | kg CFC-11 equivalents (trichlorofluoromethane) | Food (Chocolate Production) | [54] |
| Photochemical Ozone Formation Potential (POFP) | Quantitative | POFP=∑​(Emissionsi​ ×Characterization Factori​), where: Emissionsi​: The amount of each ozone precursor pollutant i (e.g., VOCs, NOx) emitted, Characterization Factori​: The relative ozone formation potential of pollutant i compared to a reference substance, typically ethylene (C2H4 equivalents). POFP assesses the contribution of emissions (e.g., VOCs, NOx) to the formation of ground-level (tropospheric) ozone, a key component of smog. This indicator measures how pollutants react in the presence of sunlight to form ozone at lower atmospheric levels (troposphere). | kilogram ethylene equivalents (kg C₂H₄-eq.) | Electrical Equipment & Components | [52] |
| Acidification Potential (AP) | Quantitative | AP = Σ (Εi**Api), Ei​: the emission amount of each acidifying substance iii (e.g., NOx, SO2​, NH3​), APi​: the acidification potential factor of that substance, typically expressed in terms of SO₂-equivalents. | Mass (kg) | Agri-food, Machining | [55,56] |
| Indicator Group E2: Climate Change |  |  |  |  |  |
| CO2 efficiency indicator (CEI) | Composite | CEI= Total CO₂ emissions associated with the product or system​/ Predicted service life or operational duration | mass of CO₂ (e.g., kg, tons) per unit of time (e.g., year) | Construction | [57] |
| GHG emissions from fossil fuels, transportation, purchased electricity, waste, and secondary materials | Quantitative | Total GHG Emissions = GHGstationary​+GHGmobile​+GHGtransport​+GHGwaste​+GHGprocess​ | Kilograms of CO₂ equivalent (kg CO₂e) | Furniture Manufacturing, Chemical and Petrochemical, Wine | [46,58,59] |
| Climate Change (biogenic, fossil, land use change) | Quantitative | Climate Change Impact = ∑(Biogenic Emissions + Fossil Emissions + Land Use Change Emissions), where: Biogenic Emissions = (CH₄ emissions from biogenic sources) × Global Warming Potential (GWP of CH₄) + CO₂ from natural sources, Fossil Emissions = Total CO₂ from fossil fuel combustion, Land Use Change Emissions = Net CO₂ released due to deforestation or other land alterations. | kilograms of carbon dioxide equivalents (kg CO₂e) | Packaging, Food, Construction, Automotive | [52–54,60–62] |
| Indicator Group E3: Environmental – Economic Trade-Off Metrics |  |  |  |  |  |
| Cost-Benefit Ratio | Composite | Cost-Benefit Ratio (CBR) = Total Benefits/Total Costs, ​ where: Total Benefits is the sum of all monetary, environmental, or other measurable benefits gained from the investment (e.g., money saved, CO₂ emissions reduced, resources conserved), Total Costs is the sum of all expenses incurred to implement the action or measure (e.g., indicator application costs, production costs, and associated operational costs). | Dimensionless Measure | Food Packaging | [63] |
| Net Environmental Benefit | Composite | NEB=Total Environmental Savings−Total Environmental Costs | Monetary value, Energy Consumption (MJ,kWh), Resource Use (kg) or Carbon Footprint (COe) | Food Packaging | [63] |
| Indicator Group E4: Packaging |  |  |  |  |  |
| Package-to-Product (PtP) Indicator | Composite | PtPCC​(%) = (∑CCPa​​/ΣCCPr)×100, where: PtPCC​ (%): The Package-to-Product indicator for the climate change category, expressed as a percentage. CCPa​: The climate change impact of the packaging materials. CCPr​: The climate change impact of the product itself. | Percentage (%) | Construction, Food Packaging (paper, plastic, glass, metal) | [60,61] |
| Marine Litter Indicator (MLI) | Quantitative | MLI = (F1×F4)/(F2×F3), where each component is defined as follows: F1​: The potential number of dispersed containers, calculated as the ratio of the estimated number of dispersed containers (Bd) to the maximum number of dispersed containers: F1=Bd/Bdmax, F2​: The incentive for returning bottles, expressed as a ratio of the current incentive (Pi) to the maximum incentive available (Pimax) F2=Pi/Pimax, F3​: The weight of the packaging material, calculated as the ratio of the bottle's weight (Wb) to the maximum weight of packaging material among all options (Wbmax) F3=Wb/Wbmax, F4​: The degradation time of the material, expressed as a ratio of the degradation time (Dt) of the specific material to the longest degradation time among the options F4=Dt/Dtmax | Dimensionless Unit | Food Packaging | [64] |
| Material-Specific Efficiency Packaging Indicators (Plastic, Fiber, Glass, Metal) | Quantitative | Measure packaging efficiency at company and sector levels by calculating the amount of each primary packaging material (plastic, fiber, glass, metal) used per unit of economic turnover. Sector indicators aggregate these values, weighting each company’s material usage by its turnover share, to provide an overall view of material efficiency across the packaging sector. | kg/monetary value | Packaging | [65] |
| Indicator Group E5: Material Sustainability |  |  |  |  |  |
| Biodegradability | Composite | The ability of a substance to break down into simpler, non-toxic components through natural processes involving microorganisms, such as bacteria or fungi, thus minimizing long-term environmental impact | Percentage over a specified period (%), CO₂ or O₂ Consumption (mg/g or mL/g), Biological Oxygen Demand (BOD), Half-life (days) | Chemical & Petrochemical | [46] |
| Material Degradation Time | Quantitative | F4 = Dt/(Dt)max​, where: Dt represents the degradation time of the material (in years), (Dt)max​ is the maximum degradation time among the materials being compared, applicable to any product lifecycle | Dimensionless Ratio | Food Packaging | [64] |
| Indicator Group E6: Resource Use, Scarcity & Depletion |  |  |  |  |  |
| Abiotic Depletion Potential (ADP) | Quantitative | ADP=∑​(​Qi/Ri​​×CFi​), where: Qi​: Quantity of each resource i used in the production process (e.g., kg or MJ for fossil fuels), Ri: Total known reserves of resource i (in kg or MJ), which represents the resource's availability in the Earth's crust, CFi​: Characterization factor of resource i, which converts the quantity of the resource used into a depletion potential in terms of a reference substance, usually antimony (Sb eq), to normalize scarcity impacts across different resources. | kg Sb eq (kilograms of antimony equivalent) | Agri-food, Food | [52,56,66] |
| Fossil Resource Scarcity | Quantitative | Fossil Resource Scarcity=∑​(Qi​×EFi​), where: Qi​: Quantity of each fossil resource i used (e.g., kg or MJ), EFi​: Characterization factor for resource i (often measured in kg oil equivalent per unit of Qi​) | kg oil equivalent (kg oil-eq) | Food Packaging | [64] |
| Material Intensity of Use (MIU) | Quantitative | MIU= Material Use (mass)/ Economic Output ​, where: Material Use is the amount of a particular material (often in mass units like tons), Economic Output is typically measured in terms of GDP or another economic indicator | kg/EU (mass/monetary value) | Metal, chemical processes, construction materials | [67] |
| Share of Renewable Sources | Composite | (Mass or volume of renewable feedstocks​/ Total mass or volume of all feedstocks) × 100 | Percentage (%) | Chemical & Petrochemical | [46] |
| Total Energy consumption (renewable/non-renewable) | Quantitative | Represents the total energy used in the production, operation, or lifecycle of a product, normalized either per unit mass or per individual product. | kilowatt-hours per kilogram or kilowatt-hours per item (kWh/kg or kWh/item) | Railway | [68] |
| Land Requirement | Quantitative | The total area of land needed to establish, operate, and maintain manufacturing facilities, including spaces for production lines, storage, logistics, waste management, and auxiliary operations | Square meters per unit of output (m²/unit) or Square meters per production capacity (m²/capacity unit) or Square meters (m²) or hectares (ha) | Energy | [69] |
| Indicator Group E7: Toxicity & Eutrophication |  |  |  |  |  |
| Weighted Hazard Exposure | Quantitative | Weighted Hazard Exposure=∑ (​mass of materiali​​/ Permissible Exposure Limit (PEL)i), where: mass of materiali​ is the mass (in kg) of each material i used in the process, PELi is the Permissible Exposure Limit (in ppm) for material i. | Dimensionless unit | Chemical and Petrochemical | [46] |
| Ecotoxicity Potential (Fresh Water Aquatic -FAWP, Marine Aquatic-MAEP, Terrestrial-TEP) | Quantitative | ETPcompartment​ = ∑Ei​×Fi,compartment​×EFi,compartment​, where: Ei​: Emission quantity of substance i (e.g., kg of emitted chemical), Fi,compartment​: Fate Factor of substance i in the specified compartment (e.g., freshwater, marine water, terrestrial), EFi,compartment: Factor for ecotoxicity of substance i in the specified compartment. | kilograms of dichlorobenzene equivalents (kg DCB-eq) | Agri-food, Food, Machining, Energy, Automotive, Disposable diapers | [54–56,62,66,70,71] |
| Human Toxicity Potential (HTP) (via soil, water, air) (carcinogenic/non-carcinogenic) | Quantitative | HTPcompartment​=∑​(Ei​×CFi,compartment​), where: Ei​: Emission of substance i (e.g., kg or g released per unit of time or activity), CFi,compartment​: Characterization factor for substance i in a given environmental compartment (soil, water, or air), representing the potential impact per unit of emission on human health. | kilograms of 1,4-dichlorobenzene equivalents | Machining, Agri-food | [55,56] |
| Eutrophication Potential (EP) (Freshwater (EPFW), Marine (EPM) and Terrestrial (EPT)) | Quantitative | EP=∑​(mi​×EFi​), where: mi​ is the mass of emitted substance i, EFi​ is the eutrophication characterization factor for substance i, which is specific to the type of eutrophication (marine, freshwater, or terrestrial) and the impact model used. | kg PO4(3−)​ equivalents or kg N equivalents | Electrical Equipment & Components, Agri-food, Food, Disposable diapers | [52,56,66,70] |
| Indicator Group E8: Water Management |  |  |  |  |  |
| Biochemical Oxygen Demand (BOD) Grey Water Footprint | Quantitative | BOD Grey Water Footprint= BOD load​/(BOD limit−natural BOD concentration), where BOD Load: The total amount of BOD introduced into the water system, typically measured in units, such as mg of BOD per liter (mg/L) or kg/day. BOD load reflects the concentration of organic matter that, when decomposing, consumes oxygen from the water. High BOD levels indicate more organic pollutants, which means more oxygen is needed for breakdown, BOD Limit: This is the maximum acceptable concentration of BOD in water according to water quality standards, Natural BOD Concentration: This is the baseline or “natural” concentration of BOD in water bodies. | L/day (liters per day) or m³/day (cubic meters per day) | Electronics | [72] |
| Chemical Oxygen Demand (COD) Grey Water Footprint | Quantitative | COD Grey Water Footprint= COD load​/(COD limit−natural COD concentration), where COD Load: The total amount of COD introduced into the water system, typically measured in kg/day or mg/day. COD Limit: This is the maximum acceptable concentration of COD in water according to water quality standards. Natural COD Concentration: This is the baseline or “natural” concentration of COD in water bodies. | L/day (liters per day) or m³/day (cubic meters per day) | Electronics | [72] |
| Suspended Solids (SS) Grey Water Footprint | Quantitative | SS Grey Water Footprint=SSload/(SSlimit−natural SSconcentration), where: SS Load: This is the total amount of suspended solids introduced into the water system, typically measured in units such as mg/L or kg/day. The SS load represents the concentration of particulate matter (such as soil, silt, organic and inorganic particles) in the water, SS Limit: The maximum acceptable concentration of suspended solids in the water, as defined by water quality standards, Natural SS Concentration: This is the baseline or “natural” concentration of suspended solids in water bodies, representing typical background levels without additional pollution. | cubic meters per day (m³/day) | Electronics | [72] |
| Total Volume of Wastewater Treated | Quantitative | Measures the quantity of wastewater processed in a treatment system to reduce pollutants and contaminants | cubic meters per year (m³/year) | Energy and Plastics | [22] |
| Volume of Water Recycled and Reused | Quantitative | (Volume of water recycled or reused/Total water used)​×100 | Percentage (%) | Energy and Plastics | [22] |
| Water Footprint (WF) | Quantitative | Blue Water Footprint: Refers to the volume of surface and groundwater consumed, calculated as: Blue Water Footprint = Blue Water Evaporation + Blue Water Incorporation + Lost Return Flow (volume/time), Green Water Footprint: Refers to the volume of rainwater stored in the soil that is consumed, expressed as: Green Water Footprint = Green Water Evaporation + Green Water Incorporation (volume/time), Gray Water Footprint: Refers to the volume of freshwater needed to dilute pollutants, based on existing water quality standards, calculated as: Gray Water Footprint=L/(Cmax−Cnat) | cubic meters (m³) | Electronics | [72] |

Table A3: Categorization of Social Sustainability Indicators: Types and Definitions

| Social Sustainability Indicator | Indicator Type | Definition/ Description | Measurement Unit | Manufacturing Sector(s) from reference(s) | Reference(s) |
| --- | --- | --- | --- | --- | --- |
| Indicator Group S0: Employment Practices |  |  |  |  |  |
| Job Creation & Qualification | Quantitative | Employment opportunities created in a circular economy, emphasizing both quantity and quality, including the development of skilled jobs and addressing the risk of insecure, exploitative roles | Number of jobs created | Textile & Fashion | [73] |
| Formalized work | Quantitative | Employment agreements that adhere to legal standards, ensuring workers have formal contracts and benefits, reducing risks of informal labor and increasing traceability in the workforce | Percentage of formal contracts; Number of workers under legal agreements | Textile & Fashion | [73] |
| Inclusive Employment Practices | Quantitative | Practices aimed at increasing diversity in hiring, including employment of women, migrants, individuals with disabilities, and those in vulnerable situations, ensuring equity across all levels of employment | Percentage of diverse hires (e.g., women, migrants, etc.); Diversity index | Textile & Fashion, General | [73,74] |
| Social security | Quantitative | Coverage provided to workers through formal systems ensuring access to basic needs during periods of unemployment, illness, or retirement | Percentage of workers covered | Various manufacturing sectors, Electronics Manufacturing, Chemical Manufacturing | [75,76] |
| Equal pay | Quantitative | Guaranteeing equal compensation for all workers, irrespective of gender, race, ethnicity, disability, or other personal characteristics, ensuring pay equity across roles and responsibilities | Gender pay gap; Percentage of roles with pay parity | Textile & Fashion | [73] |
| Equal Leadership Opportunities | Quantitative | Ensuring women, migrants, people of different races and ethnicities, people in social vulnerability situations, people with disabilities, and LGBTQIA+ individuals are represented in tactical and strategic positions and leadership | Percentage of underrepresented groups in leadership positions; Representation index for tactical/strategic roles | Textile & Fashion, Electronics Manufacturing, Chemical Manufacturing | [73,76] |
| Educational Background of Persons Employed | Quantitative | Tracks the education levels (vocational, university, etc.) of employees | Number of employees by education level | General | [77] |
| Subsidized Employment of Vulnerable Groups | Quantitative | Measures the proportion of employment opportunities specifically created for vulnerable or disadvantaged groups (e.g., long-term unemployed, disabled workers, refugees, and marginalized communities) through government, NGO, or corporate subsidies. | Percentage of subsidized employees | General | [77] |
| Encouragement of Local Labor | Semi-Quantitative | Evaluates the extent to which companies prioritize hiring from the local community, supporting regional economies and reducing economic disparities. It assesses both direct employment policies and indirect incentives for subcontractors to hire locally. | Scored numerically based on a response scale (e.g., Scale: None, Low, Moderate, High, Very High) | Textile & Fashion | [73] |
| Career Plan Programs for Workers | Semi-Quantitative | Evaluates the presence and effectiveness of structured career development pathways, including mentorship, promotion tracks, leadership programs, and succession planning. This indicator reflects an organization's commitment to long-term employee growth and retention. | Scored numerically based on a response scale (Yes/No, or Scale: None to Extensive) | Textile & Fashion | [73] |
| Qualification/Requalification of workers in the value chain to develop new specialties and skills | Semi-Quantitative | Tracks investment in reskilling and upskilling initiatives designed to prepare workers for new technologies, evolving industry needs, and emerging job roles. This includes certification programs, workforce adaptation strategies, and skill conversion efforts to ensure long-term employability and competitiveness. | Scored numerically based on a response scale (Scale: None to High investment) | Textile & Fashion | [73] |
| Training | Semi-Quantitative | Measures the frequency and availability of job-related training programs that help workers improve their current skills and maintain operational efficiency. Unlike requalification, this indicator tracks routine training on existing job functions rather than skill transformation. | Scored numerically based on a response scale (Scale: None to Frequent training programs) | Various Manufacturing Sectors | [75] |
| Indicator Group S1: Labour Standards & Workplace Conditions |  |  |  |  |  |
| Working hour control measures | Quantitative | Policies to regulate and monitor working hours, preventing excessive workloads and ensuring compliance with labor standards for a healthy work-life balance | Average weekly hours worked per employee; Percentage compliance with legal working hour limits | Textile & Fashion, Electronics Manufacturing, Chemical Manufacturing, Food Manufacturing, Automotive manufacturing, Home Appliance Manufacturing, Leather Industry | [73,76,78–80] |
| Cognitive parameters (Mental Workload & Cognitive Impact) | Quantitative | Mental Workload, Reaction Time, Concentration Levels, Stress Levels | Subjective rating (e.g., NASA-TLX), Seconds (s), Percentage (%)/Index, Cortisol level (ng/mL) | Woodworking, General | [81,82] |
| Ergonomics and Physical Parameters (Musculoskeletal & Physical Workload) | Quantitative | Posture, Force Exertion, Repetition Rate, Reach Zones, Energy Expenditure, Dust Exposure, Tool Accessibility, Recovery Time | Angles (°), Newton (N), Cycles per minute (cpm), Distance (cm), Kilocalories (kcal)/Oxygen Consumption (mL/min), Dust concentration (mg/m³), Position accuracy (mm), Minutes (min) | Woodworking, General, Automotive manufacturing, Home Appliance Manufacturing, Leather Industry | [79,81,82] |
| Psycho-physical Interaction Parameters | Quantitative | Heat Stress, Noise Exposure, Lighting, Vibration | Wet Bulb Globe Temperature (°C), Decibels dB, Lux lx, Acceleration m/s² | Woodworking, Automotive manufacturing, Home Appliance Manufacturing, Leather Industry | [79,81] |
| Policies against harassment and abuse | Semi-Quantitative | Existence and enforcement of anti-harassment policies | Scored numerically based on a response scale (Yes/No, or Strength Scale: Weak to Strong) | Textile & Fashion | [73] |
| Worker Satisfaction | Semi-Quantitative | Employee satisfaction level based on surveys | Scored numerically based on a response scale (Scale: -5 to +5, or Satisfaction % index) | Various Manufacturing Sectors | [75] |
| Indicator Group S2: Fair Compensation & Benefits |  |  |  |  |  |
| Pay Levels | Quantitative | Evaluates the median monthly salary of employees | Median monthly salary (€) | General, Electronics Manufacturing, Chemical Manufacturing | [73,76,77,80] |
| Living Wage Compliance | Quantitative | Providing wages that meet or exceed a middle-class living standard, ensuring economic security, personal autonomy, and career growth opportunities for workers | Average wage vs. living wage benchmark; Percentage of employees earning above the living wage | Textile & Fashion | [73] |
| Workers’ benefits (e.g., transport, food, health, etc.) | Quantitative | Provision of essential benefits such as transportation, food, and healthcare to support workers' overall well-being | Percentage of employees receiving essential benefits | Textile & Fashion | [73] |
| Additional workers benefit (e.g., bonuses, profit sharing, etc.) | Quantitative | Offering supplementary benefits, including bonuses and profit-sharing schemes, to enhance worker satisfaction and motivation beyond the basic salary | Percentage of employees receiving additional benefits; Value of bonuses/profit-sharing as a percentage of salary | Textile & Fashion | [73] |
| Indicator Group S3: Ethical Practices |  |  |  |  |  |
| Child Labour | Quantitative | The use of children (under the legal working age) in economic activities that may hinder their education, health, and development | Number of children involved; percentage of total labor force | General, Electronics Manufacturing, Chemical Manufacturing | [76,83,84] |
| Forced or Compulsory Labour | Quantitative | Work or services that individuals are compelled to perform against their will, under threat of penalty or harm, often without adequate compensation | Number of cases reported; percentage of workforce affected | General, Electronics Manufacturing, Chemical Manufacturing | [76,84] |
| Ethical Sourcing & Supplier Responsibility | Semi-Quantitative | A set of questions related to suppliers, including social responsibility, employee human rights, contract issues, health, safety, education, and training | Scored numerically based on responses (e.g., Yes/No/Not Known, -5 to +5 scale) | Process Industry, Metal manufacturing | [85–87] |
| Freedom of association and collective bargaining | Quantitative | Measures the extent to which workers have the right to form and join labor unions, participate in collective bargaining, and engage in worker representation without fear of retaliation. | % of workers covered by collective agreements | Textile & Fashion | [73] |
| Indicator Group S4: Health & Safety |  |  |  |  |  |
| Work-related injuries | Quantitative | Physical harm sustained by workers due to workplace activities, regardless of the cause (accident or repetitive strain), potentially impacting their health or ability to work | Number of injuries/year | Various manufacturing sectors | [75] |
| Work accidents | Quantitative | Unplanned events occurring in the workplace that may or may not result in injury, including near-misses and unsafe incidents. | Number of accidents/year | Various manufacturing sectors | [75,88] |
| Personal protective equipment (PPE) | Quantitative | Equipment worn by workers to minimize exposure to occupational hazards that could cause injury or illness | Percentage of workers using PPE | Various manufacturing sectors, Metallurgical Manufacturing | [75,89] |
| Lost workdays | Quantitative | Days of work missed by workers due to injuries, accidents, or illnesses occurring as a result of workplace activities | Days lost/year | Various manufacturing sectors | [75,88] |
| Workplace safety control | Quantitative | Implementation of safety protocols and regular monitoring to prevent workplace accidents and ensure the health and well-being of all workers in the production chain | Number of accidents/incidents reported; Compliance rate with safety standards | Textile & Fashion, Automotive manufacturing, Home Appliance Manufacturing, Leather Industry | [73,79] |
| Indicator Group S5: Governance & Reporting |  |  |  |  |  |
| Reporting Indicator | Semi-quantitative | Covers CSR and local community matters, including the extent and quality of policies, standards, compliance programs, third-party certification and auditing, transparency in reporting activities, and proactive communication (e.g., risk communication and change policies) | Scored numerically based on responses to qualitative and quantitative aspects (e.g., -5 to +5 scale). | Process Industry, Metal Manufacturing | [85–87] |
| Location Indicator | Semi-quantitative | Encompasses governance dimensions such as voice and accountability, political stability, government effectiveness, regulatory quality, rule of law, and control of corruption, supporting cross-country comparisons and trend evaluations | Reported in standard normal units (-2.5 to 2.5) and percentile rank (0 to 100) | Process Industry, Metal Manufacturing | [85–87] |
| Indicator Group S6: Impacts on Society |  |  |  |  |  |
| Number of Workplaces and Personnel in CE Industries | Quantitative | Monitors the total number of workplaces and employees in CE industries, reflecting sector growth and economic contribution | Count of workplaces and employees | General | [77] |
| Consumer Health & Safety | Semi-Quantitative | Evaluates the extent to which a company ensures that its products and manufacturing processes do not pose health or safety risks to consumers. This includes compliance with health and safety regulations, use of non-toxic materials, adherence to product safety standards, and implementation of quality control measures. It also considers product recalls, reported safety incidents, and the presence of consumer protection policies. | Scored numerically based on response scale (Scale: No Safeguards to Strong Safeguards) | General | [84] |
| Relationship and actions with local communities | Semi-Quantitative | Assesses the extent and nature of a company's engagement with local communities, focusing on corporate initiatives that support social development, economic growth, and environmental well-being (e.g., partnerships with local organizations, investment in community projects) | Scored numerically based on response scale (Scale: No Engagement to Active Programs) | Textile & Fashion | [73] |
| Opportunities to situate facilities improving future residents' lives | Semi-Quantitative | Evaluates the extent to which companies consider social and economic benefits when selecting locations for new facilities. | Scored numerically based on response scale (Scale: No Consideration to Strong Integration) | Industrial Material Manufacturing | [90] |
| Potential to attract government attention to ongoing community needs | Semi-Quantitative | Measures the company's influence in drawing government attention to pressing local community issues, encouraging policy changes, and securing public support for infrastructure or social development programs. | Scored numerically based on response scale (Scale: None to Strong Policy Influence) | Industrial Material Manufacturing | [90] |
| Fair Trading Relationships | Semi-Quantitative | Examines a company's commitment to ethical business practices, particularly in supplier and customer relationships. | Scored numerically based on response scale (Scale: Not Considered to Fully Implemented) | General | [84] |
| Philanthropy | Semi-Quantitative | Assesses the extent to which a company engages in philanthropic activities, including charitable donations, CSR programs, and employee volunteerism. This indicator captures both financial contributions and non-monetary efforts to support social and environmental causes. | Scored numerically based on response scale (Scale: None to High Contribution) | General | [91] |

References

1. 	Figge, F.; Thorpe, A.S.; Givry, P.; Canning, L.; Franklin-Johnson, E. Longevity and Circularity as Indicators of Eco-Efficient Resource Use in the Circular Economy. Ecological Economics 2018, 150, 297–306, doi:10.1016/j.ecolecon.2018.04.030.

2. 	Saidani, M.; Kreuder, A.; Babilonia, G.; Benavides, P.T.; Blume, N.; Jackson, S.; Koffler, C.; Kumar, M.; Minke, C.; Richkus, J.; et al. Clarify the Nexus between Life Cycle Assessment and Circularity Indicators: A SETAC/ACLCA Interest Group. Int J Life Cycle Assess 2022, 27, 916–925, doi:10.1007/s11367-022-02061-w.

3. 	Saidani, M.; Kim, H. Nexus Between Life Cycle Assessment, Circularity, and Sustainability Indicators—Part I: A Review. Circ.Econ.Sust. 2022, 2, 1143–1156, doi:10.1007/s43615-022-00159-9.

4. 	De Pascale, A.; Arbolino, R.; Szopik-Depczyńska, K.; Limosani, M.; Ioppolo, G. A Systematic Review for Measuring Circular Economy: The 61 Indicators. Journal of Cleaner Production 2021, 281, 124942, doi:10.1016/j.jclepro.2020.124942.

5. 	Halada, K.; Tahara, K.; Matsumoto, M. New Indicators ‘Acircularity’ and ‘Resource Efficiency Account’ to Evaluate the Efforts of Eco-Design in Circular Economy. International Journal of Automation Technology 2022, 16, 684–695, doi:10.20965/ijat.2022.p0684.

6. 	Maio, F.D.; Rem, P.C. A Robust Indicator for Promoting Circular Economy through Recycling. Journal of Environmental Protection 2015, 6, 1095–1104, doi:10.4236/jep.2015.610096.

7. 	Matos, J.; Martins, C.I.; Simoes, R. Circularity Micro-Indicators for Plastic Packaging and Their Relation to Circular Economy Principles and Design Tools. Sustainability 2024, 16, 2182, doi:10.3390/su16052182.

8. 	Mesa, J.A.; Gonzalez-Quiroga, A.; Aguiar, M.F.; Jugend, D. Linking Product Design and Durability: A Review and Research Agenda. Heliyon 2022, 8, doi:10.1016/j.heliyon.2022.e10734.

9. 	dos Santos Gonçalves, P.V.; Barbosa, R.F.M.; Allan, V.; Raspini, P.; Sehnem, S.; Campos, L.M. The Use of Circular Economy Indicators to Improve Sustainability in the Recycling Aluminium Context.". 2021.

10. 	Hatzfeld, T.; Backes, J.G.; Guenther, E.; Traverso, M. Modeling Circularity as Functionality Over Use-Time to Reflect on Circularity Indicator Challenges and Identify New Indicators for the Circular Economy. Journal of Cleaner Production 2022, 379, 134797, doi:10.1016/j.jclepro.2022.134797.

11. 	Hummen, T.; Hellweg, S.; Roshandel, R. Optimizing Lifespan of Circular Products: A Generic Dynamic Programming Approach for Energy-Using Products. Energies 2023, 16, 6711, doi:10.3390/en16186711.

12. 	Incelli, F.; Cardellicchio, L.; Rossetti, M. Circularity Indicators as a Design Tool for Design and Construction Strategies in Architecture. Buildings 2023, 13, 1706, doi:10.3390/buildings13071706.

13. 	Khadim, N.; Agliata, R.; Marino, A.; Thaheem, M.J.; Mollo, L. Critical Review of Nano and Micro-Level Building Circularity Indicators and Frameworks. Journal of Cleaner Production 2022, 357, 131859.

14. 	Kounani, A.; PAVLOUDI, A.; AGGELOPOULOS, S.; KONTOGEORGOS, A. Circular Economy: Assessment of Circularity in Olive Oil Mills. 2023.

15. 	Kravchenko, M.; McAloone, T.C.; Pigosso, D.C. To What Extent Do Circular Economy Indicators Capture Sustainability? Procedia CIRP 2020, 90, 31–36.

16. 	Lavallais, C.M.; Dunn, J.B. Developing Product Level Indicators to Advance the Nitrogen Circular Economy. Resources, Conservation and Recycling 2023, 198, 107167, doi:10.1016/j.resconrec.2023.107167.

17. 	Matos, J.; Martins, C.; Simões, C.L.; Simoes, R. Comparative Analysis of Micro Level Indicators for Evaluating the Progress towards a Circular Economy. Sustainable Production and Consumption 2023, 39, 521–533, doi:10.1016/j.spc.2023.06.002.

18. 	Matos, J.; Santos, S.; Simões, C.L.; Martins, C.I.; Simoes, R. Practical Application of Circularity Micro-Indicators to Automotive Plastic Parts in an Industrial Context. Sustainable Production and Consumption 2023, 43, 155–167, doi:10.1016/j.spc.2023.11.009.

19. 	Mesa, J.; González-Quiroga, A.; Maury, H. Developing an Indicator for Material Selection Based on Durability and Environmental Footprint: A Circular Economy Perspective. Resources, Conservation and Recycling 2020, 160, 104887, doi:10.1016/j.resconrec.2020.104887.

20. 	Moraga, G.; Huysveld, S.; De Meester, S.; Dewulf, J. Development of Circularity Indicators Based on the In-Use Occupation of Materials. Journal of Cleaner Production 2021, 279, 123889.

21. 	Niero, M.; Kalbar, P.P. Coupling Material Circularity Indicators and Life Cycle Based Indicators: A Proposal to Advance the Assessment of Circular Economy Strategies at the Product Level. Resources, Conservation and Recycling 2019, 140, 305–312.

22. 	Niyommaneerat, W.; Suwanteep, K.; Chavalparit, O. Sustainability Indicators to Achieve a Circular Economy: A Case Study of Renewable Energy and Plastic Waste Recycling Corporate Social Responsibility (CSR) Projects in Thailand. Journal of Cleaner Production 2023, 391, 136203, doi:10.1016/j.jclepro.2023.136203.

23. 	Roos Lindgreen, E.; Mondello, G.; Salomone, R.; Lanuzza, F.; Saija, G. Exploring the Effectiveness of Grey Literature Indicators and Life Cycle Assessment in Assessing Circular Economy at the Micro Level: A Comparative Analysis. Int J Life Cycle Assess 2021, 26, 2171–2191, doi:10.1007/s11367-021-01972-4.

24. 	Saidani, M.; Cluzel, F.; Leroy, Y.; Yannou, B. Testing the Robustness of Circularity Indicators: Empirical Insights from Workshops on an Industrial Product. Proceedings of the Design Society: International Conference on Engineering Design 2019, 1, 3401–3410, doi:10.1017/dsi.2019.347.

25. 	Saidani, M.; Kim, H.; Cluzel, F.; Leroy, Y.; Yannou, B. PRODUCT CIRCULARITY INDICATORS: WHAT CONTRIBUTIONS IN DESIGNING FOR A CIRCULAR ECONOMY? Proceedings of the Design Society: DESIGN Conference 2020, 1, 2129–2138, doi:10.1017/dsd.2020.76.

26. 	Silva, N.R.; Rodrigues, T.O.; Braga, T.E.N.; Christoforo, A.L.; Silva, D.A.L. Re-Refining of Lubricant Oil Used and Contaminated (LOUC) or Its Combustion in Cement Plants? An Exploratory Study in Brazil Based on the Life Cycle Assessment and Circularity Indicators. Sustainable Production and Consumption 2022, 33, 360–371.

27. 	Syu, F.-S.; Vasudevan, A.; Despeisse, M.; Chari, A.; Bekar, E.T.; Gonçalves, M.M.; Estrela, M.A. Usability and Usefulness of Circularity Indicators for Manufacturing Performance Management. Procedia CIRP 2022, 105, 835–840.

28. 	Vadoudi, K.; Deckers, P.; Demuytere, C.; Askanian, H.; Verney, V. Comparing a Material Circularity Indicator to Life Cycle Assessment: The Case of a Three-Layer Plastic Packaging. Sustainable production and consumption 2022, 33, 820–830.

29. 	Vural Gursel, I.; Elbersen, B.; Meesters, K.P.H. Monitoring Circular Biobased Economy – Systematic Review of Circularity Indicators at the Micro Level. Resources, Conservation and Recycling 2023, 197, 107104, doi:10.1016/j.resconrec.2023.107104.

30. 	Wiedemann, S.G.; Nguyen, Q.V.; Clarke, S.J. Using LCA and Circularity Indicators to Measure the Sustainability of Textiles—Examples of Renewable and Non-Renewable Fibres. Sustainability 2022, 14, 16683.

31. 	Huysman, S.; De Schaepmeester, J.; Ragaert, K.; Dewulf, J.; De Meester, S. Performance Indicators for a Circular Economy: A Case Study on Post-Industrial Plastic Waste. Resources, Conservation & Recycling 2017, 120, 46–54.

32. 	Bobba, S.; Eynard, U.; Maury, T.; Ardente, F.; Blengini, G.A.; Mathieux, F. Circular Input Rate: Novel Indicator to Assess Circularity Performances of Materials in a Sector – Application to Rare Earth Elements in e-Vehicles Motors. Resources, Conservation and Recycling 2023, 197, 107037, doi:10.1016/j.resconrec.2023.107037.

33. 	Lee, H.M.; Lu, W.F.; Song, B. A Framework for Assessing Product End-Of-Life Performance: Reviewing the State of the Art and Proposing an Innovative Approach Using an End-of-Life Index. Journal of Cleaner production 2014, 66, 355–371.

34. 	Pollard, J.; Osmani, M.; Cole, C.; Grubnic, S.; Colwill, J.; Díaz, A.I. Developing and Applying Circularity Indicators for the Electrical and Electronic Sector: A Product Lifecycle Approach. Sustainability 2022, 14, 1154, doi:10.3390/su14031154.

35. 	Kowalski, Z.; Kulczycka, J.; Makara, A.; Verhé, R.; De Clercq, G. Assessment of Energy Recovery from Municipal Waste Management Systems Using Circular Economy Quality Indicators. Energies 2022, 15, 8625, doi:10.3390/en15228625.

36. 	Kowalski, Z.; Kulczycka, J.; Banach, M.; Makara, A. A Complex Circular-Economy Quality Indicator for Assessing Production Systems at the Micro Level. Sustainability 2023, 15, 13495, doi:10.3390/su151813495.

37. 	Mesa, J.; Esparragoza, I.; Maury, H. Developing a Set of Sustainability Indicators for Product Families Based on the Circular Economy Model. Journal of cleaner production 2018, 196, 1429–1442.

38. 	Kravchenko, M.; Pigosso, D.CA.; McAloone, T.C. Towards the Ex-Ante Sustainability Screening of Circular Economy Initiatives in Manufacturing Companies: Consolidation of Leading Sustainability-Related Performance Indicators. Journal of Cleaner Production 2019, 241, 118318, doi:10.1016/j.jclepro.2019.118318.

39. 	Das, S.K.; Yedlarajiah, P.; Narendra, R. An Approach for Estimating the End-of-Life Product Disassembly Effort and Cost. International Journal of Production Research 2000, 38, 657–673, doi:10.1080/002075400189356.

40. 	Rossi, E.; Bertassini, A.C.; Ferreira, C. dos S.; Neves do Amaral, W.A.; Ometto, A.R. Circular Economy Indicators for Organizations Considering Sustainability and Business Models: Plastic, Textile and Electro-Electronic Cases. Journal of Cleaner Production 2020, 247, 119137, doi:10.1016/j.jclepro.2019.119137.

41. 	Park, J.Y.; Chertow, M.R. Establishing and Testing the “Reuse Potential” Indicator for Managing Wastes as Resources. Journal of Environmental Management 2014, 137, 45–53, doi:10.1016/j.jenvman.2013.11.053.

42. 	Saidani, M.; Kim, H. Design for Product Circularity: Circular Economy Indicators With Tools Mapped Along the Engineering Design Process. In Proceedings of the ASME 2021 International Design Engineering Technical Conferences and Computers and Information in Engineering Conference; American Society of Mechanical Engineers: Virtual, United States, December 2021.

43. 	Tanguay, X.; Essoua Essoua, G.G.; Amor, B. Attributional and Consequential Life Cycle Assessments in a Circular Economy with Integration of a Quality Indicator: A Case Study of Cascading Wood Products. Journal of Industrial Ecology 2021, 25, 1462–1473, doi:10.1111/jiec.13167.

44. 	Di Maio, F.; Rem, P.C.; Baldé, K.; Polder, M. Measuring Resource Efficiency and Circular Economy: A Market Value Approach. Resources, Conservation and Recycling 2017, 122, 163–171, doi:10.1016/j.resconrec.2017.02.009.

45. 	Bracquené, E.; Lindemann, J.; Duflou, J. Implementation of Circularity Indicators in a Household Product Manufacturing Company. Procedia CIRP 2022, 105, 660–665, doi:10.1016/j.procir.2022.02.110.

46. 	Broeren, M.L.M.; Zijp, M.C.; Waaijers-van der Loop, S.L.; Heugens, E.H.W.; Posthuma, L.; Worrell, E.; Shen, L. Environmental Assessment of Bio-Based Chemicals in Early-Stage Development: A Review of Methods and Indicators. Biofuels, Bioproducts and Biorefining 2017, 11, 701–718, doi:10.1002/bbb.1772.

47. 	Espinoza, L.A.T.; Soulier, M. Defining Regional Recycling Indicators for Metals. Resources, Conservation and Recycling 2018, 129, 120–128.

48. 	Glöser, S.; Soulier, M.; Tercero Espinoza, L.A. Dynamic Analysis of Global Copper Flows. Global Stocks, Postconsumer Material Flows, Recycling Indicators, and Uncertainty Evaluation. Environ. Sci. Technol. 2013, 47, 6564–6572, doi:10.1021/es400069b.

49. 	Saidani, M.; Kendall, A.; Yannou, B.; Leroy, Y.; Cluzel, F. Closing the Loop on Platinum from Catalytic Converters: Contributions from Material Flow Analysis and Circularity Indicators. Journal of Industrial Ecology 2019, doi:10.1111/jiec.12852.

50. 	Lou, E.; Mativenga, P.T. What Should Be Recycled: An Integrated Model for Product Recycling Desirability. Journal of cleaner production 2017, 154, 51–60.

51. 	Smirlis, Y.; Bonazountas, M. LCA/LCC. 2022.

52. 	Carvalho, M.L.; Cusenza, M.A.; Mela, G.; Temporelli, A.; Quinzeni, I.; Girardi, P. LCA and C-LCC Indicator as Tools for Sodium-Ion Batteries’ Eco-Design. Energies 2023, 16, 6220, doi:10.3390/en16176220.

53. 	Scipioni, A.; Niero, M.; Mazzi, A.; Manzardo, A.; Piubello, S. Significance of the Use of Non-Renewable Fossil CED as Proxy Indicator for Screening LCA in the Beverage Packaging Sector. Int J Life Cycle Assess 2013, 18, 673–682, doi:10.1007/s11367-012-0484-x.

54. 	Vesce, E.; Olivieri, G.; Pairotti, M.B.; Romani, A.; Beltramo, R. Life Cycle Assessment as a Tool to Integrate Environmental Indicators in Food Products: A Chocolate LCA Case Study. International Journal of Environment and Health 2016, 8, 21–37, doi:10.1504/IJENVH.2016.077660.

55. 	Filleti, R.A.P.; Silva, D.A.L.; Silva, E.J. da; Ometto, A.R. Productive and Environmental Performance Indicators Analysis by a Combined LCA Hybrid Model and Real-Time Manufacturing Process Monitoring: A Grinding Unit Process Application. Journal of Cleaner Production 2017, 161, 510–523, doi:10.1016/j.jclepro.2017.05.158.

56. 	Namdari, M.; Rafiee, S.; Notarnicola, B.; Tassielli, G.; Renzulli, P.A.; Hosseinpour, S. Use of LCA Indicators to Assess Iranian Sugar Production Systems: Case Study — Hamadan Province. Biomass Conv. Bioref. 2024, 14, 6759–6772, doi:10.1007/s13399-022-02982-4.

57. 	Elkhaldi, I.; Roziere, E.; Turcry, P.; Loukili, A. Towards Global Indicator of Durability Performance and Carbon Footprint of Clinker-Slag-Limestone Cement-Based Concrete Exposed to Carbonation. Journal of Cleaner Production 2022, 380, 134876, doi:10.1016/j.jclepro.2022.134876.

58. 	Hussain, M.; Naseem Malik, R.; Taylor, A. Carbon Footprint as an Environmental Sustainability Indicator for the Particleboard Produced in Pakistan. Environmental Research 2017, 155, 385–393, doi:10.1016/j.envres.2017.02.024.

59. 	Rugani, B.; Vázquez-Rowe, I.; Benedetto, G.; Benetto, E. A Comprehensive Review of Carbon Footprint Analysis as an Extended Environmental Indicator in the Wine Sector. Journal of Cleaner Production 2013, 54, 61–77, doi:10.1016/j.jclepro.2013.04.036.

60. 	Pešta, J.; Šerešová, M.; Kočí, V. Carbon Footprint Assessment of Construction Waste Packaging Using the Package-to-Product Indicator. Sustainability 2020, 12, 10094, doi:10.3390/su122310094.

61. 	Šerešová, M.; Kočí, V. Proposal of Package-to-Product Indicator for Carbon Footprint Assessment with Focus on the Czech Republic. Sustainability 2020, 12, 3034, doi:10.3390/su12073034.

62. 	Tintelecan, A.; Dobra, A.C.; Marţiş, C. LCA Indicators in Electric Vehicles Environmental Impact Assessment. In Proceedings of the 2019 Electric Vehicles International Conference (EV); July 2019; pp. 1–5.

63. 	Lehn, F.; Goossens, Y.; Schmidt, T. Economic and Environmental Assessment of Food Waste Reduction Measures – Trialing a Time-Temperature Indicator on Salmon in HelloFresh Meal Boxes. Journal of Cleaner Production 2023, 392, 136183, doi:10.1016/j.jclepro.2023.136183.

64. 	Stefanini, R.; Borghesi, G.; Ronzano, A.; Vignali, G. Plastic or Glass: A New Environmental Assessment with a Marine Litter Indicator for the Comparison of Pasteurized Milk Bottles. Int J Life Cycle Assess 2021, 26, 767–784, doi:10.1007/s11367-020-01804-x.

65. 	Hanssen, O.J.; Olsen, A.; Møller, H.; Rubach, S. National Indicators for Material Efficiency and Waste Minimization for the Norwegian Packaging Sector 1995–2001. Resources, Conservation and Recycling 2003, 38, 123–137, doi:10.1016/S0921-3449(02)00104-0.

66. 	Pak, S.-C.; O, N.-C.; Ri, R.-J.; Ro, J.-S.; Ri, P.-C. Applicability of Carbon Footprint as Indicator for Environmental Performance of Food Products. Int J Environ Res 2023, 18, 5, doi:10.1007/s41742-023-00553-7.

67. 	Zhang, C.; Chen, W.-Q.; Ruth, M. Measuring Material Efficiency: A Review of the Historical Evolution of Indicators, Methodologies and Findings. Resources, Conservation and Recycling 2018, 132, 79–92, doi:10.1016/j.resconrec.2018.01.028.

68. 	Vandermeulen, B.; Dewulf, W.; Duflou, J.; Ander, Å.; Zimmermann, T. The Use of Performance Indicators for Environmental Assessment within the Railway Business: The RAVEL Workbench Prototype, a Web-Based Tool. Journal of Cleaner Production 2003, 11, 779–785, doi:10.1016/S0959-6526(02)00149-X.

69. 	Bortoluzzi, M.; Correia de Souza, C.; Furlan, M. Bibliometric Analysis of Renewable Energy Types Using Key Performance Indicators and Multicriteria Decision Models. Renewable and Sustainable Energy Reviews 2021, 143, 110958, doi:10.1016/j.rser.2021.110958.

70. 	Adhitya, A.; Halim, I.; Srinivasan, R. Decision Support for Green Supply Chain Operations by Integrating Dynamic Simulation and LCA Indicators: Diaper Case Study. Environ. Sci. Technol. 2011, 45, 10178–10185, doi:10.1021/es201763q.

71. 	Lamnatou, Chr.; Cristofari, C.; Chemisana, D.; Canaletti, J.L. Payback Times and Multiple Midpoint/Endpoint Impact Categories about Building-Integrated Solar Thermal (BIST) Collectors. Science of The Total Environment 2019, 658, 1039–1055, doi:10.1016/j.scitotenv.2018.12.110.

72. 	Kuo, T.C.; Huang, M.-L.; Hsu, C.W.; Lin, C.J.; Hsieh, C.-C.; Chu, C.-H. Application of Data Quality Indicator of Carbon Footprint and Water Footprint. Int. J. of Precis. Eng. and Manuf.-Green Tech. 2015, 2, 43–50, doi:10.1007/s40684-015-0006-y.

73. 	Galatti, L.G.; Baruque-Ramos, J. Circular Economy Indicators for Measuring Social Innovation in the Brazilian Textile and Fashion Industry. Journal of Cleaner Production 2022, 363, 132485.

74. 	Riesen, T.; Morgan, R.L.; Griffin, C. Customized Employment: A Review Of&nbsp;The&nbsp;Literature. Journal of Vocational Rehabilitation 2015, 43, 183–193, doi:10.3233/JVR-150768.

75. 	Ahmad, S.; Wong, K.Y.; Rajoo, S. Sustainability Indicators for Manufacturing Sectors: A Literature Survey and Maturity Analysis from the Triple-Bottom Line Perspective. Journal of Manufacturing Technology Management 2019, 30, 312–334.

76. 	Sutherland, J.W.; Richter, J.S.; Hutchins, M.J.; Dornfeld, D.; Dzombak, R.; Mangold, J.; Robinson, S.; Hauschild, M.Z.; Bonou, A.; Schönsleben, P.; et al. The Role of Manufacturing in Affecting the Social Dimension of Sustainability. CIRP Annals 2016, 65, 689–712, doi:10.1016/j.cirp.2016.05.003.

77. 	Pitkänen, K.; Karppinen, T.K.M.; Kautto, P.; Pirtonen, H.; Salmenperä, H.; Savolahti, H.; Schubin, E.; Myllymaa, T. How to Measure the Social Sustainability of the Circular Economy? Developing and Piloting Social Circular Economy Indicators in Finland. Journal of Cleaner Production 2023, 392, 136238.

78. 	Ahmad, S.; Wong, K.Y. Development of Weighted Triple-Bottom Line Sustainability Indicators for the Malaysian Food Manufacturing Industry Using the Delphi Method. Journal of Cleaner Production 2019, 229, 1167–1182, doi:10.1016/j.jclepro.2019.04.399.

79. 	Bhanot, N.; Qaiser, F.H.; Alkahtani, M.; Rehman, A.U. An Integrated Decision-Making Approach for Cause-And-Effect Analysis of Sustainable Manufacturing Indicators. Sustainability 2020, 12, 1517, doi:10.3390/su12041517.

80. 	Suhányi, L.; Suhányiová, A.; Kádárová, J.; Janeková, J. Relationships between Average Wages in the Manufacturing Sector and Economic Indicators of the Manufacturing Sector in the Region of Visegrad Group Countries. Sustainability 2023, 15, 4164, doi:10.3390/su15054164.

81. 	Gregori, F.; Papetti, A.; Pandolfi, M.; Peruzzini, M.; Germani, M. Digital Manufacturing Systems: A Framework to Improve Social Sustainability of a Production Site. Procedia CIRP 2017, 63, 436–442.

82. 	Lin, C.J.; Belis, T.T.; Kuo, T.C. Ergonomics-Based Factors or Criteria for the Evaluation of Sustainable Product Manufacturing. Sustainability 2019, 11, 4955, doi:10.3390/su11184955.

83. 	Dammert, A.C.; de Hoop, J.; Mvukiyehe, E.; Rosati, F.C. Effects of Public Policy on Child Labor: Current Knowledge, Gaps, and Implications for Program Design. World Development 2018, 110, 104–123, doi:10.1016/j.worlddev.2018.05.001.

84. 	Padilla-Rivera, A.; do Carmo, B.B.T.; Arcese, G.; Merveille, N. Social Circular Economy Indicators: Selection through Fuzzy Delphi Method. Sustainable Production and Consumption 2021, 26, 101–110.

85. 	Husgafvel, R.; Pajunen, N.; Päällysaho, M.; Paavola, I.-L.; Inkinen, V.; Heiskanen, K.; Dahl, O.; Ekroos, A. Social Metrics in the Process Industry: Background, Theory and Development Work. International Journal of Sustainable Engineering 2014, 7, 171–182, doi:10.1080/19397038.2013.800166.

86. 	Husgafvel, R.; Pajunen, N.; Virtanen, K.; Paavola, I.-L.; Päällysaho, M.; Inkinen, V.; Heiskanen, K.; Dahl, O.; Ekroos, A. Social Sustainability Performance Indicators – Experiences from Process Industry. International Journal of Sustainable Engineering 2015, 8, 14–25, doi:10.1080/19397038.2014.898711.

87. 	Päällysaho, M.; Pajunen, N.; Husgafvel, R.; Wierink, M.; Paavola, I.L.; Heiskanen, K.; Dahl, O.; Ekroos, A. Metrics for Sustainable Production in Process Industry. In Proceedings of the 26th International Mineral Processing Congress IMPC 2012 New Delhi, India. 24.-28.9. 2012: Innovative Processing for Sustainable Growth-Conference Proceedings; 2012; pp. 4033–4046.

88. 	Jilcha, K.; Kitaw, D. Lean Influence on Occupational Safety and Health in Manufacturing Industries. Global Journal of Research in Engineering 2016, 16.

89. 	Abikenova, S.; Daumova, G.; Kurmanbayeva, A.; Yesbenbetova, Z.; Kazbekova, D. Relationship Between Occupational Risk and Personal Protective Equipment on the Example of Ferroalloy Production. International Journal of Safety & Security Engineering 2022, 12.

90. 	Odii, E.C.; Ebido, C.C.; Harder, M.K. A Values-Based Approach for Generating Localized Social Indicators for Use in Sustainability Assessment and Decision-Making: Test Case of Brownfield Soft Reuse in Nigeria. Science of The Total Environment 2020, 711, 135045, doi:10.1016/j.scitotenv.2019.135045.

91. 	Hutchins, M.J.; Sutherland, J.W. An Exploration of Measures of Social Sustainability and Their Application to Supply Chain Decisions. Journal of Cleaner Production 2008, 16, 1688–1698, doi:10.1016/j.jclepro.2008.06.001.