Supplementary information

Table S1: Physicochemical parameters of measured samples

| SAMPLE ID | LATITUDE | LONGITUDE | DEPTH(m) | Physicochemical parameters |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | pH | EC | alkalinity | TDS |
| S-1 | 11.73136111 | 79.76913889 | 210 | 6.53 | 604 | 3.73 | 405 |
| S-2 | 11.81352778 | 79.74630556 | 180 | 7.09 | 390 | 2.87 | 261 |
| S-3 | 11.82252778 | 79.69541667 | 45 | 7.23 | 1202 | 4.92 | 805 |
| S-4 | 11.82175 | 79.68377778 | 45 | 7.03 | 1561 | 7.79 | 1046 |
| S-5 | 11.77805556 | 79.77477778 | 240 | 6.3 | 313 | 3.47 | 210 |
| S-6 | 11.9443 | 79.7813 | 90 | 6.51 | 672 | 1.06 | 450 |
| S-7 | 11.98158333 | 79.78519444 | 75 | 5.56 | 198 | 0.90 | 133 |
| S-8 | 11.99013889 | 79.77769444 | 80 | 7.18 | 1076 | 5.29 | 721 |
| S-9 | 11.98088889 | 79.73663889 | 180 | 7.35 | 2235 | 1.90 | 1497 |
| S-10 | 12.00447222 | 79.70191667 | 168 | 7.57 | 598 | 4.39 | 401 |
| S-11 | 11.97441667 | 79.65530556 | 152 | 7.69 | 1339 | 5.33 | 897 |
| S-12 | 11.9566 | 79.6606 | 28 | 7.45 | 1142 | 5.30 | 765 |
| S-13 | 12.01783333 | 79.79775 | 200 | 7.44 | 1179 | 5.30 | 790 |
| S-14 | 12.01522222 | 79.85638889 | 183 | 6.99 | 798 | 2.44 | 535 |
| S-15 | 12.062 | 79.75852778 | 152 | 7.62 | 747 | 5.29 | 500 |
| S-16 | 12.05272222 | 79.77372222 | 80 | 7.71 | 739 | 5.08 | 495 |
| S-17 | 12.05305556 | 79.775 | 90 | 7 | 745 | 5.57 | 499 |
| S-18 | 12.04438889 | 79.8345 | 150 | 7.2 | 512 | 4.32 | 343 |
| S-19 | 11.93083333 | 79.62075 | 180 | 6.98 | 1885 | 4.18 | 1263 |
| S-20 | 11.93044444 | 79.62127778 | 37 | 7.17 | 1983 | 2.40 | 1329 |
| S-21 | 11.9017903 | 79.72374 | 13 | 8.3 | 763 | 2.90 | 511 |
| S-22 | 11.8916209 | 79.8019576 | 29.1 | 8.8 | 922 | 1.73 | 618 |
| S-23 | 11.9237257 | 79.631371 | 31 | 6.59 | 737 | 1.75 | 494 |

Table S2: Major ion data of measured samples

| Sample ID | Na+ (mg/L) | K+(mg/L) | Ca2+(mg/L) | Mg2+(mg/L) | F-(mg/L) | Cl-(mg/L) | NO3-(mg/L) | SO42-(mg/L) | HCO3-(mg/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-1 | 98.5 | 8.4 | 76.2 | 31.6 | 0.16 | 55.27 | 0.74 | 41.45 | 455 |
| S-2 | 60.6 | 5.9 | 51.8 | 24 | 0.37 | 20.77 | 0.87 | 23.06 | 350.6 |
| S-3 | 180 | 2.72 | 87.2 | 72.8 | 0.34 | 250.32 | 7.5 | 50.22 | 600 |
| S-4 | 190.6 | 3.98 | 200.2 | 97 | 0.3 | 252.62 | 64.94 | 34.14 | 950.6 |
| S-5 | 61.1 | 9 | 71.3 | 29 | 0.14 | 34.71 | 0.76 | 6.11 | 423.26 |
| S-6 | 100.5 | 14.5 | 86.2 | 35 | 0.05 | 250.78 | 15.41 | 77.19 | 129 |
| S-7 | 34.5 | 2.74 | 22.5 | 9.7 | 0.79 | 47.01 | 4.17 | 3.26 | 110 |
| S-8 | 115.12 | 0.72 | 99.2 | 124.24 | 0.62 | 245.76 | 8 | 29.78 | 645.4 |
| S-9 | 134.6 | 6.7 | 64 | 31.6 | 0.3 | 248.15 | 0 | 0.59 | 231.8 |
| S-10 | 20 | 11.17 | 81.6 | 90.5 | 0.36 | 84.44 | 2.26 | 12.7 | 535.4 |
| S-11 | 350 | 11.4 | 54.6 | 38.2 | 0.32 | 190 | 3.8 | 150.08 | 650 |
| S-12 | 218.4 | 9.2 | 87.2 | 100.2 | 0.36 | 295.48 | 2.82 | 76.38 | 646.6 |
| S-13 | 206 | 8.84 | 60.2 | 105.4 | 0.84 | 249.42 | 3.38 | 55.42 | 646.6 |
| S-14 | 102.8 | 2.8 | 85.6 | 53 | 0.62 | 233.21 | 20.86 | 10.97 | 298.2 |
| S-15 | 92.7 | 8.78 | 60 | 100 | 0.43 | 100 | 2.94 | 30 | 645 |
| S-16 | 90 | 5.9 | 60 | 100 | 0.68 | 98 | 5.6 | 38 | 620 |
| S-17 | 100 | 6 | 72 | 100 | 0.56 | 100 | 9.28 | 40 | 680 |
| S-18 | 69.12 | 69.44 | 58 | 44.7 | 0.39 | 69.37 | 4.73 | 8.68 | 527 |
| S-19 | 304 | 5.8 | 128.6 | 133.2 | 0.38 | 589.82 | 19.28 | 133.24 | 510 |
| S-20 | 283.6 | 6.1 | 88 | 33 | 0.31 | 443.12 | 24.6 | 0.65 | 292.8 |
| S-21 | 164 | 2.484 | 8.4 | 26.3 | 0.48 | 96.6 | 1.1 | 19.2 | 353.8 |
| S-22 | 96.38 | 4.094 | 31.8 | 21 | 0.28 | 108 | 1.6 | 29.6 | 210.45 |
| S-23 | 75 | 5.3 | 59.5 | 13.4 | 0.17 | 76.2 | 5.1 | 72.1 | 213.5 |

Table S3: Heavy metal data of measured samples

| Sample ID | Mn (µg/L) | Cu (µg/L) | Zn (µg/L) | Li (µg/L) | Co (µg/L) | Pb (µg/L) | Ni (µg/L) | Cr (µg/L) | Fe (µg/L) | U (µg/L) | Rn (Bq/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-1 | 62.0 | 16.6 | 40.0 | 2.00 | 1.100 | 0.11 | 1.20 | 1.55 | 2800 | 0.06 | 0.05 |
| S-2 | 4.50 | 1.30 | 42.0 | 0.40 | 0.055 | 0.12 | 0.20 | 1.56 | 200 | 0.07 | 0.15 |
| S-3 | 59.0 | 7.00 | 57.0 | 0.40 | 4.700 | 0.11 | 4.00 | 1.54 | 200 | 2.19 | 0.32 |
| S-4 | 1.10 | 6.20 | 187 | 0.70 | 0.056 | 0.13 | 0.70 | 1.52 | 200 | 2.70 | 0.86 |
| S-5 | 30.0 | 3.40 | 73.3 | 0.40 | 0.055 | 0.11 | 0.20 | 4.72 | 400 | 0.93 | 1.08 |
| S-6 | 15.4 | 3.20 | 120 | 2.20 | 0.056 | 0.14 | 1.40 | 1.55 | 200 | 0.21 | 0.07 |
| S-7 | 9.40 | 0.20 | 54.0 | 6.00 | 0.057 | 0.15 | 0.20 | 1.54 | 200 | 0.05 | 0.45 |
| S-8 | 0.60 | 3.60 | 109 | 5.60 | 0.054 | 0.11 | 3.60 | 1.52 | 200 | 0.89 | 1.16 |
| S-9 | 0.55 | Bdl | 11.5 | 0.10 | 0.52 | 0.12 | 0.20 | 1.52 | 0.5 | 9.36 | 1.71 |
| S-10 | 23.6 | 0.30 | 110 | 8.00 | 0.056 | 0.13 | 0.30 | 1.51 | 500 | 0.12 | 2.57 |
| S-11 | 99.0 | 11.7 | 113 | 38.5 | 0.052 | 0.15 | 11.7 | 1.57 | 200 | 0.15 | 3.41 |
| S-12 | 200 | 3.20 | 94.0 | 18.4 | 0.053 | 0.15 | 3.20 | 1.53 | 200 | 2.22 | 4.19 |
| S-13 | 10.0 | 6.40 | 123 | 24.0 | 0.054 | 0.12 | 6.40 | 1.52 | 200 | 10.40 | 0.94 |
| S-14 | 2.10 | 1.50 | 37.2 | 5.00 | 1.000 | 0.13 | 1.50 | 1.54 | 200 | 0.12 | 0.21 |
| S-15 | 0.10 | 2.10 | 27.3 | 21.6 | 0.052 | 0.11 | 2.10 | 1.58 | 200 | 1.89 | 2.73 |
| S-16 | 6.70 | 1.10 | 97.6 | 17.3 | 0.053 | 0.16 | 1.10 | 1.59 | 200 | 0.67 | 3.63 |
| S-17 | 181 | 1.00 | 65.0 | 2.00 | 0.055 | 0.12 | 1.00 | 1.52 | 200 | 0.83 | 2.95 |
| S-18 | 409 | 8.90 | 134 | 10.0 | 0.800 | 0.11 | 9.00 | 1.58 | 300 | 0.10 | 0.19 |
| S-19 | 8.20 | 9.00 | 40.8 | 3.40 | 0.055 | 0.13 | 9.00 | 1.56 | 200 | 0.14 | 0.08 |
| S-20 | 0.37 | 0.12 | 0.06 | 0.70 | 0.052 | 0.004 | 0.20 | 0.52 | 1255 | 4.51 | 0.06 |
| S-21 | 1.66 | 0.10 | 0.79 | 0.09 | 0.053 | 0.004 | 0.20 | 0.51 | 814.9 | 2.23 | 0.98 |
| S-22 | 0.67 | 0.13 | 0.39 | 0.12 | 0.056 | 0.3 | 0.22 | 0.53 | 3538 | 0.50 | 1.24 |
| S-23 | 0.24 | 0.08 | 0.63 | 2.48 | 0.058 | 0.002 | 0.23 | 0.55 | 798.9 | 3.76 | 0.07 |

Table S4: Calculation of irrigation water quality parameters

| Parameter | Formula |
| --- | --- |
| Sodium Absorption Ratio (SAR) | SAR= Na+Ca2++Mg2+2 |
| Sodium Percent (Na%) | Na%=Na++K+Ca2++Mg2++Na++K+×100 |
| Residual Sodium Carbonate (RSC) | RSC=HCO3-+CO32-- (Ca2++Mg2+) |
| Magnesium Hazard (MH) | MR= Mg2+Ca2++Mg2+×100 |
| Permeability Index (PI) | PI=Na++HCO3-Ca2++Mg2++Na+×100 |
| Kelley’s Ratio (KR) | KR=Na+Ca2++Mg2+ |
| Potential Salinity (PS) | PS= Cl-+12SO42- |
| Soluble Sodium Percentage (SSP) | SSP= Na+Ca2++Mg2++Na+ |

S-1. Calculation of Drinking and irrigation water quality index:

Here each relevant parameter is assigned with a weightage (wn) and a quality rating (qn)

If there is n number of parameters contributing toward quality, then each of them is assigned a quality rating (qn)

qn=Vn-ViVs-Vi×100 										(1)

Vn= Observed value of the parameter

Vi=Ideal value of the parameter

Vs=Standard value of the parameter

Vi for most parameters are 0 except pH. For pH Vi=7.0

The unit weight of nth parameter is defined as,

Wn= kSn										(2)

Sn = maximum permissible limit for that parameter

k = proportionality constant defined as  k=1Sn

Finally, WQI is calculated as

WQI= 1nqnWnWn   									 (3)

The weightage of drinking and irrigation quality parameters are given in (Table S5 and S6).

Table S5: Maximum permissible limit according to WHO 2018 and respective weight factors

| Parameter | Sn | Wn |
| --- | --- | --- |
| pH | 8.5 | 0.141 |
| EC | 1500 | 0.001 |
| TDS | 1000 | 0.001 |
| Ca | 300 | 0.004 |
| Mg | 100 | 0.012 |
| Cl | 250 | 0.005 |
| SO4 | 250 | 0.005 |
| NO3 | 50 | 0.024 |
| Fluoride | 1.5 | 0.802 |
| Total Alkalinity | 600 | 0.002 |
| TH | 500 | 0.002 |

Table S6: maximum permissible limit of the parameters, and weight factors.

| Parameter | Sn | wn |
| --- | --- | --- |
| SAR | 18 | 0.039 |
| Na% | 60 | 0.790 |
| RSC | 2.5 | 0.032 |
| MH | 50 | 0.109 |
| EC | 2000 | 0.001 |
| PI | 75 | 0.026 |
| PS | 15 | 0.1 |
| SSP | 80 | 0.02 |

S-2. Annual effective dose from NORM (Rn & U):

Annual effective dose due to ingestion is calculated by equation 4.

Ding=Cw×IR×DCF 						(4)

Where Cw is the concentration of dissolved radioactive elements expressed as Bq/L. IR is ingestion rate, typically 730 L/yr (considering 2L water consumption per day). DCF is dose conversion factor through ingestion rout. DCF values depends on radionuclides.

As radon (222Rn) is a gaseous element and can enter human body by inhalation along with ingestion, annual effective dose via inhalation rout is also calculated following equation 5.

Dinh=Cw×AWR×OCF×EF×DCF 				(5)

Here, AWR is air to water concentration ratio given by 10-4, OFC is indoor occupancy factor given by 7000 h/yr, EF is indoor radon-progeny factor typically given as 0.4.

For radon, DCF via inhalation route is 9×10-9 Sv/Bq.

Total annual effective dose is calculated as summation of all annual effective doses through inhalation and ingestion routes.

S-3. Calculation of HPI and HEI:

HPI values are calculated following equation 6.

HPI=i=1nQiWii=1nWi							(6)

Here Qi is the sub-index of ith metal and wi is unit weightage given to ith metal. n is the number of heavy/trace metals under consideration. Qi is calculated following equation 7.

Qi=[Ci(-)li]Si-li×100							(7)

Where, Ci is measured concentration of heavy/trace metals and li and Si is ideal and standard values of ith metal in groundwater.

A simpler index, Heavy metal evaluation index (HEI) is also used to understand suitability of groundwater for specific uses [30]. HEI is calculated following equation 8.

HEI=i=1nMiMMAC							(8)

Here Mi and MMAC are measured and maximum allowable concentrations of heavy metals in groundwater.

Table S7: RfD vales of heavy metals

| Metal | RfD (mg/kg/day) |
| --- | --- |
| Chromium | 3×10-3 |
| Copper | 4×10-2 |
| Manganese | 1.4×10-1 |
| Nickel | 2×10-2 |
| Lead | 3.5×10-3 |
| Lithium | 2×10-3 |
| Zinc | 0.3 |
| Iron | 0.7 |

Table S8: Classification of Children, Female and Male group based on body weight, ingestion rate and exposure duration.

| Parameter | Unit | children |  | Female |  | Male |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mean | Std. deviation | Mean | Std. deviation | Mean | Std. deviation |
| Body weight (BW) | kg | 6.00 | 1.43 | 51.2 | 11.8 | 57.6 | 11.7 |
| Ingestion rate (IR) | L/day | 0.25 | 0.100 | 3.03 | 1.11 | 3.95 | 0.790 |
| Exposure Duration (ED) | years | 6.00 | 2.070 | 72.1 | 12.4 | 68.3 | 11.2 |

S-4. Steps involved in Morris method:

The target variable (ILCR or THQ) and the input variables (contaminant concentration, body weight, water intake, etc.) are related by an unknown function (equation 9),

Y= φ X 										(9)

Where,

Y: Target variable (ILCR or THQ)

X: Input variable vector

φ: Unknown function

In the Morris method, the sensitivity is calculated as an elementary effect (EEj) of each input variable in vector (Xj) on the target variable (Y) (equation 10).

EEj=φ Xj+δj ,  Xi- φ Xj ,  Xiδj 								(10)

δj is a fixed step size for each parameter, defined as a multiple of δj = p/2(p-1), and p is the number of equally spaced values for each input parameter.

Each input parameter is changed by δj at a time to calculate the corresponding elementary effect, which is separately performed for each variable. Finally, the impact of the elementary effects for each input parameters is calculated as equation 11.

μ=r=1REEXrR 									(11)

Where, R is number of resampling done from entire input space. Higher the value of µ, higher will be the influence of that input parameter on the final output. The relative sensitivity of the target variable to the input variables may thereafter be estimated.

Table S9: Classification of DWQI

| DWQI | Category | Percentage of samples in particular category |
| --- | --- | --- |
| 0 – 25 | Excellent | 52% |
| 26 –50 | Good | 43% |
| 51 –75 | Bad | 5% |
| 76 – 100 | Very bad | Nil |
| Above 100 | Unfit | Nil |

Table S10: Classification of IWQI

| IWQI | Class | Restriction for usage in irrigation | Percentage of samples falling in particular class |
| --- | --- | --- | --- |
| <150 | Ⅰ | No restriction | 95% |
| 150 – 300 | Ⅱ | Slight restriction | 5% |
| 300 – 450 | Ⅲ | Moderate restriction | Nil |
| >450 | Ⅳ | Severe restriction | Nil |

Table S11: Total variance of components

| Component | Eigen value | % variance | Cumulative % |
| --- | --- | --- | --- |
| PC-1 | 3.6 | 24.1 | 24.1 |
| PC-2 | 3.5 | 23.5 | 48.2 |
| PC-3 | 2.0 | 13.5 | 61.7 |
| PC-4 | 1.7 | 11.4 | 73.1 |
| PC-5 | 1.2 | 8.0 | 81.2 |