A Systematic Literature Review of Digital Twin Applications for Sustainable Water Systems in Natural and Built Environment

Mehwish Qureshi, School of Built Environment, Massey University 0745, Auckland New Zealand (m.qureshi@massey.ac.nz)

# Appendix A — Summary of Included Studies: Water Systems, Motivations, Tools, Stakeholders, Challenges, and DT Implementation Tier

This appendix presents a consolidated summary of the 48 studies included in the systematic literature review. For each study, the table identifies the water system examined, the motivations for DT adoption, the tools and technologies employed, the stakeholders benefiting from the application, challenges and barriers reported, and the DT implementation tier assigned during full-text screening. Tier colour coding: Fully Operational | Transitional | Conceptual / DT-Enabling

| No. | Authors (Year) | Title | Water System Studied | Motivations for DT Adoption | Components & Tools | Stakeholders | Challenges & Barriers | DT Implementation Tier |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | (Wang et al., 2022) | Design and Application of Digital Twin Platform Based Smart Weihe River Basin | Weihe River Basin | WRB faces water resource shortage, soil/water loss, water pollution and river silation | BIM, GIS, IoT, VR/AR, UAV technology | Ministry of Water Resources, River Basin Management Agencies, provincial and local governments | - | Transitional |
| 2 | (Henriksen et al., 2023) | A New Digital Twin for Climate Change Adaptation, Water Management, and Disaster Risk Reduction (HIP Digital Twin) | Groundwater Table | High groundwater levels in urban areas causing pollutant mobilisation, infrastructure damage; drought frequency increasing; 100% groundwater-based drinking supply in Denmark | HIP DK model (MIKE SHE), ML Gradient Boosting, ML Random Forest, GIS | Municipalities, water companies, consultancies, regional/state authorities, farmers, landowners, citizens | High-quality data from diverse sources required; clear goals and implementation plan needed; complex national-scale undertaking | Transitional |
| 3 | (Remigi et al., 2022) | Practical Application of a Model-Based Digital Twin for Monitoring and Scenario Analysis of a Water Resource Recovery Facility | Water Resource Recovery Facility (Bresso-Niguarda, Italy) | Improve operational efficiency, reduce energy consumption, increase biogas recovery, optimise carbon footprint | SCADA (60 sensors, 18 energy meters), MIKE Operation | Gruppo CAP utility company (Metropolitan City of Milan) | - | Fully Operational |
| 4 | (Fargas & Cornellà, 2023) | A Digital Twin for Large Diameter Water Distribution Pipes | Water Distribution System (Tarragona, Spain) | Catastrophic failure of main pipeline led to DT implementation for structural monitoring | Inline Soundprint Acoustic Fibre Optics (AFO), GIS | Tarragona Water Consortium (direct); municipalities and industries (indirect) | - | Transitional |
| 5 | (Conejos et al., 2020) | Building and Exploiting a Digital Twin for the Management of Drinking Water Distribution Networks | Water Distribution Network (Valencia, Spain - 1.6M inhabitants) | Population growth, resource scarcity, infrastructure wear; need for asset management, leak localisation, energy efficiency | GIS, AMR, CMMS, SCADA, EPANET | - | 1. Algorithm development to fit GIS to model; 2. Demand calculation algorithm complexity; 3. Filtering/correcting raw SCADA data; 4. Limitations of EPANET elements | Fully Operational |
| 6 | (Bellandi et al., 2022) | Ozonation Digital Twin for Predicting Micropollutants Removal at the Wervershoof (NL) WWTP | Wervershoof Waste Water Treatment Plant (Netherlands) | Real-time prediction of micropollutants and bromate formation in ozonation process | AMOZONE Model, Computational Fluid Dynamics (CFD) | Local Water Authority (HHNK), Drinking Water Company (PWN) | - | Transitional |
| 7 | (Habibian et al., 2023) | Marching Towards a Digital Twin: How GLWA Built a 3D Model of Its WRRF Complex Underground Utilities | Water Resource Recovery Facility (Detroit, USA) | Support ongoing asset management program; nine decades of infrastructure requiring comprehensive digital record | Georeferenced 3D and 2D models, GIS-based InfoWater, Autodesk Plant 3D, CCTV | Facility owners, operators, and maintenance staff | Over 200 years of accumulated infrastructure; data integration across legacy systems | Transitional |
| 8 | (Payne et al., 2022) | Machine Learning for Surrogate Groundwater Modelling of a Small Carbonate Island | Groundwater Resources (Barbados) | Need for low-cost, computationally efficient groundwater modelling for small island states | FEFLOW, Deep Neural Networks (DNNs), Elastic-Net Regression, GANs, Python (Keras, Scikit-learn), DEM | Stakeholders defined generically (water managers, planners) | Framework proposed for small island; real-world deployment not confirmed; data scarcity challenges | Conceptual / DT-Enabling |
| 9 | (Chen, 2020) | In-Depth Informatization of Technology Management of Dateng Gorge Water Conservancy Project | Flood Control - Dateng Gorge Water Conservancy Project | Inadequate information integration of technology management processes | BIM Collaborated Management Platform, cloud operation centre, mobile office system, digital certificate CA | - | - | Transitional |
| 10 | (Melo et al., 2019) | Implementation of City Information Modeling (CIM) Concepts in the Process of Management of the Sewage System in Piumhi, Brazil | Sewage System (Piumhi, Brazil) | Develop a management model for sewage infrastructure using CIM concepts | GIS, Civil 3D, QGIS | - | Difficulties registering the existing network during the survey; data quality issues | Conceptual / DT-Enabling |
| 11 | (Tomić et al., 2022) | Digital Twins: Case Studies in Water Distribution Management | Water Distribution System (Las Vegas & California, USA) | Ageing infrastructure, climate change, tighter resources, increasing regulatory and customer demands | Mobile sensors, SCADA, GIS, enterprise databases, cloud-based platform, Qatium | - | Lakewood City (CA) system limited by 80,000 customer connections with constrained data granularity | Fully Operational |
| 12 | (Yang et al., 2021) | Building Information Model and Optimization Algorithms for Supporting Campus Facility Maintenance Management | Water Dispenser Maintenance (Academic Campus, Taiwan) | Provide support for facility management (FM) of water dispensers not originally designed with BIM | BIM, COBie data export, digital meters, FORTRAN, Python | FM staff and water dispenser maintenance contractors | Campus buildings not originally designed with BIM; retrofitting data model required | Transitional |
| 13 | (Marzouk & Othman, 2017) | Modelling the Performance of Sustainable Sanitation Systems Using Building Information Modelling | El-Saf Anaerobic Sewage Treatment Plant (El-Giza, Egypt) | Address challenges facing sanitation infrastructure performance assessment | Green Building Studio (BPM), BIM, Computer Simulation, Google Maps | - |  | Conceptual / DT-Enabling |
| 14 | (Ahmed & Marzouk, 2019) | BIM-Based Facility Management for Water Treatment Plants Using Laser Scanning | Sixth of October Water Treatment Plant (Egypt) | Paper-based facility management; lack of digital records in Egyptian treatment plants | 3D Laser Scanning (Z+F), Autodesk Recap 360, ArcGIS | - | Absence of procedures for refreshing the designed model over time | Conceptual / DT-Enabling |
| 15 | (Huang et al., 2023) | River Shoreline Project Management Based on BIM Technology | Green Water Wetland - Nanjing Reach, Yangtze River (China) | Need for fast, efficient communication and project management using BIM and GIS | BIM, GIS, UAV-based tilt photography, Autodesk RECAP, Context Capture, Civil 3D, Revit | - | - | Conceptual / DT-Enabling |
| 16 | (Park & You, 2023) | A Digital Twin Dam and Watershed Management Platform | Sumjin Dam and River Basin (South Korea) | Climate change and global warming increasing flood risk; need for real-time dam and watershed management | K-Twin SJ Platform, LiDAR, DEM, DSM, XDWorld Engine, AI flood prediction, GeoStudio, FLAC, AI-CCTV (YOLOv4, YOLACT) | - | - | Fully Operational |
| 17 | (Ruangpan et al., 2023) | Real Time Control of Nature-Based Solutions: Towards Smart Solutions and Digital Twins | Irrigation and Drainage System - Rangsit Area (Thailand) | Need for real-time control of nature-based solutions (NBS) for urban flood management | MIKE Hydro River, Telemetry, SCADA, AutoCal | - | Simulation of NBS for furrows of Klong 7 and Klong 8 channels; real-time control integration | Transitional |
| 18 | (Qiu et al., 2022) | Design and Development of a Web-Based Interactive Twin Platform for Watershed Management | Watershed Environment - Chaohu Lake (China) | Use Virtual Geographic Environments (VGEs) for integrated watershed management and decision support | DEM, Virtual Graphic Environment (VGE), Cull Rendering (CR), web-based interface | Integrated Watershed Management (IWM) stakeholders | 3D modelling of water surfaces; data integration complexity; VGE rendering performance | Transitional |
| 19 | (Ramos. et al., 2023) | Smart Water Grids and Digital Twin for the Management of System Efficiency in Water Distribution Networks | Water Distribution Network (Madeira Island, Portugal) | Population growth and under-investment by water utilities driving DT adoption | GIS, EPANET, hydraulic modelling, sensors, SCADA, smart metering, CMMS | Water consumers and utility operators (if implemented) | DT not yet applied practically; challenges remain theoretical at time of publication | Transitional |
| 20 | (Sinagra et al., 2023) | Energy Recovery Optimization by Means of a Turbine in a Pressure Regulation Node | Water Distribution Network -Montescuro Ovest Pipeline (Sicily, Italy) | Optimise energy recovery through turbine installation in pressure regulation node | EPANET 2.2, SCADA, MATLAB 2022b | - | - | Transitional |
| 21 | (Bonilla et al., 2022) | A Digital Twin of a Water Distribution System by Using Graph Convolutional Networks | Water Distribution System (Colombia / C-Town benchmark) | Failure in monitoring systems, remote control, cyber attacks requiring robust state estimation | T-GCN model (Keras 2.6, TensorFlow, Spektral), EPANET 2.2 | - | - | Transitional |
| 22 | (Ramos et al., 2024) | Are Digital Twins Improving Urban-Water Systems Efficiency and Sustainable Development Goals? | Water Distribution Network - Santa Cruz de Madeira (Portugal) | Population increase and limited investment in water infrastructure | GIS, EPANET, hydraulic modelling tools | Water consumers and utility managers | - | Transitional |
| 23 | (Sun et al., 2020) | Real-Time Control of Urban Water Cycle Under Cyber-Physical Systems Framework | Urban Water Cycle - Barcelona and Badalona (Spain) | Potential of CPS to generate real-time water cycle management and reduce operational costs | MATLAB/SIMULINK, TOMLAB, AMR, SCADA, GAMS, sensors | - | - | Transitional |
| 24 | (Laucelli et al., 2023) | A Complete Digital Water Experience to Support Real Leakage Management Planning | Water Distribution System (Southern Italy) | Novel processes of data integration needed for leakage management and pressure control | WDNetXL, EPANET, QGIS with custom plug-ins (DigitalWater suite) | Utility operators and network engineers | - | Transitional |
| 25 | (Lu et al., 2023) | Virtual Scene Construction of Wetlands: A Case Study of Poyang Lake, China | Wetland System - Poyang Lake (Jiangxi Province, China) | Lack of studies on construction of virtual wetland scenes; need for immersive monitoring | Unreal Engine (UE), UAVs with RTK GPS, ZENMUSE L1 cameras, DEM, Autodesk 3ds Max, Substance Designer, VR | - | Limited data collection access to vast wetland areas; rendering and computational performance | Conceptual / DT-Enabling |
| 26 | (Shafiee et al., 2020) | Streaming Smart Meter Data Integration to Enable Dynamic Demand Assignment for Real-Time Hydraulic Simulation | Water Distribution System (USA) | - | GIS, EPANET, sensors, smart meters, Hadoop-based distributed data centre | - | - | Transitional |
| 27 | (Batista et al., 2023) | BIM-IoT-FM Integration: Strategy for Implementation of Sustainable Water Management in Buildings | Water Management System — Central Market of Belo Horizonte (Brazil) | Need for integrated real-time water management in large commercial buildings | AquaBIM prototype platform, BIM, IoT, Autodesk Forge | Building facility managers and operators | - | Conceptual / DT-Enabling |
| 28 | (Green et al., 2025) | A Framework for Incorporating Rainfall Data into a Flooding Digital Twin | Stormwater / Flooding - Newcastle upon Tyne (UK) | - | SHETRAN, HiPIMS, Radar, gauge networks, PYRAMID platform | - | - | Transitional |
| 29 | (Namaki et al., 2025) | A Novel Digital Twin Framework for Improving Urban Sustainability Feasibility Assessment of Supplementary Water Supply Systems in Buildings | Residential Building Water System - Northern Tehran (Iran) | High cost, time constraints, and data scarcity hindering water supply feasibility assessment | Discrete Event Simulation (DES), BIM | Decision-makers and building developers | Data scarcity in non-developed regions; limited real-world deployment | Conceptual / DT-Enabling |
| 30 | (Zekri et al., 2022) | Smart Water Management Using Intelligent Digital Twins | Campus Water System - Sultan Qaboos University (Oman) | Fragmentation and lack of interoperability in current water management systems | CSAD loop (Inform-Control-Simulate-Advise-Deliver), GAMA platform | Asset managers and facility operators | Technical difficulty of integrating diverse data sources | Transitional |
| 31 | (Feng et al., 2024) | An Effective Digital Twin Modeling Method for Infrastructure: Application to Smart Pumping Stations | Pumping Stations - Beijing East-West Water Transfer Project (China) | Lack of established DT definitions and modelling standards for water infrastructure | Digital photogrammetry, UAVs, IFC, COBie, sensors, actuators | Facility operators (achieved RMB 2.25 million in cost savings) | Technical difficulty of integrating heterogeneous data | Transitional |
| 32 | (Ntousakis et al., 2025) | Optimizing an Urban Water Infrastructure Through a Smart Water Network Management System | Water Distribution Network - Heraklion (Greece) | High safety risks and financial losses from non-revenue water (NRW) | QGIS, EPANET Trace, Automatic Meter-reading, SmartLIK platform | Site managers, safety officers, utility operators | Multi-source data integration difficulty | Transitional |
| 33 | (Rajan & Li, 2024) | Smart Building Digital Twin for Interior Water Distribution System Management | Building Water Distribution System - Toronto Metropolitan University (Canada) | Water scarcity concerns in the region; need for smart building water monitoring | BIM (IFC), GIS (ArcGIS Pro), IoT sensors | - | None of the systems were fully connected at time of publication; integration remains incomplete | Transitional |
| 34 | (Shrivastava et al., 2024) | River Digital Twin for Water Quality Prediction | Ganga River (India) | Support water quality management and pollution monitoring efforts | MESA framework in Python, sensors | - | Simulation-based; real-world deployment challenges not quantified | Transitional |
| 35 | (Roudbari et al., 2024) | From Data to Action in Flood Forecasting Leveraging Graph Neural Networks and Digital Twin Visualization | Stormwater / Flood Forecasting - Terrebonne, Quebec (Canada) | Hydrology community seeking to predict water flow under structural uncertainty | QGIS, HRDEM, Unreal Engine (UE), Graph Neural Networks | Policy makers | Difficulty accurately modelling water flow under structural conditions | Transitional |
| 36 | (Singh & Sharma, 2025) | Proposing a Digital Twin-Based Sustainable Water Governance System for Rural Indian Villages | Groundwater - Rural Villages (Maharashtra, India) | Traditional water management inadequate for rural communities; governance gaps | QGIS, IoT sensors, Empathy maps (Voice of Stakeholders) | Local community members | Significant investment required; digital literacy barriers in rural context | Conceptual / DT-Enabling |
| 37 | (Yang et al., 2025) | High-Precision Optimization of BIM-3D GIS Models for Digital Twins: A Case Study of Santun River Basin | Santun River Basin (Xinjiang, China) | Need for high-precision BIM-GIS integration for river basin DT development | BIM-3D, GIS, AW-QEM, LOD framework, IFC, OSGB, Unity with C# programming | - | - | Conceptual / DT-Enabling |
| 38 | (Primeau et al., 2024) | Towards a Digital Twin for a Drinking Water Source in Norway | Drinking Water Source - Brusdalsvatnet (Norway) | Contamination of drinking water by runoffs and stormwater requiring monitoring | Python-based back-end, IoT sensors, Vertical Profiler System (YSI), 4G datalink | - | - | Transitional |
| 39 | (Daneshgar et al., 2024) | A Full-Scale Operational Digital Twin for a Water Resource Recovery Facility | Water Resource Recovery Facility - Eindhoven (Netherlands) | Transition from mechanistic to data-driven operational decision-making at full scale | WEST (DHI), Python-based pipeline (wwdata and wwtwin), Microsoft Azure, sensors, SCADA | Operators, process engineers, utility managers | Complex multi-level data integration; achieving full real-time synchronisation | Fully Operational |
| 40 | (Ghaith et al., 2022) | Synchronization-Enhanced Deep Learning Early Flood Risk Predictions | Stormwater / Urban Flooding - Calgary (Canada) | Monetary and life losses due to urban flooding | HEC-RAS, rainfall sensors, Bayesian Model Averaging (BMA), CNNs, LSTMs | Hydrologists and urban planners | - | Transitional |
| 41 | (Tarpanelli et al., 2023) | Flooding in the Digital Twin Earth: The Case Study of the Enza River Levee Breach | Enza River Basin - Northern Italy | Investigate 2017 levee breach and improve flood event simulation | WEC-Flood (2D model), Sentinel-1/2, DEM, remote sensing | - | Temporal misalignment between satellite overpasses and flood event timing | Transitional |
| 42 | (Xu et al., 2024) | Design of a 3D Platform for the Evaluation of Water Quality in Urban Rivers Based on a Digital Twin Model | Chuancheng River and Baihe River - Nanyang City (China) | Improve informatisation and intelligent management of urban river water quality | Kriging interpolation, ANSYS Fluent, AutoCad, SpaceClaim, GIS, ArcGIS, SketchUp, Python, MySQL, JavaScript | - | Technical difficulties establishing large environmental models | Transitional |
| 43 | (Cohen-Manrique et al., 2025) | Simulation and Optimisation Using a Digital Twin for Resilience-Based Management of Confined Aquifers | Morroa Aquifer (Colombia) | -- | 5-layered DT architecture, ESP32 nodes, LoRa connectivity, Sentinel-2, MODIS, Planet, GRACE data | - | Significant project and funding challenges during implementation | Transitional |
| 44 | (Pedersen et al., 2022) | Using Multi-Event Hydrologic and Hydraulic Signatures from Water Level Sensors in Living Digital Twins | Urban Drainage System - Odense (Denmark) | Address inherent limitations of urban drainage models using real sensor data | Water level sensors, hydraulic pipe-flow module, hydrologic surface model (general terms) | - | Water level measurements often non-unique indicators of model uncertainty | Transitional |
| 45 | (Aslam, Shu, Naz, et al., 2024) | Machine Learning-Based Wetland Vulnerability Assessment in the Sindh Province Ramsar Site | Khinjhir Lake Wetland (Pakistan) | Multifaceted vulnerabilities of Ramsar-listed wetland requiring assessment | Landsat imagery, NDWI, MNDWI, NDVI, GIS spatial analysis, ANN, CA-Markov, CA-ANN modelling | Policy makers and local community | - | Conceptual / DT-Enabling |
| 46 | (Aslam, Shu, Javid, et al., 2024) | Wetland Identification through Remote Sensing: Insights into Wetness, Greenness, Turbidity, Temperature | Wetlands - Gilgit-Baltistan (Pakistan) | Identify neglected wetlands; understand ecological change through multi-sensor analysis | Sentinel-2, MODIS, QuickBird, ASTER DEM, supervised classification, multi-sensor data fusion | Policy makers and environmental stakeholders | - | Conceptual / DT-Enabling |
| 47 | (Aslam et al., 2023) | Identification of Time-Varying Wetlands Neglected in Pakistan through Remote Sensing Techniques | Wetlands -Pakistan (multiple lakes) | Use satellite imagery to locate and monitor neglected/uncharted wetlands | Satellite imagery (Sentinel-2, Sentinel-1, MODIS, QuickBird), spectral indices (MNDWI, GDVI, NDVI, SAVI, NDTI, TWI) | - | - | Conceptual / DT-Enabling |
| 48 | (Cavalieri & Gambadoro, 2024) | Digital Twin of a Water Supply System Using the Asset Administration Shell | Water Supply System - Piemonte (Italy, ~60,000 residents) | Need for Industry 4.0-based DT framework using AAS standard for water supply infrastructure | Asset Administration Shell (AAS), AASX Package Explorer, IEC 61360, RAMI 4.0, Profinet, PLC, RTU, IoT sensors, SCADA | Water utility operators and asset managers | - | Conceptual / DT-Enabling |

Note: 'DT Implementation Tier' classification was applied during full-text screening based on functional characteristics described in each study, independent of authors' terminology. Studies using DT terminology were not automatically classified as Fully Operational unless functional criteria were clearly met. '—' indicates information not explicitly reported in the source study.

Ahmed, R., & Marzouk, M. (2019). BIM-Based Facility Management for Water Treatment Plants using Laser Scanning. Water Practice and Technology, 14(2), 325-330. https://doi.org/10.2166/wpt.2019.016

Aslam, R. W., Shu, H., Javid, K., Pervaiz, S., Mustafa, F., Raza, D., Ahmed, B., Quddoos, A., Al-Ahmadi, S., & Hatamleh, W. A. (2024). Wetland Identification through Remote Sensing: Insights into Wetness, Greenness, Turbidity, Temperature, and Changing Landscapes. Big Data Research, 35, 100416. https://doi.org/https://doi.org/10.1016/j.bdr.2023.100416

Aslam, R. W., Shu, H., Naz, I., Quddoos, A., Yaseen, A., Gulshad, K., & Alarifi, S. S. (2024). Machine Learning-Based Wetland Vulnerability Assessment in the Sindh Province Ramsar Site using Remote Sensing Data. Remote Sensing, 16(5), 928. https://doi.org/https://doi.org/10.3390/rs16050928

Aslam, R. W., Shu, H., Yaseen, A., Sajjad, A., & Abidin, S. Z. U. (2023). Identification of Time-Varying Wetlands Neglected in Pakistan through Remote Sensing Techniques. Environmental Science and Pollution Research, 30(29), 74031-74044. https://doi.org/https://doi.org/10.1007/s11356-023-27554-5

Batista, L. T., Franco, J. R. Q., Fakury, R. H., Porto, M. F., Alves, L. V. R., & Kohlmann, G. S. (2023). BIM-IoT-FM Integration: Strategy for Implementation of Sustainable Water Management in Buildings. Smart and Sustainable Built Environment, 13(5), 1096-1116. https://doi.org/10.1108/sasbe-11-2022-0250

Bellandi, G., Muoio, R., Hoekstra, M., Daza, M., Duchi, S., Spruijt, M., Versteegh, J., Rehman, U., & Audenaert, W. (2022). Ozonation Digital Twin for Predicting Micropollutants Removal at the Wervershoof (NL) WWTP, Pilot and Demonstration-Scale Testing. 95th Water Environment Federation Technical Exhibition and Conference, WEFTEC 2022,

Bonilla, C. A., Zanfei, A., Brentan, B., Montalvo, I., & Izquierdo, J. (2022). A Digital Twin of a Water Distribution System by Using Graph Convolutional Networks for Pump Speed-Based State Estimation. Water (Switzerland), 14(4), Article 514. https://doi.org/10.3390/w14040514

Cavalieri, S., & Gambadoro, S. (2024). Digital Twin of a Water Supply System Using the Asset Administration Shell. Sensors (Basel), 24(5), Article 1360. https://doi.org/10.3390/s24051360

Chen, X. M. (2020). In-depth Informatization of Technology Management of Dateng Gorge Water Conservancy Project. Proceedings - 2020 International Conference on Computer Science and Management Technology, ICCSMT 2020,

Cohen-Manrique, C. S., Villa-Ramirez, J. L., Camacho-Leon, S., Solano-Correa, Y. T., Alvarez-Month, A. A., & Coronado-Hernandez, O. E. (2025). Simulation and Optimisation Using a Digital Twin for Resilience-Based Management of Confined Aquifers. Water (Switzerland), 17(13), Article 1973. https://doi.org/10.3390/w17131973

Conejos, F. P., Martínez Alzamora, F., Hervás Carot, M., & Alonso Campos, J. C. (2020). Building and Exploiting a Digital Twin for the Management of Drinking Water Distribution Networks. Urban Water Journal, 17(8), 704-713. https://doi.org/10.1080/1573062X.2020.1771382

Daneshgar, S., Polesel, F., Borzooei, S., Sorensen, H. R., Peeters, R., Weijers, S., Nopens, I., & Torfs, E. (2024). A full-scale operational digital twin for a water resource recovery facility-A case study of Eindhoven Water Resource Recovery Facility. Water Environ Res, 96(3), e11016, Article e11016. https://doi.org/10.1002/wer.11016

Fargas, M., A., & Cornellà, G., J. (2023). A Digital Twin for Large Diameter Water Distribution Pipes. Pipelines 2023: Condition Assessment, Utility Engineering, Surveying, and Multidiscipline - Proceedings of Sessions of the Pipelines 2023 Conference,

Feng, F., Liu, Z., Shi, G., & Mo, Y. (2024). An Effective Digital Twin Modeling Method for Infrastructure: Application to Smart Pumping Stations. Buildings, 14(4), Article 863. https://doi.org/10.3390/buildings14040863

Ghaith, M., Yosri, A., & El-Dakhakhni, W. (2022). Synchronization-Enhanced Deep Learning Early Flood Risk Predictions: The Core of Data-Driven City Digital Twins for Climate Resilience Planning. Water (Switzerland), 14(22), Article 3619. https://doi.org/10.3390/w14223619

Green, A. C., Lewis, E., Tong, X., & Wardle, R. (2025). A Framework for Incorporating Rainfall Data into a Flooding Digital Twin. Journal of Hydrology, 656, Article 132893. https://doi.org/10.1016/j.jhydrol.2025.132893

Habibian, A., Broz, J., Sadatiyan, M., Nicolas, N., & Wood, B. (2023). Marching towards a Digital Twin: How GLWA Built a 3D Model of Its WRRF Complex Underground Utilities. Pipelines 2023: Condition Assessment, Utility Engineering, Surveying, and Multidiscipline - Proceedings of Sessions of the Pipelines 2023 Conference,

Henriksen, H. J., Schneider, R., Koch, J., Ondracek, M., Troldborg, L., Seidenfaden, I. K., Kragh, S. J., Bogh, E., & Stisen, S. (2023). A New Digital Twin for Climate Change Adaptation, Water Management, and Disaster Risk Reduction (HIP Digital Twin). Water (Switzerland), 15(1), Article 25. https://doi.org/10.3390/w15010025

Huang, T., Xu, H., Wang, Y., Chen, H., Zhang, L., & Fan, H. (2023). River Shoreline Project Management Based on BIM Technology: A Case Study of the Environmental Improvement Project of the Green Water Wetland in the Nanjing Reach of the Yangtze River. Lecture Notes in Civil Engineering,

Laucelli, D., Spagnuolo, S., Rinaldi, A., Perrone, G., Berardi, L., & Giustolisi, O. (2023). A Complete Digital Water Experience to Support Real Leakage Management Planning. IOP Conference Series: Earth and Environmental Science,

Lu, S., Fang, C., & Xiao, X. (2023). Virtual Scene Construction of Wetlands: A Case Study of Poyang Lake, China. ISPRS International Journal of Geo-Information, 12(2), Article 49. https://doi.org/10.3390/ijgi12020049

Marzouk, M., & Othman, A. (2017). Modeling the Performance of Sustainable Sanitation Systems using Building Information Modeling. Journal of Cleaner Production, 141, 1400-1410. https://doi.org/10.1016/j.jclepro.2016.09.226

Melo, H. C., Tomé, S. M. G., Silva, M. H., Gonzales, M. M., & Gomes, D. B. O. (2019). Implementation of City Information Modeling (CIM) Concepts in the Process of Management of the Sewage System in Piumhi, Brazil. IOP Conference Series: Earth and Environmental Science,

Namaki, P., Shahir, H., Ahani, M., & Alvanchi, A. (2025). A Novel Digital Twin Framework for Improving Urban Sustainability Feasibility Assessment of Supplementary Water Supply Systems in Buildings. Sustainable Cities and Society, 131, Article 106724. https://doi.org/10.1016/j.scs.2025.106724

Ntousakis, E., Loukakis, K., Petrou, E., Ipsakis, D., & Papaefthimiou, S. (2025). Optimizing an Urban Water Infrastructure Through a Smart Water Network Management System. Electronics, 14(12), Article 2455. https://doi.org/10.3390/electronics14122455

Park, D., & You, H. (2023). A Digital Twin Dam and Watershed Management Platform. Water (Switzerland), 15(11), Article 2106. https://doi.org/10.3390/w15112106

Payne, K., Chami, P., Odle, I., Yawson, D. O., Paul, J., Maharaj-Jagdip, A., & Cashman, A. (2022). Machine Learning for Surrogate Groundwater Modelling of a Small Carbonate Island. Hydrology, 10(1), Article 2. https://doi.org/10.3390/hydrology10010002

Pedersen, A. N., Pedersen, J. W., Borup, M., Brink-Kjaer, A., Christiansen, L. E., & Mikkelsen, P. S. (2022). Using Multi-Event Hydrologic and Hydraulic Signatures from Water Level Sensors to Diagnose Locations of Uncertainty in Integrated Urban Drainage Models used in Living Digital Twins. Water Sci Technol, 85(6), 1981-1998. https://doi.org/10.2166/wst.2022.059

Primeau, R., Seidu, R., & Li, G. (2024). Towards a Digital Twin for a Drinking Water Source in Norway. 2024 IEEE 19th Conference on Industrial Electronics and Applications, ICIEA 2024,

Qiu, Y., Duan, H., Xie, H., Ding, X., & Jiao, Y. (2022). Design and Development of a Web‐Based Interactive Twin Platform for Watershed Management. Transactions in GIS, 26(3), 1299-1317. https://doi.org/10.1111/tgis.12904

Rajan, G., & Li, S. (2024). Smart Building Digital Twin for Interior Water Distribution System Management. International Archives of the Photogrammetry, Remote Sensing and Spatial Information Sciences - ISPRS Archives,

Ramos, Helena., Kuriqi, A., Coronado-Hernández, O. E., López-Jiménez, P. A., & Pérez-Sánchez, M. (2024). Are digital twins improving urban-water systems efficiency and sustainable development goals? Urban Water Journal, 21(10), 1164-1175. https://doi.org/10.1080/1573062X.2023.2180396

Ramos., Kuriqi, A., Besharat, M., Creaco, E., Tasca, E., Coronado-Hernández, O. E., Pienika, R., & Iglesias-Rey, P. (2023). Smart Water Grids and Digital Twin for the Management of System Efficiency in Water Distribution Networks. Water (Switzerland), 15(6), Article 1129. https://doi.org/10.3390/w15061129

Remigi, E. U., Polesel, F., Flauto, M., Spinelli, L., Di Cosmo, R., & Muzzatti, M. A. (2022). Practical Application of a Model-Based Digital Twin for Monitoring and Scenario Analysis of a Water Resource Recovery Facility. 95th Water Environment Federation Technical Exhibition and Conference, WEFTEC 2022,

Roudbari, N. S., Punekar, S. R., Patterson, Z., Eicker, U., & Poullis, C. (2024). From Data to Action in Flood Forecasting Leveraging Graph Neural Networks and Digital Twin Visualization. Scientific Reports, 14(1), Article 18571. https://doi.org/10.1038/s41598-024-68857-y

Ruangpan, L., Mahgoub, M., Abebe, Y. A., Vojinovic, Z., Boonya-aroonnet, S., Sanchez Torres, A., & Weesakul, S. (2023). Real time control of nature-based solutions: Towards Smart Solutions and Digital Twins in Rangsit Area, Thailand. Journal of Environmental Management, 344, Article 118389. https://doi.org/10.1016/j.jenvman.2023.118389

Shafiee, M. E., Rasekh, A., Sela, L., & Preis, A. (2020). Streaming Smart Meter Data Integration to Enable Dynamic Demand Assignment for Real-Time Hydraulic Simulation. Journal of Water Resources Planning and Management, 146(6), Article 06020008. https://doi.org/10.1061/(asce)wr.1943-5452.0001221

Shrivastava, S., Barat, S., Kausley, S., Kulkarni, V., & Rai, B. (2024). River Digital Twin for Water Quality Prediction. Proceedings - Winter Simulation Conference,

Sinagra, M., Creaco, E., Morreale, G., & Tucciarelli, T. (2023). Energy Recovery Optimization by Means of a Turbine in a Pressure Regulation Node of a Real Water Network Through a Data-Driven Digital Twin. Water Resources Management, 37(12), 4733-4749. https://doi.org/10.1007/s11269-023-03575-0

Singh, D., & Sharma, V. (2025). Proposing a digital twin-based sustainable water governance system for rural Indian villages. International Journal of Information Technology, 17(3), 1777-1783, Article 103520. https://doi.org/10.1007/s41870-025-02414-2

Sun, C., Puig, V., & Cembrano, G. (2020). Real-Time Control of Urban Water Cycle under Cyber-Physical Systems Framework. Water (Switzerland), 12(2), Article 406. https://doi.org/10.3390/w12020406

Tarpanelli, A., Bonaccorsi, B., Sinagra, M., Domeneghetti, A., Brocca, L., & Barbetta, S. (2023). Flooding in the Digital Twin Earth: The Case Study of the Enza River Levee Breach in December 2017. Water (Switzerland), 15(9), Article 1644. https://doi.org/10.3390/w15091644

Tomić, S., Karmous‐Edwards, G., & Kamojjala, S. (2022). Digital Twins: Case Studies in Water Distribution Management. Journal AWWA, 114(8), 44-56. https://doi.org/10.1002/awwa.1979

Wang, L., Jiang, R., Chen, X., Xie, J., Liu, X., Tian, L., & Wang, M. (2022). Design and Application of Digital Twin Platform Based Smart Weihe River Basin. Proceedings of SPIE - The International Society for Optical Engineering.,

Xu, Y., Hui, M., & Qu, H. (2024). Design of a 3D Platform for the Evaluation of Water Quality in Urban Rivers Based on a Digital Twin Model. Water (Switzerland), 16(24), Article 3668. https://doi.org/10.3390/w16243668

Yang, L.-H., Xu, L., Wang, W.-C., & Wang, S.-H. (2021). Building Information Model and Optimization Algorithms for Supporting Campus Facility Maintenance Management: A Case Study of Maintaining Water Dispensers. KSCE Journal of Civil Engineering, 25(1), 12-27. https://doi.org/10.1007/s12205-020-0219-7

Yang, Z., Aihemaiti, M., Abudureheman, B., & Tao, H. (2025). High-Precision Optimization of BIM-3D GIS Models for Digital Twins: A Case Study of Santun River Basin. Sensors, 25(15), Article 4630. https://doi.org/10.3390/s25154630

Zekri, S., Jabeur, N., & Gharrad, H. (2022). Smart Water Management Using Intelligent Digital Twins. Computing and Informatics, 41(1), 135-153. https://doi.org/10.31577/cai_2022_1_135