Comparative Assessment of Energy Recovery from Landfill Gas and Incineration for Municipal Solid Waste Management in Chennai, India: A Case Study Using the IPCC Default Method and First-Order Decay Model

Madhavaraj Muthukrishnan1 and Karthikeyan Kothandapani2*

1Research Scholar, School of Civil Engineering, Vellore Institute of Technology, Chennai, India,  madhavaraj.m2021@vitstudent.ac.in /ORCID: 0009-0009-8286-5414

2Associate Professor, School of Civil Engineering, Vellore Institute of Technology, Chennai, India, karthikeyan.kothanda@vit.ac.in /ORCID: 0000-0002-4144-5802

*Corresponding Author

### 1. Calculation for the year 1993 (IPCC Default Method – Perungudi dumpsite)

1. Annual MSW disposed

Daily MSW (excluding C&D waste) = 2021.94 tonnes/day

Annual MSW:

MSWT = 2021.94 × 365

= 738,008.10 tonnes/year

= 738.0081 Gg/year

2. Methane estimation using the IPCC Default Method

Using Equation (1),

CH₄ = MSWT × MSWF × MCF × DOC × DOCF × F × (16/12)

For the minimum methane estimate,

CH₄ = 738.0081 × 0.70 × 0.40 × 0.15 × 0.77 × 0.40 × (16/12)

CH₄ = 738.0081 × 0.70 × 0.40 × 0.15 × 0.77 × 0.40 × 1.3333

CH₄ = 12.7292 Gg CH₄/year

For the maximum methane estimate,

CH₄ = 738.0081 × 0.90 × 0.40 × 0.15 × 0.77 × 0.60 × (16/12)

CH₄ = 738.0081 × 0.90 × 0.40 × 0.15 × 0.77 × 0.60 × 1.3333

CH₄ = 24.5491 Gg CH₄/year

3. Average methane emission

Average CH₄ = (Minimum CH₄ + Maximum CH₄) / 2

Average CH₄ = (12.7292 + 24.5491) / 2

Average CH₄ = 18.6391 Gg CH₄/year

Final Results

Annual MSW disposed = 738.0081 Gg/year

Minimum methane emission = 12.7292 Gg CH₄/year

Maximum methane emission = 24.5491 Gg CH₄/year

Average methane emission = 18.6391 Gg CH₄/year

### 2. Sample calculation for the year 1993 (First-Order Decay (FOD) Model – Perungudi dumpsite)

1. Annual MSW disposed

Daily MSW (excluding C&D waste) = 2021.94 tonnes/day

Annual MSW:

MSWT = 2021.94 × 365

= 738,008.10 tonnes/year

= 738.0081 Gg/year

2. Decomposable degradable organic carbon (DDOCm)

Using Equation (2),

DDOCm = W × DOC × DOCF × MCF

For the minimum estimate,

DDOCm = 738.0081 × 0.15 × 0.65 × 0.40

DDOCm = 28.7823 Gg C

For the maximum estimate,

DDOCm = 738.0081 × 0.15 × 0.80 × 0.40

DDOCm = 35.4244 Gg C

3. Accumulated degradable organic carbon (DDOCma)

Using Equation (3),

DDOCma(T) = DDOCm(T) + DDOCma(T−1) × e⁻ᵏ

For the minimum estimate (k = 0.03),

DDOCma(1993) = 28.7823 + (83.4415 × 0.9704455)

DDOCma(1993) = 109.7577 Gg C

For the maximum estimate (k = 0.10),

DDOCma(1993) = 35.4244 + (96.0551 × 0.9048374)

DDOCma(1993) = 122.3387 Gg C

4. Decomposed degradable organic carbon (DDOCmdecomp)

Using Equation (4),

DDOCmdecomp = DDOCma × (1 − e⁻ᵏ)

For the minimum estimate,

DDOCmdecomp = 109.7577 × (1 − 0.9704455)

DDOCmdecomp = 109.7577 × 0.0295545

DDOCmdecomp = 2.4661 Gg C

For the maximum estimate,

DDOCmdecomp = 122.3387 × (1 − 0.9048374)

DDOCmdecomp = 122.3387 × 0.0951626

DDOCmdecomp = 9.1409 Gg C

5. Methane estimation

Using Equation (5),

CH₄ = DDOCmdecomp × F × (16/12)

For the minimum estimate,

CH₄ = 2.4661 × 0.40 × (16/12)

CH₄ = 1.3152 Gg CH₄/year

For the maximum estimate,

CH₄ = 9.1409 × 0.60 × (16/12)

CH₄ = 7.3127 Gg CH₄/year

6. Average methane emission

Average CH₄ = (Minimum CH₄ + Maximum CH₄) / 2

Average CH₄ = (1.3152 + 7.3127) / 2

Average CH₄ = 4.3140 Gg CH₄/year

Final Results

Annual MSW disposed = 738.0081 Gg/year

Minimum DDOCm = 28.7823 Gg C

Maximum DDOCm = 35.4244 Gg C

Minimum DDOCma = 109.7577 Gg C

Maximum DDOCma = 122.3387 Gg C

Minimum DDOCmdecomp = 2.4661 Gg C

Maximum DDOCmdecomp = 9.1409 Gg C

Minimum methane emission = 1.3152 Gg CH₄/year

Maximum methane emission = 7.3127 Gg CH₄/year

Average methane emission = 4.3140 Gg CH₄/year

### 3. Sample calculation for the year 2026 (Landfill Gas-to-Energy – Perungudi dumpsite)

1. Methane generation

Methane generated using the IPCC Default Method (2026):

CH₄ = 15.2126 Gg/year

2. Conversion of methane to mass

Methane generated:

CH₄ = 15.2126 Gg/year

= 15.2126 × 1000

= 15,212.57 tonnes/year

3. Avoided methane emissions

Using Equation (12),

GHGAvoided,LF = CH₄ × 28 × 0.90

Substituting the values,

GHGAvoided,LF = 15,212.57 × 28 × 0.90

GHGAvoided,LF = 383,356.75 tCO₂e/year

4. Annual electricity generation

Using Equation (7),

Annual electricity generated from landfill gas

= 98,783.25 MWh/year

= 98,783.25 × 1000

= 98,783,247.30 kWh/year

5. Avoided emissions due to electricity generation

Using the Tamil Nadu grid emission factor of 0.75 kg CO₂e/kWh,

GHGAvoided,WtE = Electricity generated × Grid emission factor

GHGAvoided,WtE = 98,783,247.30 × 0.75

GHGAvoided,WtE = 74,087,435.47 kg CO₂e/year

= 74,087.44 tCO₂e/year

6. Total avoided greenhouse gas emissions

GHGAvoided,Total = GHGAvoided,LF + GHGAvoided,WtE

GHGAvoided,Total = 383,356.75 + 74,087.44

GHGAvoided,Total = 457,444.18 tCO₂e/year

7. Carbon credit revenue

Using a carbon price of 7 USD/tCO₂e,

Carbon Revenue = GHGAvoided,Total × Carbon price

Carbon Revenue = 457,444.18 × 7

Carbon Revenue = 3,202,109.27 USD/year

Final Results

Methane generated = 15.2126 Gg/year

Annual electricity generation = 98,783.25 MWh/year

Avoided methane emissions = 383,356.75 tCO₂e/year

Avoided emissions from electricity generation = 74,087.44 tCO₂e/year

Total avoided greenhouse gas emissions = 457,444.18 tCO₂e/year

Carbon credit revenue = 3,202,109.27 USD/year

### 4. Sample calculation for the year 2026 (Incineration Energy Recovery – Perungudi dumpsite)

1. Annual MSW available for incineration

Daily MSW (excluding C&D waste) = 2200.31 tonnes/day

Annual MSW:

MSW = 2200.31 × 365

= 803,113.15 tonnes/year

2. Thermal energy available

Using the lower calorific value (LCV) of MSW,

LCV = 5709 kJ/kg

Thermal Energy = MSW × 1000 × LCV

Thermal Energy = 803,113.15 × 1000 × 5709

Thermal Energy = 4.58497 × 10¹² kJ/year

3. Electrical power output

Using Equation (10),

PINC = (MSW × 1000 × LCV × ηINC) / (365 × 24 × 60 × 60)

where,

ηINC = 0.25

Substituting the values,

PINC = (803,113.15 × 1000 × 5709 × 0.25) / 31,536,000

PINC = 13,266,619.30 W

= 13,266.62 kW

4. Annual electricity generation

Using Equation (11),

EINC = PINC × Capacity factor × Operating hours

where,

Capacity factor = 0.85

Operating hours = 8760 h/year

Substituting the values,

EINC = 13,266.62 × 0.85 × 8760 / 1000

EINC = 98,783.25 MWh/year

Final Results

Annual MSW treated = 803,113.15 tonnes/year

Lower calorific value = 5709 kJ/kg

Electrical power output = 13,266,619.30 W

Electrical power output = 13,266.62 kW

Annual electricity generation = 98,783.25 MWh/year

s1.Temporal Methane Generation Trends Estimated by DM and FOD Models at Kodungaiyur Landfill

| Year | Default Method |  |  | First Order Decay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Minimum CH₄ emission (Gg/year) in kodungaiyur | Maximum CH₄ emission (Gg/year) in kodungaiyur | Average CH₄ emission (Gg/year) in kodungaiyur | Minimum CH₄ emission (Gg/year) in kodungaiyur | Maximum CH₄ emission (Gg/year) in kodungaiyur | Average CH₄ emission (Gg/year) in kodungaiyur |
| 1990 | 12.31919945 | 23.75845608 | 18.03882776 |  |  |  |
| 1991 | 12.35653188 | 23.83045434 | 18.09349311 | 0.439066398 | 2.610004068 | 1.524535233 |
| 1992 | 12.39386431 | 23.9024526 | 18.14815846 | 0.866486981 | 4.979542836 | 2.923014908 |
| 1993 | 12.43119675 | 23.97445087 | 18.20282381 | 1.282605936 | 7.131499603 | 4.207052769 |
| 1994 | 12.46846623 | 24.04632772 | 18.25739697 | 1.687757275 | 9.086580035 | 5.387168655 |
| 1995 | 12.50579866 | 24.11832599 | 18.31206232 | 2.082262898 | 10.86350605 | 6.472884476 |
| 1996 | 12.54313109 | 24.19032425 | 18.36672767 | 2.466439676 | 12.47924463 | 7.472842154 |
| 1997 | 12.58046353 | 24.26232252 | 18.42139302 | 2.840592873 | 13.94913478 | 8.394863827 |
| 1998 | 12.61779596 | 24.33432078 | 18.47605837 | 3.205018731 | 15.28705581 | 9.246037272 |
| 1999 | 12.65512839 | 24.40631904 | 18.53072372 | 3.560004735 | 16.50556625 | 10.03278549 |
| 2000 | 12.69246083 | 24.47831731 | 18.58538907 | 3.905829876 | 17.61602952 | 10.7609297 |
| 2001 | 12.60703062 | 24.31355905 | 18.46029484 | 4.242764899 | 18.62872766 | 11.43574628 |
| 2002 | 12.6400821 | 24.37730119 | 18.50869165 | 4.566697183 | 19.52695519 | 12.04682619 |
| 2003 | 12.67262994 | 24.44007203 | 18.55635098 | 4.882233804 | 20.3467075 | 12.61447065 |
| 2004 | 12.70511482 | 24.50272144 | 18.60391813 | 5.18960494 | 21.09534581 | 13.14247538 |
| 2005 | 12.73804039 | 24.56622076 | 18.65213057 | 5.489049674 | 21.77962417 | 13.63433692 |
| 2006 | 12.77102892 | 24.62984148 | 18.7004352 | 5.780817973 | 22.4057606 | 14.09328928 |
| 2007 | 12.80414335 | 24.69370504 | 18.74892419 | 6.065138954 | 22.97930137 | 14.52222016 |
| 2008 | 12.83719483 | 24.75744718 | 18.797321 | 6.342237205 | 23.50527831 | 14.92375776 |
| 2009 | 12.87030927 | 24.82131073 | 18.84581 | 6.612323948 | 23.98820436 | 15.30026415 |
| 2010 | 12.90348666 | 24.8852957 | 18.89439118 | 6.875608646 | 24.43218971 | 15.65389918 |
| 2011 | 12.93691587 | 24.94976632 | 18.94334109 | 7.132294576 | 24.84095338 | 15.98662398 |
| 2012 | 12.97040804 | 25.01435835 | 18.99238319 | 7.382585734 | 25.21790052 | 16.30024313 |
| 2013 | 13.0039002 | 25.07895039 | 19.0414253 | 7.626673359 | 25.56607221 | 16.59637278 |
| 2014 | 13.03751828 | 25.14378525 | 19.09065176 | 7.864740792 | 25.88820678 | 16.87647379 |
| 2015 | 13.07113636 | 25.20862011 | 19.13987823 | 8.096970446 | 26.18680869 | 17.14188957 |
| 2016 | 13.10481739 | 25.27357639 | 19.18919689 | 8.323534852 | 26.46411735 | 17.3938261 |
| 2017 | 13.13862433 | 25.33877549 | 19.23869991 | 8.544603688 | 26.72217243 | 17.63338806 |
| 2018 | 13.17249423 | 25.40409601 | 19.28829512 | 8.760343859 | 26.96283282 | 17.86158834 |
| 2019 | 13.20642708 | 25.46953794 | 19.33798251 | 8.970915096 | 27.18776718 | 18.07934114 |
| 2020 | 13.24048584 | 25.5352227 | 19.38785427 | 9.176472407 | 27.39848538 | 18.28747889 |
| 2021 | 13.27372619 | 25.59932908 | 19.43652763 | 9.377168463 | 27.59636695 | 18.48676771 |
| 2022 | 13.30702949 | 25.66355687 | 19.48529318 | 9.573117768 | 27.78246005 | 18.67778891 |
| 2023 | 13.34127712 | 25.72960587 | 19.53544149 | 9.764462853 | 27.95789985 | 18.86118135 |
| 2024 | 13.42815529 | 25.89715664 | 19.66265597 | 9.95137345 | 28.12390021 | 19.03763683 |
| 2025 | 13.46259179 | 25.96356988 | 19.71308083 | 10.13585641 | 28.29250996 | 19.21418319 |
| 2026 | 13.49709124 | 26.03010453 | 19.76359788 | 10.31611443 | 28.45237026 | 19.38424234 |
| 2027 | 13.53146478 | 26.09639636 | 19.81393057 | 10.4922746 | 28.60432706 | 19.54830083 |
| 2028 | 13.56590127 | 26.1628096 | 19.86435543 | 10.66445355 | 28.7491058 | 19.70677967 |
| 2029 | 13.60046368 | 26.22946566 | 19.91496467 | 10.83277119 | 28.88740289 | 19.86008704 |
| 2030 | 13.63508904 | 26.29624314 | 19.96566609 | 10.99734613 | 29.01986184 | 20.00860398 |
| 2031 | 13.66996622 | 26.36350628 | 20.01673625 | 11.15829122 | 29.14705154 | 20.15267138 |
| 2032 | 13.7048434 | 26.43076941 | 20.0678064 | 11.31572271 | 29.26952678 | 20.29262475 |
| 2033 | 13.73984649 | 26.49827537 | 20.11906093 | 11.46974445 | 29.38773621 | 20.42874033 |
| 2034 | 13.77491254 | 26.56590275 | 20.17040764 | 11.6204617 | 29.50211245 | 20.56128707 |
| 2035 | 13.80997858 | 26.63353012 | 20.22175435 | 11.76797436 | 29.61303361 | 20.69050399 |
| 2036 | 13.84504463 | 26.7011575 | 20.27310106 | 11.91237715 | 29.72082848 | 20.81660282 |
| 2037 | 13.88055136 | 26.76963477 | 20.32509307 | 12.05376197 | 29.82579458 | 20.93977828 |
| 2038 | 13.91612105 | 26.83823345 | 20.37717725 | 12.19223373 | 29.92829445 | 21.06026409 |
| 2039 | 13.95219438 | 26.90780345 | 20.42999891 | 12.32788077 | 30.02857614 | 21.17822845 |
| 2040 | 13.97561371 | 26.9529693 | 20.46429151 | 12.46080451 | 30.12695742 | 21.29388097 |
| 2041 | 14.01156113 | 27.02229647 | 20.5169288 | 12.59063444 | 30.22093822 | 21.40578633 |
| 2042 | 14.04757151 | 27.09174505 | 20.56965828 | 12.71790852 | 30.31359156 | 21.51575004 |
| 2043 | 14.08421143 | 27.16240776 | 20.6233096 | 12.84270452 | 30.4050571 | 21.62388081 |
| 2044 | 14.12941327 | 27.24958273 | 20.689498 | 12.96511812 | 30.49558124 | 21.73034968 |
| 2045 | 14.16586433 | 27.3198812 | 20.74287277 | 13.08552487 | 30.58706755 | 21.83629621 |
| 2046 | 14.20237834 | 27.39030109 | 20.79633972 | 13.20367222 | 30.67757048 | 21.94062135 |
| 2047 | 14.23851463 | 27.4599925 | 20.84925356 | 13.31962918 | 30.76719696 | 22.04341307 |
| 2048 | 14.27358068 | 27.52761987 | 20.90060027 | 13.43344701 | 30.85595035 | 22.14469868 |
| 2049 | 14.31116493 | 27.60010379 | 20.95563436 | 13.5451508 | 30.94368699 | 22.2444189 |
| 2050 | 14.34881214 | 27.67270913 | 21.01076063 | 13.65489278 | 31.03103717 | 22.34296498 |

s2.Temporal Methane Generation Trends Estimated by DM and FOD Models at Perungudi

| Year | Default Method |  |  | First Order Decay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Minimum CH₄ emission (Gg/year) in Perungudi | Maximum CH₄ emission (Gg/year) in Perungudi | Average CH₄ emission (Gg/year) in Perungudi | Minimum CH₄ emission (Gg/year) in Perungudi | Maximum CH₄ emission (Gg/year) in Perungudi | Average CH₄ emission (Gg/year) in Perungudi |
| 1990 | 12.64203371 | 24.38106502 | 18.51154936 |  |  |  |
| 1991 | 12.67105606 | 24.43703669 | 18.55404637 | 0.450572476 | 2.678401267 | 1.564486872 |
| 1992 | 12.70014136 | 24.49312977 | 18.59663557 | 0.888862904 | 5.108067767 | 2.998465336 |
| 1993 | 12.72916371 | 24.54910144 | 18.63913257 | 1.315236516 | 7.312683079 | 4.313959798 |
| 1994 | 12.75824901 | 24.60519452 | 18.68172177 | 1.730043264 | 9.313650319 | 5.521846791 |
| 1995 | 12.78733431 | 24.6612876 | 18.72431096 | 2.133627243 | 11.1303625 | 6.631994871 |
| 1996 | 12.81635666 | 24.71725927 | 18.76680797 | 2.526320138 | 12.78035381 | 7.653336973 |
| 1997 | 12.84544196 | 24.77335236 | 18.80939716 | 2.908441584 | 14.2794765 | 8.59395904 |
| 1998 | 12.87446431 | 24.82932403 | 18.85189417 | 3.280306258 | 15.64210095 | 9.461203604 |
| 1999 | 12.90354961 | 24.88541711 | 18.89448336 | 3.642215051 | 16.88120335 | 10.2617092 |
| 2000 | 12.93257196 | 24.94138878 | 18.93698037 | 3.994464447 | 18.00855172 | 11.00150808 |
| 2001 | 12.94440754 | 24.96421454 | 18.95431104 | 4.33733768 | 19.03476752 | 11.6860526 |
| 2002 | 12.97040804 | 25.01435835 | 18.99238319 | 4.670499307 | 19.96583352 | 12.31816641 |
| 2003 | 12.99647149 | 25.06462358 | 19.03054754 | 4.994741199 | 20.81380546 | 12.90427333 |
| 2004 | 13.05709735 | 25.18154488 | 19.11932111 | 5.310329218 | 21.58660413 | 13.44846667 |
| 2005 | 13.08366444 | 25.23278142 | 19.15822293 | 5.618750957 | 22.29870576 | 13.95872836 |
| 2006 | 13.11029449 | 25.28413937 | 19.19721693 | 5.919004329 | 22.9486706 | 14.43383746 |
| 2007 | 13.13692454 | 25.33549733 | 19.23621093 | 6.21133299 | 23.54242507 | 14.87687903 |
| 2008 | 13.19729858 | 25.45193297 | 19.32461577 | 6.49597115 | 24.0853183 | 15.29064473 |
| 2009 | 13.22392863 | 25.50329092 | 19.36360977 | 6.774348761 | 24.58933954 | 15.68184415 |
| 2010 | 13.25062163 | 25.55477029 | 19.40269596 | 7.045448188 | 25.05103879 | 16.04824349 |
| 2011 | 13.27731464 | 25.60624965 | 19.44178214 | 7.309486776 | 25.47445685 | 16.39197181 |
| 2012 | 13.33907369 | 25.7253564 | 19.53221504 | 7.566673205 | 25.86323666 | 16.71495493 |
| 2013 | 13.36576669 | 25.77683576 | 19.57130123 | 7.81845977 | 26.22810375 | 17.02328176 |
| 2014 | 13.3924597 | 25.82831513 | 19.61038741 | 8.063756278 | 26.56390445 | 17.31383036 |
| 2015 | 13.41852315 | 25.87858036 | 19.64855175 | 8.302754539 | 26.87340479 | 17.58807966 |
| 2016 | 13.48235972 | 26.00169375 | 19.74202674 | 8.535618257 | 27.15897421 | 17.84729623 |
| 2017 | 13.50905273 | 26.05317312 | 19.78111292 | 8.763875 | 27.43089282 | 18.09738391 |
| 2018 | 13.53580869 | 26.1047739 | 19.82029129 | 8.986337097 | 27.68259027 | 18.33446368 |
| 2019 | 13.56256465 | 26.15637468 | 19.85946966 | 9.20317805 | 27.91600418 | 18.55959111 |
| 2020 | 13.62721964 | 26.28106644 | 19.95414304 | 9.414563989 | 28.13287446 | 18.77371923 |
| 2021 | 13.65504584 | 26.33473125 | 19.99488854 | 9.622006886 | 28.34280493 | 18.98240591 |
| 2022 | 13.68293499 | 26.38851748 | 20.03572623 | 9.824310668 | 28.53865325 | 19.18148196 |
| 2023 | 13.71076119 | 26.44218229 | 20.07647174 | 10.02162946 | 28.72177288 | 19.37170117 |
| 2024 | 13.79512116 | 26.60487651 | 20.19999883 | 10.21410835 | 28.89336175 | 19.55373505 |
| 2025 | 13.82357691 | 26.65975546 | 20.24166618 | 10.40390529 | 29.06649469 | 19.73519999 |
| 2026 | 13.85209561 | 26.71475582 | 20.28342572 | 10.58910707 | 29.22918063 | 19.90914385 |
| 2027 | 13.88048841 | 26.76951336 | 20.32500088 | 10.76985174 | 29.38242705 | 20.0761394 |
| 2028 | 13.94293996 | 26.88995565 | 20.41644781 | 10.94626653 | 29.52710559 | 20.23668606 |
| 2029 | 13.97139572 | 26.94483459 | 20.45811515 | 11.11969331 | 29.67124742 | 20.39547037 |
| 2030 | 13.99985147 | 26.99971354 | 20.4997825 | 11.28900874 | 29.80770112 | 20.54835493 |
| 2031 | 14.02843313 | 27.05483532 | 20.54163422 | 11.45433433 | 29.9371983 | 20.69576631 |
| 2032 | 14.0922697 | 27.17794871 | 20.6351092 | 11.61579248 | 30.06042764 | 20.83811006 |
| 2033 | 14.12141796 | 27.2341632 | 20.67779058 | 11.77475401 | 30.18545488 | 20.98010445 |
| 2034 | 14.15062917 | 27.29049911 | 20.72056414 | 11.93005639 | 30.30475969 | 21.11740804 |
| 2035 | 14.17996629 | 27.34707785 | 20.76352207 | 12.08180999 | 30.41889997 | 21.25035498 |
| 2036 | 14.24336218 | 27.46934135 | 20.85635176 | 12.2301242 | 30.52839388 | 21.37925904 |
| 2037 | 14.2726993 | 27.52592008 | 20.89930969 | 12.37631455 | 30.64089941 | 21.50860698 |
| 2038 | 14.30203643 | 27.58249882 | 20.94226762 | 12.51922991 | 30.74891413 | 21.63407202 |
| 2039 | 14.3314365 | 27.63919897 | 20.98531774 | 12.65896709 | 30.85286539 | 21.75591624 |
| 2040 | 14.38551502 | 27.74349325 | 21.06450414 | 12.79562225 | 30.95315322 | 21.87438774 |
| 2041 | 14.41541874 | 27.80116471 | 21.10829173 | 12.93016605 | 31.05535473 | 21.99276039 |
| 2042 | 14.44538542 | 27.85895759 | 21.1521715 | 13.06179927 | 31.15416603 | 22.10798265 |
| 2043 | 14.47535209 | 27.91675046 | 21.19605128 | 13.19061017 | 31.24992307 | 22.22026662 |
| 2044 | 14.5505206 | 28.0617183 | 21.30611945 | 13.31668218 | 31.3429165 | 22.32979934 |
| 2045 | 14.58111683 | 28.12072531 | 21.35092107 | 13.44170726 | 31.442986 | 22.44234663 |
| 2046 | 14.61177601 | 28.17985373 | 21.39581487 | 13.56412777 | 31.54001489 | 22.55207133 |
| 2047 | 14.64249815 | 28.23910357 | 21.44080086 | 13.68402292 | 31.63430586 | 22.65916439 |
| 2048 | 14.7066495 | 28.36282403 | 21.53473676 | 13.8014696 | 31.72613279 | 22.76380119 |
| 2049 | 14.73800119 | 28.423288 | 21.58064459 | 13.91773161 | 31.82281265 | 22.87027213 |
| 2050 | 14.76941583 | 28.48387339 | 21.62664461 | 14.03167496 | 31.91693451 | 22.97430474 |

Table s3 (a) Energy recovery from Incineration in Perungudi:

| Year | MSW (tons/day) | MSW (tons/year) | PINC, (W/year) | PINC, (kW/year) | EINC (MWh/year) |
| --- | --- | --- | --- | --- | --- |
| 2026 | 2200.31 | 803,113.15 | 13,266,619.30 | 13,266.62 | 98,783.25 |
| 2027 | 2204.82 | 804,759.30 | 13,293,812.04 | 13,293.81 | 98,985.72 |
| 2028 | 2214.74 | 808,380.10 | 13,353,624.00 | 13,353.62 | 99,431.08 |
| 2029 | 2219.26 | 810,029.90 | 13,380,877.03 | 13,380.88 | 99,634.01 |
| 2030 | 2223.78 | 811,679.70 | 13,408,130.07 | 13,408.13 | 99,836.94 |
| 2031 | 2228.32 | 813,336.80 | 13,435,503.69 | 13,435.50 | 100,040.76 |
| 2032 | 2238.46 | 817,037.90 | 13,496,642.13 | 13,496.64 | 100,495.99 |
| 2033 | 2243.09 | 818,727.85 | 13,524,558.39 | 13,524.56 | 100,703.86 |
| 2034 | 2247.73 | 820,421.45 | 13,552,534.96 | 13,552.53 | 100,912.18 |
| 2035 | 2252.39 | 822,122.35 | 13,580,632.11 | 13,580.63 | 101,121.39 |
| 2036 | 2262.46 | 825,797.90 | 13,641,348.49 | 13,641.35 | 101,573.48 |
| 2037 | 2267.12 | 827,498.80 | 13,669,445.64 | 13,669.45 | 101,782.69 |
| 2038 | 2271.78 | 829,199.70 | 13,697,542.80 | 13,697.54 | 101,991.90 |
| 2039 | 2276.45 | 830,904.25 | 13,725,700.24 | 13,725.70 | 102,201.56 |
| 2040 | 2285.04 | 834,039.60 | 13,777,493.06 | 13,777.49 | 102,587.21 |
| 2041 | 2289.79 | 835,773.35 | 13,806,132.86 | 13,806.13 | 102,800.47 |
| 2042 | 2294.55 | 837,510.75 | 13,834,832.96 | 13,834.83 | 103,014.17 |
| 2043 | 2299.31 | 839,248.15 | 13,863,533.06 | 13,863.53 | 103,227.87 |
| 2044 | 2311.25 | 843,606.25 | 13,935,524.47 | 13,935.52 | 103,763.92 |
| 2045 | 2316.11 | 845,380.15 | 13,964,827.51 | 13,964.83 | 103,982.11 |
| 2046 | 2320.98 | 847,157.70 | 13,994,190.85 | 13,994.19 | 104,200.75 |
| 2047 | 2325.86 | 848,938.90 | 14,023,614.47 | 14,023.61 | 104,419.83 |
| 2048 | 2336.05 | 852,658.25 | 14,085,054.38 | 14,085.05 | 104,877.31 |
| 2049 | 2341.03 | 854,475.95 | 14,115,080.96 | 14,115.08 | 105,100.89 |
| 2050 | 2346.02 | 856,297.30 | 14,145,167.82 | 14,145.17 | 105,324.92 |
|  |  |  |  | ETINC | 2,550,794.26 |

Table s3 (b) Energy recovery from Incineration in Kodungaiyur:

| Year | MSW (tons/day) | MSW (tons/year) | PINC, (W/year) | PINC, (kW/year) | EINC (MWh/year) |
| --- | --- | --- | --- | --- | --- |
| 2026 | 2143.92 | 782,530.80 | 12,926,619.63 | 12,926.62 | 96,251.61 |
| 2027 | 2149.38 | 784,523.70 | 12,959,540.33 | 12,959.54 | 96,496.74 |
| 2028 | 2154.85 | 786,520.25 | 12,992,521.32 | 12,992.52 | 96,742.31 |
| 2029 | 2160.34 | 788,524.10 | 13,025,622.91 | 13,025.62 | 96,988.79 |
| 2030 | 2165.84 | 790,531.60 | 13,058,784.78 | 13,058.78 | 97,235.71 |
| 2031 | 2171.38 | 792,553.70 | 13,092,187.83 | 13,092.19 | 97,484.43 |
| 2032 | 2176.92 | 794,575.80 | 13,125,590.89 | 13,125.59 | 97,733.15 |
| 2033 | 2182.48 | 796,605.20 | 13,159,114.53 | 13,159.11 | 97,982.77 |
| 2034 | 2188.05 | 798,638.25 | 13,192,698.46 | 13,192.70 | 98,232.83 |
| 2035 | 2193.62 | 800,671.30 | 13,226,282.40 | 13,226.28 | 98,482.90 |
| 2036 | 2199.19 | 802,704.35 | 13,259,866.33 | 13,259.87 | 98,733.00 |
| 2037 | 2204.83 | 804,762.95 | 13,293,872.33 | 13,293.87 | 98,986.17 |
| 2038 | 2210.48 | 806,825.20 | 13,327,938.62 | 13,327.94 | 99,239.83 |
| 2039 | 2216.21 | 808,916.65 | 13,362,487.27 | 13,362.49 | 99,497.08 |
| 2040 | 2219.93 | 810,274.45 | 13,384,916.75 | 13,384.92 | 99,664.09 |
| 2041 | 2225.64 | 812,358.60 | 13,419,344.81 | 13,419.34 | 99,920.44 |
| 2042 | 2231.36 | 814,446.40 | 13,453,833.16 | 13,453.83 | 100,177.24 |
| 2043 | 2237.18 | 816,570.70 | 13,488,924.45 | 13,488.92 | 100,438.53 |
| 2044 | 2244.36 | 819,191.40 | 13,532,215.77 | 13,532.22 | 100,760.88 |
| 2045 | 2250.15 | 821,304.75 | 13,567,126.18 | 13,567.13 | 101,020.82 |
| 2046 | 2255.95 | 823,421.75 | 13,602,096.89 | 13,602.10 | 101,281.21 |
| 2047 | 2261.69 | 825,516.85 | 13,636,705.83 | 13,636.71 | 101,538.91 |
| 2048 | 2267.26 | 827,549.90 | 13,670,289.76 | 13,670.29 | 101,788.98 |
| 2049 | 2273.23 | 829,728.95 | 13,706,285.47 | 13,706.29 | 102,057.00 |
| 2050 | 2279.21 | 831,911.65 | 13,742,341.48 | 13,742.34 | 102,325.47 |
|  |  |  |  | ETINC | 2,481,060.89 |

Table s 3 (c) Energy recovery from LFGE

| Year | Method | Kodungaiyur (m³/year) | Recoverable Energy Potential (kWh/year) | Perungudi (m³/year) | Recoverable Energy Potential (kWh/year) |
| --- | --- | --- | --- | --- | --- |
| 2026 | DM | 20,702,092.76 | 29,006,116.20 | 21,246,605.15 | 29,769,043.41 |
|  | FOD | 27,072,964.17 | 37,932,471.55 | 27,806,066.83 | 38,959,636.35 |
| 2027 | DM | 20,754,815.54 | 29,079,987.15 | 21,290,154.55 | 29,830,061.35 |
|  | FOD | 27,302,096.13 | 38,253,512.93 | 28,039,30 0.83 | 39,286,425.18 |
| 2028 | DM | 20,807,634.88 | 29,153,993.39 | 21,385,943.93 | 29,964,273.76 |
|  | FOD | 27,523,435.30 | 38,563,635.66 | 28,263,528.02 | 39,600,594.38 |
| 2029 | DM | 20,860,647.35 | 29,228,270.21 | 21,429,589.90 | 30,025,427.00 |
|  | FOD | 27,737,551.74 | 38,863,638.49 | 28,485,293.81 | 39,911,314.86 |
| 2030 | DM | 20,913,756.38 | 29,302,682.34 | 21,473,235.86 | 30,086,580.23 |
|  | FOD | 27,944,977.63 | 39,154,267.05 | 28,698,819.74 | 40,210,490.31 |
| 2031 | DM | 20,967,251.66 | 29,377,635.64 | 21,517,074.95 | 30,148,004.06 |
|  | FOD | 28,146,189.07 | 39,436,188.43 | 28,904,701.56 | 40,498,955.45 |
| 2032 | DM | 21,020,746.93 | 29,452,588.94 | 21,614,988.69 | 30,285,192.95 |
|  | FOD | 28,341,654.67 | 39,710,059.20 | 29,103,505.67 | 40,777,503.87 |
| 2033 | DM | 21,074,435.33 | 29,527,812.83 | 21,659,696.84 | 30,347,834.43 |
|  | FOD | 28,531,760.24 | 39,976,419.91 | 29,301,821.85 | 41,055,368.63 |
| 2034 | DM | 21,128,220.30 | 29,603,172.02 | 21,704,501.54 | 30,410,611.20 |
|  | FOD | 28,716,881.39 | 40,235,796.85 | 29,493,586.65 | 41,324,054.12 |
| 2035 | DM | 21,182,005.26 | 29,678,531.21 | 21,749,499.38 | 30,473,658.57 |
|  | FOD | 28,897,351.93 | 40,488,657.74 | 29,679,266.74 | 41,584,214.21 |
| 2036 | DM | 21,235,790.22 | 29,753,890.39 | 21,846,737.18 | 30,609,900.40 |
|  | FOD | 29,073,467.62 | 40,735,416.95 | 29,859,300.34 | 41,836,462.89 |
| 2037 | DM | 21,290,251.12 | 29,830,196.64 | 21,891,735.01 | 30,672,947.76 |
|  | FOD | 29,245,500.38 | 40,976,455.50 | 30,039,953.88 | 42,089,580.18 |
| 2038 | DM | 21,344,808.57 | 29,906,638.19 | 21,936,732.85 | 30,735,995.12 |
|  | FOD | 29,413,776.67 | 41,212,230.76 | 30,215,184.39 | 42,335,099.15 |
| 2039 | DM | 21,400,138.53 | 29,984,162.09 | 21,981,827.24 | 30,799,177.78 |
|  | FOD | 29,578,531.36 | 41,443,071.85 | 30,385,357.88 | 42,573,532.63 |
| 2040 | DM | 21,436,059.54 | 30,034,491.75 | 22,064,773.89 | 30,915,395.99 |
|  | FOD | 29,740,057.21 | 41,669,388.96 | 30,550,820.86 | 42,805,366.12 |
| 2041 | DM | 21,491,196.37 | 30,111,745.06 | 22,110,640.78 | 30,979,661.00 |
|  | FOD | 29,896,349.63 | 41,888,373.39 | 30,716,145.80 | 43,037,006.20 |
| 2042 | DM | 21,546,429.76 | 30,189,133.67 | 22,156,604.23 | 31,044,061.31 |
|  | FOD | 30,049,930.22 | 42,103,558.24 | 30,877,070.74 | 43,262,481.35 |
| 2043 | DM | 21,602,628.77 | 30,267,875.22 | 22,202,567.68 | 31,108,461.62 |
|  | FOD | 30,200,950.85 | 42,315,156.25 | 31,033,891.93 | 43,482,206.66 |
| 2044 | DM | 21,671,960.19 | 30,365,016.87 | 22,317,862.55 | 31,270,003.58 |
|  | FOD | 30,349,650.39 | 42,523,502.15 | 31,186,870.59 | 43,696,548.12 |
| 2045 | DM | 21,727,869.52 | 30,443,352.54 | 22,364,791.62 | 31,335,756.84 |
|  | FOD | 30,497,620.41 | 42,730,825.91 | 31,344,059.54 | 43,916,788.70 |
| 2046 | DM | 21,783,875.40 | 30,521,823.50 | 22,411,817.25 | 31,401,645.39 |
|  | FOD | 30,643,325.91 | 42,934,976.80 | 31,497,306.32 | 44,131,505.84 |
| 2047 | DM | 21,839,301.92 | 30,599,482.70 | 22,458,939.45 | 31,467,669.24 |
|  | FOD | 30,786,889.76 | 43,136,126.98 | 31,646,877.64 | 44,341,073.20 |
| 2048 | DM | 21,893,086.88 | 30,674,841.89 | 22,557,335.99 | 31,605,534.61 |
|  | FOD | 30,928,350.11 | 43,334,329.91 | 31,793,018.43 | 44,545,833.98 |
| 2049 | DM | 21,950,734.32 | 30,755,612.87 | 22,605,423.81 | 31,672,911.40 |
|  | FOD | 31,067,624.16 | 43,529,469.56 | 31,941,720.85 | 44,754,183.91 |
| 2050 | DM | 22,008,478.32 | 30,836,519.14 | 22,653,608.18 | 31,740,423.49 |
|  | FOD | 31,205,258.35 | 43,722,311.58 | 32,087,017.79 | 44,957,762.36 |

Table S 4 (a) Methane Emissions, GHG Avoidance, and Carbon Revenue for Kodungaiyur

| Year | Method | Methane emission (tons/year) | GHGAvoided,LF (tCO₂e/year) | GHGAvoided,WtE (tCO₂e/year) | GHGAvoided T (tCO₂e/year) | Revenue Carbon (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| 2026 | DM | 14822.69841 | 373532.0000 | 72188.70734 | 445720.7074 | 3120044.952 |
|  | FOD | 19384.24234 | 488482.9071 | 72188.70734 | 560671.6144 | 3924701.301 |
| 2027 | DM | 14860.44793 | 374483.2877 | 72372.55298 | 446855.8407 | 3127990.885 |
|  | FOD | 19548.30083 | 492617.1809 | 72372.55298 | 564989.7339 | 3954928.137 |
| 2028 | DM | 14898.26658 | 375436.3177 | 72556.73534 | 447993.0530 | 3135951.371 |
|  | FOD | 19706.77967 | 496610.8478 | 72556.73534 | 569167.5831 | 3984173.082 |
| 2029 | DM | 14936.22350 | 376392.8323 | 72741.59112 | 449134.4234 | 3143940.964 |
|  | FOD | 19860.08704 | 500474.1935 | 72741.59112 | 573215.7846 | 4012510.492 |
| 2030 | DM | 14974.24957 | 377351.0891 | 72926.78361 | 450277.8727 | 3151945.109 |
|  | FOD | 20008.60398 | 504216.8203 | 72926.78361 | 577143.604 | 4040005.228 |
| 2031 | DM | 15012.55219 | 378316.3151 | 73113.32296 | 451429.6380 | 3160007.466 |
|  | FOD | 20152.67138 | 507847.3187 | 73113.32296 | 580960.6416 | 4066724.492 |
| 2032 | DM | 15050.85480 | 379281.5411 | 73299.86230 | 452581.4034 | 3168069.823 |
|  | FOD | 20292.62475 | 511374.1436 | 73299.8623 | 584674.0059 | 4092718.041 |
| 2033 | DM | 15089.29570 | 380250.2516 | 73487.07508 | 453737.3267 | 3176161.287 |
|  | FOD | 20428.74033 | 514804.2564 | 73487.07508 | 588291.3314 | 4118039.32 |
| 2034 | DM | 15127.80573 | 381220.7044 | 73674.62457 | 454895.3290 | 3184267.303 |
|  | FOD | 20561.28707 | 518144.4343 | 73674.62457 | 591819.0588 | 4142733.412 |
| 2035 | DM | 15166.31576 | 382191.1573 | 73862.17406 | 456053.3313 | 3192373.319 |
|  | FOD | 20690.50399 | 521400.7004 | 73862.17406 | 595262.8745 | 4166840.121 |
| 2036 | DM | 15204.82580 | 383161.6101 | 74049.72355 | 457211.3336 | 3200479.336 |
|  | FOD | 20816.60282 | 524578.391 | 74049.72355 | 598628.1145 | 4190396.802 |
| 2037 | DM | 15243.81980 | 384144.2589 | 74239.63003 | 458383.8890 | 3208687.223 |
|  | FOD | 20939.77828 | 527682.4125 | 74239.63003 | 601922.0426 | 4213454.298 |
| 2038 | DM | 15282.88294 | 385128.6500 | 74429.87323 | 459558.5233 | 3216909.663 |
|  | FOD | 21060.26409 | 530718.6551 | 74429.87323 | 605148.5284 | 4236039.698 |
| 2039 | DM | 15322.49918 | 386126.9794 | 74622.81013 | 460749.7896 | 3225248.527 |
|  | FOD | 21178.22845 | 533691.357 | 74622.81013 | 608314.1671 | 4258199.17 |
| 2040 | DM | 15348.21863 | 386775.1095 | 74748.06760 | 461523.1771 | 3230662.240 |
|  | FOD | 21293.88097 | 536605.8003 | 74748.0676 | 611353.8679 | 4279477.076 |
| 2041 | DM | 15387.69660 | 387769.9544 | 74940.33108 | 462710.2854 | 3238971.998 |
|  | FOD | 21405.78633 | 539425.8156 | 74940.33108 | 614366.1467 | 4300563.027 |
| 2042 | DM | 15427.24371 | 388766.5415 | 75132.93128 | 463899.4727 | 3247296.309 |
|  | FOD | 21515.75004 | 542196.901 | 75132.93128 | 617329.8323 | 4321308.826 |
| 2043 | DM | 15467.48220 | 389780.5514 | 75328.89860 | 465109.4500 | 3255766.150 |
|  | FOD | 21623.88081 | 544921.7963 | 75328.8986 | 620250.6949 | 4341754.865 |
| 2044 | DM | 15517.12350 | 391031.5122 | 75570.65899 | 466602.1712 | 3266215.198 |
|  | FOD | 21730.34968 | 547604.8119 | 75570.65899 | 623175.4709 | 4362228.296 |
| 2045 | DM | 15557.15457 | 392040.2953 | 75765.61618 | 467805.9115 | 3274641.380 |
|  | FOD | 21836.29621 | 550274.6646 | 75765.61618 | 626040.2808 | 4382281.965 |
| 2046 | DM | 15597.25479 | 393050.8207 | 75960.91008 | 469011.7307 | 3283082.115 |
|  | FOD | 21940.62135 | 552903.6581 | 75960.91008 | 628864.5682 | 4402051.977 |
| 2047 | DM | 15636.94017 | 394050.8924 | 76154.18370 | 470205.0760 | 3291435.532 |
|  | FOD | 22043.41307 | 555494.0093 | 76154.1837 | 631648.1929 | 4421537.351 |
| 2048 | DM | 15675.45021 | 395021.3452 | 76341.73319 | 471363.0784 | 3299541.549 |
|  | FOD | 22144.69868 | 558046.4067 | 76341.73319 | 634388.1399 | 4440716.979 |
| 2049 | DM | 15716.72577 | 396061.4894 | 76542.75122 | 472604.2406 | 3308229.685 |
|  | FOD | 22244.4189 | 560559.3562 | 76542.75122 | 637102.1075 | 4459714.752 |
| 2050 | DM | 15758.07047 | 397103.3760 | 76744.10597 | 473847.4819 | 3316932.373 |
|  | FOD | 22342.96498 | 563042.7175 | 76744.10597 | 639786.8234 | 4478507.764 |

Table S 4 (b) Methane Emissions, GHG Avoidance, and Carbon Revenue for Perungudi

| Year | Method | Methane emission (tons/year) | GHGAvoided,LF (tCO₂e/year) | GHGAvoided,WtE (tCO₂e/year) | GHGAvoided T (tCO₂e/year) | Revenue Carbon (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| 2026 | DM | 15212.56929 | 383356.7460 | 74087.43547 | 457444.1815 | 3202109.271 |
|  | FOD | 19909.14385 | 501710.4250 | 74087.43547 | 575797.8604 | 4030585.023 |
| 2027 | DM | 15243.75066 | 384142.5166 | 74239.29332 | 458381.8100 | 3208672.670 |
|  | FOD | 20076.13940 | 505918.7128 | 74239.29332 | 580158.0061 | 4061106.043 |
| 2028 | DM | 15312.33585 | 385870.8635 | 74573.31323 | 460444.1768 | 3223109.237 |
|  | FOD | 20236.68606 | 509964.4888 | 74573.31323 | 584537.8020 | 4091764.614 |
| 2029 | DM | 15343.58637 | 386658.3764 | 74725.50779 | 461383.8842 | 3229687.189 |
|  | FOD | 20395.47037 | 513965.8533 | 74725.50779 | 588691.3611 | 4120839.528 |
| 2030 | DM | 15374.83688 | 387445.8893 | 74877.70235 | 462323.5917 | 3236265.142 |
|  | FOD | 20548.35493 | 517818.5443 | 74877.70235 | 592696.2466 | 4148873.726 |
| 2031 | DM | 15406.22567 | 388236.8868 | 75030.57033 | 463267.4571 | 3242872.200 |
|  | FOD | 20695.76631 | 521533.3111 | 75030.57033 | 596563.8815 | 4175947.170 |
| 2032 | DM | 15476.33190 | 390003.5639 | 75371.99795 | 465375.5619 | 3257628.933 |
|  | FOD | 20838.11006 | 525120.3735 | 75371.99795 | 600492.3715 | 4203446.600 |
| 2033 | DM | 15508.34293 | 390810.2420 | 75527.89636 | 466338.1383 | 3264366.968 |
|  | FOD | 20980.10445 | 528698.6320 | 75527.89636 | 604226.5284 | 4229585.699 |
| 2034 | DM | 15540.42311 | 391618.6623 | 75684.13148 | 467302.7937 | 3271119.556 |
|  | FOD | 21117.40804 | 532158.6826 | 75684.13148 | 607842.8140 | 4254899.698 |
| 2035 | DM | 15572.64155 | 392430.5672 | 75841.04003 | 468271.6072 | 3277901.250 |
|  | FOD | 21250.35498 | 535508.9456 | 75841.04003 | 611349.9856 | 4279449.899 |
| 2036 | DM | 15642.26382 | 394185.0483 | 76180.11065 | 470365.1590 | 3292556.113 |
|  | FOD | 21379.25904 | 538757.3278 | 76180.11065 | 614937.4385 | 4304562.069 |
| 2037 | DM | 15674.48227 | 394996.9532 | 76337.01920 | 471333.9724 | 3299337.807 |
|  | FOD | 21508.60698 | 542016.8959 | 76337.01920 | 618353.9151 | 4328477.405 |
| 2038 | DM | 15706.70072 | 395808.8581 | 76493.92774 | 472302.7858 | 3306119.501 |
|  | FOD | 21634.07202 | 545178.6149 | 76493.92774 | 621672.5427 | 4351707.799 |
| 2039 | DM | 15738.98830 | 396622.5052 | 76651.17301 | 473273.6783 | 3312915.748 |
|  | FOD | 21755.91624 | 548249.0892 | 76651.17301 | 624900.2622 | 4374301.836 |
| 2040 | DM | 15798.37810 | 398119.1282 | 76940.41001 | 475059.5382 | 3325416.767 |
|  | FOD | 21874.38774 | 551234.5709 | 76940.41001 | 628174.9810 | 4397224.867 |
| 2041 | DM | 15831.21880 | 398946.7137 | 77100.34898 | 476047.0626 | 3332329.438 |
|  | FOD | 21992.76039 | 554217.5618 | 77100.34898 | 631317.9108 | 4419225.376 |
| 2042 | DM | 15864.12863 | 399776.0414 | 77260.62467 | 477036.6661 | 3339256.662 |
|  | FOD | 22107.98265 | 557121.1627 | 77260.62467 | 634381.7874 | 4440672.512 |
| 2043 | DM | 15897.03846 | 400605.3691 | 77420.90035 | 478026.2695 | 3346183.886 |
|  | FOD | 22220.26662 | 559950.7188 | 77420.90035 | 637371.6192 | 4461601.334 |
| 2044 | DM | 15979.58959 | 402685.6576 | 77822.93642 | 480508.5940 | 3363560.158 |
|  | FOD | 22329.79934 | 562710.9434 | 77822.93642 | 640533.8798 | 4483737.159 |
| 2045 | DM | 16013.19080 | 403532.4082 | 77986.57924 | 481518.9874 | 3370632.912 |
|  | FOD | 22442.34663 | 565547.1351 | 77986.57924 | 643533.7143 | 4504736.000 |
| 2046 | DM | 16046.86115 | 404380.9011 | 78150.55878 | 482531.4598 | 3377720.219 |
|  | FOD | 22552.07133 | 568312.1975 | 78150.55878 | 646462.7562 | 4525239.294 |
| 2047 | DM | 16080.60064 | 405231.1362 | 78314.87503 | 483546.0112 | 3384822.079 |
|  | FOD | 22659.16439 | 571010.9426 | 78314.87503 | 649325.8176 | 4545280.723 |
| 2048 | DM | 16151.05257 | 407006.5248 | 78657.98621 | 485664.5110 | 3399651.577 |
|  | FOD | 22763.80119 | 573647.7901 | 78657.98621 | 652305.7763 | 4566140.434 |
| 2049 | DM | 16185.48344 | 407874.1828 | 78825.66959 | 486699.8524 | 3406898.967 |
|  | FOD | 22870.27213 | 576330.8576 | 78825.66959 | 655156.5272 | 4586095.690 |
| 2050 | DM | 16219.98346 | 408743.5831 | 78993.68969 | 487737.2728 | 3414160.91 |
|  | FOD | 22974.30474 | 578952.4793 | 78993.68969 | 657946.1690 | 4605623.183 |