Online resource 1: Table, details about sources

Title: Violet-blue light exposure of the skin: is there need for protection?

Journal: Photochemical Photobiological Sciences

Authors: Terje Christensen, a,b Bjørn J. Johnsen b and Ellen M. Bruzell c Affiliations: aUniversity of South-Eastern Norway. bNorwegian Radiation and Nuclear Safety Authority cNordic Institute of Dental Materials (NIOM) Corresponding author e-mail: terje.christensen222@gmail.com

Radiometric and photometric exposure values and corresponding time or time range to reach the estimated lowest radiant exposure (Hb = 65 J/cm2) to induce a biological effect, Tthres, for selected light sources, ambient indoor light conditions and outdoor conditions in winter. Eb: blue light hazard-weighted irradiance; E400-500: irradiance between 400 and 500 nm; Ev: illuminance; Eb/500: blue light hazard-weighted irradiance per 500 lx; ND: not determined; NA: not applicable; a: laboratory measurement; b: survey measurement.*: obtained from SG Armaturen AS, Norway. Ambient measurements indoors/outdoors were made at latitude 59°55’N.

| Light source or ambient light condition (details), measurement distance | Eb (µWcm-2) | E400-500 (µWcm-2) | Ev (lx) |  | Eb/500 (µW cm-2lx-1) | Tthres (h) |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor luminaires/lamps (distance 10 cm)a Fluorescent |  |  |  |  |  |  |
| Foshan Lighting Fluorescent (FSL T8 36 W/765, 6500 K) | 787 | 1113 | 8315 | 47 |  | 384 |
| Osram Lumilux Interna (36 W/827, 2700 K) | 142 | 228 | 4929 | 14 |  | 1290 |
| Osram FQ Lumilux (49 W/830 Plus, 3000 K) | 648 | 994 | 16137 | 20 |  | 903 |
| Osram FH Lumilux (28 W/830 HE, 3000 K) | 628 | 951 | 16175 | 19 |  | 951 |
| Aura light T5 eco saver HE longlife (25 W/830, 3000 K) | 523 | 759 | 13620 | 19 |  | 951 |
| Indoor luminaires/lamps (distance 10 cm)a LED |  |  |  |  |  |  |
| Linus LED* (D/I 22 W, 3000 K) | 338 | 475 | 10065 | 17 |  | 1062 |
|  | Eb (µWcm-2) |  | Ev (lx) |  |  | Tthres (h) |
| Indoor luminaires/lampsb |  |  |  |  |  |  |
| Office lighting (Inspire*, 36 W LED, 3000 K), 110 cm | 50 |  | 500 |  |  | 364 |
| Dimmable LED (Gyro Cube*, dim to warm, 2000–2800 K), 120 cm | High brightness: |  |  |  |  |  |
|  | 21 |  | 250 |  |  | 865 |
|  | Medium brightness: |  |  |  |  |  |
|  | 7.5 |  | 110 |  |  | 2360 |
|  | Low brightness: |  |  |  |  |  |
|  | 1.33 |  | 32 |  |  | 14326 |
| LED decoration (Gnosjö Konstsmide AB, Gothenburg, Sweden; 120×0.03W, warm white), 20 cm | 35 |  | 450 |  |  | 689 |
| Ambient indoor light conditionsb |  |  |  |  |  |  |
| Bathroom, head level, face position | 2–5 |  | ND |  |  | 2275–3575 |
| Staircase, 1 m above stairs | 2 |  | ND |  |  | 2275 |
| At dining table, table surface | 4 |  | ND |  |  | 4550 |
| By window 7:45 a.m., December 12, 2019 | 0.2 |  | <10 |  |  | 91000 |
| By window, cloudy dusk, 8:45 a.m., December 12, 2019 | 5.5 |  | 20 |  |  | 3309 |
| Mobile phone screens (measured indoors)b |  |  |  |  |  |  |
| Motorola G6 (white screen, held against skin at ear/cheek), 0.5 cm | 5.5 |  | 20 |  |  | 3309 |
| Motorola G6 (different degrees of bright and dark backgrounds), 20 cm | 0.08–26 |  | ND |  |  | 696–225000 |
| Huawei P20 Lite (highest light setting), 20 cm | 6.7–8.5 |  | ND |  |  | 2698–2145 |
| Laptop screens (measured indoors, distance 40 cm)b |  |  |  |  |  |  |
| HP® (blue/white background) | 6.5 |  | 11 |  |  | 2795 |
| HP® (white background) | 10 |  | 40 |  |  | 1820 |
| Acer (blue background) | 2.8 |  | 7 |  |  | 6500 |
| Acer (white background) | 4 |  | 13 |  |  | 4550 |
| Outdoorsb |  |  |  |  |  |  |
| Cloudy, rain, 7:45 a.m., December 12, 2019 | 54 |  | 120 |  |  | 332 |
| Foggy, under streetlight, 08:45 a.m., November 15, 2019 | 25 |  | ND |  |  | 715 |
| Foggy, 09:05 a.m. November 15, 2019 | 63 |  | ND |  |  | 286 |
| Misty, 12:40 a.m., November 15, 2019 | 570 |  | ND |  |  | 32 |
| Medical operating light (LED)b |  |  |  |  |  |  |
| Faro EDI TS, Faro S.p.A, Ornago, Italy (95 W, halogen, 5000 K), 70 cm | 400 |  | Maximum 25000 |  |  | 45 |
| Dental material light curing devices (LEDs)a (from 1). Measured close to emission area. Tthres with units of s in parentheses. |  |  |  |  |  |  |
| Elipar FreeLight 2 | 0.8 · 106 |  | NA |  |  | 0.023 (84) |
| Flashmax | 2.8 · 106 |  | NA |  |  | 0.006 (23) |
| Bluephase 16i | 1.8 · 106 |  | NA |  |  | 0.010 (36) |
| Bluephase 20i | 1.0 · 106 |  | NA |  |  | 0.017 (62) |
| Demi Plus | 1.3 · 106 |  | NA |  |  | 0.014 (49) |
| Valo | 0.7 · 106 |  | NA |  |  | 0.026 (93) |
| FlashSoft | 1.0 · 106 |  | NA |  |  | 0.018 (66) |
| SmartLite iQ2 | 0.4 · 106 |  | NA |  |  | 0.041 (146) |
| LEDemetron II | 1.3 · 106 |  | NA |  |  | 0.014 (50) |

Reference:

- Bruzell, E.M., Christensen T., & Johnsen B. (2015). Appropriate eye protection filters prevent ocular damage from curing light. Tandlaegebladet, 119(5), 368-378. (In Danish with English summary).