Pollution Status and Health Risk Assessment of Groundwater Aquifers in Ekpan Community, Delta State, Nigeria

1, 2James Chikelue Ozoemenam, 3*Vincent Nwalieji Okafor, 3John Kanayochukwu Nduka, 3,4 Daniel Omeodisemi Omokpariola and 1Bernard Enefeanya Akudinobi

1Department of Geological Sciences, Nnamdi Azikiwe University, P.M.B 5025 Awka, Anambra State, Nigeria.

2Logistics Unit, Renaissance Industrial Office Area, Rumuobiakani, P. O. Box 263, Port Harcourt, Rivers State, Nigeria.

3Department of Pure and Industrial Chemistry, Nnamdi Azikiwe University, P.M.B 5025 Awka, Anambra State, Nigeria.

4Production and Technical Department, OCP Africa Fertilizers Nigeria Limited, Maitama, Abuja, Nigeria.

* Corresponding author email: vnw.okafor@unizik.edu.ng

ORCID: Vincent N. Okafor https://orcid.org/0000-0002-3838-9635

ORCID: John Kanayochukwu Nduka https://orcid.org/0000-0002-5323-3153

ORCID: Daniel O. Omokpariola https://orcid.org/0000-0003-1360-4340

LIST OF SUPPLEMENTARY TABLES

Table S1: Instrument Working Conditions for the Analysis of Heavy Metals

| Parameters | Zn | Cu | Fe | Pb | Cd | Ni | Mn | Cr | As | Al | Hg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wavelength (nm) | 214 | 324 | 248 | 283 | 229 | 232 | 276 | 358 | 194 | 332 | 254 |
| Silt width (nm) | 1 | 0.5 | 0.2 | 1 | 0.5 | 0.2 | 0.2 | 0.2 | 0.2 | 0.2 | 0.3 |
| Lamp current (mA) | 10 | 15 | 5 | 12 | 10 | 10 | 10 | 15 | 10 | 5 | 15 |
| Fuel | C2H2 | C2H2 | C2H2 | C2H2 | C2H2 | C2H2 | C2H2 | C2H2 | C2H2 | C2H2 | C2H2 |
| Support | O2 | O2 | O2 | O2 | O2 | O2 | O2 | O2 | O2 | O2 | O2 |
| Detection limit | 0.0005 | 0.03 | 0.01 | 0.005 | 0.0002 | 0.0005 | 0.003 | 0.001 | 0.0001 | 0.04 | 0.0002 |
| Quantification limit | 0.00165 | 0.099 | 0.033 | 0.0165 | 0.00066 | 0.00165 | 0.0099 | 0.0033 | 0.00033 | 0.132 | 0.00066 |

Table S2: Parameters Used to Evaluate Health Risk Assessment

| Parameter | Unit | Adults | Children | References |
| --- | --- | --- | --- | --- |
| C | mg/L | Laboratory data | Laboratory data |  |
| IR | L/day | 2.88 | 1.704 | [11, 61 - 63] |
| EF | Day/year | 350 | 350 | [11, 61 - 63] |
| ED | Year | 30 | 6 | [11, 61 - 63] |
| BW | Kg | 80 | 15 | [11, 61 - 63] |
| AT – non-cancer | Day | 10950 | 2160 | [11, 61 - 63] |
| AT – cancer | Day | 25550 | 25550 | [11, 61 - 63] |
| SA | cm2 | 19652 | 6365 | [11, 61 - 63] |
| AF | mg/cm2 | 0.07 | 0.2 | [11, 61 - 63] |
| ABS | Unitless | See Table S3 | See Table S3 | - |
| RfD | mg/kg/day | See Table S3 | See Table S3 | - |
| CSF | (mg/day/day)-1 | See Table S3 | See Table S3 | - |

Table S3: Reference Dose (RfD) and Cancer Slope Factor (CSF)

| Parameters | ABS | RfD (mg/kg-day) | CSF (mg/kg-day)⁻¹ | Comment |
| --- | --- | --- | --- | --- |
| K (mg/L) | 1 | 1.36 | - | K salts as inorganic phosphate |
| Na (mg/L) | 1 | 1 | - | Na salts as inorganic phosphate |
| NO₃⁻ (mg/L) | 1 | 1.6 | - | Nitrate in water |
| Zn | 1 | 0.3 | - | - |
| Cu | 1 | 0.04 | - | - |
| Fe | 1 | 0.7 | - | - |
| Pb | 1 | 0.0035 | 0.0085 | - |
| Cd | 0.05 | 0.0001 | 6.3 | - |
| Ni | 0.04 | 0.02 | 0.84 | Nickel soluble salts |
| Mn | 1 | 0.024 | - | - |
| Cr (III) | 0.013 | 1.5 | - | - |
| Cr (VI) | 0.025 | 0.003 | 0.5 | - |
| As | 1 | 0.0003 | 0.0043 | As inorganic |
| Al | 1 | 1 | - | - |
| Hg | 1 | 0.0003 | - | Mercuric chloride (and other mercury salts) |

Table S4: Mean Values of the Physicochemical Parameters of All the Groundwater Samples and Permissible Limits of WHO and FME

| Parameter | Mean value | Regulatory body Permissible limit |
| --- | --- | --- |
| pH | 6.23 | 6.5 – 8.5a, b |
| Temperature | 24.51 | 22 – 35a |
| Electrical conductivity (EC), µS/cm | 160 | 100 – 1000a |
| Total dissolved solids (TDS), mg/L | 100.75 | 600 – 1000a, b |
| Biochemical Oxygen Demand (BOD), mg/L | 4.23 | 6 – 10a |
| Salinity (%) | 31.81 | - |
| K | 5.30 | 5a |
| Na | 13.47 | 200b |
| Ca | 16.71 | 75a |
| Mg | 3.11 | 50a |
| Total Hardness (TH), mg/L | 38.01 | 200b |
| HCO3- | 29.77 | - |
| SO42- | 14.03 | 500b |
| NO3- | 0.70 | 50a , 10b |

a = WHO, b = FME

Table S5: Pearson and Spearman Correlations of Borehole Locations

| Spearman Correlation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pearson correlation |  | BH 1 | BH 2 | BH 3 | BH 4 | BH 5 | BH 6 | BH 7 | BH 8 | BH 9 | BH 10 | BH 11 | BH 12 |
|  | BH 1 |  | 0.96 | 0.99 | 0.95 | 0.96 | 0.97 | 0.92 | 0.92 | 0.90 | 0.92 | 0.92 | 0.94 |
|  | BH 2 | 0.96 |  | 0.97 | 0.94 | 0.96 | 0.98 | 0.94 | 0.93 | 0.92 | 0.94 | 0.94 | 0.95 |
|  | BH 3 | 0.97 | 0.99 |  | 0.96 | 0.98 | 0.99 | 0.93 | 0.93 | 0.91 | 0.93 | 0.93 | 0.95 |
|  | BH 4 | 0.89 | 0.97 | 0.95 |  | 0.97 | 0.96 | 0.87 | 0.87 | 0.86 | 0.88 | 0.87 | 0.90 |
|  | BH 5 | 0.97 | 0.99 | 0.98 | 0.93 |  | 0.99 | 0.93 | 0.91 | 0.91 | 0.92 | 0.93 | 0.95 |
|  | BH 6 | 0.76 | 0.81 | 0.80 | 0.80 | 0.81 |  | 0.95 | 0.93 | 0.92 | 0.94 | 0.94 | 0.96 |
|  | BH 7 | 0.78 | 0.80 | 0.79 | 0.73 | 0.81 | 0.63 |  | 0.97 | 0.98 | 0.97 | 0.97 | 0.98 |
|  | BH 8 | 0.60 | 0.62 | 0.61 | 0.54 | 0.63 | 0.46 | 0.95 |  | 0.99 | 0.99 | 0.99 | 0.98 |
|  | BH 9 | 0.73 | 0.75 | 0.74 | 0.70 | 0.76 | 0.57 | 0.99 | 0.97 |  | 0.99 | 0.98 | 0.98 |
|  | BH 10 | 0.74 | 0.77 | 0.77 | 0.72 | 0.77 | 0.54 | 0.98 | 0.96 | 0.99 |  | 0.98 | 0.99 |
|  | BH 11 | 0.68 | 0.68 | 0.69 | 0.59 | 0.69 | 0.53 | 0.97 | 0.99 | 0.97 | 0.97 |  | 0.98 |
|  | BH 12 | 0.83 | 0.86 | 0.85 | 0.82 | 0.86 | 0.62 | 0.98 | 0.91 | 0.98 | 0.98 | 0.93 |  |

Note: Correlation: (Bold: >0.70 – 1.00; italics: 0.40 – 0.69) at significance p > 0.05.

Table S6a: Pearson and Spearman Correlation Matrix for Physicochemical Parameters of Borehole Locations

| Spearman Correlation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pearson correlation |  | pH | Temp. (°C) | EC (µS/cm) | TDS (mg/L) | BOD (mg/L) | Salinity (%) | K (mg/L) | Na (mg/L) | Ca (mg/L) | Mg (mg/L) | TH (mg/L) | HCO₃ˉ (mg/L) | SO₄²ˉ (mg/L) |
|  | pH |  | 0.34 | 0.19 | 0.12 | 0.21 | 0.14 | 0.00 | 0.21 | 0.08 | 0.22 | 0.33 | 0.19 | 0.26 |
|  | Temp. (°C) | 0.46 |  | 0.81 | 0.79 | 0.72 | 0.78 | 0.72 | 0.84 | 0.71 | 0.67 | 0.76 | 0.83 | 0.70 |
|  | EC (µS/cm) | -0.12 | 0.59 |  | 0.95 | 0.75 | 0.91 | 0.87 | 0.88 | 0.92 | 0.76 | 0.82 | 0.87 | 0.85 |
|  | TDS (mg/L) | 0.17 | 0.70 | 0.86 |  | 0.81 | 0.94 | 0.95 | 0.87 | 0.92 | 0.76 | 0.79 | 0.89 | 0.81 |
|  | BOD (mg/L) | 0.39 | 0.86 | 0.45 | 0.53 |  | 0.79 | 0.79 | 0.78 | 0.84 | 0.79 | 0.71 | 0.85 | 0.81 |
|  | Salinity (%) | 0.45 | 0.97 | 0.64 | 0.77 | 0.88 |  | 0.93 | 0.82 | 0.86 | 0.78 | 0.78 | 0.80 | 0.77 |
|  | K (mg/L) | -0.11 | 0.61 | 1.00 | 0.85 | 0.48 | 0.66 |  | 0.78 | 0.89 | 0.77 | 0.80 | 0.82 | 0.75 |
|  | Na (mg/L) | -0.08 | 0.61 | 1.00 | 0.86 | 0.49 | 0.66 | 1.00 |  | 0.84 | 0.79 | 0.71 | 0.90 | 0.89 |
|  | Ca (mg/L) | -0.05 | 0.66 | 0.99 | 0.88 | 0.52 | 0.72 | 0.99 | 0.99 |  | 0.86 | 0.73 | 0.82 | 0.91 |
|  | Mg (mg/L) | -0.17 | 0.52 | 0.99 | 0.84 | 0.40 | 0.57 | 0.99 | 0.99 | 0.98 |  | 0.73 | 0.77 | 0.95 |
|  | TH (mg/L) | 0.34 | 0.93 | 0.71 | 0.67 | 0.80 | 0.89 | 0.73 | 0.72 | 0.74 | 0.64 |  | 0.87 | 0.72 |
|  | HCO₃ˉ (mg/L) | 0.09 | 0.80 | 0.91 | 0.87 | 0.72 | 0.82 | 0.92 | 0.93 | 0.93 | 0.90 | 0.87 |  | 0.83 |
|  | SO₄²ˉ (mg/L) | -0.07 | 0.57 | 0.95 | 0.85 | 0.53 | 0.65 | 0.95 | 0.97 | 0.96 | 0.97 | 0.64 | 0.92 |  |
|  | NO₃ˉ (mg/L) | 0.43 | 0.89 | 0.60 | 0.65 | 0.87 | 0.93 | 0.62 | 0.63 | 0.68 | 0.55 | 0.85 | 0.79 | 0.65 |
|  | Zn (mg/L) | -0.39 | -0.62 | 0.03 | -0.13 | -0.73 | -0.56 | 0.00 | 0.00 | -0.05 | 0.10 | -0.47 | -0.23 | 0.02 |
|  | Cu (mg/L) | 0.16 | 0.79 | 0.73 | 0.76 | 0.68 | 0.79 | 0.75 | 0.74 | 0.77 | 0.68 | 0.77 | 0.78 | 0.67 |
|  | Fe (mg/L) | 0.19 | 0.70 | 0.91 | 0.78 | 0.59 | 0.72 | 0.92 | 0.93 | 0.92 | 0.90 | 0.82 | 0.91 | 0.89 |
|  | Pb (mg/L) | 0.11 | 0.82 | 0.62 | 0.51 | 0.66 | 0.80 | 0.65 | 0.62 | 0.67 | 0.54 | 0.88 | 0.70 | 0.50 |
|  | Cd (mg/L) | 0.52 | 0.86 | 0.71 | 0.79 | 0.72 | 0.84 | 0.72 | 0.73 | 0.73 | 0.67 | 0.91 | 0.85 | 0.67 |
|  | Ni (mg/L) | 0.40 | 0.79 | 0.70 | 0.68 | 0.75 | 0.82 | 0.73 | 0.75 | 0.74 | 0.69 | 0.84 | 0.85 | 0.76 |
|  | Mn (mg/L) | 0.46 | 0.97 | 0.60 | 0.68 | 0.91 | 0.98 | 0.63 | 0.64 | 0.68 | 0.54 | 0.91 | 0.81 | 0.63 |
|  | Cr (mg/L) | -0.04 | 0.45 | 0.50 | 0.56 | 0.61 | 0.54 | 0.50 | 0.51 | 0.54 | 0.49 | 0.32 | 0.58 | 0.62 |
|  | As (mg/L) | 0.40 | 0.35 | -0.28 | -0.10 | 0.53 | 0.35 | -0.25 | -0.23 | -0.21 | -0.29 | 0.22 | 0.05 | -0.12 |
|  | Al (mg/L) | 0.70 | 0.64 | 0.02 | 0.33 | 0.67 | 0.66 | 0.03 | 0.03 | 0.09 | -0.08 | 0.49 | 0.23 | -0.01 |
|  | Hg (mg/L) | 0.35 | 0.42 | -0.09 | 0.02 | 0.31 | 0.33 | -0.09 | -0.11 | -0.07 | -0.19 | 0.42 | 0.09 | -0.24 |

Note: Correlation: (Bold: >0.70 – 1.00; italics: 0.40 – 0.69; other: 0.00 – 0.39) at significance p > 0.05.

Table S6b: Pearson and Spearman Correlation for Heavy Metal Concentrations of Borehole Locations

| Spearman Correlation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pearson correlation |  | NO₃ˉ (mg/L) | Zn (mg/L) | Cu (mg/L) | Fe (mg/L) | Pb (mg/L) | Cd (mg/L) | Ni (mg/L) | Mn (mg/L) | Cr (mg/L) | As (mg/L) | Al (mg/L) | Hg (mg/L) |
|  | Ph | 0.23 | -0.23 | 0.06 | 0.32 | 0.20 | 0.30 | 0.37 | 0.24 | -0.27 | 0.32 | 0.47 | 0.45 |
|  | Temp. (°C) | 0.63 | -0.56 | 0.81 | 0.81 | 0.79 | 0.78 | 0.72 | 0.76 | 0.41 | 0.28 | 0.67 | 0.38 |
|  | EC (µS/cm) | 0.80 | -0.30 | 0.77 | 0.81 | 0.79 | 0.88 | 0.73 | 0.90 | 0.68 | 0.14 | 0.69 | 0.38 |
|  | TDS (mg/L) | 0.76 | -0.45 | 0.86 | 0.82 | 0.72 | 0.88 | 0.78 | 0.89 | 0.73 | 0.17 | 0.68 | 0.31 |
|  | BOD (mg/L) | 0.83 | -0.56 | 0.62 | 0.75 | 0.70 | 0.75 | 0.71 | 0.90 | 0.59 | 0.54 | 0.68 | 0.32 |
|  | Salinity (%) | 0.79 | -0.45 | 0.81 | 0.79 | 0.71 | 0.88 | 0.75 | 0.83 | 0.62 | 0.37 | 0.74 | 0.18 |
|  | K (mg/L) | 0.75 | -0.46 | 0.86 | 0.83 | 0.76 | 0.84 | 0.79 | 0.81 | 0.62 | 0.23 | 0.62 | 0.23 |
|  | Na (mg/L) | 0.84 | -0.29 | 0.71 | 0.87 | 0.69 | 0.76 | 0.75 | 0.91 | 0.72 | 0.19 | 0.56 | 0.10 |
|  | Ca (mg/L) | 0.84 | -0.29 | 0.77 | 0.82 | 0.78 | 0.82 | 0.70 | 0.94 | 0.66 | 0.13 | 0.59 | 0.31 |
|  | Mg (mg/L) | 0.91 | -0.10 | 0.58 | 0.87 | 0.79 | 0.82 | 0.86 | 0.89 | 0.44 | 0.35 | 0.38 | 0.13 |
|  | TH (mg/L) | 0.73 | -0.41 | 0.66 | 0.89 | 0.80 | 0.93 | 0.88 | 0.74 | 0.32 | 0.33 | 0.68 | 0.39 |
|  | HCO₃ˉ (mg/L) | 0.78 | -0.44 | 0.72 | 0.88 | 0.71 | 0.89 | 0.83 | 0.88 | 0.66 | 0.26 | 0.63 | 0.25 |
|  | SO₄²ˉ (mg/L) | 0.93 | -0.09 | 0.59 | 0.87 | 0.76 | 0.80 | 0.78 | 0.96 | 0.58 | 0.21 | 0.46 | 0.18 |
|  | NO₃ˉ (mg/L) |  | -0.11 | 0.45 | 0.81 | 0.77 | 0.74 | 0.76 | 0.93 | 0.56 | 0.47 | 0.51 | 0.14 |
|  | Zn (mg/L) | -0.41 |  | -0.62 | -0.33 | -0.27 | -0.38 | -0.28 | -0.27 | -0.12 | -0.38 | -0.74 | -0.39 |
|  | Cu (mg/L) | 0.60 | -0.59 |  | 0.75 | 0.62 | 0.74 | 0.63 | 0.64 | 0.46 | -0.04 | 0.62 | 0.25 |
|  | Fe (mg/L) | 0.70 | -0.14 | 0.75 |  | 0.80 | 0.87 | 0.92 | 0.83 | 0.39 | 0.21 | 0.55 | 0.18 |
|  | Pb (mg/L) | 0.78 | -0.41 | 0.70 | 0.64 |  | 0.73 | 0.74 | 0.78 | 0.28 | 0.31 | 0.47 | 0.52 |
|  | Cd (mg/L) | 0.77 | -0.38 | 0.75 | 0.86 | 0.64 |  | 0.87 | 0.82 | 0.46 | 0.28 | 0.70 | 0.32 |
|  | Ni (mg/L) | 0.81 | -0.31 | 0.65 | 0.86 | 0.61 | 0.86 |  | 0.76 | 0.34 | 0.35 | 0.46 | 0.21 |
|  | Mn (mg/L) | 0.96 | -0.57 | 0.75 | 0.72 | 0.83 | 0.82 | 0.83 |  | 0.69 | 0.31 | 0.59 | 0.31 |
|  | Cr (mg/L) | 0.55 | -0.36 | 0.55 | 0.40 | 0.25 | 0.31 | 0.41 | 0.52 |  | 0.01 | 0.33 | -0.04 |
|  | As (mg/L) | 0.40 | -0.48 | -0.11 | -0.10 | 0.10 | 0.16 | 0.33 | 0.38 | 0.08 |  | 0.44 | 0.12 |
|  | Al (mg/L) | 0.56 | -0.77 | 0.48 | 0.18 | 0.40 | 0.54 | 0.33 | 0.63 | 0.31 | 0.42 |  | 0.47 |
|  | Hg (mg/L) | 0.33 | -0.37 | 0.21 | -0.04 | 0.51 | 0.26 | 0.11 | 0.38 | -0.11 | 0.10 | 0.52 |  |

Note: Correlation: (Bold: >0.70 – 1.00; italics: 0.40 – 0.69; other: 0.00 – 0.39) at significance p > 0.05.

Table S7: Ecological Risk Indices

| Parameters | BH 1 | BH 2 | BH 3 | BH 4 | BH 5 | BH 6 | BH 7 | BH 8 | BH 9 | BH 10 | BH 11 | BH 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K (mg/L) | 0.46 | 0.57 | 0.55 | 2.34 | 0.50 | 0.50 | 0.08 | 0.05 | 0.07 | 0.04 | 0.07 | 0.07 |
| Na (mg/L) | 0.07 | 0.08 | 0.08 | 0.35 | 0.09 | 0.08 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Ca (mg/L) | 0.20 | 0.34 | 0.30 | 1.13 | 0.27 | 0.32 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 | 0.02 |
| Mg (mg/L) | 0.05 | 0.06 | 0.05 | 0.40 | 0.08 | 0.07 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 | 0.01 |
| Zn (mg/L) | 0.01 | 0.01 | 0.01 | 0.02 | 0.01 | 0.01 | 0.00 | 0.02 | 0.01 | 0.02 | 0.02 | 0.02 |
| Cu (mg/L) | 0.04 | 0.04 | 0.05 | 0.06 | 0.03 | 0.05 | 0.04 | 0.02 | 0.03 | 0.01 | 0.02 | 0.01 |
| Fe (mg/L) | 0.17 | 0.07 | 0.23 | 0.47 | 0.23 | 0.13 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 | 0.03 |
| Pb (mg/L) | 15.00 | 15.00 | 13.00 | 14.00 | 5.00 | 6.00 | 1.00 | 2.00 | 2.00 | 2.00 | 2.00 | 2.00 |
| Cd (mg/L) | 170.00 | 10.00 | 140.00 | 213.33 | 140.00 | 176.67 | 6.67 | 3.33 | 3.33 | 3.33 | 6.67 | 10.00 |
| Ni (mg/L) | 2.00 | 1.50 | 2.00 | 3.00 | 3.00 | 2.00 | 0.50 | 0.50 | 1.00 | 0.50 | 1.50 | 1.00 |
| Mn (mg/L) | 0.50 | 0.53 | 0.50 | 0.53 | 0.53 | 0.53 | 0.17 | 0.17 | 0.17 | 0.17 | 0.13 | 0.20 |
| Cr (mg/L) | 0.80 | 2.20 | 1.00 | 2.20 | 2.00 | 2.00 | 2.00 | 0.40 | 1.20 | 1.20 | 0.60 | 1.40 |
| As (mg/L) | 0.30 | 0.30 | 0.20 | 0.10 | 0.40 | 0.30 | 0.20 | 0.20 | 0.10 | 0.20 | 0.30 | 0.20 |
| Al (mg/L) | 2.10 | 1.80 | 2.55 | 1.20 | 1.80 | 2.85 | 1.90 | 1.05 | 1.05 | 1.15 | 1.10 | 1.65 |
| Hg (mg/L) | 0.30 | 0.20 | 0.20 | 0.10 | 0.10 | 0.20 | 0.10 | 0.10 | 0.20 | 0.10 | 0.10 | 0.20 |
| N | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Cdeg | 192.00 | 32.70 | 160.72 | 239.23 | 154.04 | 191.71 | 12.73 | 7.91 | 9.23 | 8.79 | 12.58 | 16.82 |
| MCdeg | 12.80 | 2.18 | 10.71 | 15.95 | 10.27 | 12.78 | 0.85 | 0.53 | 0.62 | 0.59 | 0.84 | 1.12 |
| PLI | 0.46 | 0.39 | 0.47 | 0.78 | 0.47 | 0.49 | 0.00 | 0.11 | 0.13 | 0.11 | 0.13 | 0.15 |
| NPI | 9.56 | 2.93 | 8.68 | 10.71 | 8.67 | 9.73 | 1.94 | 1.39 | 1.40 | 1.40 | 1.94 | 2.36 |

Table S8: Water Quality Assessment

| Parameters | BH 1 | BH 2 | BH 3 | BH 4 | BH 5 | BH 6 | BH 7 | BH 8 | BH 9 | BH 10 | BH 11 | BH 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WQI | 149.15 | 34.86 | 138.22 | 146.36 | 141.17 | 140.36 | 69.50 | 65.64 | 73.86 | 62.68 | 78.12 | 94.73 |
| EC/TDS | 1.94 | 2.00 | 2.00 | 2.00 | 1.82 | 0.57 | 1.93 | 2.00 | 2.27 | 3.00 | 1.80 | 2.85 |
| CBE | -20.35 | -5.40 | 15.39 | 9.06 | -25.52 | -15.19 | -46.96 | -36.74 | -44.25 | -35.77 | -35.25 | -49.13 |
| TH | 46.43 | 75.93 | 65.68 | 292.93 | 66.40 | 73.94 | 5.49 | 5.99 | 5.19 | 5.04 | 4.84 | 6.30 |
| TA | 40.02 | 30.01 | 20.01 | 79.21 | 41.25 | 37.72 | 10.00 | 5.00 | 8.61 | 6.23 | 5.41 | 9.43 |
| SAR | 4.58 | 4.18 | 4.35 | 9.73 | 5.34 | 4.32 | 2.26 | 1.63 | 2.31 | 2.71 | 1.64 | 2.08 |
| Percent Na | 37.15 | 30.86 | 32.80 | 34.65 | 38.16 | 32.36 | 43.77 | 38.57 | 45.77 | 53.00 | 36.95 | 41.59 |

Table S9: Geo-Accumulation Index

| Parameter | BH 1 | BH 2 | BH 3 | BH 4 | BH 5 | BH 6 | BH 7 | BH 8 | BH 9 | BH 10 | BH 11 | BH 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pH | -0.74 | -0.81 | -0.67 | -0.81 | -0.69 | -0.67 | -0.81 | -0.78 | -0.76 | -0.76 | -0.78 | -0.76 |
| Temp. (°C) | -1.07 | -1.08 | -1.08 | -1.08 | -1.08 | -1.07 | -1.12 | -1.12 | -1.12 | -1.12 | -1.12 | -1.13 |
| EC (µS/cm) | -3.20 | -2.97 | -3.06 | -0.77 | -3.27 | -2.90 | -5.80 | -6.55 | -5.91 | -5.80 | -6.38 | -5.34 |
| TDS (mg/L) | -3.16 | -2.97 | -3.06 | -0.77 | -3.14 | -1.10 | -5.74 | -6.55 | -6.09 | -6.38 | -6.23 | -5.85 |
| BOD (mg/L) | -0.56 | -0.53 | -0.71 | -0.64 | -0.37 | -0.55 | -0.84 | -1.18 | -1.27 | -1.42 | -1.32 | -1.02 |
| Salinity (%) | 5.03 | 5.26 | 5.18 | 5.40 | 5.12 | 5.54 | 2.43 | 1.68 | 1.09 | 1.74 | 2.28 | 2.24 |
| K (mg/L) | -1.70 | -1.39 | -1.46 | 0.64 | -1.59 | -1.58 | -4.31 | -5.01 | -4.53 | -5.23 | -4.35 | -4.40 |
| Na (mg/L) | -4.49 | -4.25 | -4.29 | -2.09 | -4.02 | -4.23 | -7.06 | -7.49 | -7.08 | -6.87 | -7.64 | -7.09 |
| Ca (mg/L) | -2.93 | -2.15 | -2.33 | -0.41 | -2.49 | -2.22 | -6.10 | -6.10 | -6.23 | -6.29 | -6.46 | -5.97 |
| Mg (mg/L) | -5.01 | -4.59 | -5.01 | -1.92 | -4.23 | -4.49 | -7.79 | -7.35 | -7.74 | -7.74 | -7.55 | -7.41 |
| TH (mg/L) | -2.20 | -3.18 | -2.78 | -2.27 | -2.91 | -3.03 | -6.13 | -6.61 | -6.44 | -6.43 | -6.09 | -6.01 |
| HCO₃ˉ (mg/L) | -1.62 | -2.04 | -2.62 | -0.63 | -1.58 | -1.71 | -3.62 | -4.62 | -3.84 | -4.30 | -4.51 | -3.71 |
| SO₄²ˉ (mg/L) | -6.36 | -4.31 | -5.42 | -2.39 | -3.64 | -4.23 | -7.57 | -6.92 | -7.55 | -7.40 | -7.70 | -6.79 |
| NO₃ˉ (mg/L) | -6.20 | -5.90 | -6.19 | -5.91 | -5.85 | -6.03 | -10.29 | -8.79 | -10.55 | -7.59 | -9.87 | -6.94 |
| Zn (mg/L) | -8.14 | -7.81 | -7.97 | -6.44 | -7.81 | -7.97 | -8.81 | -6.44 | -6.68 | -6.18 | -6.49 | -6.23 |
| Cu (mg/L) | -5.23 | -5.23 | -5.01 | -4.64 | -5.49 | -5.01 | -5.23 | -6.23 | -5.49 | -6.81 | -6.23 | -6.81 |
| Fe (mg/L) | -3.17 | -4.49 | -2.68 | -1.68 | -2.68 | -3.49 | -5.49 | -5.49 | -5.49 | -5.49 | -5.49 | -5.49 |
| Pb (mg/L) | 3.32 | 3.32 | 3.12 | 3.22 | 1.74 | 2.00 | -0.58 | 0.42 | 0.42 | 0.42 | 0.42 | 0.42 |
| Cd (mg/L) | 6.82 | 2.74 | 6.54 | 7.15 | 6.54 | 6.88 | 2.15 | 1.15 | 1.15 | 1.15 | 2.15 | 2.74 |
| Ni (mg/L) | 0.42 | 0.00 | 0.42 | 1.00 | 1.00 | 0.42 | -1.58 | -1.58 | -0.58 | -1.58 | 0.00 | -0.58 |
| Mn (mg/L) | -1.58 | -1.49 | -1.58 | -1.49 | -1.49 | -1.49 | -3.17 | -3.17 | -3.17 | -3.17 | -3.49 | -2.91 |
| Cr (mg/L) | -0.91 | 0.55 | -0.58 | 0.55 | 0.42 | 0.42 | 0.42 | -1.91 | -0.32 | -0.32 | -1.32 | -0.10 |
| As (mg/L) | -2.32 | -2.32 | -2.91 | -3.91 | -1.91 | -2.32 | -2.91 | -2.91 | -3.91 | -2.91 | -2.32 | -2.91 |
| Al (mg/L) | 0.49 | 0.26 | 0.77 | -0.32 | 0.26 | 0.93 | 0.34 | -0.51 | -0.51 | -0.38 | -0.45 | 0.14 |
| Hg (mg/L) | -2.32 | -2.91 | -2.91 | -3.91 | -3.91 | -2.91 | -3.91 | -3.91 | -2.91 | -3.91 | -3.91 | -2.91 |

Table S10: Cancer Risk Evaluation for Heavy Metals in Adult Population

|  |  |  | Shallow | Borehole |  |  |  |  |  | Deep | Borehole |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Adult | BH 1 | BH 2 | BH 3 | BH 4 | BH 5 | BH 6 | BH 7 | BH 8 | BH 9 | BH 10 | BH 11 | BH 12 | Σ (sum) |
| Pb | 2.57E-07 | 2.57E-07 | 2.23E-07 | 2.40E-07 | 8.58E-08 | 1.03E-07 | 1.72E-08 | 3.43E-08 | 3.43E-08 | 3.43E-08 | 3.43E-08 | 3.43E-08 | 1.36E-06 |
| Cd | 6.49E-04 | 3.82E-05 | 5.34E-04 | 8.14E-04 | 5.34E-04 | 6.74E-04 | 2.54E-05 | 1.27E-05 | 1.27E-05 | 1.27E-05 | 2.54E-05 | 3.82E-05 | 3.37E-03 |
| Ni | 6.78E-06 | 5.09E-06 | 6.78E-06 | 1.02E-05 | 1.02E-05 | 6.78E-06 | 1.70E-06 | 1.70E-06 | 3.39E-06 | 1.70E-06 | 5.09E-06 | 3.39E-06 | 6.28E-05 |
| Cr VI | 4.04E-06 | 1.11E-05 | 5.05E-06 | 1.11E-05 | 1.01E-05 | 1.01E-05 | 1.01E-05 | 2.02E-06 | 6.06E-06 | 6.06E-06 | 3.03E-06 | 7.07E-06 | 8.58E-05 |
| As | 2.60E-09 | 2.60E-09 | 1.74E-09 | 8.68E-10 | 3.47E-09 | 2.60E-09 | 1.74E-09 | 1.74E-09 | 8.68E-10 | 1.74E-09 | 2.60E-09 | 1.74E-09 | 2.43E-08 |
| ΣCR | 6.60E-04 | 5.46E-05 | 5.46E-04 | 8.36E-04 | 5.55E-04 | 6.91E-04 | 3.73E-05 | 1.65E-05 | 2.22E-05 | 2.05E-05 | 3.36E-05 | 4.87E-05 | 3.52E-03 |

Table S11: Cancer Risk Evaluation for Heavy Metals in Children Population

|  |  |  | Shallow | Borehole |  |  |  |  |  | Deep | Borehole |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Children | BH 1 | BH 2 | BH 3 | BH 4 | BH 5 | BH 6 | BH 7 | BH 8 | BH 9 | BH 10 | BH 11 | BH 12 | Σ (sum) |
| Pb | 4.00E-07 | 4.00E-07 | 3.47E-07 | 3.74E-07 | 1.33E-07 | 1.60E-07 | 2.67E-08 | 5.34E-08 | 5.34E-08 | 5.34E-08 | 5.34E-08 | 5.34E-08 | 2.11E-06 |
| Cd | 1.01E-03 | 5.93E-05 | 8.31E-04 | 1.27E-03 | 8.31E-04 | 1.05E-03 | 3.96E-05 | 1.98E-05 | 1.98E-05 | 1.98E-05 | 3.96E-05 | 5.93E-05 | 5.24E-03 |
| Ni | 1.05E-05 | 7.91E-06 | 1.05E-05 | 1.58E-05 | 1.58E-05 | 1.05E-05 | 2.64E-06 | 2.64E-06 | 5.27E-06 | 2.64E-06 | 7.91E-06 | 5.27E-06 | 9.76E-05 |
| Cr VI | 6.28E-06 | 1.73E-05 | 7.85E-06 | 1.73E-05 | 1.57E-05 | 1.57E-05 | 1.57E-05 | 3.14E-06 | 9.42E-06 | 9.42E-06 | 4.71E-06 | 1.10E-05 | 1.33E-04 |
| As | 4.05E-09 | 4.05E-09 | 2.70E-09 | 1.35E-09 | 5.40E-09 | 4.05E-09 | 2.70E-09 | 2.70E-09 | 1.35E-09 | 2.70E-09 | 4.05E-09 | 2.70E-09 | 3.78E-08 |
| ΣCR | 1.03E-03 | 8.49E-05 | 8.49E-04 | 1.30E-03 | 8.62E-04 | 1.07E-03 | 5.79E-05 | 2.56E-05 | 3.45E-05 | 3.19E-05 | 5.22E-05 | 7.56E-05 | 5.47E-03 |

Table S12: Non-Carcer Risk Evaluation of Heavy Metals and Non-Metal in Adult Population

|  |  |  |  | Shallow | Borehole |  |  |  | Deep | Borehole |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Adult | BH 1 | BH 2 | BH 3 | BH 4 | BH 5 | BH 6 | BH 7 | BH 8 | BH 9 | BH 10 | BH 11 | BH 12 | Σ (sum) |
| K | 2.88E-03 | 3.58E-03 | 3.41E-03 | 1.46E-02 | 3.11E-03 | 3.12E-03 | 4.73E-04 | 2.91E-04 | 4.05E-04 | 2.49E-04 | 4.57E-04 | 4.42E-04 | 3.30E-02 |
| Na | 9.45E-03 | 1.11E-02 | 1.08E-02 | 4.97E-02 | 1.31E-02 | 1.13E-02 | 1.59E-03 | 1.18E-03 | 1.57E-03 | 1.81E-03 | 1.06E-03 | 1.55E-03 | 1.14E-01 |
| NO3- | 4.51E-04 | 5.57E-04 | 4.55E-04 | 5.52E-04 | 5.74E-04 | 5.08E-04 | 2.65E-05 | 7.51E-05 | 2.21E-05 | 1.72E-04 | 3.53E-05 | 2.69E-04 | 3.70E-03 |
| Zn | 1.88E-04 | 2.36E-04 | 2.12E-04 | 6.12E-04 | 2.36E-04 | 2.12E-04 | 1.18E-04 | 6.12E-04 | 5.18E-04 | 7.30E-04 | 5.89E-04 | 7.07E-04 | 4.97E-03 |
| Cu | 1.06E-03 | 1.06E-03 | 1.24E-03 | 1.59E-03 | 8.83E-04 | 1.24E-03 | 1.06E-03 | 5.30E-04 | 8.83E-04 | 3.53E-04 | 5.30E-04 | 3.53E-04 | 1.08E-02 |
| Fe | 5.05E-05 | 2.02E-05 | 7.07E-05 | 1.41E-04 | 7.07E-05 | 4.04E-05 | 1.01E-05 | 1.01E-05 | 1.01E-05 | 1.01E-05 | 1.01E-05 | 1.01E-05 | 4.54E-04 |
| Pb | 3.03E-02 | 3.03E-02 | 2.62E-02 | 2.83E-02 | 1.01E-02 | 1.21E-02 | 2.02E-03 | 4.04E-03 | 4.04E-03 | 4.04E-03 | 4.04E-03 | 4.04E-03 | 1.60E-01 |
| Cd | 3.60E+00 | 2.12E-01 | 2.97E+00 | 4.52E+00 | 2.97E+00 | 3.75E+00 | 1.41E-01 | 7.07E-02 | 7.07E-02 | 7.07E-02 | 1.41E-01 | 2.12E-01 | 1.87E+01 |
| Ni | 1.41E-03 | 1.06E-03 | 1.41E-03 | 2.12E-03 | 2.12E-03 | 1.41E-03 | 3.53E-04 | 3.53E-04 | 7.07E-04 | 3.53E-04 | 1.06E-03 | 7.07E-04 | 1.31E-02 |
| Mn | 4.42E-03 | 4.71E-03 | 4.42E-03 | 4.71E-03 | 4.71E-03 | 4.71E-03 | 1.47E-03 | 1.47E-03 | 1.47E-03 | 1.47E-03 | 1.18E-03 | 1.77E-03 | 3.65E-02 |
| Cr III | 1.88E-05 | 5.18E-05 | 2.36E-05 | 5.18E-05 | 4.71E-05 | 4.71E-05 | 4.71E-05 | 9.42E-06 | 2.83E-05 | 2.83E-05 | 1.41E-05 | 3.30E-05 | 4.00E-04 |
| Cr VI | 9.42E-03 | 2.59E-02 | 1.18E-02 | 2.59E-02 | 2.36E-02 | 2.36E-02 | 2.36E-02 | 4.71E-03 | 1.41E-02 | 1.41E-02 | 7.07E-03 | 1.65E-02 | 2.00E-01 |
| As | 7.07E-03 | 7.07E-03 | 4.71E-03 | 2.36E-03 | 9.42E-03 | 7.07E-03 | 4.71E-03 | 4.71E-03 | 2.36E-03 | 4.71E-03 | 7.07E-03 | 4.71E-03 | 6.60E-02 |
| Al | 2.97E-04 | 2.54E-04 | 3.60E-04 | 1.70E-04 | 2.54E-04 | 4.03E-04 | 2.69E-04 | 1.48E-04 | 1.48E-04 | 1.63E-04 | 1.55E-04 | 2.33E-04 | 2.86E-03 |
| Hg | 7.07E-04 | 4.71E-04 | 4.71E-04 | 2.36E-04 | 2.36E-04 | 4.71E-04 | 2.36E-04 | 2.36E-04 | 4.71E-04 | 2.36E-04 | 2.36E-04 | 4.71E-04 | 4.48E-03 |
| HI | 3.67E+00 | 2.98E-01 | 3.03E+00 | 4.65E+00 | 3.04E+00 | 3.81E+00 | 1.77E-01 | 8.90E-02 | 9.74E-02 | 9.91E-02 | 1.65E-01 | 2.44E-01 | 1.94E+01 |

Table S13: Non-Carcer Risk Evaluation of Heavy Metals and Non-Metal in Children Population

|  |  |  |  | Shallow | Borehole |  |  |  | Deep | Borehole |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Children | BH 1 | BH 2 | BH 3 | BH 4 | BH 5 | BH 6 | BH 7 | BH 8 | BH 9 | BH 10 | BH 11 | BH 12 | Σ (sum) |
| K | 4.14E-03 | 5.15E-03 | 4.91E-03 | 2.10E-02 | 4.47E-03 | 4.49E-03 | 6.81E-04 | 4.19E-04 | 5.83E-04 | 3.59E-04 | 6.58E-04 | 6.36E-04 | 4.75E-02 |
| Na | 1.36E-02 | 1.60E-02 | 1.56E-02 | 7.15E-02 | 1.88E-02 | 1.63E-02 | 2.29E-03 | 1.70E-03 | 2.26E-03 | 2.60E-03 | 1.53E-03 | 2.24E-03 | 1.64E-01 |
| NO3- | 6.49E-04 | 8.01E-04 | 6.55E-04 | 7.95E-04 | 8.27E-04 | 7.31E-04 | 3.81E-05 | 1.08E-04 | 3.18E-05 | 2.48E-04 | 5.09E-05 | 3.88E-04 | 5.32E-03 |
| Zn | 2.71E-04 | 3.39E-04 | 3.05E-04 | 8.82E-04 | 3.39E-04 | 3.05E-04 | 1.70E-04 | 8.82E-04 | 7.46E-04 | 1.05E-03 | 8.48E-04 | 1.02E-03 | 7.15E-03 |
| Cu | 1.53E-03 | 1.53E-03 | 1.78E-03 | 2.29E-03 | 1.27E-03 | 1.78E-03 | 1.53E-03 | 7.63E-04 | 1.27E-03 | 5.09E-04 | 7.63E-04 | 5.09E-04 | 1.55E-02 |
| Fe | 7.27E-05 | 2.91E-05 | 1.02E-04 | 2.03E-04 | 1.02E-04 | 5.81E-05 | 1.45E-05 | 1.45E-05 | 1.45E-05 | 1.45E-05 | 1.45E-05 | 1.45E-05 | 6.54E-04 |
| Pb | 4.36E-02 | 4.36E-02 | 3.78E-02 | 4.07E-02 | 1.45E-02 | 1.74E-02 | 2.91E-03 | 5.81E-03 | 5.81E-03 | 5.81E-03 | 5.81E-03 | 5.81E-03 | 2.30E-01 |
| Cd | 5.19E+00 | 3.05E-01 | 4.27E+00 | 6.51E+00 | 4.27E+00 | 5.39E+00 | 2.03E-01 | 1.02E-01 | 1.02E-01 | 1.02E-01 | 2.03E-01 | 3.05E-01 | 2.70E+01 |
| Ni | 2.03E-03 | 1.53E-03 | 2.03E-03 | 3.05E-03 | 3.05E-03 | 2.03E-03 | 5.09E-04 | 5.09E-04 | 1.02E-03 | 5.09E-04 | 1.53E-03 | 1.02E-03 | 1.88E-02 |
| Mn | 6.36E-03 | 6.78E-03 | 6.36E-03 | 6.78E-03 | 6.78E-03 | 6.78E-03 | 2.12E-03 | 2.12E-03 | 2.12E-03 | 2.12E-03 | 1.70E-03 | 2.54E-03 | 5.26E-02 |
| Cr III | 2.71E-05 | 7.46E-05 | 3.39E-05 | 7.46E-05 | 6.78E-05 | 6.78E-05 | 6.78E-05 | 1.36E-05 | 4.07E-05 | 4.07E-05 | 2.03E-05 | 4.75E-05 | 5.76E-04 |
| Cr VI | 1.36E-02 | 3.73E-02 | 1.70E-02 | 3.73E-02 | 3.39E-02 | 3.39E-02 | 3.39E-02 | 6.78E-03 | 2.03E-02 | 2.03E-02 | 1.02E-02 | 2.37E-02 | 2.88E-01 |
| As | 1.02E-02 | 1.02E-02 | 6.78E-03 | 3.39E-03 | 1.36E-02 | 1.02E-02 | 6.78E-03 | 6.78E-03 | 3.39E-03 | 6.78E-03 | 1.02E-02 | 6.78E-03 | 9.49E-02 |
| Al | 4.27E-04 | 3.66E-04 | 5.19E-04 | 2.44E-04 | 3.66E-04 | 5.80E-04 | 3.87E-04 | 2.14E-04 | 2.14E-04 | 2.34E-04 | 2.24E-04 | 3.36E-04 | 4.11E-03 |
| Hg | 1.02E-03 | 6.78E-04 | 6.78E-04 | 3.39E-04 | 3.39E-04 | 6.78E-04 | 3.39E-04 | 3.39E-04 | 6.78E-04 | 3.39E-04 | 3.39E-04 | 6.78E-04 | 6.44E-03 |
| HI | 5.29E+00 | 4.30E-01 | 4.37E+00 | 6.70E+00 | 4.37E+00 | 5.49E+00 | 2.55E-01 | 1.28E-01 | 1.40E-01 | 1.43E-01 | 2.37E-01 | 3.51E-01 | 2.79E+01 |