# Supplementary Data

Supplement 7: Calcium carbonate content and UCS of sand SH treated with MICP

Supplement 8: Tasklist 1 - without filter

| Step | Plug-in | Parameters |
| --- | --- | --- |
| 1 | Reload image |  |
| 2 | Reload Region of Interest |  |
| 3 | Reload Clipboard |  |
| 4 | Comment: Sand and calcite material |  |
| 5 | Filtering | Contrast enhancement (3D space) |
| 6 | Threshold | Lower grey threshold: 50; upper grey threshold: 255 |
| 7 | Despeckle | Sweep (3D Space); Remove all except the largest object |
| 8 | Despeckle | Remove black spackles (3D space); volume: less than 16 voxels; apply to: image |
| 9 | Despeckle | Remove black speckles (2D space); area: less than 2 pixels); apply to: image |
| 10 | Save bitmaps |  |
| 11 | 3D analysis |  |
| 12 | Comment: extract pore system |  |
| 13 | Bitwise operations | Image = NOT Image |
| 14 | Mode: Shrink-wrap (3D space) |  |
| 15 | Save bitmaps |  |

Supplement 9: Tasklist 1 – gauss filter

| Step | Plug-in | Parameters |
| --- | --- | --- |
| 1 | Reload image |  |
| 2 | Reload Region of Interest |  |
| 3 | Reload Clipboard |  |
| 4 | Comment: Sand and calcite material |  |
| 5 | Filtering | Gaussian blur (3D space); Kernel: round; radius 1.0 |
| 6 | Filtering | Contrast enhancement (3D space) |
| 7 | Threshold | Lower grey threshold: 50; upper grey threshold: 255 |
| 8 | Despeckle | Sweep (3D Space); Remove all except the largest object |
| 9 | Despeckle | Remove black spackles (3D space); volume: less than 16 voxels; apply to: image |
| 10 | Despeckle | Remove black speckles (2D space); area: less than 2 pixels); apply to: image |
| 11 | Save bitmaps |  |
| 12 | 3D analysis |  |
| 13 | Comment: extract pore system |  |
| 14 | Bitwise operations | Image = NOT Image |
| 15 | Mode: Shrink-wrap (3D space) |  |
| 16 | Save bitmaps |  |

Supplement 10: Tasklist 1 – non local means filter. non-local means filtering was applied before use in CTAn by using Fiji and NLM plug-in. There sigma=15 was applied

| Step | Plug-in | Parameters |
| --- | --- | --- |
| 1 | Reload image |  |
| 2 | Reload Region of Interest |  |
| 3 | Reload Clipboard |  |
| 4 | Comment: Sand and calcite material |  |
| 5 | Filtering | Contrast enhancement (3D space) |
| 6 | Threshold | Lower grey threshold: 50; upper grey threshold: 255 |
| 7 | Despeckle | Sweep (3D Space); Remove all except the largest object |
| 8 | Despeckle | Remove black spackles (3D space); volume: less than 16 voxels; apply to: image |
| 9 | Despeckle | Remove black speckles (2D space); area: less than 2 pixels); apply to: image |
| 10 | Save bitmaps |  |
| 11 | 3D analysis |  |
| 12 | Comment: extract pore system |  |
| 13 | Bitwise operations | Image = NOT Image |
| 14 | Mode: Shrink-wrap (3D space) |  |
| 15 | Save bitmaps |  |

Supplement 11: Tasklist 2- all samples

| Step | Plug-in | Parameters |
| --- | --- | --- |
| 1 | Comment: Analysis pore system |  |
| 2 | Thresholding: global | Lower grey threshold: 1; Upper grey threshold: 255 |
| 3 | Despeckle | Remove white speckles (2D space); Area: more than 350 000 pixels; apply to: image |
| 4 | Despekle | Remove white speckles (3D); Volume: less than 50 voxels; Apply to: image |
| 5 | Save bitmaps |  |
| 6 | Comment: Conneced ore system |  |
| 7 | Bitwise operations | Clipboard = COPY Image |
| 8 | Despeckle | Sweep (3D Space); Remove: all exept the largest object; Apply to: Image |
| 9 | 3D-Analysis |  |
| 10 | Save bitmaps |  |
| 11 | Comment: Analysis not connected pores |  |
| 12 | Bitwise operations | Image = COPY Clipboard |
| 13 | Despeckle | Sweep (3D Space); Remove: the largest object; Apply to: image |
| 14 | Save bitmaps |  |
| 15 | Individual object analysis |  |