Supplementary tables

| Center | MR scanner | Sequence | TR(ms) | TE(ms) | FOV(mm) | ST(mm) | Matrix | FA° |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Center 1 | GE SIGNA EXCITE 1.5 T | T1WI | 660 | 11.68 | 280–320 | 6 | 320×224 | 90 |
|  |  | T2WI | 4640 | 81.7 | 280–320 | 6 | 320×256 | 90 |
|  |  | axi lava 3d + C | 3.8 | 1.8 | 320–380 | 3 | 288×288 | 12 |
|  | GE SIGNA HDx 1.5 T | T1WI | 583 | 12.47 | 260–350 | 5 | 320×224 | 90 |
|  |  | T2WI | 6000 | 39.2 | 260–350 | 5 | 320×192 | 90 |
|  |  | axi Vibrant + C | 5.43 | 2.67 | 260–340 | 1.8 | 384×224 | 15 |
|  | GE DISCOVERY MR750 (750W) 3.0 T | T1WI | 603 | 7 | 320 | 5 | 384×256 | 111 |
|  |  | T2WI | 5266 | 81.28 | 320 | 5 | 320×256 | 111 |
|  |  | axi WATER Vibrant + C | 3.9 | 1.72 | 360 | 1.4 | 320×320 | 5 |
|  | GE SIGNA Pioneer 3.0 T | T1WI | 420 | 5.71 | 340 | 5 | 320×256 | 111 |
|  |  | T2WI | 3382 | 107.8 | 340 | 5 | 416×256 | 111 |
|  |  | axi WATER Disco + C | 4.11 | 1.69 | 320 | 2 | 288×320 | 12 |
|  | SIMENS TrioTim 3.0 T | T1WI | 6.04 | 2.45 | 280–360 | 1.3 | 448×336 | 18.6 |
|  |  | T2WI | 5690 | 83 | 280–360 | 4 | 512×384 | 140 |
|  |  | fl3d_tra T1+ C | 4.4 | 1.75 | 280–350 | 0.7 | 384×416 | 10 |
|  | SIMENS TrioTim 1.5 T | T1WI | 8.6 | 4.7 | 300 | 1.5 | 384×326 | 20 |
|  |  | T2WI | 5000 | 65 | 300 | 5 | 320×288 | 142 |
|  |  | fl3d_tra T1+ C | 4.2 | 1.53 | 370 | 1 | 384×307 | 10 |
|  | Phillips Achieva 3.0 T | T1WI | 627 | 10 | 341 | 4 | 425×424 | 90 |
|  |  | T2WI | 3640 | 60 | 341 | 4 | 424×421 | 90 |
|  |  | axi eTHRIVE + C | 4.8 | 2.4 | 340 | 2 | 444×443 | 10 |
|  | United Imaging 3.0 T | T1WI | 680 | 10.5 | 340 | 5 | 448×448 | 120 |
|  |  | T2WI | 3700 | 74.62 | 340 | 5 | 448×358 | 120 |
|  |  | axi T1WI + C | 4.32 | 1.99 | 340 | 0.67 | 336×335 | 10 |
| Center 2 | GE DISCOVERY MR750 (750W) 3.0T | T1WI | 500 | 6.5 | 320 | 4 | 320×256 | 111 |
|  |  | T2WI | 5349 | 85.97 | 320 | 5 | 320×256 | 111 |
|  |  | axi WATER Vibrant + C | 6.66 | 1.71 | 360 | 0.8 | 320×320 | 15 |
|  | Phillips Achieva 3.0T | T1WI | 542 | 8 | 350 | 4 | 341×285 | 90 |
|  |  | T2WI | 5000 | 65 | 341 | 4 | 341×324 | 90 |
|  |  | axi eTHRIVE + C | 4.5 | 2.3 | 340 | 1.5 | 405×400 | 12 |

Table S1. MRI acquisition parameters in two centers

Abbreviations: T1WI, T1-weighted imaging; T2WI, T2-weighted imaging; TR, repetition time; TE, echo time; FOV, field of view; ST, slice thickness; FA, flip angle.

Table S2. The definition of key radiomic features of combined model

| Key features | Feature type | Image processing methods | Definition of features |
| --- | --- | --- | --- |
| RF1 | First order | lbp-3D-k | Minimum |
| RF2 | First order | lbp-3D-m1 | Median |
| RF3 | First order | square | Range |
| RF4 | First order | wavelet_HLH | Mean |
| RF5 | First order | wavelet_LHL | Robust Mean Absolute Deviation |
| RF6 | First order | wavelet_LLL | Skewness |
| RF7 | First order | wavelet-HLH | Mean |
| RF8 | First order | wavelet-LHL | Skewness |
| RF9 | First order | wavelet-LLH | Skewness |
| RF10 | Shape | original | Elongation_y |
| RF11 | GLCM | lbp-3D-k | Correlation |
| RF12 | GLCM | lbp-3D-k | MCC |
| RF13 | GLCM | logarithm | Cluster Shade |
| RF14 | GLCM | log-sigma-3-0-mm-3D | Inverse Variance |
| RF15 | GLCM | wavelet_LHL | Idn |
| RF16 | GLCM | wavelet_LLL | Imc2 |
| RF17 | GLDM | lbp-3D-k | Small Dependence High Gray Level Emphasis |
| RF18 | GLRLM | lbp-3D-m1 | Gray Level Variance |
| RF19 | GLRLM | wavelet_LHH | Gray Level Variance |
| RF20 | GLRLM | wavelet_LHH | Run Variance |
| RF21 | GLSZM | lbp-3D-k | Small Area Low Gray Level Emphasis |
| RF22 | GLSZM | lbp-3D-m2 | Zone Entropy |
| RF23 | GLSZM | wavelet-HHH | Small Area Low Gray Level Emphasis |
| RF24 | NGTDM | exponential | Strength |
| RF25 | NGTDM | wavelet_LLL | Busyness |

Note: RF, radiomic feature; GLCM, gray-level co-occurrence matrix; GLDM, gray-level dependence matrix; GLRLM, gray-level run-length matrix; GLSZM, gray-level size zone matrix; NGTDM, neighborhood gray-tone difference matrix; LOG, Laplacian of Gaussian; Wavelet, wavelet transform; LBP, local binary pattern.

Supplementary figures

Figure S1. Key radiomic features weights of the combined model

Figure S2. Ten-fold cross-validation results of each machine learning model

To evaluate the robustness of models, ten-fold cross-validation was conducted on the training set. As depicted in the figure, the average performance across the different machine learning models was comparable, however, the models built with LR, SVM, KNN, and XGBoost demonstrated the highest stability.