Effects of microplastics on soil carbon pool and terrestrial plant performance

Yalan Chen 1, Yang Li 1, Xinru Liang 1, Siyuan Lu 2, Jiaqi Ren 1, Yuqin Zhang 1, Zichen Han 1, Bo Gao 3, Ke Sun 1,*

a State Key Laboratory of Water Environment Simulation, School of Environment, Beijing Normal University, Beijing 100875, China

b State Key Laboratory of Simulation and Regulation of Water Cycle in River Basin, China Institute of Water Resources and Hydropower Research, Beijing 100038, China

*Corresponding authors: (86)‐10‐58807493 (phone), (86)‐10‐58807493 (fax).

Email: sunke@bnu.edu.cn (K. SUN)

Fig. S1 Global microplastics abundance in different soils. Here, the data expressed as mass concentration (mg per kilogram soil).

Table S1 Occurrence and characteristics of microplastics in different soils.

| Soil type | Study area | Sampling depth | Extraction method* | Digestion method | Identification method | Abundance (items kg−1) | Mass concentration (mg kg−1) | Size distribution | Polymer constitution | Morphology | Sources | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Industrial soils | Sydney, Australia | N/A | Pressurized fluid extraction | N/A | Microscope, FTIR | N/A | 300–67,500 | N/A | PVC (>80%), PE, PS | N/A | N/A | (Fuller and Gautam, 2016) |
| Floodplain soils | Switzerland | 5 cm | NaCl (1.2 g cm–3) | HNO3 | FTIR > Raman | 0–593 | 5 (0–55.5) | 125–5,000 µm (88% <500 µm) | PE (88%) > PS > PVC > PP | N/A | Plastic waste, diffuse aeolian transport | (Scheurer and Bigalke, 2018) |
| Coastal soils | Shandong, China | 2 cm | NaCl (1.2 g cm−3), NaI (1.6 g cm−3) | N/A | Microscope, FTIR | 740 ± 2,458 (1.3–14,713) | N/A | <5 mm (∼ 60% < 1 mm, ~30% of 100–250 μm) | PE, PP, PS, PEU | Flake (69.0%), foam (27.8%), fragment (1.1%), fiber (1.0%) | Human activity, degradation | (Zhou et al., 2018) |
| Coastal soils | Hebei, China | 2 cm | NaCl (1.2 g cm−3), NaI (1.8 g cm−3) | 2 M HCl, 24 h | Microscope | 634 | N/A | 120–4,670 μm (49.8% <1 mm) | N/A | Granules (76.3%), fragment (20.5%), fiber (2.2%), film (1.0%) | N/A | (Zhou et al., 2016) |
| Riparian soils (2019) | Three Gorges Reservoir, China | 10 cm | ZnCl2 (1.6 g cm−3) | 30%H2O2+0.05M FeSO4 | μ-FTIR | 7,633 ± 4,036 | 44 ± 41 | 40–3,610 µm (97% < 1 mm, 83.3% <300 µm) | PE (60.2%), PP (26.5%), PS, PVC, PET, PU, PAM, PA | Fragment (85.5%), fiber (11.7%), film (2.8%) | N/A | (Chen et al., 2022) |
| Riparian soil (2020) | Three Gorges Reservoir, China | 10 cm | ZnCl2 (1.6 g cm−3) | 30%H2O2+0.05M FeSO4 | μ-FTIR | 4,875 ± 1,722 | 18 ± 7 | 40–2,330 µm (87.4% <300 µm) | PE (75.5%), PP (9.9%), PS, PVC, PET, PU, PAM, PA | Fragment (88.3%), fiber (11.4%), film (0.3%) | N/A | (Chen et al., 2022) |
| Riparian soils | Xiangxi River, China | 2 cm | Potassium formate (1.5 g cm−3) | 30%H2O2 | FTIR | 10.6–280.4 | N/A | 100–5,000 µm (41–77% > 1 mm) | PP (26–99%), PE, PS, PP-PE copolymer | Sheet (55–94%), fragment, line, foam | Dumping site, consumer behavior of the local residents | (Zhang et al., 2019) |
| Agricultural soils | Wuhan, China | 5 cm | ZnCl2 (1.6 g cm−3) | N/A | micro-Raman | 2020 (320‒12,560) | N/A | <200 µm (70%), 200‒500 µm (9%), 500‒1,000 µm (13%), 1,000‒5,000 (7%) | PA (32.5%), PP (28.8%), PS (16.9%), PE (4.2%), PVC (1.9%) | Bead (48%), fiber (37%), fragment (15%), foam (1%) | Littering, wastewater, green nets, tire tread | (Chen et al., 2020) |
| Agricultural soils | Franconia, Germany | 5 cm | Size fractioning (1 mm and 5mm sieve) | H2O2 | Microscope, FTIR | 0.34 ± 0.36 (0–1.25) | 0.027 | 1–5,000 µm | PE (62.5%), PP (25%), PS (12.5%) | Film (43.75%), fragment (43.75%), fiber (12.5%) | Atmospheric transport, degradation | (Piehl et al., 2018) |
| Agricultural soils | Shanghai, China | 3 cm | NaCl (1.2 g cm−3) | 30%H2O2, 50 °C, 72 h | Microscope, μ-FTIR | 78.00 ± 12.91 | N/A | 300–5,000 µm (48.79% <1 mm) | PP (50.51%), PE (43.43%), PES (6.06%) | Fiber (53.33%), fragment (37.58%), film (6.67%) | Plastic mulching, sewage sludge, degradation | (Liu et al., 2018) |
| Agricultural soils | Shanghai, China | 3–6 cm | NaCl (1.2 g cm−3) | 30%H2O2, 50 °C, 72 h | Microscope, μ-FTIR | 62.50 ± 12.97 | N/A | 300–5,000 µm (59.81% <1 mm) | PP (50.51%), PE (43.43%), PES (6.06%) | Fiber (37.62%), film (33.76%), fragment (28.03%), pellets (0.32%) | Plastic mulching, sewage sludge, degradation | (Liu et al., 2018) |
| Agricultural soils | Mellipilla, Chile | 25 cm | NaCl (1.2 g cm−3), ZnCl2 (1.6 g cm−3) | N/A | Microscope | 600–10,400 | 0.57–12.9 | 20 µm× 0.97 mm | N/A | Fiber (97 ± 3%) | Sewage sludge | (Corradini et al., 2019) |
| Agricultural soils | Kunming, China | 10 cm | NaI (1.8 g cm−3) | 35%H2O2+10%FeSO4, 50 °C | Microscope | 18,760 (7,100–42,960) | N/A | 50–250 µm (82%), 250–1,000 µm (13%), 1–10 mm (5%) | N/A | Fiber (92.0%), fragment (4.0%), film (3.7%) | Sewage sludge, wastewater, atmospheric deposition | (Zhang and Liu, 2018) |
| Agricultural soils | Northern Jutland, Denmark | 15 cm | ZnCl2 (1.6 g cm−3) | N/A | FTIR | 82,000 (0–165,000) | 6.26 (0–16.5) | 10–130 µm | PP (56%), PE (39%), Nylon (5%) | N/A | Sewage sludge | (Vollertsen and Hansen, 2017) |
| Agricultural soils | Northern Jutland, Denmark | 15 cm | ZnCl2 (1.6 g cm−3) | N/A | FTIR | 236,000 (53,000–528,000) | 51.1 (0.1–224.3) | 20–245 µm | PE (89%), Nylon (12%), PP (1%) | N/A | Sewage sludge | (Vollertsen and Hansen, 2017) |
| Agricultural soils | Loess plateau, China | 10 cm | Flotation | N/A | Microscope, heating at 130 °C | 40 ± 126 | 8 ± 25 | >100 µm | N/A | N/A | Plastic mulching | (Zhang et al., 2018) |
| Agricultural soils | Loess plateau, China | 10–30 cm | Flotation | N/A | Microscope, heating at 130 °C | 100 ± 141 | 368 ± 740 | >100 µm | N/A | N/A | Plastic mulching | (Zhang et al., 2018) |
| Agricultural soil | Hebei, China | N/A | N/A | 30%H2O2 | TOF-SIMS | N/A | N/A | <25 μm | PA6 (28%) > PET (25%) > PVC (12.5%) | N/A | N/A | (Du et al., 2020b) |
| Agricultural soil | Hebei, China | 30 cm | Distilled water | N/A | Microscope, FTIR | 107 | 0.002‒0.27 | N/A | PE | N/A | N/A | (Zhang et al., 2020) |
| Agricultural soil | Shihezi, China | 20 cm | Distilled water | 98% H2SO4 | ATR-FTIR | N/A | 369.55 ± 6.15 | 100 μm (fiber), 125‒800 μm (fragment) | PE | Fiber, fragment | Plastic mulching | (Li et al., 2020) |
| Agricultural soil | Nanjing and Wuxi, China | N/A | NaCl (1.2 g cm−3), NaI (1.8 g cm−3), ZnCl2 (1.6 g cm−3) | 30%H2O2, 70 °C | μ-FTIR | 200‒740 | N/A | 20‒5,000 μm (>70% of 20‒250 μm) | PE, PP | Bulk, fragment, fiber | Plastic mulching | (Li et al., 2019) |
| Agricultural soil | Xinjiang, China | 40 cm | NaI (1.8 g cm−3) | 30% H2O2, 50 °C, 72 h | μ-FTIR | 80.3 ± 49.3‒1075.6 ± 346.8 | N/A | N/A | PE | N/A | Plastic mulching | (Huang et al., 2020) |
| Agricultural soil | Shaanxi, China | 10 cm | NaCl and CaCl2 | H2O2, 65 °C, 12 h | FTIR | 1,430‒3,410 | N/A | 0‒5,000 μm (81% of <490 μm) | PS, PE, PP, PVC, PET | Film (23.8%), fragment (21.9%) | Agricultural plastic waste | (Ding et al., 2020) |
| Agricultural soil | Yunnan, China | 30 cm | NaI (1.8 g cm−3) | Fenton | Microscope | 9,800 (900‒40,800) | N/A | <500 µm (89.3%) | – | Fragment (78.3%) | N/A | (Huang et al., 2021) |
| Agricultural soil | Mellipilla, Chile | N/A | Water, NaCl, ZnCl2 | N/A | Microscope, μ-FTIR | 306 ± 360 (crop land), 184 ± 266 (pasture) | N/A | 0.076 ± 0.292 mm2 | PUR, PE, PP, PS. PES, PA, etc. | Fiber (68%), film (23%), fragment (7%), pellet (2%) | N/A | (Corradini et al., 2021) |
| Agricultural soil | Ontario, Canada | 15 cm | N/A | Fenton | ATR-FTIR, μ-FTIR | 18‒541 | N/A | N/A | PP, PE, PES | Fiber > fragment | Biosolids | (Crossman et al., 2020) |
| Agricultural soil | Tibetan Plateau, China | 3 cm | NaCl (1.2 g cm−3) | 0.05 M Fe(II), 30%H2O2, 75 °C, 72 h | Raman | 52.35 ± 20.41 | N/A | <50 μm (38.2%), 50‒100 μm (25.84%), 100‒500 μm (23.59%) | PE (43.75%), PA (26.25%), PP (10%), PS (7.5%) | Film (49.44%), fiber (24.72%), fragment (8.99%), sphere (8.99%), foam (7.86%) | Facility agriculture and previous secondary industry | (Feng et al., 2020) |
| Agricultural soil | Tibetan Plateau, China | 3‒6 cm | NaCl (1.2 g cm−3) | 0.05 M Fe(II), 30%H2O2, 75 °C, 72 h | Raman | 43.53 ± 19.45 | N/A | <50 μm (38.2%), 50‒100 μm (25.84%), 100‒500 μm (23.59%) | PE (49.27%), PA (21.74%), PS (8.69%), PP (7.25%) | Film (52.7%), fiber (13.51%), fragment (12.16%), sphere (6.76%), foam (14.86%) | Facility agriculture and previous secondary industry | (Feng et al., 2020) |
| Fruit soils | Loess plateau, China | 10 cm | Flotation | N/A | Microscope, heating at 130 °C | 320 ± 329 | 540 ± 603 | >100 µm (87.5%), 50–100 µm (12.5%) | N/A | N/A | Plastic mulching | (Zhang et al., 2018) |
| Fruit soils | Loess plateau, China | 10–30 cm | Flotation | N/A | Microscope, heating at 130 °C | 120 ± 169 | 460 ± 735 | >100 µm (83.3%), 50–100 µm (16.7%) | N/A | N/A | Plastic mulching | (Zhang et al., 2018) |
| Greenhouse soils | Loess plateau, China | 10 cm | Flotation | N/A | Microscope, heating at 130 °C | 100 ± 254 | 130 ± 307 | >100 µm (80%), 50–100 µm (20%) | N/A | N/A | Plastic mulching | (Zhang et al., 2018) |
| Greenhouse soils | Loess plateau, China | 10–30 cm | Flotation | N/A | Microscope, heating at 130 °C | 80 ± 193 | 24 ± 51 | >100 µm (50%), <50 µm (50%) | N/A | N/A | Plastic mulching | (Zhang et al., 2018) |
| Agricultural soils on a coastal plain | Hangzhou Bay, China | 10 cm | Sodium hexametaphosphate (0.5 mol L−1), NaCl (1.2 g cm−3), NaI (1.6 g cm−3) | 30%H2O2, 70 °C, 72 h | μ-FTIR | Overall: 503.3 (0-2760), mulching: 571, non-mulching: 263 | N/A | ≥ 60 μm | PE, PP-PE copolymer, PP, PES, nylon | Fragment (domain), fiber, film | Plastic mulching, irrigation, plastic wastes, compost | (Zhou et al., 2020) |
| Paddy soils (rice soils) | Shanghai, China | 10 cm | NaCl (1.2 g cm−3) | N/A | Microscope, μ-FTIR | 16.1 ± 3.5 (1.7–41.3) | N/A | 20–1,000 µm (domain) | PE (61.4%), PP (35.1%), PVC (3.5%) | Fiber (domain), fragment, film, granule | Agriculture fertilizers, fish diets, atmospheric deposition | (Lv et al., 2019) |
| Paddy soils (aquaculture soils) | Shanghai, China | 10 cm | NaCl (1.2 g cm−3) | N/A | Microscope, μ-FTIR | 4.5 ± 1.2 (1.7–13.3) | N/A | 20–1,000 µm (domain) | PE (61.4%), PP (35.1%), PVC (3.5%) | Fiber (domain), fragment, film, granule | Agriculture fertilizers, fish diets, atmospheric deposition | (Lv et al., 2019) |
| Home garden soils | Campeche, Mexico | 20 cm | H2O | N/A | Microscope | 870 ± 1,900 | N/A | 10–20 µm (59.3%), 20-50 µm (34.4%), >50 µm (4.8%) | N/A | N/A | Domestic waste | (Huerta Lwanga et al., 2017) |
| Wetland soil | Washington, D.C., USA | 5 cm | Wet sieving | 30%H2O2+0.05 M FeSO4, 70 °C | Microscope, FTIR | 1,270 ± 150 (334–3,068) | N/A | N/A | PS (29%), PE, synthetic rubber (8%) | Fiber and fragment | N/A | (Helcoski et al., 2020) |
| Construction land, Farmland, Forest land | Baoding City, China | 10 cm | N/A | 30%H2O2 | TOF-SIMS | N/A | N/A | <35 μm | PA6 ~ PET > PP > PVC | N/A | N/A | (Du et al., 2020a) |
| Vacant land | Valencia, Spain | 30 cm | NaI (1.8 g cm−3) | N/A | Microscope, μ-FTIR | 2,030‒5,190 | N/A | Mainly of 150–250 μm | PP, PVC | Fragment, fiber, film | Sewage sludge | (van den Berg et al., 2020) |
| Vacant land | Hesse, Germany | 30 cm | N/A | N/A | ATR-FTIR | 2.06 (0.31‒8.59) | N/A | 2,000‒5,000 μm | PE (16%), PP (6%), PA (5%), PS (4%), POM (4%), PET (3%) | Fragment (32%), film (32%), fiber (19%) | N/A | (Weber and Opp, 2020) |
| Vacant land, woodland, farmland | Wuhan, China | 5 cm | NaCl (1.2 g cm−3) | 30% KOH: NaClO | Raman | 220,000 (22,000‒690,000) | N/A | 10–50 µm (46.1%), 50–100 µm (35.3%), 100–500 µm (18.1%), >500 µm (0.2%) | PE (36.1%), PP (11.5%), PS (8.9%), PA (17.3%), PVC (8.5%) | Fragment (52%), bead (14%), fiber (13.8%), foam (9.7%), film (1.6%) | Plastic mulching, sewage sludge, wastewater | (Zhou et al., 2019) |

*Unless otherwise stated, the extraction method refers to density separation method and the solution type and density are provided. “N/A” represents the information is not provided.

Table S2 Density information of commonly used microplastics.

| Polymer type | Acronym | Density (g·cm‒3) |
| --- | --- | --- |
| Polypropylene | PP | 0.88‒0.91 |
| Polystyrene | PS | 1.04‒1.05 |
| Polyethylene | PE | 0.92‒0.97 |
| Low-density polyethylene | LDPE | 0.92‒0.94 |
| High-density polyethylene | HDPE | 0.94‒0.97 |
| Polyvinyl chloride | PVC | 1.45‒1.70 |
| Polyethylene terephthalate | PET | 1.40‒1.60 |
| Polyamide/Nylon | PA | 1.05‒1.10 |
| Polycarbonate | PC | 1.18‒1.22 |
| Polyurethane | PUR | 1.20‒1.40 |
| Acrylonitrile butadiene styrene | ABS | 1.01‒1.08 |
| Polymethyl methacrylate | PMMA | 1.16‒1.21 |
| Cellulose acetate | CA | 1.30‒1.35 |
| Polylactic acid | PLA | 1.20‒1.30 |

References:

Chen, Y., Gao, B., Xu, D., Sun, K. and Li, Y. (2022) Catchment-wide flooding significantly altered microplastics organization in the hydro-fluctuation belt of the reservoir. iScience 25(6), 104401.https://doi.org/10.1016/j.isci.2022.104401

Chen, Y., Leng, Y., Liu, X. and Wang, J. (2020) Microplastic pollution in vegetable farmlands of suburb Wuhan, central China. Environmental Pollution 257, 113449.https://doi.org/10.1016/j.envpol.2019.113449

Corradini, F., Casado, F., Leiva, V., Huerta-Lwanga, E. and Geissen, V. (2021) Microplastics occurrence and frequency in soils under different land uses on a regional scale. Science of the Total Environment 752, 141917.https://doi.org/10.1016/j.scitotenv.2020.141917

Corradini, F., Meza, P., Eguiluz, R., Casado, F., Huerta-Lwanga, E. and Geissen, V. (2019) Evidence of microplastic accumulation in agricultural soils from sewage sludge disposal. Science of The Total Environment 671, 411-420.https://doi.org/10.1016/j.scitotenv.2019.03.368

Crossman, J., Hurley, R.R., Futter, M. and Nizzetto, L. (2020) Transfer and transport of microplastics from biosolids to agricultural soils and the wider environment. Science of the Total Environment 724, 138334.https://doi.org/10.1016/j.scitotenv.2020.138334

Ding, L., Zhang, S., Wang, X., Yang, X., Zhang, C., Qi, Y. and Guo, X. (2020) The occurrence and distribution characteristics of microplastics in the agricultural soils of Shaanxi Province, in north-western China. Science of the Total Environment 720, 137525.https://doi.org/10.1016/j.scitotenv.2020.137525

Du, C., Liang, H., Li, Z. and Gong, J. (2020a) Pollution Characteristics of Microplastics in Soils in Southeastern Suburbs of Baoding City, China. International Journal of Environmental Research and Public Health 17(3).10.3390/ijerph17030845

Du, C., Wu, J., Gong, J., Liang, H. and Li, Z. (2020b) ToF-SIMS characterization of microplastics in soils. Surface and Interface Analysis 52(5), 293-300.https://doi.org/10.1002/sia.6742

Feng, S., Lu, H., Tian, P., Xue, Y., Lu, J., Tang, M. and Feng, W. (2020) Analysis of microplastics in a remote region of the Tibetan Plateau: Implications for natural environmental response to human activities. Science of the Total Environment 739, 140087.https://doi.org/10.1016/j.scitotenv.2020.140087

Fuller, S. and Gautam, A. (2016) A Procedure for Measuring Microplastics using Pressurized Fluid Extraction. Environmental Science & Technology 50(11), 5774-5780.10.1021/acs.est.6b00816

Helcoski, R., Yonkos, L.T., Sanchez, A. and Baldwin, A.H. (2020) Wetland soil microplastics are negatively related to vegetation cover and stem density. Environmental Pollution 256, 113391.https://doi.org/10.1016/j.envpol.2019.113391

Huang, B., Sun, L., Liu, M., Huang, H., He, H., Han, F., Wang, X., Xu, Z., Li, B. and Pan, X. (2021) Abundance and distribution characteristics of microplastic in plateau cultivated land of Yunnan Province, China. Environmental Science and Pollution Research 28(2), 1675-1688.10.1007/s11356-020-10527-3

Huang, Y., Liu, Q., Jia, W., Yan, C. and Wang, J. (2020) Agricultural plastic mulching as a source of microplastics in the terrestrial environment. Environmental Pollution 260, 114096.https://doi.org/10.1016/j.envpol.2020.114096

Huerta Lwanga, E., Mendoza Vega, J., Ku Quej, V., Chi, J.d.l.A., Sanchez del Cid, L., Chi, C., Escalona Segura, G., Gertsen, H., Salánki, T., van der Ploeg, M., Koelmans, A.A. and Geissen, V. (2017) Field evidence for transfer of plastic debris along a terrestrial food chain. Scientific Reports 7(1), 14071.10.1038/s41598-017-14588-2

Li, Q., Wu, J., Zhao, X., Gu, X. and Ji, R. (2019) Separation and identification of microplastics from soil and sewage sludge. Environmental Pollution 254, 113076.https://doi.org/10.1016/j.envpol.2019.113076

Li, W., Wufuer, R., Duo, J., Wang, S., Luo, Y., Zhang, D. and Pan, X. (2020) Microplastics in agricultural soils: Extraction and characterization after different periods of polythene film mulching in an arid region. Science of the Total Environment 749, 141420.https://doi.org/10.1016/j.scitotenv.2020.141420

Liu, M., Lu, S., Song, Y., Lei, L., Hu, J., Lv, W., Zhou, W., Cao, C., Shi, H., Yang, X. and He, D. (2018) Microplastic and mesoplastic pollution in farmland soils in suburbs of Shanghai, China. Environmental Pollution 242, 855-862.https://doi.org/10.1016/j.envpol.2018.07.051

Lv, W., Zhou, W., Lu, S., Huang, W., Yuan, Q., Tian, M., Lv, W. and He, D. (2019) Microplastic pollution in rice-fish co-culture system: A report of three farmland stations in Shanghai, China. Science of The Total Environment 652, 1209-1218.https://doi.org/10.1016/j.scitotenv.2018.10.321

Piehl, S., Leibner, A., Löder, M.G.J., Dris, R., Bogner, C. and Laforsch, C. (2018) Identification and quantification of macro- and microplastics on an agricultural farmland. Scientific Reports 8(1), 17950.10.1038/s41598-018-36172-y

Scheurer, M. and Bigalke, M. (2018) Microplastics in Swiss Floodplain Soils. Environmental Science & Technology 52(6), 3591-3598.10.1021/acs.est.7b06003

van den Berg, P., Huerta-Lwanga, E., Corradini, F. and Geissen, V. (2020) Sewage sludge application as a vehicle for microplastics in eastern Spanish agricultural soils. Environmental Pollution 261, 114198.https://doi.org/10.1016/j.envpol.2020.114198

Vollertsen, J. and Hansen, A. (2017) Microplastic in Danish wastewater: Sources, occurrences and fate.

Weber, C.J. and Opp, C. (2020) Spatial patterns of mesoplastics and coarse microplastics in floodplain soils as resulting from land use and fluvial processes. Environmental Pollution 267, 115390.https://doi.org/10.1016/j.envpol.2020.115390

Zhang, G.S. and Liu, Y.F. (2018) The distribution of microplastics in soil aggregate fractions in southwestern China. Science of The Total Environment 642, 12-20.https://doi.org/10.1016/j.scitotenv.2018.06.004

Zhang, K., Chen, X., Xiong, X., Ruan, Y., Zhou, H., Wu, C. and Lam, P.K.S. (2019) The hydro-fluctuation belt of the Three Gorges Reservoir: Source or sink of microplastics in the water? Environmental Pollution 248, 279-285.https://doi.org/10.1016/j.envpol.2019.02.043

Zhang, S., Liu, X., Hao, X., Wang, J. and Zhang, Y. (2020) Distribution of low-density microplastics in the mollisol farmlands of northeast China. Science of the Total Environment 708, 135091.https://doi.org/10.1016/j.scitotenv.2019.135091

Zhang, S., Yang, X., Gertsen, H., Peters, P., Salánki, T. and Geissen, V. (2018) A simple method for the extraction and identification of light density microplastics from soil. Science of The Total Environment 616-617, 1056-1065.https://doi.org/10.1016/j.scitotenv.2017.10.213

Zhou, B., Wang, J., Zhang, H., Shi, H., Fei, Y., Huang, S., Tong, Y., Wen, D., Luo, Y. and Barceló, D. (2020) Microplastics in agricultural soils on the coastal plain of Hangzhou Bay, east China: Multiple sources other than plastic mulching film. Journal of Hazardous Materials 388, 121814.https://doi.org/10.1016/j.jhazmat.2019.121814

Zhou, Q., Zhang, H., Fu, C., Zhou, Y., Dai, Z., Li, Y., Tu, C. and Luo, Y. (2018) The distribution and morphology of microplastics in coastal soils adjacent to the Bohai Sea and the Yellow Sea. Geoderma 322, 201-208.https://doi.org/10.1016/j.geoderma.2018.02.015

Zhou, Q., Zhang, H., Zhou, Y., Li, Y., Xue, Y., Fu, C., Tu, C. and Luo, Y. (2016) Separation of microplastics from a coastal soil and their surface microscopic features (in Chinese). Chinese Science Bulletion 61, 1604–1611.10.1360/N972015-01098

Zhou, Y., Liu, X. and Wang, J. (2019) Characterization of microplastics and the association of heavy metals with microplastics in suburban soil of central China. Science of the Total Environment 694, 133798.https://doi.org/10.1016/j.scitotenv.2019.133798