Supplement material

Table S1 Activity data and emission factor for land carbon balance assessment of Guangdong province used in this study

| Boundary | Activity data | Emission factor | Duration | Source |
| --- | --- | --- | --- | --- |
| Forest ecosystem carbon sink | Forest standing live timber volume | Wood density: 0.474 Mg m-3 Biomass conversion factor (BEFtot): 1.915 carbon conversion factor (Fc): 0.5 | 2015-2021 | Provincial Greenhouse Gas Inventory Compilation Guide (Trial), Guangdong Statistical Yearbook |
| Energy activities emission | Terminal energy consumption | Raw coal: 1.981 t CO2 t-1 Oil products: 3.018 t CO2 t-1 Other energy: 2.773 t CO2 t-1 standard coal Electricity: 6.69 t CO2 per ten thousand kilowatt-hours | 2016-2021 | As above |
| Industrial processes emission | Production of industrial products: cement and steel are mainly considered | Cement: 0.538 t CO2 t-1 cement Steel: 0.213 t CO2 t-1 steel | 2016-2021 | As above and (Li et al. 2016) |
| Agricultural activities emission | CH4 from paddy field: Rice type, sowing area | Early rice: 241.0 kg CH4 ha-1 Late rice: 273.2 kg CH4 ha-1 | 2016-2021 | Provincial Greenhouse Gas Inventory Compilation Guide (Trial), Guangdong Statistical Yearbook |
|  | N2O emission from cropland: nitrogen fertilizer amount, animal type and number | N2O from nitrogen fertilizer: 0.0178 kg N2O-N kg-1 N input N2O from atmospheric nitrogen deposition: 0.001 kg N2O-N kg-1 N N2O from leaching and runoff: 0.0075 kg N2O-N kg-1 N | 2016-2021 | As above |
|  | CH4 emission from animal intestinal fermentation: animal type and number | Buffalo: 70.5 kg CH4 head-1 year-1 Cow: 88.1 kg CH4 head-1 year-1 Goat: 8.9 kg CH4 head-1 year-1 Pig: 1 kg CH4 head-1 year-1 | 2016-2021 | As above |
|  | CH4 and N2O from animal excrement management: animal type and number | Buffalo: 8.24 kg CH4 head-1 year-1, 0.86 kg N2O head-1 year-1 Cow: 8.45 kg CH4 head-1 year-1, 1.71 kg N2O head-1 year-1 Goat: 0.31 kg CH4 head-1 year-1, 0.106 kg N2O head-1 year-1 Pig: 5.85 kg CH4 head-1 year-1, 0.157 kg N2O head-1 year-1 Poultry: 0.02 kg CH4 head-1 year-1, 0.007 kg N2O head-1 year-1 | 2016-2021 | As above |
| Waste management emission | Solid waste amount, chemical oxygen demand (COD) and nitrogen content in wastewater | Solid waste: 0.27 t CO2 t-1 solid waste Wastewater: 0.126 kg CH4 kg-1 COD, 0.005 kg N2O-N kg-1 N | 2016-2021 | As above |

Table S2 Parameters and hypothesized values in the life cycle analysis of biochar

| Parameters | Value |  |  | Source |
| --- | --- | --- | --- | --- |
| Feedstock for biochar production | Agricultural residues | Forestry residues | Bioenergy crops | This study |
| The emission factor of feedstock cultivation and collection (kg CO2e t-1 dry biomass) | 68 | 151.8 | 151.8 | (Robert et al. 2010; Gvein et al. 2023) |
| Biochar yield (%) | 31.5 | 37.6 | 33.825 | (Jiang et al. 2013; Deng et al. 2024) |
| Bio-oil (%) | 38 | 35.2 | 36.6 | (Jiang et al. 2013) |
| Bio-gas (%) | 30.5 | 27.2 | 28.85 | (Jiang et al. 2013) |
| Biochar carbon content (%) | 64 | 76.4 | 63.095 | (Jiang et al. 2013) |
| Biochar stable carbon (%) | 63.6 | 72.1 | 74.95 | (Jiang et al. 2013) |
| Transport vehicle | Diesel truck |  |  | (Roberts et al. 2010) |
| Transport loss of feedstock | 0.01 |  |  | (Hammond et al. 2011) |
| Transport loss of biochar | 0.03 |  |  | (Hammond et al. 2011) |
| Heat value of diesel (MJ kg-1) | 42 |  |  | (Liu et al. 2014) |
| Conversion Factors (kg CO2 (dm. km)-1), diesel | 0.272 |  |  | (Defra 2008) |
| Biogas heat value (MJ kg-1) | 6 |  |  | (Laird et al. 2009) |
| Emission factors of coal tar (t CO2e t-1) | 1.62 |  |  | (Yang et al. 2021) |
| Power conversion factor (%) | 35 |  |  | (Gaunt and Lehmann 2008) |
| CO2e emissions from coal-fired power generation (kg kWh-1) | 1.1 |  |  | (Gaunt and Lehmann 2008) |
| CO2e emissions from biochar apply (kg t-1 biochar) | 45.32 |  |  | (West and Marland 2002) |
| Biochar application rate (t ha-1) | 20 |  |  | This study |
| Distance (km) | — |  |  | Calculated in this study |
| Conversion of C to CO2 | 3.6 |  |  | - |
| 100-year GWP of N2O | 273 |  |  | (IPCC 2022) |
| Nfer/Pfer/Kfer (kg ha-1) | Global Database |  |  | (Mueller et al. 2012) |
| N content of straw biochar (g kg-1) | 1.54 |  |  | (Ippolito et al. 2020) |
| P content of straw biochar (g kg-1) | 8 |  |  | (Ippolito et al. 2020) |
| K content of straw biochar (g kg-1) | 40.9 |  |  | (Ippolito et al. 2020) |
| Nfer_decre | 0.1 |  |  | (Hammond et al. 2011) |
| Pfer_decre | 0.05 |  |  | (Hammond et al. 2011) |
| Kfer_decre | 0.05 |  |  | (Hammond et al. 2011) |
| Nfer_co2e (kg kg-1) | 3 |  |  | (Roberts et al. 2010) |
| Pfer_co2e (kg kg-1) | 0.7 |  |  | (Roberts et al. 2010) |
| Kfer_co2e (kg kg-1) | 1 |  |  | (Roberts et al. 2010) |
| N2O remission rate from Nfer (%) | 1.325 |  |  | (Roberts et al. 2010) |
| Inhibition rate of N2O (%) | 25 |  |  | (Hammond et al. 2011) |
| SOC change with biochar | Assumed to be biochar stable carbon content |  |  | (Roberts et al. 2010) |

Table S3 The biomass resource of Guangdong province

| Biomass types |  | Types | Biomass (Tg biomass) | Rate |
| --- | --- | --- | --- | --- |
| Agricultural residues |  | Rice | 9.21 | 0.75 |
|  |  | Maize | 0.65 | 0.05 |
|  |  | Other grain | 0.01 | 0.00 |
|  |  | Canola | 0.03 | 0.00 |
|  |  | Peanuts | 1.35 | 0.11 |
|  |  | Soybeans | 0.07 | 0.01 |
|  |  | Potatoes | 0.98 | 0.08 |
|  |  | Sum | 12.29 | 1.00 |
| Forestry residues | Wood exploitation and process | Merchant | 1.44 | 0.07 |
|  |  | Fire | 4.86 | 0.25 |
|  |  | Log | 0.29 | 0.02 |
|  | Bamboo exploitation and process | Bamboo | 3.36 | 0.18 |
|  | Forest logging | Fuel | 0.11 | 0.01 |
|  |  | Timber | 1.98 | 0.10 |
|  |  | Protect | 0.19 | 0.01 |
|  |  | Economic | 6.30 | 0.33 |
|  |  | Special purpose | 0.04 | 0.00 |
|  |  | Shrub | 0.17 | 0.01 |
|  |  | Sparse | 0.45 | 0.02 |
|  |  | Sum | 19.19 | 1.00 |
| Bioenergy crops cultivated on marginal land I |  | Miscanthus | 19.14 | 0.25 |
|  |  | Eucalypt | 21.29 | 0.28 |
|  |  | Poplar | 12.31 | 0.16 |
|  |  | Switchgrass | 12.00 | 0.16 |
|  |  | Willow | 12.48 | 0.16 |
| Bioenergy crops cultivated on marginal land II |  | Miscanthus | 29.13 | 0.25 |
|  |  | Eucalypt | 31.96 | 0.27 |
|  |  | Poplar | 18.68 | 0.16 |
|  |  | Switchgrass | 18.18 | 0.16 |
|  |  | Willow | 18.88 | 0.16 |

Table S4 Key parameters for uncertainty analyses

| Parameters | Range | Mean | Standard deviation | References |
| --- | --- | --- | --- | --- |
| Biochar yield (%) | 23.2 - 34.8 | 29 | 3.53 | Deng et al. (2024) |
| Biochar carbon (%) | 53 - 79 | 66 | 8.03 | Deng et al. (2024) |
| Biochar stable carbon (%) | 60.3 - 89.6 | 67.8 | 10.54 | Estimated from Jiang et al. (2013) |
| Biogas yield (%) | 13 - 41.7 | 27.6 | 9.42 | Estimated from Jiang et al. (2023) |
| Biochar application rate (t ha-1) | 0.9 - 120 | 20 | 24 | Estimated from Han et al. (2021) |

Fig. S1 Administrative division (a) and regional division (b) of Guangdong province

Fig. S2 Annual mean temperature (a), annual precipitation (b), and annual mean sunshine hours (c) of Guangdong province from 2000 to 2021

Fig. S3 Framework for life cycle analysis of biochar in this study

Fig. S4 Carbon emission from energy consumption (a) and industrial processes of cement and steel production (b) in Guangdong province from 2016 to 2021

Fig. S5 The CH4 emissions from rice paddies of Guangdong province from 2016 to 2021 (a) and in four regions for 2021 (b)

Fig. S6 Direct and indirect N2O emission from cropland in Guangdong province from 2016 to 2021 (a) and in four regions for 2021(b)

Fig. S7 The CH4 emission from animal intestinal fermentation (a), CH4 (b), and N2O emission (c) from animal excrement management of Guangdong province from 2016 to 2021. (d) is the proportion of CO2 emissions from animal intestinal fermentation and excrement management

Fig. S8 The CO2 emission from livestock activities grouped by animal type (a) and intestinal fermentation/ excrement management (b) in four regions of Guangdong province for 2021

Fig. S9 The CO2 emission from wastewater (a, b) and solid waste (c, d) in Guangdong province from 2016 to 2021 (a, c) and in four regions (b, d) for 2021

Fig. S10 Distribution of marginal land I (a) and marginal land II (i.e., abandoned cropland) (b) in Guangdong province

Fig. S11 Yield of five bioenergy crops of (a) Miscanthus, (b) Eucalypt, (c) Poplar, (d) Switchgrass, (e) Willow and (f) the best crop type with maximum yield in each grid cell on marginal land I of Guangdong province

Fig. S12 Yield of five bioenergy crops of (a) Miscanthus, (b) Eucalypt, (c) Poplar, (d) Switchgrass, (e) Willow and (f) best crop type with maximum yield in each grid cell on marginal land II (i.e., abandoned cropland) of Guangdong province

Fig. S13 The CO2 emission and removal of biochar derived from five bioenergy crops (i.e., Miscanthus, Eucalypt, Poplar, Switchgrass and Willow) in Guangdong province

Fig. S14 The shortest distance from cropland to marginal land I (a), marginal land II (c) and forestry land (e). The corresponding frequency distribution of the shortest distance is shown in (b), (d) and (f), respectively. The mean and median values are represented by dashed and solid lines, respectively

Fig. S15 Total CDR potential of biochar derived from forestry residues and best bioenergy crops on marginal land I and II (i.e., abandoned cropland). Two scenarios of applying biochar to the nearest cropland and in situ were compared

Fig. S16 CDR of biochar derived from five bioenergy crops planted on marginal land I (a) and marginal land II (i.e., abandoned cropland) (b) in four regions of Guangdong province

Fig. S17 Spatial distribution of CDR of biochar derived from (a) Miscanthus, (b) Eucalypt, (c) Poplar, (d) Switchgrass, (e) Willow and (f) the best crop (best crop type with maximum yield in each grid cell) planted on the marginal land I of Guangdong province

Fig. S18 Spatial distribution of CDR of biochar derived from (a) Miscanthus, (b) Eucalypt, (c) Poplar, (d) Switchgrass, (e) Willow and (f) the best crop (best crop type with maximum yield in each grid cell) planted on the marginal land II (i.e., abandoned cropland) of Guangdong province

Fig. S19 The spatial map (a) and frequency distribution (b) of forest age in Guangdong province. The grey line and dashed line are the mean and median of the forest age, respectively

References

Defra 2008. Guidelines to Defra's GHG Conversion Factors. http://www.aef.org.uk/downloads/ghg-cf-guidelines-annexes2008.pdf . Accessed 23 April 2024

Deng X, Teng F, Chen M, Du Z, Wang B, Li R, Wang P (2024) Exploring negative emission potential of biochar to achieve carbon neutrality goal in China. Nat Commun 15: 1085. https://doi.org/10.1038/s41467-024-45314-y

Gaunt JL, Lehmann J (2008) Energy balance and emissions associated with biochar sequestration and pyrolysis bioenergy production. Environ Sci Technol 42: 4152-4158

Gvein MH, Hu X, Næss JS, Watanabe MDB, Cavalett O, Malbranque M, Kindermann G, Cherubini F (2023) Potential of land-based climate change mitigation strategies on abandoned cropland. Commun Earth Environ 4: 39. https://doi.org/10.1038/s43247-023-00696-7

Hammond J, Shackley S, Sohi S, Brownsort P (2011) Prospective life cycle carbon abatement for pyrolysis biochar systems in the UK. Energy Policy 39: 2646-2655. https://doi.org/10.1016/j.enpol.2011.02.033

Han M, Zhao Q, Li W, Ciais P, Wang YP, Goll DS, Zhu L, Zhao Z, Wang J, Wei Y, Wu F (2021) Global soil organic carbon changes and economic revenues with biochar application. Glob Change Biol Bioenergy 14: 364-377. https://doi.org/10.1111/gcbb.12915

IPCC (2022) Climate Change 2022: Mitigation of Climate Change. Contribution of Working Group III to the Sixth Assessment Report of the Intergovernmental Panel on Climate Change. https://www.ipcc.ch/report/sixth-assessment-report-working-group-3/. Accessed 23 April 2024

Ippolito JA, Cui L, Kammann C, Wrage-Mönnig N, Estavillo JM, Fuertes-Mendizabal T, Cayuela ML, Sigua G, Novak J, Spokas K (2020) Feedstock choice, pyrolysis temperature and type influence biochar characteristics: a comprehensive meta-data analysis review. Biochar 2: 421-438

Jiang Z-X, Zheng H, Li F-M, Wang Z-Y (2013) Preliminary assessment of the potential of biochar technology in mitigating the greenhouse effect in China. Environ Sci 34: 2486-2492

Laird DA, Brown RC, Amonette JE, Lehmann J (2009) Review of the pyrolysis platform for coproducing bio-oil and biochar. Biofuel Bioprod Biorefin 3: 547-562. https://doi.org/10.1002/bbb.169

Li X, Xie H, Kou W, Wang W, Zhang N (2016) Discussion and case study on the CO2 emission calculation methods in iron and steel industry. J Saf Environ 16: 320-324

Liu X, Li Z, Zhang Y, Feng R, Mahmood IB (2014) Characterization of human manure-derived biochar and energy-balance analysis of slow pyrolysis process. Waste Manage 34: 1619-1626

Mueller ND, Gerber JS, Johnston M, Ray DK, Ramankutty N, Foley JA (2012) Closing yield gaps through nutrient and water management. Nature 490: 254-257

Roberts KG, Gloy BA, Joseph S, Scott NR, Lehmann J (2010) Life Cycle Assessment of Biochar Systems: Estimating the Energetic, Economic, and Climate Change Potential. Environ Sci Technol 44: 827-833. https://doi.org/10.1021/es902266r

West TO, Marland G (2002) A synthesis of carbon sequestration, carbon emissions, and net carbon flux in agriculture: comparing tillage practices in the United States. Agric Ecosyst Environ 91: 217-232

Yang Q, Zhou H, Bartocci P, Fantozzi F, Masek O, Agblevor FA, Wei Z, Yang H, Chen H, Lu X, Chen G, Zheng C, Nielsen CP, McElroy MB (2021) Prospective contributions of biomass pyrolysis to China's 2050 carbon reduction and renewable energy goals. Nat Commun 12: 1698. https://doi.org/10.1038/s41467-021-21868-z