Supporting information

for

Source discrimination of sedimentary PAHs in the Orkhon River Basin, Central Asia: A geochemical record of anthropogenic contributions

Shi-Ting Zhai1,2, Jing Chen1,2,, Anarmaa Sharkhuu3, Soyol-Erdene Tseren-Ochir4,5

1 State Key Joint Laboratory of Environment Simulation and Pollution Control, School of Environment, Beijing Normal University, Beijing 100875, China

2 Center for Atmospheric Environmental Studies, Beijing Normal University, Beijing 100875, China

3 Department of Biology, School of Arts and Sciences, National University of Mongolia, Ulaanbaatar 14201, Mongolia

4 Laboratory of Environmental Chemistry and Geochemistry, National University of Mongolia, Ulaanbaatar 14201, Mongolia

5 Department of Green Energy and Engineering, School of Engineering and Technology, National University of Mongolia, Ulaanbaatar 14201, Mongolia

Summary of supporting information contents:

Number of pages: 16

Number of texts: 4

Number of figures: 2

Number of tables: 7

# Text S1. PAH analysis

Before chemical analysis, sediment samples were freeze-dried to a constant weight using a vacuum freeze-dryer. The dried sediment samples were then lightly crushed, sieved through a 100-mesh screen, and stored in sealed polyethylene bags. The extraction of the sediment samples followed methods described in the literature ( Martinez et al. 2004; Bahia et al. 2021; Pérez and Albero 2023). Briefly, 100 ng of surrogate standard, hexamethylbenzene, was added to approximately 10 g of sediment. The sample was then sonicated three times for 30 minutes each with 20 mL of n-hexane-dichloromethane (1:1, v:v). The extracts were combined and concentrated under a gentle flow of pure nitrogen to about 1 mL. The extract was then purified using a silica gel column. The PAH-containing fraction was eluted with 50 mL of n-hexane-dichloromethane (3:2) and concentrated to 1 mL. Finally, 10 μL of internal standard was added to the sample before GC-MS analysis.

16 PAHs were analyzed using a Shimadzu TQ8040 gas chromatography-triple quadrupole mass spectrometry (GC/MS/MS) system. The chromatographic column was a DB-5MS capillary column (30 m × 0.25 mm, 0.25 μm film thickness), with high-purity helium as the carrier gas at a constant flow rate of 1 mL·min-1. The injector temperature was set to 280 °C. The column temperature program was as follows: start at 60°C (hold for 1.5 min), ramp to 160 °C at 15 °C·min-1 (hold for 1 min), then ramp to 200 °C at 3°C·min-1, and finally ramp to 300 °C at 8°C·min-1 (hold for 7 min). Multiple reaction monitoring (MRM) mode was used for identification and quantification. PAHs concentrations were quantified using the internal standard method, with calibration curve regression coefficients greater than 0.998.

The analysis process followed strict quality control procedures. In addition to the sample analysis, field, laboratory, and reagent blanks were also analyzed. The average recovery of hexamethylbenzene in sediment samples was 83.3 ± 12.4 %. The detection limit was calculated as the average blank sample value plus three times of the standard deviation, ranging from 0.02 to 0.3 ng·g-1. The relative standard deviation (RSD) of replicate samples at each site was less than 5%.

# Text S2. Diagnostic ratios of PAHs

The ratios of Ant /(Ant + Phe) and BaA /(BaA + Chr) are commonly used to distinguish between petrogenic and combustion sources. When Ant /(Ant + Phe) < 0.1, it indicates a petrogenic source, while Ant /(Ant + Phe) > 0.1 suggests a combustion-dominated source. For BaA /(BaA + Chr), values < 0.2 indicate a petrogenic source, 0.2 to 0.35 suggest petroleum combustion, and > 0.35 indicate biomass combustion. The IcdP/(IcdP + BghiP) ratio is used to differentiate petrogenic (unburned petroleum), petroleum combustion (vehicle engine related), and biomass/coal combustion sources, corresponding to the IcdP/(IcdP + BghiP) ratio < 0.2, 0.2 – 0.5, and > 0.5, respectively (Zhao et al. 2017; Bateni et al. 2022; Kong et al. 2023).

# Text S3. Absolute principal component score-multiple linear regression (APCS-MLR)

The APCS-MLR model was applied to calculate the contribution of each source to each component (%) after determining the number of potential sources. The multiple linear regression process is shown in the following formula ( Zhang et al. 2020; Wang et al. 2024):

(1)

where, Cj is the j-th polycyclic aromatic hydrocarbon (ng·g⁻¹ for sediment), bi0 and bpi are the regression coefficients for each factor’s constant term obtained from multiple linear regression, APCSp is the adjusted score for factor p, and bpiAPCSp represents the contribution of source P to pollutant i. The average value of bpiAPCSp represents the average absolute contribution of the pollution source corresponding to factor p.

# Text S4. PMF analysis

The PMF model can be described by the following equation (Hopke 2003):

(2)

where i represents the number of measured samples; j is the type of pollutant being measured; p is the number of appropriate factors identified by the PMF model; f refers to the factor matrix for each source; g is the contribution matrix of sources to the samples; and eij is the residual for each pollutant type in each sample. The goal of the PMF model iteration is to minimize the objective function Q (the accumulated residuals). The calculation of the Q value is as follows:

(3)

where n is the number of samples, m is the number of species, and uij is the uncertainty of the concentration of species j in the i-th sample. During the calculation process, the PMF model imposes non-negative constraints on the factor loadings and weights them by the uncertainty. The uncertainty of uij in this study was estimated using the following equation:

(4)

where DLj represents the detection limit of the j-th PAH compound.

Supporting Figures:

Fig. S1 Geochronology estimated by 210Pbex CRS model for sediments from Orkhon River (a) and Ishgent Lake (b).

Fig. S2 Factor loadings of latent variables based on PLS-SEM analysis.

Supporting Tables

Table S1. Detailed parameters of CRS model calculations.

| Sample depth (cm) | 226Ra (Bq·kg–1) | 210Pb (Bq·kg–1) | ²¹⁰Pbex (Bq·kg–1) | Deposition age (a) | Sedimentation rate (cm·a–1) |
| --- | --- | --- | --- | --- | --- |
| RW 0–1 | 34.83 | 42.33 | 7.50 | 1.42 | 0.69 |
| RW 1–2 | 39.50 | 46.50 | 7.00 | 2.90 | 0.71 |
| RW 2–3 | 50.67 | 55.00 | 4.33 | 4.35 | 1.09 |
| RW 3–4 | 55.33 | 59.17 | 3.83 | 5.28 | 1.20 |
| RW 4–5 | 42.00 | 45.67 | 3.67 | 6.12 | 1.22 |
| RW 5–6 | 29.67 | 30.67 | 1.00 | 6.95 | 4.36 |
| RW 6–7 | 26.83 | 34.00 | 7.17 | 7.18 | 0.60 |
| RW 7–8 | 24.67 | 36.17 | 11.50 | 8.88 | 0.36 |
| RW 8–9 | 25.17 | 39.33 | 14.17 | 11.81 | 0.26 |
| RW 9–10 | 25.67 | 36.33 | 10.67 | 15.83 | 0.31 |
| RW 10–11 | 23.83 | 32.67 | 8.83 | 19.23 | 0.34 |
| RW 11–12 | 23.50 | 32.33 | 8.83 | 22.35 | 0.31 |
| RW 12–13 | 25.00 | 41.67 | 16.67 | 25.80 | 0.15 |
| RW 13–14 | 25.33 | 49.83 | 24.50 | 33.54 | 0.08 |
| RW 14–15 | 25.00 | 45.33 | 20.33 | 49.98 | 0.06 |
| RW 15–16 | 23.67 | 40.00 | 16.33 | 75.94 | 0.03 |
| LW 0–1 | 11.50 | 107.45 | 95.95 | 4.56 | 0.20 |
| LW 1–2 | 5.57 | 103.77 | 98.20 | 9.88 | 0.17 |
| LW 2–3 | 8.33 | 90.42 | 82.09 | 16.42 | 0.16 |
| LW 3–4 | 8.36 | 93.40 | 85.04 | 23.15 | 0.13 |
| LW 4–5 | 13.58 | 76.74 | 63.16 | 32.02 | 0.13 |
| LW 5–6 | 10.07 | 64.79 | 54.72 | 40.67 | 0.12 |
| LW 6–7 | 8.61 | 33.63 | 25.02 | 50.69 | 0.19 |
| LW 7–8 | 11.54 | 33.58 | 22.04 | 56.57 | 0.18 |
| LW 8–9 | 13.00 | 44.85 | 31.85 | 62.83 | 0.10 |
| LW 9–10 | 13.31 | 22.03 | 8.72 | 74.79 | 0.25 |
| LW 10–11 | 10.14 | 16.18 | 6.04 | 79.03 | 0.32 |
| LW 11–12 | 10.95 | 19.50 | 8.55 | 82.33 | 0.20 |
| LW 12–13 | 8.48 | 14.96 | 6.48 | 87.67 | 0.23 |
| LW 13–14 | 9.60 | 15.56 | 5.96 | 92.40 | 0.21 |
| LW 14–15 | 11.02 | 15.76 | 4.74 | 97.48 | 0.23 |
| LW 15–16 | 12.99 | 13.38 | 0.39 | 102.18 | 2.40 |
| LW 16–17 | 12.32 | 20.48 | 8.16 | 102.59 | 0.11 |
| LW 17–18 | 14.48 | 25.14 | 10.66 | 112.91 | 0.06 |
| LW 18–19 | 12.35 | 23.22 | 10.87 | 134.86 | 0.03 |

Table S2. Comparison of sedimentation rates between this study and references.

| Study site | Sedimentation rate range (cm·a–1) | Data source |
| --- | --- | --- |
| OR, Mongolia | 0.03–4.36 | This study |
| IL, Mongolia | 0.03–2.40 | This study |
| Olgoy lake, Mongolia | 0.10–26.70 | Udaanjargal et al. 2022 |
| Boontsagaan lake, Mongolia | 0.20–16.00 | Udaanjargal et al. 2022 |
| Orog lake, Mongolia | 0.30–43.40 | Udaanjargal et al. 2022 |
| Nur sphagnum bog, Mongolia | 0.33–0.46 | Fukumoto et al. 2012 |

Table S3. Description and acquisition sources of anthropogenic and environmental factors.

| Variable | Unit | Abbreviation | Scale |
| --- | --- | --- | --- |
| Average air temperature | ℃ | AT | Dakhan province |
| pH | / | pH | Study area |
| Clay | %, < 2μm | Clay | Study area |
| Silt | %, 2 – 63 μm | Silt | Study area |
| Sand | %, > 63 μm | Sand | Study area |
| Total carbon | g·kg–1 | TC | Study area |
| Methane | Gg | CH4 | Mongolia nationwide |
| Carbon dioxide | Gg | CO2 | Mongolia nationwide |
| Nitrous oxide | Gg | N2O | Mongolia nationwide |
| Black carbon | Gg | BC | Mongolia nationwide |
| Carbon monoxide | Gg | CO | Mongolia nationwide |
| Ammonia | Gg | NH3 | Mongolia nationwide |
| Non-methane volatile organic compounds | Gg | NMVOC | Mongolia nationwide |
| Nitrogen oxides | Gg | NOX | Mongolia nationwide |
| Sulfur dioxide | Gg | SO2 | Mongolia nationwide |
| PM10 | Gg | PM10 | Mongolia nationwide |
| PM2.5 | Gg | PM2.5 | Mongolia nationwide |
| Resident population | individuals | RP | Selenge province |
| Local GDP | million tugrik | LGDP | Selenge province |
| Per-capita GDP | thousand tugrik | PGDP | Selenge province |
| Electricity consumption | Gwh | EC | Mongolia nationwide |
| Coal consumption | kt | CC | Mongolia nationwide |
| Passenger automobile | Units | PA | Selenge province |
| Special purpose vehicle | Units | SPV | Selenge province |
| Bus | Units | Bus | Selenge province |
| Truck | Units | Truck | Selenge province |

Note: The data collected from the National Statistical Office of Mongolia, World Bank, and Global Carbon Emissions Database were classified. The carbon emissions data for Mongolia (e.g., CH4, CO2, N2O) and pollutants (e.g., BC, CO, NH3, NMVOC, NOX, SO2, PM10, and PM2.5) came from the Global CarbonEmissions Database (EDGAR, https://edgar.jrc.ec.europa.eu/dataset_ghg80#p1m) (Friedlingstein et al., 2023). Economic data such as electricity consumption, coal consumption, and GDP were from the National Statistical Office of Mongolia (https://en.nso.mn/home/). Population data were based on Selenge Province, and other socioeconomic indicators came from the World Bank (https://data.worldbank.org.cn/). Traffic data (e.g., buses, trucks, passenger cars, and special vehicles) were from the annual vehicle census by the National Statistical Office of Mongolia, where buses refer to passenger vehicles with 9 or more seats (including buses and trolleybuses), trucks are vehicles used for freight transport (including combinations of trailers and trucks), passenger cars are vehicles that carry up to 8 passengers (including taxis), and special vehicles are those equipped with specialized equipment for specific purposes. Sediment property data (e.g., clay particles, silt, sand, pH, and total carbon) were obtained through sampling and analysis in the study area.

Table S4. Statistical analysis of PAH concentrations (ng·g–1) in sediments.

| PAHs | Ring number | Orkhon River |  |  |  | Ishgent Lake |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Min | Max | Mean | CVa | Min | Max | Mean | CVa |
| Nap | 2 | 9.28 | 20.53 | 15.10 | 0.23 | 12.36 | 23.16 | 17.42 | 0.20 |
| Acy | 3 | 1.06 | 1.30 | 1.17 | 0.06 | 1.04 | 1.29 | 1.15 | 0.06 |
| Ace | 3 | 0.82 | 1.27 | 1.06 | 0.13 | 0.85 | 1.29 | 1.07 | 0.10 |
| Flu | 3 | 2.11 | 3.47 | 2.64 | 0.15 | 2.48 | 4.42 | 3.14 | 0.20 |
| Phe | 3 | 5.33 | 11.28 | 7.51 | 0.22 | 7.57 | 22.24 | 12.08 | 0.37 |
| Ant | 3 | 1.62 | 2.29 | 1.88 | 0.11 | 1.54 | 3.27 | 2.18 | 0.23 |
| Fla | 4 | 0.17 | 1.28 | 0.54 | 0.48 | 0.20 | 2.48 | 1.00 | 0.61 |
| Pyr | 4 | 0.98 | 3.54 | 2.55 | 0.24 | 1.70 | 6.50 | 3.32 | 0.36 |
| BaA | 4 | 1.05 | 1.57 | 1.19 | 0.09 | 1.08 | 1.69 | 1.32 | 0.15 |
| Chr | 4 | 0.02 | 0.45 | 0.16 | 0.90 | 0.06 | 2.37 | 1.05 | 0.63 |
| BbF | 5 | 1.56 | 2.06 | 1.74 | 0.08 | 1.59 | 3.32 | 2.17 | 0.28 |
| BkF | 5 | 0.16 | 0.33 | 0.20 | 0.22 | 0.16 | 0.47 | 0.29 | 0.31 |
| BaP | 5 | 0.99 | 1.39 | 1.13 | 0.10 | 0.97 | 1.95 | 1.35 | 0.25 |
| InP | 6 | 1.89 | 2.05 | 1.95 | 0.03 | 1.88 | 2.57 | 2.13 | 0.11 |
| DahA | 5 | 3.27 | 3.34 | 3.29 | 0.01 | 3.26 | 3.38 | 3.31 | 0.01 |
| BghiP | 6 | 0.03 | 0.19 | 0.06 | 0.78 | 0.02 | 0.65 | 0.23 | 0.94 |
| ∑PAHs | 2–6 | 33.26 | 50.99 | 42.18 | 0.14 | 38.76 | 73.74 | 53.21 | 0.21 |

a CV: coefficient of variation.

Table S5. PAH concentrations and peak years in reference lakes and rivers.

| Region |  | Concentration (ng·g–1) | Year range |  | Peak year |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lake |  |  |  |  |  |  |  |
| Ishgent lake, Northern Mongolia |  | Σ16PAHs 38.76 ~ 73.74 | 1888 ~ 2018 |  | 2018 |  | This study |
| Lianhuan lake, Northeastern China |  | Σ16PAHs 198 ~ 1056 | 1845 ~ 2005 |  | 1845 |  | Sun and Zang 2012 |
| Daihai lake, Northern China |  | Σ16PAHs 254.5 ~ 353.2 | 1985 ~ 2010 |  | ~ 2000 AD |  | Sun et al. 2021 |
| SLNG04 lake, Southeastern Russia |  | Σ19PAHs 1.78 ~ 188.3 | 1800 ~ 2014 |  | ~ 2010 AD |  | Adams et al. 2018 Adams et al. 2018 |
| Black lake, Southeastern Russia |  | Σ16PAHs 50 ~ 138 | 1900 ~ 2014 |  | ~ 1970 AD |  |  |
| Lake Baikal tributaries, South-central Russia |  | Σ14PAHs 20 ~ 150 | Recent records |  |  |  | Semenov et al. 2023 |
| Lake Baikal, South-central Russia |  | Σ16PAHs 50 ~ 700 | Recent records |  |  |  | Semenov et al. 2023 |
| River |  |  |  |  |  |  |  |
| Orkhon river, Northern Mongolia |  | Σ16PAHs 33.26 ~ 50.99 | 1947 ~ 2022 |  | 2014 |  | This study |
| Yellow River Estuary, Northeastern China |  | Σ16PAHs 39.8 ~ 285.6 | 1934 ~ 2015 |  | 2006 |  | Wang et al. 2020 |
| Liaodong Bay, Northeastern China |  | Σ16PAHs 12.18 ~ 430.96 | 1815 ~ 2014 |  | ~ 2010 AD |  | Guo et al. 2022 |
| Yangtze Estuary, Southeastern China |  | Σ16PAHs 181.8 ~ 330 | 1955 ~ 2009 |  | 2009 |  | Li et al. 2016 |
| White Sea, Northwest Russia |  | Σ16PAHs 13 ~ 208 | 1904 ~ 1994 |  | ~ 1970 AD |  | Savinov et al. 2000 |

Table S6. Rotating component matrix by APCS-MLR model.

| Component | Factor 1 | Factor 2 |
| --- | --- | --- |
| Nap | 0.406 | 0.763 |
| Acy | 0.103 | 0.839 |
| Ace | 0.099 | 0.927 |
| Flu | 0.733 | 0.593 |
| Phe | 0.853 | 0.399 |
| Ant | 0.808 | 0.383 |
| Fla | 0.881 | 0.295 |
| Pyr | 0.214 | 0.536 |
| BaA | 0.904 | 0.165 |
| Chr | 0.902 | 0.141 |
| BbF | 0.965 | 0.192 |
| BkF | 0.929 | 0.189 |
| BaP | 0.954 | 0.174 |
| InP | 0.947 | 0.257 |
| DahA | 0.780 | 0.478 |
| BghiP | 0.940 | 0.296 |

Table S7. Factor loadings of PAH congeners for pollution sources identified by APCS-MLR and PMF.

| PAHs | APCS-MLR(%) |  | PMF(%) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Coal combustion | Mixed sources | Coal | Traffic | Biomass | Petroleum |
| Nap | 2.97 | 97.03 | 12.30 | 8.31 | 26.88 | 52.52 |
| Acy | 41.32 | 58.68 | 48.42 | 15.97 | 12.13 | 23.48 |
| Ace | 27.68 | 72.32 | 32.59 | 9.25 | 23.20 | 34.96 |
| Flu | 26.13 | 73.87 | 26.80 | 18.58 | 20.40 | 34.22 |
| Phe | 13.86 | 86.14 | 10.56 | 28.92 | 25.61 | 34.91 |
| Ant | 36.75 | 63.25 | 39.07 | 25.12 | 11.04 | 24.78 |
| Fla | 25.80 | 74.20 | 16.52 | 45.66 | 37.82 | 0.00 |
| Pyr | 8.30 | 91.70 | 9.20 | 0.00 | 64.89 | 25.91 |
| BaA | 47.60 | 52.40 | 54.52 | 27.81 | 4.86 | 12.82 |
| Chr | 30.75 | 69.25 | 0.00 | 69.78 | 11.52 | 18.70 |
| BbF | 44.00 | 56.00 | 51.50 | 36.22 | 3.43 | 8.85 |
| BkF | 41.78 | 58.22 | 50.76 | 38.67 | 5.04 | 5.53 |
| BaP | 45.60 | 54.40 | 54.05 | 34.87 | 2.99 | 8.09 |
| InP | 47.06 | 52.94 | 53.03 | 24.58 | 6.38 | 16.01 |
| DahA | 49.38 | 50.62 | 56.32 | 19.20 | 7.03 | 17.46 |
| BghiP | 33.82 | 66.18 | 1.27 | 79.60 | 18.93 | 0.19 |

References:

Adams JK, Martins CC, Rose NL, Shchetnikov AA, Mackay AW (2018) Lake sediment records of persistent organic pollutants and polycyclic aromatic hydrocarbons in southern Siberia mirror the changing fortunes of the Russian economy over the past 70 years. Environ Pollut 242:528–538. https://doi.org/10.1016/j.envpol.2018.07.005

Bahia PVB, Nascimento MM, Hatje V, De Andrade JB, Machado ME (2021) Microscale extraction combined with gas chromatography/mass spectrometry for the simultaneous determination of polycyclic aromatic hydrocarbons and polycyclic aromatic sulfur heterocycles in marine sediments. J Chromatogr A 1653:462414. https://doi.org/10.1016/j.chroma.2021.462414

Bateni F, Mehdinia A, Lundin L, Hashtroudi MS (2022) Distribution, source and ecological risk assessment of polycyclic aromatic hydrocarbons in the sediments of northern part of the Persian Gulf. Chemosphere 295:133859. https://doi.org/10.1016/j.chemosphere.2022.133859

Friedlingstein P, O'Sullivan M, Jones MW, et al (2023) Global carbon budget 2023. Earth Syst Sci Data 15:5301–5369. https://doi.org/10.5194/essd-15-5301-2023

Fukumoto Y, Kashima K, Orkhonselenge A, Ganzorig U (2012) Holocene environmental changes in northern Mongolia inferred from diatom and pollen records of peat sediment. Quat Int 254:83–91. https://doi.org/10.1016/j.quaint.2011.10.014

Guo F, Gao M, Dong J, Sun J, Hou G, Liu S, Du X, Yang S, Liu J, Huang Y (2022) The first high resolution PAH record of industrialization over the past 200 years in Liaodong Bay, northeastern China. Water Res 224:119103. https://doi.org/10.1016/j.watres.2022.119103

Hopke PK (2003) Recent developments in receptor modeling. J Chemom 17:255–265. https://doi.org/10.1002/cem.796

Kong J, Ma T, Cao X, Li W, Zhu F, He H, Sun C, Yang S, Li S, Xian Q (2023) Occurrence, partition behavior, source and ecological risk assessment of nitro-PAHs in the sediment and water of Taige Canal, China. J Environ Sci 124:782–793. https://doi.org/10.1016/j.jes.2022.02.034

Li R, Feng C, Wang D, He M, Hu L, Shen Z (2017) Effect of water flux and sediment discharge of the Yangtze River on PAHs sedimentation in the estuary. Environ Monit Assess 189:10. https://doi.org/ 10.1007/s10661-016-5729-x

Martinez E, Gros M, Lacorte S, Barceló D (2004) Simplified procedures for the analysis of polycyclic aromatic hydrocarbons in water, sediments and mussels. J Chromatogr A 1047:181–188. https://doi.org/10.1016/j.chroma.2004.07.003

Pérez RA, Albero B (2023) Ultrasound-assisted extraction methods for the determination of organic contaminants in solid and liquid samples. TrAC Trends Anal Chem 166:117204. https://doi.org/10.1016/j.trac.2023.117204

Savinov VM, Savinova TN, Carroll J, Matishov GG, Dahle S, Næs K (2000) Polycyclic aromatic hydrocarbons (PAHs) in sediments of the White Sea, Russia. Mar Pollut Bull 40:807–818. https://doi.org/10.1016/S0025-326X(00)00004-7

Semenov MY, Marinaite II, Silaev AV, Begunova LA (2023) Composition, concentration and origin of polycyclic aromatic hydrocarbons in waters and bottom sediments of Lake Baikal and its tributaries. Water 15:2324. https://doi.org/10.3390/w15132324

Sun L, Zang S (2012) History of fuel consumption inferred from polycyclic aromatic hydrocarbons in sediments from the South Lianhuan Lake, northeast China. Bull Environ Contam Toxicol 88:1027–1032. https://doi.org/ 10.1007/s00128-012-0600-4

Sun Y, Zhang R, Ma R, Zhou H, Zhang F, Guo G, Li H, Lü C (2021) Distribution, sources, and ecological risk assessment of polycyclic aromatic hydrocarbons in the sediments of Daihai Lake in Inner Mongolia, China. Environ Sci Pollut Res 28:23123–23132. https://doi.org/ 10.1007/s11356-021-12349-3

Udaanjargal U, Hasebe N, Davaasuren D, Fukushi K, Tanaka Y, Gankhurel B, Katsuta N, Ochiai S, Miyata Y, Gerelmaa T (2022) Characteristics of lake sediment from southwestern Mongolia and comparison with meteorological data. Geosciences 12:7. https://doi.org/10.3390/geosciences12010007

Wang C, Yu L, Zou Y, Ji L, Li Y (2020) Sources and historical sedimentary record: Temporal variability of n-alkane and PAHs from the Yellow River estuary, China. Appl Geochem 114:104475. https://doi.org/10.1016/j.apgeochem.2019.104475

Wang H, Liu D, Lv Y, Wang W, Wu Q, Huang L, Zhu L (2024) Ecological and health risk assessments of polycyclic aromatic hydrocarbons (PAHs) in soils around a petroleum refining plant in China: a quantitative method based on the improved hybrid model. J Hazard Mater 461:132476. https://doi.org/10.1016/j.jhazmat.2023.132476

Zhang H, Cheng S, Li H, Fu K, Xu Y (2020) Groundwater pollution source identification and apportionment using PMF and PCA-APCA-MLR receptor models in a typical mixed land-use area in southwestern China. Sci Total Environ 741:140383. https://doi.org/10.1016/j.scitotenv.2020.140383

Zhao Z, Jiang Y, Li Q, Cai Y, Yin H, Zhang L, Zhang J (2017) Spatial correlation analysis of polycyclic aromatic hydrocarbons (PAHs) and organochlorine pesticides (OCPs) in sediments between Taihu Lake and its tributary rivers. Ecotoxicol Environ Saf 142:117–128. https://doi.org/10.1016/j.ecoenv.2017.03.039

Corresponding author. E-mail: jingchen@bnu.edu.cn (J. Chen);

Tel.: +86-10-58801848; Fax: +86-10-58801848.