Supplementary file

Embodied energy and associated carbon emission of key building materials in Nepal

Ajay Kumar K.C. 1, Anish Ghimire2,*, Bikash Adhikari1, Ashmin Aryal3, and Bivek Baral4

1 Department of Environmental Science and Engineering, School of Science, Kathmandu University, Dhulikhel, Kavre, Nepal

2 Environmental Engineering and Management, Asian Institute of Technology, Pathum Thani 12120, Thailand

3 Department of Built Environment and Energy Technology, Linnaeus University, Vaxjo, Sweden

4 Department of Mechanical Engineering, School of Engineering, Kathmandu University, Dhulikhel, Kavre, Nepal

*Corresponding author:

Anish Ghimire, E-mail:  anish-ghimire@ait.asia; anishghimire@gmail.com, Office Telephone: +66 2 524 6186

Table A1 Emission from Nepal Electricity Mix

| Electricity source | Energy Mix (GWh)[1] | Energy Mix (%) | Emission factor gCO2-eq/kWh |  |
| --- | --- | --- | --- | --- |
| Nepal generated | 7280 | 82.51% | 24.51 | Our World in Data, Energy, https://ourworldindata.org/energy#country-profiles |
| Imported from India | 1543 | 17.49% | 2100 | From Bihar [2] |
| Total Consumed | 8823 | 100% | 454. 05 | Including 17.18 % transmission and distribution loss [1] |

Table A2 The energy and CO2-eq emission from transportation.

| Database: India [3] |  |  |
| --- | --- | --- |
| Transportation mode | Emission (kg CO2-eq) | Unit of Transportation of material |
| transport, freight train | 0.044 | tonne*km |
| transport, freight, inland waterways, barge | 0.0323 | tonne*km |
| transport, freight, light commercial vehicle | 0.018 | tonne*km |
| transport, freight, lorry, unspecified | 0.0567 | tonne*km |
| transport, pipeline, onshore, petroleum | 0.559 | tonne*km |

Table A3 Emission factor for Diesel

| Processes | Unit | Emission Factor (kgGHG-100/ kg) | Remarks |
| --- | --- | --- | --- |
| Extraction of Crude Oil | kg CO2-eq/kg | 0.07 | Emission associated with crude oils that are exported to India [3] |
| Transportation of oil | kg CO2-eq/kg | 0.23 | Considering Kandla as a predominant terminal in India, Kandla to Pune, Pune to Raxual and Raxual to Kathmandu [3] |
| Crude Oil Refining | kg CO2-eq/kg | 0.95 | Taking average of low and high conversion capacity converters [3] |
| Total Upstream Emission | kg CO2-eq/kg | 1.25 |  |
|  | kg CO2-eq/liter | 1.06 |  |
| Emission from combustion | kg CO2-eq/liter | 2.69 | [4] |
| Total | kg CO2-eq/liter | 3.75 |  |

Table A4 Emission factor for Coal

| Processes | Unit | Emission Factor (kgCO2-eq/ kg) | Remarks |
| --- | --- | --- | --- |
| Extraction of coal | kg CO2-eq/kg | 0.70 | [3] |
| Land Transportation (900) | kg CO2-eq/kg-km | 0.02 |  |
| Total Upstream Emission | kg CO2-eq/kg | 0.72 | Extraction and transportation to Kathmandu (900 km) [3] |
| CO2 emission for burning of coal | kg CO2-eq/kg | 2.88 | [5] |
| Total Emission | kg CO2-eq/kg | 3.60 |  |

Table A5 Emission factor for Furnace oil (FO)

| Processes | Unit | Emission Factor (kgGHG-100/ kg) | Remarks |
| --- | --- | --- | --- |
| Extraction of Crude Oil | kg CO2-eq/kg | 0.07 | [3] |
| Transportation of oil | kg CO2-eq/kg | 0.23 | Considering Kandla as a predominant terminal in India, Kandla to Pune, Pune to Raxual and Raxual to Kathmandu [3] |
| Crude Oil Refining | kg CO2-eq/kg | 0.75 | Taking average of low and high conversion capacity converters [3] |
| Total Upstream Emission | kg CO2-eq/kg | 1.05 |  |
|  | kg CO2-eq/liter | 0.89 |  |
| Emission from combustion | kg CO2-eq/liter | 3.18 | [5] |
| Total | kg CO2-eq/liter | 4.07 |  |

Table A6 Emission factor for LPG

| Processes | Unit | Emission Factor (kgGHG-100/ kg) | Remarks |
| --- | --- | --- | --- |
| Extraction of Crude Oil | kg CO2-eq/kg | 0.07 | [3] |
| Transportation of oil | kg CO2-eq/kg | 0.23 | Considering Kandla as a predominant terminal in India, Kandla to Pune, Pune to Raxual and Raxual to Kathmandu [3] |
| Crude Oil Refining | kg CO2-eq/kg | 1.47 | Taking average of low and high conversion capacity converters [3] |
| Total Upstream Emission | kg CO2-eq/kg | 1.77 |  |
| Emission from combustion | kg CO2-eq/kg | 0.57 | [5] |
| Total | kg CO2-eq/kg | 2.35 |  |

Table A7 Emission factor for firewood

| Processes | Unit | Emission Factor (kgGHG-100/ kg) | Remarks |
| --- | --- | --- | --- |
| Firewood production | kg CO2-eq/kg | 0.10 | [3] |
| Transportation | kg CO2-eq/kg | 0.01 | Distance 300 km [3] |
| Total Upstream Emission | kg CO2-eq/kg | 0.11 |  |
| Emission from combustion | kg CO2-eq/kg | 1.80 | [5] |
| Total | kg CO2-eq/kg | 1.91 |  |

Cement Production

Table A8 Energy use in the production of one tonne of cement

| Energy Source (Per metric ton) | Unit | Energy Use | Energy Use | Energy Use | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  |  | (MJ) | (%) |  |
| Electricity - Grid | kWh | 115.93 | 417.348 | 6.90% | Electricity- 78.04 % |
| Electricity (Diesel) | kWh (Liters) | 32.63 (5.17) | 196.46 | 3.25% | Electricity- 21.96 % |
| FO | Liters | 0.63 | 26.00 | 0.43% |  |
| Coal | MJ | 5,411.26 | 5,411.26 | 89.43% |  |
| Total |  |  | 6,051.07 | 100.00% |  |

Table A9 Emission from the production of one tonne of cement

| Per metric ton | Emission (kgCO2-eq) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Base Case | % | Scenario I | Scenario II | Scenario III | Scenario IV |
| Electricity - Grid | 52.64 | 7.12% | 3.33 | 14.12 | 24.91 | 46.50 |
| Diesel | 19.39 | 2.62% | 19.39 | 19.39 | 19.39 | 19.39 |
| FO | 2.58 | 0.35% | 2.58 | 2.58 | 2.58 | 2.58 |
| Coal | 664.88 | 89.91% | 664.88 | 498.66 | 332.44 |  |
| Total | 739.49 | 100.00% | 690.18 | 534.75 | 379.32 | 68.47 |
| Emission per kg | 0.739 |  | 0.690 | 0.535 | 0.379 | 0.068 |
| Reduction from base case (kgCO2-eq) |  |  | 49.31 | 204.74 | 360.16 | 671.02 |
| Reduction from base case (%) |  |  | 6.67% | 27.69% | 48.70% | 90.74% |

Brick Production

Table A10 Energy use in the production of 1000 pieces of bricks

| Per 1000 pcs | Unit | Energy Use | Embodied Energy |  |
| --- | --- | --- | --- | --- |
|  |  |  | MJ | % |
| Electricity | kWh | 21.27 | 76.572 | 1.86% |
| Diesel | liter | 2.55 | 96.9 | 2.35% |
| Coal | MJ | 3863 | 3863.00 | 93.66% |
| Firewood | MJ | 88.09 | 88.09 | 2.14% |
| Total |  |  | 4124.56 | 100.00% |

Table A11 Emission from the production of 1000 pieces of bricks

| Per 1000 pcs | Emission (kgCO2-eq) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Base Case | % | Scenario I | Scenario II | Scenario III | Scenario IV |
| Electricity | 9.66 | 1.92% | 0.61 | 8.32 | 16.02 | 31.43 |
| Diesel | 9.56 | 1.90% | 9.56 | 9.56 | 9.56 | 9.56 |
| Coal | 474.65 | 94.38% | 474.65 | 355.98 | 237.32 | 0.00 |
| Firewood | 9.02 | 1.79% | 9.02 | 9.02 | 9.02 | 9.02 |
| Total | 502.89 | 100.00% | 493.84 | 382.89 | 271.93 | 50.02 |
| Reduction from base case (kgCO2-eq) |  |  | 9.05 | 120.00 | 230.96 | 452.87 |
| Reduction from base case (%) |  |  | 1.80% | 23.86% | 45.93% | 90.05% |

Reinforcement Steel Production

Table A12 Energy use in production of one tonne of reinforcement steel from billet

| Energy Source (Per metric tonne) | Unit | Energy Use | Embodied Energy |  |
| --- | --- | --- | --- | --- |
|  |  |  | MJ | % |
| Electricity - Grid | kWh | 120.7 | 434.52 | 20.40% |
| Electricity -Diesel | kWh | 28.47 |  |  |
| Diesel (Including electricity) | Liters | 12.9874 | 493.52 | 23.17% |
| Coal | MJ | 462.72 | 462.72 | 21.72% |
| Firewood | MJ | 4.66 | 4.66 | 0.22% |
| FO | MJ | 663.1 | 663.10 | 31.13% |
| Rice husk | MJ | 69.21 | 69.21 | 3.25% |
| LPG | MJ | 2.48 | 2.48 | 0.12% |
| Total |  |  | 2130.21 | 100.00% |

Table A13 Emission from the production of one tonne of reinforcement steel from billet

| Emission (Per metric tonne) | kg CO2-eq /MT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Base Case | % | Scenario I | Scenario II | Scenario III | Scenario IV |
| Electricity - Grid | 54.80 | 23.52% | 3.47 | 4.39 | 5.31 | 7.16 |
| Diesel | 48.70 | 20.90% | 48.70 | 48.70 | 48.70 | 48.70 |
| Coal | 56.85 | 24.40% | 56.85 | 42.64 | 28.43 | 0.00 |
| Firewood | 0.48 | 0.20% | 0.48 | 0.48 | 0.48 | 0.48 |
| FO | 63.26 | 27.15% | 63.26 | 63.26 | 63.26 | 63.26 |
| Rice husk | 8.79 | 3.77% | 8.79 | 8.79 | 8.79 | 8.79 |
| LPG | 0.12 | 0.05% | 0.12 | 0.12 | 0.12 | 0.12 |
|  |  |  |  | 0.00 |  | 0.00 |
| Total | 233.01 | 100.00% | 181.67 | 168.38 | 155.09 | 128.51 |
| Total | 0.23 |  | 0.18 | 0.17 | 0.16 | 0.13 |
| Reduction from base case (kgCO2-eq) |  |  | 51.34 | 64.63 | 77.92 | 104.50 |
| Reduction from base case (%) |  |  | 22.03% | 27.74% | 33.44% | 44.85% |

Table A14 Scenarios where the furnace oil (FO) is replaced by electricity.

| Emission (Per metric tonne) | kg CO2-eq /MT |  |  |  |
| --- | --- | --- | --- | --- |
|  | Base Case | Replacing 50 % FO with electricity | Replacing FO with electricity | Replacing FO & Coal with electricity |
| Electricity - Grid | 54.80 | 6.11 | 8.76 | 12.45 |
| Diesel | 48.70 | 48.70 | 48.70 | 48.70 |
| Coal | 56.85 | 56.85 | 56.85 |  |
| Firewood | 0.48 | 0.48 | 0.48 | 0.48 |
| FO | 63.26 | 31.63 |  |  |
| Rice husk | 8.79 | 8.79 | 8.79 | 8.79 |
| LPG | 0.12 | 0.12 | 0.12 | 0.12 |
| Total | 233.01 | 152.69 | 123.70 | 70.54 |
| Reduction from base case (kgCO2-eq) |  | 80.32 | 109.31 | 162.47 |
| Reduction from base case (%) |  | 34.47% | 46.91% | 69.73% |

Table A15 The energy and CO2-eq emission from different phase in material production.

| Materials / Phase | Material production |  | Transportation |  | Total |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Energy Use (MJ) | Emission (kg CO2-eq) | Energy Use (MJ) | Emission (kg CO2-eq) | Energy Use (MJ) | Emission (kg CO2-eq) |
| Cement | 5437.26 | 667.46 | 613.81 | 72.02 | 6051.07 | 739.49 |
| Brick | 3951.09 | 483.67 | 173.47 | 19.22 | 4124.56 | 502.89 |
| Reinforcement steel | 1636.69 | 184.31 | 493.52 | 48.70 | 2130.21 | 233.01 |

References

1. NEA. Nepal Electricity Authority - A year in Review Fiscal Year 2021/22 [Internet]. Kathmandu; 2022.

2. CBalance. GHG Inventory report for Electricity Generation and Consumption in India [Internet]. cBalance Solut. Pvt. Ltd. 2012.

3. ecoinvent. ecoinvent 3.9.1 Dataset [Internet]. ecoinvent 3.9.1 Dataset. 2024. https://ecoquery.ecoinvent.org/3.9.1/cutoff/search.

4. EPA. Greenhouse Gases Equivalencies Calculator - Calculations and References [Internet]. U.S. Environ. Prot. Agency. 2024. https://www.epa.gov/greenvehicles/greenhouse-gas-emissions-typical-passenger-vehicle#burning. Accessed 24 Mar 2024.

5. UNFCCC. Secretariat. Greenhouse Gas Emissions Calculator | 2021 Emission Factors [Internet]. United Nation Clim. Chang. 2024. https://unfccc.int/documents/271269. Accessed 12 Mar 2024.