Table S1 Contamination characteristics, ecological and toxicity indicators

| Indices | Equations |  | Classification |  |
| --- | --- | --- | --- | --- |
| Contamination factor (Cf) | Cf = Content of metal in sampleBackground content of metal (Hakanson, 1980). |  | - Cf < 1: Low contamination. - 1 < Cf < 3: Moderate contamination. - 3 < Cf < 6: Considerable contamination. - Cf > 6: High contamination. |  |
| Contamination degree (Cdeg) | Cdeg=i=18CI (Hakanson, 1980) |  | - Cdeg < 6: Low contamination. - 6 < Cdeg < 12: Moderate contamination. - 12 < Cdeg < 24: Considerable contamination. - Cdeg >24: Significant contamination. |  |
| Modified contamination degree (mCdeg) | mCdeg = ∑CFn (Hakanson, 1980) |  | - mCdeg <1.5 < 1: Nil to low contamination - 1.5-2: Low contamination - 2-4: Moderate contamination -4-8 : signifiant contamination -8-16: High contamination -16-32: Very high contamination -mCdeg > 32 : Ultra high contamination degree |  |
| The pollution load index (PLI) | PLI = (CF1 × CF2 × .... × CFn)1/n (Tomlinson et al., 1980) |  | - PLI =0 : Baseline. - PLI < 1: No pollution. - PlI > 1: Pollution. |  |
| Enrichment factor (EF) | EF = me/Fe2O3) sample / (me/Fe2O3) sample background (Xu et al., 2018) |  | - EF < 1: No enrichment. - EF < 2: Minimal enrichment. - 2 = EF < 5: Moderate enrichment. - 5 = EF < 20: Significant enrichment. - 20 = EF < 40: Severe enrichment. |  |
| Geo-accumulation index (Igeo) | Igeo = Log2 (Cn1.5×Bn) (Müller, 1981) |  | - Igeo = 0: Low pollution. - Igeo < 1: Low to moderate pollution. - Igeo < 2: Moderate pollution. - Igeo = 2 – 3: Moderate to strong pollution. - Igeo = 3 – 4: Strong pollution. - Igeo = 4 - 5: Strong to extreme pollution. - Igeo > 5: Extreme pollution. |  |
| Toxic risk index (TRI) Toxic units analysis (TUs) | TRIi=Ci∕TELi2+Ci∕PELi22 (Ustaoğlu et al., 2020) TUs =Ci/Pi (Zhang et al., 2016) |  | - TRI < 5: No toxic risk -5-10: Low -10-15: Moderate -15-20: Significant TRI > 20: Very high toxic risk. TUs <4 low toxicity 4≤TUs≤6, moderately toxicity TUs>6, heavily toxicity |  |
| Potential ecological risk index (RI) | = PERI= i=1nEri= i=1nTri× Cri (Hakanson, 1980) |  | - RI < 150: Low ecological risk. - RI = 150 < 300: Moderate. - RI = 300 < 600: High. - RI ≥ 600: Very high environmental risk index. |  |
| Modified hazard quotient (mHQ) |  | mHQ=Ci1TELi+1PELi+1SELi12 (Emenike et al., 2020) |  | - mHQ < 0.5: Nil severity of contamination. - 0.5 < mHQ < 1: Very low severity. - 1 < mHQ < 1.5: Low severity. - 1.5 < mHQ < 2: Moderate severity - 2 < mHQ < 2.5: Considerable severity - 2.5 < mHQ < 3: High severity - 3 < mHQ < 3.5: Very high severity - mHQ > 3.5: Extreme severity of contamination |  |

Table S2 Input values for exposure assessment of PTEs through ingestion and dermal routes

| Exposure Items | Adults | Children | Units |  |
| --- | --- | --- | --- | --- |
| C | - | - | mg/L |  |
| IngR | 100 | 200 | mg/day |  |
| InhR | 20 | 7.6 | m3/day |  |
| EF | 365 | 365 | Days/year |  |
| ED | 24 | 6 | Years |  |
| BW | 70 | 15 | Kg |  |
| AT | 8760 | 2190 | Days |  |
| SA | 5700 | 2800 | cm2 |  |
| ABS | 0.001 | 0.001 | unitless |  |
| AF | 0.07 | 0.2 | mg/cm3/day |  |
| PEF | 1.36 × 109 | 1.36 × 109 | m3/kg |  |

Table S3 Rfds and CSFs used for health risk assessment in this Study

|  | Cd | Co | Cr | Cu | Mn | Ni | Pb | Zn | As |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RfDing | 0.001 | 0.02 | 0.003 | 0.04 | 0.047 | 0.02 | 0.0035 | 0.3 | 0.0003 |
| RfDderm | 0.00001 | 0.016 | 0.00005 | 0.012 | 0.00184 | 0.0054 | 0.000525 | 0.06 | 0.000301 |
| CSFing | 0.0038 | – | 0.5 | – | – | 1.70E + 00 | 0.0085 | – | 1.50E + 00 |
| CSFderm | – | – | 2.00E + 01 | – | – | 4.25E + 01 | – | – | 3.66E + 00 |

Table S4 Sample location and geographical coordinates of the study area

| Sample code |  | Location | Latitude | Longitude | Elevation |
| --- | --- | --- | --- | --- | --- |
| ST1 and ST2 |  | Ngodilom | 04°1'20.9" | 010°42'52.3" | 303 |
| ST3and ST4 |  | Ngodilom | 0401'14.7" | 01043'01.8" | 281 |
| ST5 |  | Ngodilom | 04°01'15.3" | 010°43'04.1" | 290 |
| ST6 |  | Ngodilom | 04°01'53.5" | 010°43'53.7" | 300 |
| ST7&ST8 |  | sokla | 04°01'35.2" | 010°43'46.9" | 336 |
| ST9&ST10 |  | Lepaha | 04°01'34.5" | 010°43'47.7" | 313 |
| ST11A &ST11B |  | Lepaha | 04°01'35.8" | 010°43'46.8" | 406 |
| ST12 |  | Pogue | 04°01'66.7" | 010°43'27.9" | 293 |
| S13 |  | Pogue | 04°00'23.0" | 010°41'39.9" | 294 |