Geochemistry, sources, human health and ecological risk of trace elements in soil from the Mirsarai export processing zone, southeastern coast of Bangladesh

Md. Shazzadur Rahman a,b,c,*, Md. Ripaj Uddin d,*, Mohammad Moniruzzaman e, Md. Golam Mostafa a,c, Lutfa Ara Takeya a, Md. Aminur Rahman c, Md. Nakib Hossen c, Md. Ashikuzzaman f, Ashiqul Amin Abid g

a Department of Geology and Mining, University of Rajshahi, Rajshahi-6205, Bangladesh

b Institute of Environmental Sciences, University of Rajshahi, Rajshahi-6205, Bangladesh

cInstitute of Mining, Mineralogy and Metallurgy (IMMM), Bangladesh Council of Scientific and Industrial Research (BCSIR), Joypurhat, Bangladesh.

d Institute of National Analytical Research and Service (INARS), Bangladesh Council of Scientific and Industrial Research (BCSIR), Dhanmondi, Dhaka 1205, Bangladesh

e Central Analytical and Research Facilities (CARF), BCSIR, Dhaka-1205

f Department of Environmental Science, Bangladesh Agriculture University, Mymensingh 2202, Bangladesh

g Department of Chemistry, University of Dhaka, Dhaka-1000, Bangladesh

*Corresponding author email address: shazzad_geol@yahoo.com (Md. Shazzadur Rahman); md.ripajuddin@gmail.com (Md. Ripaj Uddin)

Supplementary information

Table S1. Pollution indices, standard formula, and Ranking scale for sediment.

| Index | Formula | Ranking scale | References |
| --- | --- | --- | --- |
| Enrichment factor (EF) | EF = MetalFesampleMetalFebackground | EF < 2: Minimal enrichment 2 ≤ EF < 5: Moderate enrichment 5≤EF < 20: Significant enrichment 20≤EF< 40: Very high enrichment EF≥40: Extremely high enrichment | Hasan et al., 2024 Taylor, 1964 |
| Geoaccumulation index (Igeo) | Igeo = Log2 (Cn)1.5 (Bn) | Igeo ≤ 0: Unpolluted 0<Igeo≤1: Unpolluted to moderately polluted 1<Igeo≤2: Moderately polluted 2<Igeo≤3: Moderately to heavily polluted 3<Igeo≤4: Heavily polluted 4<Igeo≤5: Heavily to extremely polluted Igeo>5: Extremely polluted | Muller, 1969 Loska et al., 2004 |
| Contamination factor (CF) | CF = CnBn | CF < 1: Low contamination 1≤CF<3: Moderate contamination 3≤CF<6: Considerable contamination CF ≥ 6: Very high contamination | Hakanson, 1980 Siddique et al., 2020 |
| Pollution load index (PLI) | PLI = (CF1 × CF2 × CF3……× CFn)1/n | PLI 1: Baseline level of pollutants PLI < 1: No pollution (permissible level) PLI > 1: Deterioration of site quality | Tomlinson et al., 1980 |
| Nemerow integrated pollution index (NIPI) | NIPI = CFMean2+ CFMaximum22 | NIPI ≤ 0.7: Clean 0.7 < NIPI ≤ 1: Alert level 1 < NIPI ≤ 2: Slight pollution 2 < NIPI ≤ 3: Moderate pollution NIPI > 3: Heavy pollution | Nemerow, 1985 |
| Toxic unit | TUi = CiPECi | TU < 1: Low ecological risk TU ≥ 1: Potential ecological risk | MacDonald et al., 2000 |
| Ecological risk factor (ERF) | ERF=Tr× CFi | Eᵣ < 40: Low ecological risk 40 ≤ Eᵣ < 80: Moderate risk 80 ≤ Eᵣ < 160: Considerable risk 160 ≤ Eᵣ < 320: High risk Eᵣ ≥ 320: Very high risk | Hakanson, 1980 |
| Potential ecological risk index (PERI) | PERI=∑ERF=∑ (Tr × CFi) | PERI < 150: Low ecological risk 150 ≤ PERI < 300: Moderate risk 300 ≤ PERI < 600: Considerable risk PERI ≥ 600: Very high risk | Hakanson, 1980 |

Table S2. Exposure parameters and their values were adopted from USEPA

| Parameter | Unit | Children | Adults | References |
| --- | --- | --- | --- | --- |
| Metal concentration (c) | mg/kg |  |  | USEPA, 1989, 1991, 2004, 2020 |
| Body weight (BW) | kg | 15 | 70 |  |
| Exposure time (ET) | h/day | 0.71 | 0.54 |  |
| Exposure frequency (EF) | days/year | 350 | 350 |  |
| Exposure duration (ED) | years | 6 | 30 |  |
| Ingestion rate (IngR) | mg/day | 200 | 100 |  |
| Inhalation rate (InhR) | m3/day | 10 | 20 |  |
| Skin surface area (SA) | cm2 | 2100 | 5800 |  |
| Soil adherence factor (AF) | mg/ cm2 | 0.2 | 0.07 |  |
| Dermal absorption factor (ABS) | — | 0.1 | 0.1 |  |
| Particulate emission factor (PEF) | m3/kg | 1.3 × 109 | 1.3 × 109 |  |
| Conversion factor (CF) | kg/mg | 10⁻6 | 10⁻6 |  |
| Average time (AT) for carcinogens | days | 365 × 70 | 365 × 70 |  |
| Average time (AT) for non-carcinogens | days | 365 × 6 | 365 × 30 |  |

Table S3. Reference dose (RfD), Dermal coefficient (Kp) and Carcinogenic slope factor value for health risk calculation

| Metals | Reference does value (RfD) |  |  | Carcinogenic slope factor (CSF) |  |  | Dermal coefficient (Kp) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ingestion | Dermal | Inhalation | Ingestion | Dermal | Inhalation |  |
| Fe | 0.7 | 0.35 | - | - | - | - | 0.001 |
| Mn | 0.14 | 0.0041 | 0.00005 | - | - | - | 0.001 |
| Ni | 0.02 | 0.0054 | 0.0001 | 1.7 | 0.84 | 0.91 | 0.0002 |
| Cu | 0.04 | 0.0012 | 0.01 | - | - | - | 0.001 |
| Zn | 0.3 | 0.06 | 0.005 | - | - | - | 0.0006 |
| Cr | 0.003 | 0.0006 | 0.0001 | 0.501 | 2.0 | 0.42 | 0.002 |
| Cd | 0.001 | 0.0025 | 0.00001 | 0.38 | 6.3 | 6.3 | 0.001 |
| Pb | 0.0035 | 0.000525 | - | 0.0085 | 0.042 | 0.0000085 | 0.0001 |
| As | 0.0003 | 0.000123 | - | 1.5 | 1.5 | 0.0043 | 0.001 |
| Se | 0.005 | 0.0015 | - | - | - | - | 0.001 |
| Hg | 0.0003 | 0.000021 | - | - | - | - | 0.001 |
| Co | 0.02 | 0.0049 | 0.000143 | - | - | - | 0.0004 |

[USEPA, 2009, 2010, 2017]

Table S4. Principal component analysis of trace element in soil from Mirsarai EPZ, Chittagong, Bangladesh

| Trace elements | Sediment |  |  |
| --- | --- | --- | --- |
| Principal component | PC1 | PC2 | PC3 |
| As | 0.25064 | -0.38745 | -0.25744 |
| Se | 0.07715 | 0.57783 | -0.09148 |
| Pb | 0.34981 | 0.02394 | 0.16828 |
| Cd | -0.02873 | -0.29778 | 0.8094 |
| Co | 0.35315 | 0.14775 | 0.04293 |
| Cr | 0.33038 | 0.05398 | -0.23562 |
| Cu | 0.34797 | -0.11172 | 9.90E-04 |
| Mn | 0.33707 | -0.132 | 0.0731 |
| Ni | 0.33279 | 0.25212 | 0.0026 |
| Zn | 0.33743 | 0.08392 | 0.18191 |
| Hg | -0.03522 | 0.53298 | 0.3752 |
| Fe | 0.33707 | -0.132 | 0.0731 |
| Eigenvalue | 7.50722 | 2.10569 | 1.14127 |
| Total variance % | 62.56% | 17.55% | 9.51% |
| Cumulative % of variance | 62.56% | 80.11% | 89.62% |

Figure S1. Biplot of principal component (a) and K-means cluster (b) of studied sediment in Mirsarai EPZ, Bangladesh

(a)

(b)

Figure S2. The Monte Carlo simulation generates probabilistic distributions for soil in both children and adults, specifically assessing carcinogenic risks. These risks are evaluated separately for: (a) child and (b) adults

(a)

(b)

(a)

(b)

References

Hakanson, L., 1980. An ecological risk index for aquatic pollution control.a sedimentological approach. Water Res 14, 975–1001. https://doi.org/10.1016/ 0043-1354(80)90143-8.

Hasan, A.B., Reza, A.S., Siddique, M.A.B., Akbor, M.A., Nahar, A., Hasan, M., Uddin, M.R., Zaman, M.N. and Islam, I., 2024. Origin, spatial distribution, sediment contamination, ecological and health risk evaluation of trace metals in sediments of ship breaking area of Bangladesh. Journal of Hazardous Materials, 465, p.133214.

Horton, R.K., 1965. An index number system for rating water quality. J Water Pollut Control Fed, 37(3), pp.300-306.

Linjin, L., Baohui, M. and Rui, P., 2021. Water quality evaluation of Wenyu River based on single factor evaluation and comprehensive pollution index method. Nature Environment and Pollution Technology, 20(3), pp.1041-1046.

Loska, K., Wiechuła, D., Korus, I., 2004. Metal contamination of farming soils affected by industry. Environ Int 30, 159–165. https://doi.org/10.1016/S0160- 4120(03)00157-0.

MacDonald, D.D., Ingersoll, C.G., Berger, T.A., 2000. Development and evaluation of consensus-based sediment quality guidelines for freshwater ecosystems. Arch Environ Contam Toxicol 39, 20–31.

Mohan, S.V., Nithila, P. and Reddy, S.J., 1996. Estimation of heavy metals in drinking water and development of heavy metal pollution index. Journal of Environmental Science & Health Part A, 31(2), pp.283-289.

Muller, G., 1969. Index of geoaccumulation in sediments of the Rhine River. Geo J 2, 108–118

Nemerow, N.L., 1985. Stream, Lake, Estuary, and Ocean Pollution. Van Nostrand Reinhold Publishing, New York, NY, USA.

Prasanna, M.V., Praveena, S.M., Chidambaram, S., Nagarajan, R. and Elayaraja, A., 2012. Evaluation of water quality pollution indices for heavy metal contamination monitoring: a case study from Curtin Lake, Miri City, East Malaysia. Environmental Earth Sciences, 67(7), pp.1987-2001.

Saha, P. and Paul, B., 2019. Assessment of heavy metal toxicity related with human health risk in the surface water of an industrialized area by a novel technique. Human and ecological risk assessment: an international journal.

Siddique, M.A.B., Alam, M.K., Islam, S., Diganta, M.T.M., Akbor, M.A., Bithi, U. H., et al., 2020. Apportionment of some chemical elements in soils around the coal mining area in northern Bangladesh and associated health risk assessment. Environ Nanotechnol, Monit Manag 14, 100366. https://doi.org/10.1016/j. enmm.2020.100366

Taylor, S.R., 1964. Abundance of chemical elements in the continental crust: a new table. GeochimCosmochimActa 28(8):1273–1285

Tomlinson, D.L., Wilson, J.G., Harris, C.R., Jeffrey, D.W., 1980. Problems in the assessment of heavy-metal levels in estuaries and the formation of a pollution index. Helgol¨ander Meeresunters 33, 566–575.

USEPA, 1989. Risk assessment guidance for Superfund. Volume I: Human health evaluation manual (Part A). Interim Final. Office of Emergency and Remedial Response. EPA/540/1-89/002. 1989.

USEPA, 1991. Human health evaluation manual, supplemental guidance: "Standard default exposure factors ". OSWER Directive 9285.6-03.

USEPA, 2004. Risk Assessment Guidance for Superfund Volume I: Human Health Evaluation Manual (Part E, Supplemental Guidance for Dermal Risk Assessment) Final. OSWER 9285.7-02EP. 2004. July 2004.

USEPA, 2009. Risk-based Concentration Table. United States Environmental Protection Agency, Washington D.C., USA, Philadelphia PA.

USEPA, 2010. Integrated risk information system (IRIS). United States Environmental Protection Agency, Washington D.C., USA.

USEPA, 2017. Integrated Risk Information System National Center for Environmental Assessment Office of Research and Development U.S.; Environmental Protection Agency: Washington, DC, USA.

USEPA, 2020. Exposure Assessment Tools by Media - Water and Sediment. https://www.epa.gov/expobox/exposure-assessment-tools-media-water-andsediment, 2020. Accessed date: 08 January 2020.

USEPA 2022. Basic Information about the Integrated Risk Information System (IRIS). EPA IRIS