Additional Table 1 Summary of executing HRV parameters

| Parameter | Units | Description |
| --- | --- | --- |
| Time domain |  |  |
| Mean RR | ms | The mean of RR intervals |
| SDNN | ms | Standard deviation of RR intervals |
| Mean HR | min−1 | The mean heart rate |
| STD HR | min−1 | Standard deviation of instantaneous heart rate values |
| Min HR | min−1 | Minimum HR computed using N beat moving average |
| Max HR | min−1 | Maximum HR computed using N beat moving average |
| RMSSD | ms | Square root of the mean squared differences between successive RR intervals |
| NN50 | Beats | Number of successive RR interval pairs that differ more than 50 ms |
| pNN50 | % | NNxx divided by the total number of RR intervals |
| HRV triangular index |  | The integral of the RR interval histogram divided by the height of the histogram |
| TINN | ms | Baseline width of the RR interval histogram |
| Frequency-domain—spectrum Welch’s periodogram |  |  |
| Peak frequency | Hz | VLF, LF, and HF band peak frequencies |
| Absolute power | ms2 | Absolute powers of VLF, LF, and HF bands |
| Absolute power | log | Natural logarithm transformed values of absolute powers of VLF, LF, and HF bands |
| Relative power | % | Relative powers of VLF, LF, and HF bands |
| Normalized power | n.u | Powers of LF and HF bands in normalized units (VLF not included) |
| Total power | ms2 | Total absolute powers (VLF + LF + HF) |
| LF HF ratio |  | Ratio between LF and HF band powers |

Additional Table 2 Summary of audiometry statistical data analysis repot

|  | Dunnett's multiple comparisons test | Predicted (LS) mean diff. | 95.00% CI of diff. | Adjusted P Value | Predicted (LS) mean diff. | 95.00% CI of diff. | Adjusted P Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0.125 kHz | Normal vs. Mild | -4.62 | -6.36 to -2.88 | <0.0001 | -3.33 | -6.79 to 0.133 | 0.0633 |
|  | Normal vs. Moderate | -1.68 | -3.72 to 0.374 | 0.1419 | -3.16 | -8.42 to 2.10 | 0.3777 |
|  | Normal vs. Severe | -1.98 | -4.25 to 0.283 | 0.1044 | -3.13 | -8.13 to 1.88 | 0.3433 |
| 0.250 kHz | Normal vs. Mild | -4.87 | -6.61 to -3.13 | <0.0001 | -3.32 | -6.78 to 0.141 | 0.0642 |
|  | Normal vs. Moderate | -1.6 | -3.65 to 0.451 | 0.1719 | -3.61 | -8.87 to 1.65 | 0.2668 |
|  | Normal vs. Severe | -0.91 | -3.18 to 1.36 | 0.6947 | -2.77 | -7.78 to 2.23 | 0.4472 |
| 0.500 kHz | Normal vs. Mild | -3.02 | -4.76 to -1.28 | 0.0001 | -1.04 | -4.51 to 2.42 | 0.8421 |
|  | Normal vs. Moderate | -2.3 | -4.35 to -0.248 | 0.0225 | -1.49 | -6.75 to 3.77 | 0.8646 |
|  | Normal vs. Severe | -0.672 | -2.94 to 1.60 | 0.849 | -1.94 | -6.95 to 3.06 | 0.7158 |
| 0.750 kHz | Normal vs. Mild | -2.91 | -4.65 to -1.17 | 0.0002 | -2.04 | -5.50 to 1.42 | 0.3944 |
|  | Normal vs. Moderate | -1.83 | -3.88 to 0.217 | 0.0938 | -1.88 | -7.14 to 3.38 | 0.7625 |
|  | Normal vs. Severe | -1.46 | -3.73 to 0.803 | 0.3154 | -3.13 | -8.14 to 1.87 | 0.3414 |
| 1 kHz | Normal vs. Mild | -2.26 | -4.00 to -0.520 | 0.006 | -1.64 | -5.10 to 1.82 | 0.5763 |
|  | Normal vs. Moderate | -1.22 | -3.27 to 0.833 | 0.387 | -2.21 | -7.47 to 3.05 | 0.6654 |
|  | Normal vs. Severe | -0.834 | -3.10 to 1.43 | 0.7478 | -3.99 | -9.00 to 1.01 | 0.157 |
| 1.5 kHz | Normal vs. Mild | -2.15 | -3.89 to -0.412 | 0.0097 | -2.24 | -5.71 to 1.22 | 0.3133 |
|  | Normal vs. Moderate | -2.25 | -4.30 to -0.201 | 0.0263 | -2.06 | -7.32 to 3.20 | 0.7093 |
|  | Normal vs. Severe | -0.941 | -3.21 to 1.33 | 0.6728 | -5.04 | -10.0 to -0.0332 | 0.048 |
| 2 kHz | Normal vs. Mild | -2.25 | -3.99 to -0.514 | 0.0062 | -2.09 | -5.55 to 1.38 | 0.374 |
|  | Normal vs. Moderate | -2.61 | -4.66 to -0.562 | 0.0073 | -4.98 | -10.2 to 0.283 | 0.0694 |
|  | Normal vs. Severe | -2.25 | -4.52 to 0.0138 | 0.0519 | -6.52 | -11.5 to -1.51 | 0.0059 |
| 3 kHz | Normal vs. Mild | -5.57 | -7.31 to -3.83 | <0.0001 | -3.45 | -6.91 to 0.0126 | 0.0511 |
|  | Normal vs. Moderate | -6.62 | -8.67 to -4.57 | <0.0001 | -11.4 | -16.6 to -6.10 | <0.0001 |
|  | Normal vs. Severe | -10.3 | -12.6 to -8.04 | <0.0001 | -13.5 | -18.5 to -8.47 | <0.0001 |
| 4 kHz | Normal vs. Mild | -8.84 | -10.6 to -7.10 | <0.0001 | -5.13 | -8.60 to -1.67 | 0.0013 |
|  | Normal vs. Moderate | -12.7 | -14.8 to -10.7 | <0.0001 | -18.4 | -23.6 to -13.1 | <0.0001 |
|  | Normal vs. Severe | -26.7 | -28.9 to -24.4 | <0.0001 | -23 | -28.0 to -18.0 | <0.0001 |
| 6 kHz | Normal vs. Mild | -12.7 | -14.4 to -10.9 | <0.0001 | -9.75 | -13.2 to -6.29 | <0.0001 |
|  | Normal vs. Moderate | -19.9 | -22.0 to -17.9 | <0.0001 | -23.3 | -28.6 to -18.0 | <0.0001 |
|  | Normal vs. Severe | -41.4 | -43.7 to -39.1 | <0.0001 | -40.6 | -45.6 to -35.6 | <0.0001 |
| 8 kHz | Normal vs. Mild | -15.9 | -17.7 to -14.2 | <0.0001 | -6.35 | -9.81 to -2.88 | <0.0001 |
|  | Normal vs. Moderate | -26.4 | -28.4 to -24.3 | <0.0001 | -20.8 | -26.1 to -15.6 | <0.0001 |
|  | Normal vs. Severe | -39.5 | -41.7 to -37.2 | <0.0001 | -44.7 | -49.7 to -39.7 | <0.0001 |

Additional Table 3. Individual Wave I-V Latency Statistical Analysis. The table presents the mean latencies, standard deviations, and results from statistical tests comparing latencies between normal and impaired hearing groups (mild, moderate, severe). For each wave, latency values are displayed along with their corresponding 95% confidence intervals (CI) and p-values, reflecting differences in wave latencies at each severity level.

|  | Peak I |  |  |  | Peak III |  |  | Peak V |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Decibel | Dunnett's multiple comparisons test | Predicted (LS) mean diff. | 95.00% CI of diff. | Adjusted P Value | Predicted (LS) mean diff. | 95.00% CI of diff. | Adjusted P Value | Predicted (LS) mean diff. | 95.00% CI of diff. | Adjusted P Value |
| 20 | No vs. Mild | -0.0197 | -0.0644 to 0.0251 | 0.62 | -0.373 | -0.427 to -0.319 | <0.001 | -0.462 | -0.537 to -0.387 | <0.001 |
|  | Normal vs. Moderate | -0.0647 | -0.113 to -0.0165 | 0.004 | -0.852 | -0.910 to -0.793 | <0.001 | -0.906 | -0.987 to -0.825 | <0.001 |
|  | Normal vs. Severe | -0.119 | -0.180 to -0.0578 | <0.001 | -1.05 | -1.13 to -0.978 | <0.001 | -1.02 | -1.12 to -0.912 | <0.001 |
| 25 | Normal vs. Mild | -0.0197 | -0.0644 to 0.0251 | 0.62 | -0.32 | -0.374 to -0.266 | <0.001 | -0.409 | -0.484 to -0.334 | <0.001 |
|  | Normal vs. Moderate | -0.0647 | -0.113 to -0.0165 | 0.004 | -0.767 | -0.825 to -0.708 | <0.001 | -0.821 | -0.902 to -0.740 | <0.001 |
|  | Normal vs. Severe | -0.119 | -0.180 to -0.0578 | <0.001 | -0.867 | -0.941 to -0.793 | <0.001 | -0.83 | -0.933 to -0.727 | <0.001 |
| 30 | Normal vs. Mild | -0.0197 | -0.0644 to 0.0251 | 0.62 | -0.495 | -0.549 to -0.441 | <0.001 | -0.484 | -0.560 to -0.409 | <0.001 |
|  | Normal vs. Moderate | -0.0647 | -0.113 to -0.0165 | 0.004 | -0.886 | -0.944 to -0.828 | <0.001 | -0.84 | -0.921 to -0.759 | <0.001 |
|  | Normal vs. Severe | -0.119 | -0.180 to -0.0578 | <0.001 | -0.916 | -0.990 to -0.842 | <0.001 | -0.779 | -0.882 to -0.676 | <0.001 |
| 35 | Normal vs. Mild | -0.0197 | -0.0644 to 0.0251 | 0.62 | -0.447 | -0.501 to -0.393 | <0.001 | -0.386 | -0.461 to -0.311 | <0.001 |
|  | Normal vs. Moderate | -0.0647 | -0.113 to -0.0165 | 0.004 | -0.871 | -0.929 to -0.813 | <0.001 | -0.775 | -0.856 to -0.694 | <0.001 |
|  | Normal vs. Severe | -0.119 | -0.180 to -0.0578 | <0.001 | -0.931 | -1.00 to -0.857 | <0.001 | -0.744 | -0.847 to -0.641 | <0.001 |
| 40 | Normal vs. Mild | -0.0197 | -0.0644 to 0.0251 | 0.62 | -0.55 | -0.604 to -0.496 | <0.001 | -0.509 | -0.585 to -0.434 | <0.001 |
|  | Normal vs. Moderate | -0.0647 | -0.113 to -0.0165 | 0.004 | -0.732 | -0.791 to -0.674 | <0.001 | -0.657 | -0.738 to -0.576 | <0.001 |
|  | Normal vs. Severe | -0.119 | -0.180 to -0.0578 | <0.001 | -0.88 | -0.954 to -0.806 | <0.001 | -0.713 | -0.816 to -0.610 | <0.001 |
| 45 | Normal vs. Mild | -0.0197 | -0.0644 to 0.0251 | 0.62 | -0.443 | -0.497 to -0.389 | <0.001 | -0.432 | -0.507 to -0.356 | <0.001 |
|  | Normal vs. Moderate | -0.0647 | -0.113 to -0.0165 | 0.004 | -0.706 | -0.764 to -0.647 | <0.001 | -0.66 | -0.741 to -0.579 | <0.001 |
|  | Normal vs. Severe | -0.119 | -0.180 to -0.0578 | <0.001 | -0.766 | -0.840 to -0.692 | <0.001 | -0.629 | -0.732 to -0.526 | <0.001 |
| 50 | Normal vs. Mild | -0.0197 | -0.0644 to 0.0251 | 0.62 | -0.57 | -0.625 to -0.516 | <0.001 | -0.459 | -0.535 to -0.384 | <0.001 |
|  | Normal vs. Moderate | -0.0647 | -0.113 to -0.0165 | 0.004 | -0.794 | -0.852 to -0.736 | <0.001 | -0.649 | -0.730 to -0.568 | <0.001 |
|  | Normal vs. Severe | -0.119 | -0.180 to -0.0578 | <0.001 | -0.854 | -0.928 to -0.780 | <0.001 | -0.618 | -0.721 to -0.515 | <0.001 |
| 55 | Normal vs. Mild | -0.0197 | -0.0644 to 0.0251 | 0.62 | -0.326 | -0.380 to -0.272 | <0.001 | -0.215 | -0.290 to -0.140 | <0.001 |
|  | Normal vs. Moderate | -0.0647 | -0.113 to -0.0165 | 0.004 | -0.727 | -0.785 to -0.669 | <0.001 | -0.581 | -0.662 to -0.500 | <0.001 |
|  | Normal vs. Severe | -0.119 | -0.180 to -0.0578 | <0.001 | -0.797 | -0.871 to -0.723 | <0.001 | -0.561 | -0.663 to -0.458 | <0.001 |
| 60 | Normal vs. Mild | -0.0197 | -0.0644 to 0.0251 | 0.62 | -0.294 | -0.348 to -0.240 | <0.001 | -0.0833 | -0.159 to -0.00800 | 0.03 |
|  | Normal vs. Moderate | -0.0647 | -0.113 to -0.0165 | 0.004 | -0.614 | -0.673 to -0.556 | <0.001 | -0.369 | -0.450 to -0.288 | <0.001 |
|  | Normal vs. Severe | -0.119 | -0.180 to -0.0578 | <0.001 | -0.814 | -0.888 to -0.740 | <0.001 | -0.478 | -0.581 to -0.375 | <0.001 |
| 65 | Normal vs. Mild | -0.0197 | -0.0644 to 0.0251 | 0.62 | -0.113 | -0.167 to -0.0592 | <0.001 | -0.0123 | -0.0876 to 0.0629 | 0.97 |
|  | Normal vs. Moderate | -0.0647 | -0.113 to -0.0165 | 0.004 | -0.444 | -0.502 to -0.385 | <0.001 | -0.308 | -0.389 to -0.227 | <0.001 |
|  | Normal vs. Severe | -0.119 | -0.180 to -0.0578 | <0.001 | -0.644 | -0.718 to -0.570 | <0.001 | -0.417 | -0.520 to -0.314 | <0.001 |
| 70 | Normal vs. Mild | -0.0197 | -0.0644 to 0.0251 | 0.62 | -0.103 | -0.157 to -0.0491 | <0.001 | -0.0322 | -0.107 to 0.0431 | 0.64 |
|  | Normal vs. Moderate | -0.0647 | -0.113 to -0.0165 | 0.004 | -0.349 | -0.407 to -0.290 | <0.001 | -0.243 | -0.324 to -0.162 | <0.001 |
|  | Normal vs. Severe | -0.119 | -0.180 to -0.0578 | <0.001 | -0.449 | -0.523 to -0.375 | <0.001 | -0.252 | -0.355 to -0.149 | <0.001 |

Additional Table 4. Individual Wave I-V Amplitude Statistical Analysis. The table presents the mean amplitude, standard deviations, and results from statistical tests comparing latencies between normal and impaired hearing groups (mild, moderate, severe). For each wave, amplitude values are displayed along with their corresponding 95% confidence intervals (CI) and p-values, reflecting differences in wave amplitude at each severity level

|  | Amplitude I-Ia |  |  |  | Amplitude III-IIIa |  |  | Amplitude V-Va |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Decibel | Dunnett's multiple comparisons test | Predicted (LS) mean diff. | 95.00% CI of diff. | Adjusted P Value | Predicted (LS) mean diff. | 95.00% CI of diff. | Adjusted P Value | Predicted (LS) mean diff. | 95.00% CI of diff. | Adjusted P Value |
| 20 | No vs. Mild | 0.921 | 0.732 to 1.11 | <0.001 | 0.184 | 0.108 to 0.261 | <0.001 | 0.153 | 0.0656 to 0.239 | <0.001 |
|  | Normal vs. Moderate | 1.28 | 1.07 to 1.48 | <0.001 | 0.258 | 0.175 to 0.340 | <0.001 | 0.41 | 0.316 to 0.504 | <0.001 |
|  | Normal vs. Severe | 1.28 | 1.02 to 1.54 | <0.001 | 0.258 | 0.153 to 0.363 | <0.001 | 0.41 | 0.291 to 0.529 | <0.001 |
| 25 | Normal vs. Mild | 0.987 | 0.798 to 1.18 | <0.001 | 0.197 | 0.120 to 0.273 | <0.001 | 0.159 | 0.0721 to 0.246 | <0.001 |
|  | Normal vs. Moderate | 1.62 | 1.41 to 1.82 | <0.001 | 0.326 | 0.244 to 0.409 | <0.001 | 0.428 | 0.334 to 0.521 | <0.001 |
|  | Normal vs. Severe | 1.62 | 1.36 to 1.87 | <0.001 | 0.326 | 0.221 to 0.431 | <0.001 | 0.428 | 0.309 to 0.547 | <0.001 |
| 30 | Normal vs. Mild | 1.09 | 0.903 to 1.28 | <0.001 | 0.217 | 0.140 to 0.293 | <0.001 | 0.135 | 0.0481 to 0.222 | <0.001 |
|  | Normal vs. Moderate | 1.66 | 1.46 to 1.86 | <0.001 | 0.329 | 0.246 to 0.411 | <0.001 | 0.179 | 0.0859 to 0.273 | <0.001 |
|  | Normal vs. Severe | 2.02 | 1.76 to 2.28 | <0.001 | 0.407 | 0.302 to 0.512 | <0.001 | 0.458 | 0.339 to 0.576 | <0.001 |
| 35 | Normal vs. Mild | 1.18 | 0.991 to 1.37 | <0.001 | 0.233 | 0.157 to 0.310 | <0.001 | 0.141 | 0.0539 to 0.228 | <0.001 |
|  | Normal vs. Moderate | 1.72 | 1.52 to 1.92 | <0.001 | 0.336 | 0.254 to 0.419 | <0.001 | 0.132 | 0.0380 to 0.225 | 0.003 |
|  | Normal vs. Severe | 1.99 | 1.74 to 2.25 | <0.001 | 0.374 | 0.269 to 0.479 | <0.001 | 0.187 | 0.0683 to 0.306 | <0.001 |
| 40 | Normal vs. Mild | 0.678 | 0.489 to 0.866 | <0.001 | 0.129 | 0.0518 to 0.205 | <0.001 | 0.46 | 0.373 to 0.547 | <0.001 |
|  | Normal vs. Moderate | 1.73 | 1.53 to 1.93 | <0.001 | 0.333 | 0.250 to 0.415 | <0.001 | 0.465 | 0.371 to 0.559 | <0.001 |
|  | Normal vs. Severe | 2.03 | 1.77 to 2.28 | <0.001 | 0.359 | 0.254 to 0.464 | <0.001 | 0.575 | 0.456 to 0.694 | <0.001 |
| 45 | Normal vs. Mild | 0.592 | 0.403 to 0.780 | <0.001 | 0.109 | 0.0323 to 0.186 | 0.002 | 0.408 | 0.321 to 0.495 | <0.001 |
|  | Normal vs. Moderate | 1.92 | 1.72 to 2.13 | <0.001 | 0.367 | 0.285 to 0.450 | <0.001 | 0.414 | 0.320 to 0.508 | <0.001 |
|  | Normal vs. Severe | 2.17 | 1.91 to 2.43 | <0.001 | 0.364 | 0.259 to 0.469 | <0.001 | 0.488 | 0.369 to 0.607 | <0.001 |
| 50 | Normal vs. Mild | 1.51 | 1.32 to 1.70 | <0.001 | 0.294 | 0.217 to 0.370 | <0.001 | 0.573 | 0.486 to 0.659 | <0.001 |
|  | Normal vs. Moderate | 2.23 | 2.03 to 2.43 | <0.001 | 0.415 | 0.333 to 0.498 | <0.001 | 0.578 | 0.485 to 0.672 | <0.001 |
|  | Normal vs. Severe | 3.04 | 2.78 to 3.30 | <0.001 | 0.52 | 0.415 to 0.625 | <0.001 | 0.565 | 0.446 to 0.684 | <0.001 |
| 55 | Normal vs. Mild | 1.49 | 1.30 to 1.68 | <0.001 | 0.286 | 0.210 to 0.363 | <0.001 | 0.515 | 0.428 to 0.602 | <0.001 |
|  | Normal vs. Moderate | 2.4 | 2.20 to 2.60 | <0.001 | 0.44 | 0.358 to 0.523 | <0.001 | 0.522 | 0.429 to 0.616 | <0.001 |
|  | Normal vs. Severe | 2.92 | 2.67 to 3.18 | <0.001 | 0.433 | 0.328 to 0.538 | <0.001 | 0.46 | 0.341 to 0.579 | <0.001 |
| 60 | Normal vs. Mild | 1.63 | 1.44 to 1.82 | <0.001 | 0.314 | 0.237 to 0.391 | <0.001 | 0.473 | 0.386 to 0.559 | <0.001 |
|  | Normal vs. Moderate | 2.13 | 1.92 to 2.33 | <0.001 | 0.373 | 0.291 to 0.456 | <0.001 | 0.481 | 0.388 to 0.575 | <0.001 |
|  | Normal vs. Severe | 2.6 | 2.34 to 2.85 | <0.001 | 0.309 | 0.204 to 0.414 | <0.001 | 0.49 | 0.371 to 0.609 | <0.001 |
| 65 | Normal vs. Mild | 1.53 | 1.34 to 1.72 | <0.001 | 0.29 | 0.213 to 0.367 | <0.001 | 0.365 | 0.278 to 0.452 | <0.001 |
|  | Normal vs. Moderate | 2.16 | 1.96 to 2.37 | <0.001 | 0.366 | 0.283 to 0.448 | <0.001 | 0.376 | 0.282 to 0.470 | <0.001 |
|  | Normal vs. Severe | 2.75 | 2.50 to 3.01 | <0.001 | 0.284 | 0.180 to 0.389 | <0.001 | 0.387 | 0.268 to 0.506 | <0.001 |
| 70 | Normal vs. Mild | 0.741 | 0.552 to 0.930 | <0.001 | 0.126 | 0.0490 to 0.202 | <0.001 | 0.568 | 0.481 to 0.654 | <0.001 |
|  | Normal vs. Moderate | 1.4 | 1.20 to 1.60 | <0.001 | 0.193 | 0.111 to 0.276 | <0.001 | 0.58 | 0.486 to 0.673 | <0.001 |
|  | Normal vs. Severe | 2.8 | 2.54 to 3.06 | <0.001 | 0.274 | 0.170 to 0.379 | <0.001 | 0.591 | 0.473 to 0.710 | <0.001 |