# Supplementary material

Table S1. MRI acquisition parameters for prospective testing at Philips 1.5T and Philips 3T

| Parameter | Philips 1.5T | Philips 3T |
| --- | --- | --- |
| Basic sequence | 3D FFE with Dixon option | 3D FFE with Dixon option |
| FOV (mm2) | 560x560 | 530x530 |
| TR (ms) | 6.7 | 4.6 |
| TE1/TE2 (ms) | 1.98/4.5 | 1.34/3.0 |
| Averages | 1 | 2 |
| Water-Fat shift (pixels) | 0.5 | 0.6 |
| Acquired Matrix | 280x280 | 264x266 |
| Acquired slice thickness (mm) | 6 | 6 |

Abbreviations: FFE, Fast Field Echo; FOV, Field of View; TR, Repetition Time; TE, Echo Time

Table S2. Demographics of all recruited individuals within the prospective repeatability, reproducibility, and inter/intra-analyst testing

| Age (years) | Sex (M/F) | Weight (kg) | Height (m) | BMI (kg/m2) |
| --- | --- | --- | --- | --- |
| 20 | M | 81 | 1.85 | 23.7 |
| 21 | M | 66 | 1.76 | 21.3 |
| 21 | F | 70 | 1.58 | 28.0 |
| 22 | F | 62 | 1.67 | 22.2 |
| 24 | F | 44 | 1.56 | 18.1 |
| 29 | M | 115 | 1.93 | 30.9 |
| 31 | M | 78 | 1.74 | 25.8 |
| 34 | F | 61 | 1.67 | 21.9 |
| 42 | M | 71 | 1.82 | 21.4 |
| 43 | F | 60 | 1.63 | 22.6 |
| 44 | M | 58 | 1.82 | 17.5 |
| 46 | M | 111 | 1.93 | 29.8 |

Table S3. Demographics of individuals included in the retrospective inter-device and inter-reader (analyst vs. radiologist) assessments

| Age (years) | Sex (M/F) | Weight (kg) | Height (m) | BMI (kg/m2) | Scanner | Field Strength (T) |
| --- | --- | --- | --- | --- | --- | --- |
| 28 | F | 55 | 1.60 | 21.5 | Siemens | 3 |
| 22 | F | 40 | 1.52 | 17.3 | GE | 1.5 |
| 42 | M | 71 | 1.82 | 21.4 | Philips | 3 |
| 41 | M | 78 | 1.82 | 23.5 | Siemens | 3 |
| 31 | F | 69 | 1.59 | 27.3 | GE | 1.5 |
| 32 | F | 55 | 1.50 | 24.4 | GE | 3 |
| 24 | M | 77 | 1.73 | 25.7 | Siemens | 1.5 |
| 34 | F | 61 | 1.65 | 22.4 | Siemens | 3 |
| 24 | F | 45 | 1.66 | 16.3 | Siemens | 1.5 |
| 54 | M | 83 | 1.81 | 25.3 | GE | 1.5 |
| 47 | M | 136 | 1.75 | 44.4 | Philips | 3 |
| 21 | F | 70 | 1.58 | 28.0 | Siemens | 3 |
| 27 | F | 70 | 1.73 | 23.4 | GE | 3 |
| 21 | M | 58 | 1.67 | 20.8 | GE | 1.5 |
| 21 | M | 66 | 1.76 | 21.3 | Philips | 1.5 |
| 31 | M | 78 | 1.74 | 25.8 | Philips | 1.5 |
| 29 | M | 115 | 1.93 | 30.9 | Philips | 1.5 |
| 24 | M | 68 | 1.76 | 22.0 | Siemens | 1.5 |
| 55 | M | 81 | 1.73 | 27.1 | GE | 3 |
| 24 | F | 61 | 1.77 | 19.5 | Siemens | 1.5 |
| 24 | M | 69 | 1.66 | 25.0 | Siemens | 1.5 |
| 24 | F | 44 | 1.56 | 18.1 | Philips | 1.5 |
| 42 | M | 81 | 1.80 | 25.0 | GE | 3 |
| 24 | F | 99 | 1.52 | 42.8 | Siemens | 1.5 |
| 27 | F | 80 | 1.64 | 29.7 | GE | 3 |
| 46 | M | 111 | 1.92 | 30.1 | Siemens | 3 |
| 44 | M | 59 | 1.80 | 18.2 | Siemens | 3 |
| 44 | M | 60 | 1.81 | 18.3 | GE | 3 |
| 22 | F | 62 | 1.67 | 22.2 | Philips | 3 |
| 64 | F | 79 | 1.62 | 30.1 | Philips | 3 |
| 43 | F | 60 | 1.63 | 22.6 | Philips | 1.5 |
| 31 | F | 62 | 1.58 | 24.8 | GE | 1.5 |
| 65 | M | 78 | 1.75 | 25.5 | Philips | 3 |
| 39 | M | 89 | 1.72 | 30.1 | GE | 1.5 |
| 21 | F | 70 | 1.58 | 28.0 | Philips | 1.5 |
| 63 | F | 78 | 1.62 | 29.7 | Philips | 3 |

Table S4. Summary of scan-rescan repeatability results for single-slice body composition measurements for Philips 3T

| Measurement | Scanner | Bias | Lower LoA | Upper LoA | SD | ICC | RC | CoV (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Psoas muscle CSA (cm2) | Philips 3T | -0.092 | -2.0 | 1.9 | 0.99 | 1.00 | 1.9 | 2.7 |
| Psoas muscle Index (cm2/m2) |  | -0.007 | -0.59 | 0.57 | 0.30 | 0.99 | 0.56 | 2.7 |
| Skeletal muscle CSA (cm2) |  | 1.2 | -9.4 | 12 | 5.4 | 0.99 | 10 | 2.3 |
| Skeletal muscle Index (cm2/m2) |  | 0.32 | -2.8 | 3.5 | 1.6 | 0.99 | 3.1 | 2.3 |
| SAT CSA (cm2) |  | -1.0 | -12 | 9.9 | 5.6 | 1.00 | 11 | 2.0 |
| SAT Index (cm2/m2) |  | -0.31 | -3.7 | 3.1 | 1.7 | 1.00 | 3.3 | 2.1 |
| VAT CSA (cm2) |  | -4.1 | -27 | 18 | 11 | 0.98 | 23 | 7.8 |
| VAT Index (cm2/m2) |  | -1.2 | -7.6 | 5.3 | 3.3 | 0.97 | 6.6 | 7.8 |
| VAT/SAT Ratio |  | -0.024 | -0.15 | 0.098 | 0.062 | 0.98 | 0.13 | 7.9 |

Abbreviations: CoV, coefficient of variation (%); CSA, cross-sectional area, ICC, intraclass coefficient; LoA, limits of agreement; RC, repeatability coefficient; SAT, subcutaneous adipose tissue; SD, standard deviation; VAT, visceral adipose tissue

Table S5. Summary of inter-analyst variability for single-slice body composition measurements (Analyst 1 vs. Analyst 2).

| Measurement | Scanner | Bias | Lower LoA | Upper LoA | SD | ICC | RC | CoV (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Psoas muscle CSA (cm2) | Philips 1.5T | 0.17 | -2.3 | 2.6 | 1.3 | 0.99 | 2.4 | 3.3 |
| Psoas muscle Index (cm2/m2) |  | 0.062 | -0.79 | 0.92 | 0.44 | 0.99 | 0.83 | 3.4 |
| Skeletal muscle CSA (cm2) |  | -1.4 | -6.7 | 3.9 | 2.7 | 1.00 | 5.7 | 1.3 |
| Skeletal muscle Index (cm2/m2) |  | -0.42 | -2.1 | 1.2 | 0.83 | 1.00 | 1.8 | 1.3 |
| SAT CSA (cm2) |  | -3.2 | -6.1 | -0.33 | 1.5 | 1.00 | 6.9 | 1.7 |
| SAT Index (cm2/m2) |  | -1.1 | -2.1 | -0.052 | 0.52 | 1.00 | 2.3 | 1.7 |
| VAT CSA (cm2) |  | -0.84 | -3.1 | 1.4 | 1.1 | 1.00 | 2.7 | 1.2 |
| VAT Index (cm2/m2) |  | -0.27 | -0.87 | 0.34 | 0.31 | 1.00 | 0.78 | 1.2 |
| VAT/SAT Ratio |  | 0.0083 | -0.018 | 0.035 | 0.013 | 1.00 | 0.030 | 1.7 |
| Psoas muscle CSA (cm2) | Philips 3T | 1.1 | -0.61 | 2.8 | 0.87 | 0.99 | 2.7 | 5.0 |
| Psoas muscle Index (cm2/m2) |  | 0.39 | -0.24 | 1.0 | 0.32 | 0.98 | 0.97 | 5.0 |
| Skeletal muscle CSA (cm2) |  | 0.82 | -3.8 | 5.5 | 2.4 | 1.00 | 4.7 | 1.2 |
| Skeletal muscle Index (cm2/m2) |  | 0.21 | -1.3 | 1.8 | 0.79 | 1.00 | 1.5 | 1.2 |
| SAT CSA (cm2) |  | -3.0 | -8.3 | 2.3 | 2.7 | 1.00 | 7.8 | 1.7 |
| SAT Index (cm2/m2) |  | -1.1 | -2.5 | 0.36 | 0.74 | 1.00 | 2.5 | 1.8 |
| VAT CSA (cm2) |  | -1.4 | -4.7 | 2.0 | 1.7 | 1.00 | 4.1 | 1.5 |
| VAT Index (cm2/m2) |  | -0.46 | -1.5 | 0.54 | 0.51 | 1.00 | 1.3 | 1.6 |
| VAT/SAT Ratio |  | 0.0042 | -0.019 | 0.027 | 0.012 | 1.00 | 0.023 | 1.5 |

Abbreviations: CoV, coefficient of variation (%); CSA, cross-sectional area, ICC, intraclass coefficient; LoA, limits of agreement; RC, repeatability coefficient; SAT, subcutaneous adipose tissue; SD, standard deviation; VAT, visceral adipose tissue

Table S6. Summary of intra-analyst variability for single-slice body composition measurements for Philips 1.5T for all three analysts.

| Measurement | Bias | Lower LoA | Upper LoA | SD | ICC | RC | CoV (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Analyst 1 |  |  |  |  |  |  |  |
| Psoas muscle CSA (cm2) | 0.050 | -1.8 | 1.9 | 0.94 | 1.00 | 1.8 | 2.5 |  |
| Psoas muscle Index (cm2/m2) | 0.023 | -0.58 | 0.63 | 0.31 | 0.99 | 0.58 | 2.6 |  |
| Skeletal muscle CSA (cm2) | -0.15 | -2.2 | 1.9 | 1.1 | 1.00 | 2.0 | 0.6 |  |
| Skeletal muscle Index (cm2/m2) | -0.017 | -0.71 | 0.68 | 0.35 | 1.00 | 0.66 | 0.6 |  |
| SAT CSA (cm2) | 0.37 | -2.9 | 3.6 | 1.7 | 1.00 | 3.2 | 0.8 |  |
| SAT Index (cm2/m2) | 0.083 | -0.90 | 1.1 | 0.50 | 1.00 | 0.95 | 0.7 |  |
| VAT CSA (cm2) | -0.70 | -4.1 | 2.7 | 1.8 | 1.00 | 3.6 | 2.0 |  |
| VAT Index (cm2/m2) | -0.21 | -1.5 | 1.1 | 0.66 | 1.00 | 1.3 | 1.9 |  |
| VAT/SAT Ratio | -0.0017 | -0.027 | 0.023 | 0.013 | 1.00 | 0.024 | 2.6 |  |
|  | Analyst 2 |  |  |  |  |  |  |  |
| Psoas muscle CSA (cm2) | 0.27 | -2.1 | 2.6 | 1.2 | 0.99 | 2.3 | 3.0 |  |
| Psoas muscle Index (cm2/m2) | 0.057 | -0.74 | 0.85 | 0.40 | 0.99 | 0.77 | 3.0 |  |
| Skeletal muscle CSA (cm2) | 1.2 | -3.6 | 5.9 | 2.4 | 1.00 | 5.1 | 1.1 |  |
| Skeletal muscle Index (cm2/m2) | 0.36 | -1.1 | 1.8 | 0.75 | 1.00 | 1.6 | 1.1 |  |
| SAT CSA (cm2) | -0.35 | -2.3 | 1.6 | 0.98 | 1.00 | 2.0 | 0.5 |  |
| SAT Index (cm2/m2) | -0.067 | -0.91 | 0.77 | 0.43 | 1.00 | 0.82 | 0.5 |  |
| VAT CSA (cm2) | -0.033 | -1.7 | 1.6 | 0.85 | 1.00 | 1.6 | 0.7 |  |
| VAT Index (cm2/m2) | 0.025 | -0.50 | 0.55 | 0.27 | 1.00 | 0.50 | 0.7 |  |
| VAT/SAT Ratio | 0.0017 | -0.012 | 0.016 | 0.0072 | 1.00 | 0.014 | 1.1 |  |
|  | Analyst 3 |  |  |  |  |  |  |  |
| Psoas muscle CSA (cm2) | 0.23 | -1.6 | 2.1 | 0.94 | 1.00 | 1.8 | 2.3 |  |
| Psoas muscle Index (cm2/m2) | 0.089 | -0.53 | 0.71 | 0.32 | 0.99 | 0.62 | 2.4 |  |
| Skeletal muscle CSA (cm2) | -0.77 | -5.9 | 4.4 | 2.6 | 1.00 | 5.1 | 1.2 |  |
| Skeletal muscle Index (cm2/m2) | -0.22 | -2.0 | 1.5 | 0.89 | 1.00 | 1.7 | 1.2 |  |
| SAT CSA (cm2) | -0.21 | -3.4 | 3.0 | 1.6 | 1.00 | 3.1 | 0.8 |  |
| SAT Index (cm2/m2) | 0.058 | -1.1 | 1.2 | 0.59 | 1.00 | 1.1 | 0.7 |  |
| VAT CSA (cm2) | -0.50 | -2.9 | 1.9 | 1.2 | 1.00 | 2.5 | 1.6 |  |
| VAT Index (cm2/m2) | -0.13 | -0.88 | 0.62 | 0.38 | 1.00 | 0.76 | 1.6 |  |
| VAT/SAT Ratio | -0.0017 | -0.024 | 0.020 | 0.011 | 1.00 | 0.021 | 1.9 |  |

| Measurements | Scanner | Bias | Lower LoA | Upper LoA | Pearson | ICC | RC | CoV (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Psoas muscle CSA (cm2) | Philips 1.5T | -0.83 | -3.5 | 1.8 | 0.99* | 0.99 | 3.1 | 5.8 |
| Psoas muscle Index (cm2/m2) |  | -0.31 | -1.3 | 0.68 | 0.99* | 0.98 | 1.1 | 5.8 |
| Skeletal muscle CSA (cm2) |  | 0.53 | -3.2 | 4.3 | 1.00* | 1.00 | 3.8 | 1.0 |
| Skeletal muscle Index (cm2/m2) |  | 0.16 | -1.1 | 1.4 | 1.00* | 1.00 | 1.2 | 0.9 |
| SAT CSA (cm2) |  | 3.0 | -3.1 | 9.2 | 1.00* | 1.00 | 8.4 | 1.2 |
| SAT Index (cm2/m2) |  | 0.91 | -0.77 | 2.6 | 1.00* | 1.00 | 2.4 | 1.1 |
| VAT CSA (cm2) |  | -10 | -26 | 5.9 | 0.97* | 1.00 | 25 | 26 |
| VAT Index (cm2/m2) |  | -3.4 | -8.8 | 2.0 | 0.94* | 1.00 | 8.5 | 26 |
| VAT/SAT Ratio |  | -0.092 | -0.30 | 0.11 | 0.61* | 0.97 | 0.27 | 27 |
| Psoas muscle CSA (cm2) | GE 1.5T | -0.11 | -1.8 | 1.5 | 0.99* | 0.99 | 1.6 | 4.6 |
| Psoas muscle Index (cm2/m2) |  | -0.024 | -0.69 | 0.64 | 0.99* | 0.98 | 0.65 | 4.7 |
| Skeletal muscle CSA (cm2) |  | -2.0 | -9.6 | 5.7 | 0.99* | 0.99 | 8.4 | 3.6 |
| Skeletal muscle Index (cm2/m2) |  | -1.1 | -3.8 | 1.7 | 0.99* | 0.98 | 3.4 | 3.6 |
| SAT CSA (cm2) |  | 0.94 | -6.1 | 8.0 | 0.99* | 1.00 | 7.1 | 1.5 |
| SAT Index (cm2/m2) |  | 0.21 | -2.5 | 2.9 | 1.00* | 1.00 | 2.7 | 1.5 |
| VAT CSA (cm2) |  | -8.2 | -27 | 11 | 1.00* | 0.99 | 25 | 6.1 |
| VAT Index (cm2/m2) |  | -2.3 | -6.9 | 2.3 | 1.00* | 0.99 | 6.4 | 5.4 |
| VAT/SAT Ratio |  | -0.047 | -0.13 | 0.036 | 0.99* | 0.98 | 0.12 | 6.7 |
| Psoas muscle CSA (cm2) | Siemens 1.5T | -0.11 | -1.8 | 1.6 | 0.97* | 0.97 | 1.7 | 3.8 |
| Psoas muscle Index (cm2/m2) |  | -0.046 | -0.63 | 0.54 | 0.97* | 0.97 | 0.58 | 3.8 |
| Skeletal muscle CSA (cm2) |  | -5.3 | -15 | 4.5 | 0.97* | 0.94 | 14 | 5.0 |
| Skeletal muscle Index (cm2/m2) |  | -1.8 | -5.0 | 1.3 | 0.98* | 0.95 | 4.8 | 5.0 |
| SAT CSA (cm2) |  | -2.4 | -11 | 6.4 | 1.00* | 1.00 | 9.8 | 1.7 |
| SAT Index (cm2/m2) |  | -0.76 | -3.8 | 2.3 | 1.00* | 1.00 | 3.3 | 1.7 |
| VAT CSA (cm2) |  | -10 | -30 | 9.6 | 1.00* | 0.99 | 28 | 6.2 |
| VAT Index (cm2/m2) |  | -4.0 | -12 | 4.5 | 1.00* | 0.99 | 11 | 6.2 |
| VAT/SAT Ratio |  | -0.042 | -0.100 | 0.016 | 1.00* | 0.99 | 0.099 | 7.2 |

Abbreviations: CoV, coefficient of variation (%); CSA, cross-sectional area, ICC, intraclass coefficient; LoA, limits of agreement; Pearson, Pearson correlation coefficient; RC, repeatability coefficient; SAT, subcutaneous adipose tissue; SD, standard deviation; VAT, visceral adipose tissue

| Measurement | Scanner | Bias | Lower LoA | Upper LoA | Pearson | ICC | RC | CoV (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Psoas muscle CSA (cm2) | Philips 3T | -0.38 | -2.4 | 1.7 | 0.99* | 0.99 | 2.1 |  | 4.0 |  |
| Psoas muscle Index (cm2/m2) |  | -0.12 | -0.83 | 0.58 | 0.98* | 0.98 | 0.73 |  | 3.9 |  |
| Skeletal muscle CSA (cm2) |  | -0.14 | -4.3 | 4.0 | 1.00* | 1.00 | 4.0 |  | 1.4 |  |
| Skeletal muscle Index (cm2/m2) |  | -0.11 | -1.6 | 1.3 | 1.00* | 1.00 | 1.4 |  | 1.5 |  |
| SAT CSA (cm2) |  | 0.28 | -12 | 12 | 1.00* | 1.00 | 12 |  | 1.5 |  |
| SAT Index (cm2/m2) |  | 0.061 | -4.3 | 4.4 | 1.00* | 1.00 | 4.2 |  | 1.5 |  |
| VAT CSA (cm2) |  | -13 | -27 | 0.68 | 1.00* | 1.00 | 29 |  | 8.2 |  |
| VAT Index (cm2/m2) |  | -4.5 | -10 | 1.1 | 1.00* | 1.00 | 10 |  | 8.3 |  |
| VAT/SAT Ratio |  | -0.081 | -0.17 | 0.0073 | 1.00* | 0.97 | 0.18 |  | 8.5 |  |
| Psoas muscle CSA (cm2) | GE 3T | -0.36 | -1.9 | 1.2 | 0.99* | 0.98 | 1.6 |  | 3.0 |  |
| Psoas muscle Index (cm2/m2) |  | -0.11 | -0.63 | 0.40 | 0.98* | 0.98 | 0.55 |  | 3.0 |  |
| Skeletal muscle CSA (cm2) |  | -4.3 | -12 | 3.2 | 0.99* | 0.97 | 11 |  | 3.3 |  |
| Skeletal muscle Index (cm2/m2) |  | -1.5 | -4.2 | 1.1 | 0.98* | 0.96 | 4.0 |  | 3.5 |  |
| SAT CSA (cm2) |  | 1.6 | -3.8 | 7.0 | 1.00* | 1.00 | 6.1 |  | 1.5 |  |
| SAT Index (cm2/m2) |  | 0.58 | -1.3 | 2.4 | 1.00* | 1.00 | 2.1 |  | 1.4 |  |
| VAT CSA (cm2) |  | -4.1 | -15 | 7.0 | 1.00* | 0.99 | 13 |  | 4.1 |  |
| VAT Index (cm2/m2) |  | -1.3 | -5.0 | 2.3 | 1.00* | 0.99 | 4.4 |  | 4.1 |  |
| VAT/SAT Ratio |  | -0.044 | -0.13 | 0.039 | 1.00* | 0.99 | 0.12 |  | 4.7 |  |
| Psoas muscle CSA (cm2) | Siemens 3T | 0.34 | -2.0 | 2.7 | 0.99* | 0.99 | 2.4 |  | 5.0 |  |
| Psoas muscle Index (cm2/m2) |  | 0.14 | -0.71 | 0.99 | 0.97* | 0.97 | 0.87 |  | 4.9 |  |
| Skeletal muscle CSA (cm2) |  | -1.5 | -7.4 | 4.5 | 0.99* | 0.99 | 6.4 |  | 2.0 |  |
| Skeletal muscle Index (cm2/m2) |  | -0.41 | -2.5 | 1.7 | 0.97* | 0.96 | 2.2 |  | 1.9 |  |
| SAT CSA (cm2) |  | 1.8 | -2.1 | 5.7 | 1.00* | 1.00 | 5.2 |  | 1.3 |  |
| SAT Index (cm2/m2) |  | 0.64 | -0.68 | 2.0 | 1.00* | 1.00 | 1.8 |  | 1.3 |  |
| VAT CSA (cm2) |  | -7.8 | -16 | 0.72 | 1.00* | 0.99 | 17 |  | 16 |  |
| VAT Index (cm2/m2) |  | -2.6 | -5.6 | 0.36 | 1.00* | 0.98 | 5.9 |  | 16 |  |
| VAT/SAT Ratio |  | -0.063 | -0.15 | 0.018 | 0.99* | 0.97 | 0.15 |  | 17 |  |

Table S7. Summary of retrospective inter-reader (analysts vs. radiologists) variability assessment for single-slice body composition measurements obtained from a range of scanner manufacturers (Philips, GE, Siemens) and field strengths (1.5T, 3T). * p<0.001

Abbreviations: CoV, coefficient of variation (%); CSA, cross-sectional area, ICC, intraclass coefficient; LoA, limits of agreement; Pearson, Pearson correlation coefficient; RC, repeatability coefficient; SAT, subcutaneous adipose tissue; SD, standard deviation; VAT, visceral adipose tissue

Table S8. Summary of retrospective inter-device (study software vs. reference software) variability assessment for body composition measurements obtained from a range of scanner manufacturers (Philips, GE, Siemens) and field strengths (1.5T, 3T).

| Measurement | Scanner | Bias | Lower LoA | Upper LoA | SD | ICC | RC | CoV (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Psoas muscle CSA (cm2) | Philips 1.5T | 2.4 | 0.88 | 4.0 | 0.80 | 0.98 | 5 | 1.0 |
| Skeletal muscle CSA (cm2) |  | -5.9 | -18 | 6.1 | 6.1 | 0.99 | 16 | 5.3 |
| SAT CSA (cm2) |  | -1.1 | -9.7 | 7.5 | 4.4 | 1.00 | 8.1 | 3.8 |
| VAT CSA (cm2) |  | 0.96 | -20 | 22 | 11 | 0.96 | 19 | 2.6 |
| Psoas muscle CSA (cm2) | GE 1.5T | 2.2 | -0.95 | 5.3 | 1.6 | 0.90 | 5.1 | 1.7 |
| Skeletal muscle CSA (cm2) |  | -3.2 | -32 | 26 | 15 | 0.95 | 27 | 1.2 |
| SAT CSA (cm2) |  | 4.5 | -6.7 | 16 | 5.7 | 1.00 | 14 | 3.3 |
| VAT CSA (cm2) |  | 0.97 | -14 | 16 | 7.7 | 1.00 | 14 | 9.1 |
| Psoas muscle CSA (cm2) | Siemens 1.5T | 2.5 | -0.19 | 5.2 | 1.4 | 0.84 | 5.5 | 1.7 |
| Skeletal muscle CSA (cm2) |  | -12 | -73 | 49 | 31 | 0.60 | 60 | 1.4 |
| SAT CSA (cm2) |  | 4.2 | -35 | 44 | 20 | 0.99 | 37 | 8.1 |
| VAT CSA (cm2) |  | 6.8 | -31 | 44 | 19 | 0.99 | 37 | 2.1 |
| Psoas muscle CSA (cm2) | Philips 3T | 1.2 | -4.3 | 6.7 | 2.8 | 0.92 | 5.6 | 1.3 |
| Skeletal muscle CSA (cm2) |  | -17 | -59 | 25 | 21 | 0.67 | 50 | 1.7 |
| SAT CSA (cm2) |  | 3.9 | -18 | 25 | 11 | 1.00 | 21 | 3.3 |
| VAT CSA (cm2) |  | 21 | -13 | 54 | 17 | 0.99 | 51 | 1.6 |
| Psoas muscle CSA (cm2) | GE 3T | 3.5 | -0.77 | 7.8 | 2.2 | 0.65 | 7.9 | 1.7 |
| Skeletal muscle CSA (cm2) |  | -6.6 | -31 | 18 | 12 | 0.90 | 26 | 6.7 |
| SAT CSA (cm2) |  | 7.1 | -14 | 28 | 11 | 0.99 | 24 | 5.4 |
| VAT CSA (cm2) |  | 8.6 | -19 | 36 | 14 | 0.97 | 30 | 2.9 |
| Psoas muscle CSA (cm2) | Siemens 3T | 3.5 | -1.0 | 8.0 | 2.3 | 0.88 | 8 | 1.6 |
| Skeletal muscle CSA (cm2) |  | 0.63 | -13 | 15 | 7.2 | 0.98 | 13 | 4.3 |
| SAT CSA (cm2) |  | -2.1 | -11 | 6.4 | 4.3 | 1.00 | 8.7 | 2.0 |
| VAT CSA (cm2) |  | 1.7 | -2.4 | 27 | 13 | 0.99 | 24 | 2.0 |

Abbreviations: CoV, coefficient of variation (%); CSA, cross-sectional area, ICC, intraclass coefficient; LoA, limits of agreement; RC, repeatability coefficient; SAT, subcutaneous adipose tissue; SD, standard deviation; VAT, visceral adipose tissue

Table S9. Measurement discrepancies for inter-device (VitruvianScan vs. OsirixMD) variability assessment for skeletal muscle cross-sectional area (CSA) and skeletal muscle index (SMI)

| Case no. | Summary of radiologist interviews |
| --- | --- |
| 1 | All radiologists agreed that the segmentation performed with OsirixMD was incorrect, and that the segmentation by VitruvianScan segmentation was more accurate |
| 2 | There was consensus agreement among the radiologists that OsirixMD had overestimated the quantity of skeletal muscle, while VitruvianScan provided a more accurate estimation |
| 3 | The radiologists observed very high levels of fatty infiltration, which they identified as the primary factor influencing the discrepancies between the two segmentation approaches. OsirixMD was noted to overestimate the amount of muscle, whereas VitruvianScan underestimated it. In both cases, all radiologists rated VitruvianScan’s performance as equal to or better than that of OsirixMD |
| 4 |  |