# Impurity Profiling HPLC-UV/PDA Method Development, Validation and Relative Response Factor Estimation for Budesonide, Glycopyrronium and Formoterol Fumarate Combination Product

Shrikant V. Kulkarni,1 Rasika R. Korhale,1 Bhaskar D. Musmade,1 Shrinivas G. Bhope,1 Mitesh Nagar,1 Sharad P. Panchgalle,2 Vijaykumar S. More3*

*Corresponding author: Vijaykumar More, e-mail: vijaymore@gmail.com, Phone: +07350060162.

Supplementary Table 1 Known impurities RRF study results

| For GLY and impurities | Concentration (µg/mL) |  |  |  |  |  |  |  | Slope | RRF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.016 | 0.031 | 0.062 | 0.125 | 0.250 | 0.374 | 0.499 | 0.624 |  |  |
|  | Peak areas |  |  |  |  |  |  |  |  |  |
| GLY | 776 | 1265 | 3223 | 6235 | 12751 | 20175 | 26850 | 33407 | 54156.38 | ---- |
| GLY-A | 3561 | 7104 | 14570 | 28559 | 56207 | 84500 | 104307 | 134523 | 212922.99 | 0.25 |
| GLY-B | 625 | 1564 | 3256 | 6002 | 12280 | 19086 | 24944 | 30569 | 49365.00 | 1.10 |
| GLY-C | 1496 | 2812 | 5534 | 12533 | 25267 | 33467 | 46367 | 58071 | 87385.86 | 0.62 |
| For FOF and impurities | Concentration (µg/mL) |  |  |  |  |  |  |  | Slope | RRF |
|  | 0.0125 | 0.025 | 0.050 | 0.100 | 0.200 | 0.300 | 0.400 | 0.500 |  |  |
|  | Peak areas |  |  |  |  |  |  |  |  |  |
| FOF | 3690 | 6390 | 13775 | 28735 | 57730 | 89097 | 120521 | 141433 | 296931.08 | ---- |
| FOF-A | 1838 | 3675 | 7938 | 16841 | 34253 | 52058 | 69622 | 85376 | 178612.513 | 1.66 |
| FOF-B | 2380 | 4556 | 8942 | 19166 | 38742 | 56424 | 73039 | 95289 | 181619.943 | 1.63 |
| FOF-C | 3021 | 5854 | 12727 | 26536 | 52872 | 77399 | 102063 | 131086 | 239124.623 | 1.24 |
| FOF-D | 3269 | 6344 | 14152 | 29120 | 58781 | 89191 | 115299 | 146114 | 289354.83 | 1.03 |
| FOF-E | 3132 | 5556 | 12456 | 24372 | 49257 | 74887 | 96341 | 124313 | 244892.90 | 1.21 |
| FOF-F | ND | 2699 | 4763 | 8536 | 17728 | 27549 | 36206 | 45659 | 95975.13 | 3.09 |

GLY Glycopyrronium, FOF Formoterol Fumarate, RRF Relative Response Factor