Supplementary material

|  |  | Percentage contribution of different chlorine species (%) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Category | Sub-category | HCl (361 Gg) | Fine particulate Cl- (174Gg) | Cl2 (18Gg) | HOCl (79Gg) |
| Coal combustion | Power | 5 | 1 | 4 |  |
|  | Industry | 32 | 8 | 28 |  |
|  | Resident | 9 | 2 | 7 |  |
|  | Agriculture | 1 |  | 2 |  |
| Industry Process | Cement production | 10 | 5 |  |  |
|  | Iron production | 1 | 3 |  |  |
|  | Steel production |  | 4 |  |  |
|  | Hydrochloric acid production |  |  |  |  |
|  | Glass production |  |  |  |  |
| MSW incineration | MSW incineration station | 1 |  |  |  |
|  | MSW open burning | 3 | 3 |  |  |
|  | Medical waste incineration |  |  |  |  |
| Biomass burning | Biomass household burning | 13 | 29 |  |  |
|  | Biomass open burning | 25 | 44 |  |  |
| Disinfectant | Cooling tower |  |  | 1 | 2 |
|  | Water treatment |  |  | 18 | 30 |
|  | Waste water treatment |  |  | 25 | 42 |
|  | Swimming pool |  |  | 6 | 9 |
|  | Environmental disinfection |  |  | 8 | 13 |
|  | Tap water use |  |  | 1 | 2 |
|  | Insecticide |  |  |  | 1 |
| Other |  |  | 1 |  | 1 |

Table S1: Contribution of different source categories to the anthropogenic emissions of HCl, Particulate Cl-, Cl2, and HCl in 2019 in China (Li et al., 2024).

|  | Cl- ion concentration in μg/m3 |  |  |
| --- | --- | --- | --- |
| Sample ID | 1st cycle | 2nd cycle | Percentage of Cl- in 2nd cycle |
| AJ8 | 0.61 | 0.13 | 20.86 |
| AJ15 | 0.30 | 0.07 | 21.89 |
| AJ45 | 0.62 | 0.11 | 18.19 |
| AJ66 | 0.18 | 0.03 | 17.26 |
|  |  | Average factor (%) | 19.55 |
|  |  | Estimated Factor | 0.1955 |

Table S2. This table shows the efficiency factor calculation for the particulate Cl- concentration calculation. This factor (0.1955) was multiplied with the all the readings to get the final concentration.

|  | Cl- ion concentration in μg/m3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sample ID | 1st impinger | 2nd impinger | 3rd impinger | 4th impinger | Sum (2nd+3rd +4th) | Percentage (2nd+3rd +4th) |
| GJNU65 | 0.137143 | 0.106286 | 0.083714 | 0.045352 | 0.235352 | 171.6108 |
| GJNU08 | 0.068584 | 0.067446 | 0.042305 | 0.027698 | 0.137449 | 200.4101 |
| GJNU30 | 0.088167 | 0.088457 | 0.034569 | 0.012695 | 0.135721 | 153.9363 |
|  |  |  |  |  | Average factor (%) | 175.3191 |
|  |  |  |  |  | Estimated factor | 1.75 |

Table S3. This table shows the efficiency factor calculation for the gaseous Cl2 concentration. This factor (1.75) was multiplied with all the reading to get the final concentration.

Chromatogram of a particulate sample (Sample ID: AJNU24-14)

Chromatogram of a particulate sample (Sample ID: AJNU24-14)

Chromatogram for the standard solution of 2 PPM for the anions

Chromatogram for the standard solution of 2 PPM for the anions

Chromatogram of a gaseous sample (Sample ID: GJNU24-14)

Chromatogram of a gaseous sample (Sample ID: GJNU24-14)

Figure S1. This figure shows the chromatogram of the two samples (both gaseous and particulate) along with the standard solution of 2 PPM concentration.

Figure S2. Representation of the changes in the ozone concentration due to changes in the concentration of the particulate Cl- and chlorine in the troposphere.

References:

Li, S., Liu, Y., Zhu, Y., Jin, Y., Hong, Y., Shen, A., ... & Fan, Q. (2024). ACEIC: a comprehensive anthropogenic chlorine emission inventory for China. Atmospheric Chemistry and Physics, 24(20), 11521-11544.