Supplementary Table S1. Analytical characteristics of biomarker assays

| Biomarker | Method / Kit | Manufacturer | Sample dilution | LoD / Sensitivity | Measurement range | Intra-assay CV (%) | Inter-assay CV (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Aggrecan (ACAN) | Human Aggrecan ELISA (REF RIS001R) | BioVendor, Czech Republic | Serum 1:20; SF 1:500 | 0.9 ng/mL | 10–250 ng/mL | 2.11 | 5.56 |
| Periostin | Human Periostin ELISA (REF RAG019R) | BioVendor, Czech Republic | 1:100 | 15 pg/mL | 78–5000 pg/mL | 7.12 | 7.62 |
| CRTAC1 | Human CRTAC1 ELISA (REF RK10738) | ABclonal Technology, China | Undiluted | 0.078 ng/mL | 0.16–10 ng/mL | 5.37 | 6.90 |
| Leptin | Human Leptin ELISA (REF RD191001100) | BioVendor, Czech Republic | 1:3 | 0.2 ng/mL | 1–50 ng/mL | 5.90 | 5.55 |
| HIF-1 | Human HIF-1 ELISA (REF E-EL-H6066) | Elabscience, USA | 1:2 | 37.5 pg/mL | 62.5–4000 pg/mL | 4.73 | 4.98 |
| CTX-II | Human CTX-II ELISA (REF RK09083) | ABclonal Technology, China | Undiluted | 32 pg/mL | 78–5000 pg/mL | 4.27 | 6.97 |
| CXCL12 (SDF-1) | Human CXCL12 ELISA (REF E-EL-H0052) | Elabscience, USA | Undiluted | 0.1 ng/mL | 0.16–10 ng/mL | 5.47 | 7.57 |
| hsCRP | CardioPhase® hsCRP | Siemens Healthineers, Germany | Undiluted | < 0.160 mg/L* | 0.16–10 mg/L* | 2.73 | 3.28 |

*, Depending on reagent lot; CV, coefficient of variation; LoD, limit of detection; SF, synovial fluid.