Supplementary Material

- Fiducial points detection

In a complete cardiac cycle, it can be divided into a systolic phase and a diastolic phase. Four key characteristic points can be identified in each PPG signal, including: (1) onset O, marking the cycle start and minimum vascular volume; (2) systolic peak S, corresponding to end-systole and maximum volume; (3) dicrotic notch N, reflecting the transition from systole to diastole and associated hemodynamic changes; and (4) diastolic peak D, representing the local volume maximum during diastole.

In addition to the original PPG waveform, its first derivative (VPG) and second derivative (APG) are also commonly used for feature extraction. The VPG typically includes the main peak U, main trough V, and secondary peak W; this study also annotates the secondary trough L. The APG consists of six characteristic waves labeled sequentially as a, b, c, d, e, and f. The complete set of fiducial points used in this work is illustrated in Fig. 1.

In this study, a derivative-based approach was employed to detect the aforementioned key feature points. In general, zero-crossing points before the derivative peak are correlated with local minima of the original waveform, whereas zero-crossing points after the derivative peak correspond to local maxima. Specifically, owing to the rapid variation characteristics of the PPG systolic upslope, the fiducial points in this phase can be reliably identified, including peak points S, U, a and b. Using these well-defined points as references, the locations of points V, W and L can be determined by searching for local extrema in either the forward or backward direction.

Due to inter-individual physiological variations, certain PPG waveforms may lack a distinct diastolic peak. We adopt an estimation approach using derivative marking to determine point f which denotes the estimated D point.

- Feature description

This study involves 235 features, which are divided into six categories based on the analytical method. Their specific names, calculation methods, and descriptions are provided in Table. 1 to 6.

Table 1 Temporal feature

| Feature name | Description |
| --- | --- |
| TS | The time span between the start point O and the systolic peak S |
| TN | The time span between the start point O and the diastolic notch N |
| TD | The time span between the start point O and the diastolic peak D |
| TU | The time span between the start point O and the main peak U |
| TV | The time span between the start point O and the first valley V |
| TW | The time span between the start point O and the second peak W |
| TL | The time span between the start point O and the second valley L |
| Ta | The time span between the start point O and the wave a |
| Tb | The time span between the start point O and the wave b |
| Tc | The time span between the start point O and the wave c |
| Td | The time span between the start point O and the wave d |
| Te | The time span between the start point O and the wave e |
| Tf | The time span between the start point O and the wave f |
| TSD | TD - TS |
| TSN | TN - TS |
| TND | TD – TN |
| TUV | TV – TU |
| TUW | TW – TU |
| TUL | TL – TU |
| Tab | Tb – Ta |
| Tac | Tc – Ta |
| Tad | Td – Ta |
| Tae | Te – Ta |
| Taf | Tf – Ta |
| Tbe | Te – Tb |
| Tbf | Tf – Tb |
| TOO | The time span between adjacent start points O |
| Normalized TS | TS / TOO |
| Normalized TN | TN / TOO |
| Normalized TD | TD / TOO |
| Normalized TU | TU / TOO |
| Normalized TV | TV / TOO |
| Normalized TW | TW / TOO |
| Normalized TL | TL / TOO |
| Normalized Ta | Ta / TOO |
| Normalized Tb | Tb / TOO |
| Normalized Tc | Tc / TOO |
| Normalized Td | Td / TOO |
| Normalized Te | Te / TOO |
| Normalized Tf | Tf / TOO |
| Normalized TSD | TSD / TOO |
| Normalized TSN | TSN / TOO |
| Normalized TND | TND / TOO |
| Normalized TUV | TUV / TOO |
| Normalized TUW | TUW / TOO |
| Normalized TUL | TUL / TOO |
| Normalized Tab | Tab / TOO |
| Normalized Tac | Tac / TOO |
| Normalized Tad | Tad / TOO |
| Normalized Tae | Tae / TOO |
| Normalized Taf | Taf / TOO |
| Normalized Tbe | Tbe / TOO |
| Normalized Tbf | Tbf / TOO |

Table 2 Amplitude feature

| Feature name | Description |
| --- | --- |
| AS | The amplitude of the systolic peak S |
| AN | The amplitude of the diastolic notch N |
| AD | The amplitude of the diastolic peak D |
| AU | The amplitude of the main peak U |
| AV | The amplitude of the first valley V |
| AW | The amplitude of the second peak W |
| AL | The amplitude of the second valley L |
| Aa | The amplitude of the wave a |
| Ab | The amplitude of the wave b |
| AC | The amplitude of the wave c |
| Ad | The amplitude of the wave d |
| Ae | The amplitude of the wave e |
| Af | The amplitude of the wave f |
| (Reflection Index) AD/S | AD / AS |
| Alternative AD/S | 1 - AD/S |
| AN/S | AN / AS |
| AN/D | AN / AD |
| AV/U | AV / AU |
| AW/V | AW / AV |
| AL/W | AL / AW |
| Ab/a | Ab / Aa |
| Ac/a | Ac / Aa |
| Ad/a | Ad / Aa |
| Ae/a | Ae / Aa |
| Af/a | Af / Aa |
| (PPG Augmentation Index) Ad/b | Ad / Ab |
| Age index | (Ab – Ac – Ad – Ae) / Aa |
| Substitute age index | (Ab – Ae) / Aa |
| Modified age index | (Ab – Ac – Ad) / Aa |
| Slope OS | AS / TS |
| Slope ON | AN / TN |
| Slope OD | AD / TD |
| Slope ND | AD – AN / TD – TN |
| Slope SN | AN – AS / TN – TS |
| Slope UV | AV – AU / TV – TU |
| Slope VW | AV – AW / TV – TW |
| Slope ab | Ab – Aa / Tb – Ta |
| Slope bc | Ac – Ab/ Tc – Tb |
| Slope be | Ae – Ab / Te – Tb |
| Slope bd | Ad – Ab/ Td – Tb |

Table 3 Area feature

| Feature name | Description |
| --- | --- |
| Area1 | Area between the O and S |
| Area2 | Area between S and N |
| Area3 | Area between N and D |
| Area4 | Area between D and the next O |
| K | Average blood volume from the O to the next O divided by AS |
| K1 | Average blood volume from the O to N divided by AS |
| K2 | Average blood volume from the N to the next O divided by AS |
| Area34/12 | (Area3 + Area4) / (Area1 + Area2) |
| Area23/1 | (Area2 + Area3) / Area1 |
| Area234/1 | (Area2 + Area3 + Area4) / Area1 |
| Area2/1 | Area2 / Area1 |
| Area3/1 | Area3 / Area1 |
| Area4/1 | Area4 / Area1 |
| Area3/12 | Area3 / (Area1 + Area2) |
| Area4/12 | Area4 / (Area1 + Area2) |

Table 4 Pulse width feature

| Feature name | Description |
| --- | --- |
| W5 | The maximum time span of 5% systolic peak amplitude |
| W10 | The maximum time span of 10% systolic peak amplitude |
| W15 | The maximum time span of 15% systolic peak amplitude |
| W20 | The maximum time span of 20% systolic peak amplitude |
| W25 | The maximum time span of 25% systolic peak amplitude |
| W30 | The maximum time span of 30% systolic peak amplitude |
| W35 | The maximum time span of 35% systolic peak amplitude |
| W40 | The maximum time span of 40% systolic peak amplitude |
| W45 | The maximum time span of 45% systolic peak amplitude |
| W50 | The maximum time span of 50% systolic peak amplitude |
| W55 | The maximum time span of 55% systolic peak amplitude |
| W60 | The maximum time span of 60% systolic peak amplitude |
| W65 | The maximum time span of 65% systolic peak amplitude |
| W70 | The maximum time span of 70% systolic peak amplitude |
| W75 | The maximum time span of 75% systolic peak amplitude |
| W80 | The maximum time span of 80% systolic peak amplitude |
| W85 | The maximum time span of 85% systolic peak amplitude |
| W90 | The maximum time span of 90% systolic peak amplitude |
| W95 | The maximum time span of 95% systolic peak amplitude |
| W5R | The maximum time span from S to right side 5%*AS point |
| W10R | The maximum time span from S to right side 10%*AS point |
| W15R | The maximum time span from S to right side 15%*AS point |
| W20R | The maximum time span from S to right side 20%*AS point |
| W25R | The maximum time span from S to right side 25%*AS point |
| W30R | The maximum time span from S to right side 30%*AS point |
| W35R | The maximum time span from S to right side 35%*AS point |
| W40R | The maximum time span from S to right side 40%*AS point |
| W45R | The maximum time span from S to right side 45%*AS point |
| W50R | The maximum time span from S to right side 50%*AS point |
| W55R | The maximum time span from S to right side 55%*AS point |
| W60R | The maximum time span from S to right side 60%*AS point |
| W65R | The maximum time span from S to right side 65%*AS point |
| W70R | The maximum time span from S to right side 70%*AS point |
| W75R | The maximum time span from S to right side 75%*AS point |
| W80R | The maximum time span from S to right side 80%*AS point |
| W85R | The maximum time span from S to right side 85%*AS point |
| W90R | The maximum time span from S to right side 90%*AS point |
| W95R | The maximum time span from S to right side 95%*AS point |
| W5L | The maximum time span from S to left side 5%*AS point |
| W10L | The maximum time span from S to left side 10%*AS point |
| W15L | The maximum time span from S to left side 15%*AS point |
| W20L | The maximum time span from S to left side 20%*AS point |
| W25L | The maximum time span from S to left side 25%*AS point |
| W30L | The maximum time span from S to left side 30%*AS point |
| W35L | The maximum time span from S to left side 35%*AS point |
| W40L | The maximum time span from S to left side 40%*AS point |
| W45L | The maximum time span from S to left side 45%*AS point |
| W50L | The maximum time span from S to left side 50%*AS point |
| W55L | The maximum time span from S to left side 55%*AS point |
| W60L | The maximum time span from S to left side 60%*AS point |
| W65L | The maximum time span from S to left side 65%*AS point |
| W70L | The maximum time span from S to left side 70%*AS point |
| W75L | The maximum time span from S to left side 75%*AS point |
| W80L | The maximum time span from S to left side 80%*AS point |
| W85L | The maximum time span from S to left side 85%*AS point |
| W90L | The maximum time span from S to left side 90%*AS point |
| W95L | The maximum time span from S to left side 95%*AS point |
| W5_Ratio | W5 / TOO |
| W10_Ratio | W10 / TOO |
| W15_Ratio | W15 / TOO |
| W20_Ratio | W20 / TOO |
| W25_Ratio | W25 / TOO |
| W30_Ratio | W30 / TOO |
| W35_Ratio | W35 / TOO |
| W40_Ratio | W40 / TOO |
| W45_Ratio | W45 / TOO |
| W50_Ratio | W50 / TOO |
| W55_Ratio | W55 / TOO |
| W60_Ratio | W60 / TOO |
| W65_Ratio | W65 / TOO |
| W70_Ratio | W70 / TOO |
| W75_Ratio | W75 / TOO |
| W80_Ratio | W80 / TOO |
| W85_Ratio | W85 / TOO |
| W90_Ratio | W90 / TOO |
| W95_Ratio | W95 / TOO |
| W5_Ratio_LR | W5L / W5R |
| W10_Ratio_LR | W10L / W10R |
| W15_Ratio_LR | W15L / W15R |
| W20_Ratio_LR | W20L / W20R |
| W25_Ratio_LR | W25L / W25R |
| W30_Ratio_LR | W30L / W30R |
| W35_Ratio_LR | W35L / W35R |
| W40_Ratio_LR | W40L / W40R |
| W45_Ratio_LR | W45L / W45R |
| W50_Ratio_LR | W50L / W50R |
| W55_Ratio_LR | W55L / W55R |
| W60_Ratio_LR | W60L / W60R |
| W65_Ratio_LR | W65L / W65R |
| W70_Ratio_LR | W70L / W70R |
| W75_Ratio_LR | W75L / W75R |
| W80_Ratio_LR | W80L / W80R |
| W85_Ratio_LR | W85L / W85R |
| W90_Ratio_LR | W90L / W90R |
| W95_Ratio_LR | W95L / W95R |
| W5_Ratio_RL | W5R / W5L |
| W10_Ratio_RL | W10R / W10L |
| W15_Ratio_RL | W15R / W15L |
| W20_Ratio_RL | W20R / W20L |
| W25_Ratio_RL | W25R / W25L |
| W30_Ratio_RL | W30R / W30L |
| W35_Ratio_RL | W35R / W35L |
| W40_Ratio_RL | W40R / W40L |
| W45_Ratio_RL | W45R / W45L |
| W50_Ratio_RL | W50R / W50L |
| W55_Ratio_RL | W55R / W55L |
| W60_Ratio_RL | W60R / W60L |
| W65_Ratio_RL | W65R / W65L |
| W70_Ratio_RL | W70R / W70L |
| W75_Ratio_RL | W75R / W75L |
| W80_Ratio_RL | W80R / W80L |
| W85_Ratio_RL | W85R / W85L |
| W90_Ratio_RL | W90R / W90L |
| W95_Ratio_RL | W95R / W95L |

Table 5 Frequency feature

| Feature name | Description |
| --- | --- |
| Fdominant | The frequency component with the highest power |
| Fcenter | The Center of Gravity Frequency |
| Froot | The Root Mean Square Frequency |
| Fsecond | The second most prominent frequency |
| Peak Frequency Ratio | FDominent / FSecond |
| Peak Frequency Difference | The difference in frequency between the dominant and the secondary frequency |

Table 6 Entropy feature

| Feature name | Description |
| --- | --- |
| eEn | Energy Entropy |
| ApEn | Approximate entropy |
| SpEn | Sample entropy |
| FuzzyEn | Fuzzy entropy |
| PeEn | Permutation entropy |

- Robust classification of features

High-robustness features are characterized by their stability against fiducial point localization errors or physiological absences. This category encompasses two types of features: those independent of any fiducial points, calculated from complete waveform segments, such as the microcirculatory feature K, and those derived from highly stable fiducial points. As discussed in Section 3.2, fiducial points S, U, a and b, benefitting from the rapid upslope characteristics of the PPG signal, exhibit highly reproducible localization. Therefore, features directly derived from these points, such as upslope time and systolic slope, are thus considered highly robust. Similarly, features that use above points as reference points also maintain excellent stability. A representative example is the pulse width feature, which utilizes the stable fiducial point S as a reference, with percentage amplitude points identified relative to it. This approach ensures consistent reliability across diverse physiological conditions.

In contrast, low-robustness features typically depend on the precise localization of specific fiducial points that are challenging to identify consistently. First, due to inter-subject physiological variations, fiducial points such as N, D, c, and d may be absent in waveforms from certain individuals, as illustrated in Fig. 2. Second, higher-order differentiation amplifies signal noise, further compromising the stable identification of feature points such as e and f in the second derivative. Consequently, the application of these features remains largely confined to specific populations or controlled laboratory settings with stringent signal quality requirements, thereby limiting their broad deployment in clinical practice.

Fig. 2 Absence of fiducial points: (a) Absence of N and D; (b) Fusion of c and d.

- Simulated Annealing-Guided Acceptance Criterion

To balance exploitation and exploration and avoid local optima, a simulated annealing-inspired acceptance criterion is adopted. The probability of accepting a new solution is defined as:

|  | (1) |
| --- | --- |

where  represents the change in feature cost,  denotes the change in mean squared error, and the temperature function  controls the annealing process.

|  | (2) |
| --- | --- |

where  represents the initial temperature,  represents the annealing decay coefficient, and  indicates the current generation number.

- Five-folds results in Section 4.5

Solution 4 achieved a correlation coefficient of 0.90±0.01, ME of -0.01±0.18 m/s, MAE of 0.59±0.06 m/s, and SDE of 0.74 m/s. Table. 7 shows its detailed fold-wise results.

Table 7 AS prediction results of the solution 4

| Fold | MAE | ME | R | R² |
| --- | --- | --- | --- | --- |
| 1 | 0.54 | 0.13 | 0.90 | 0.80 |
| 2 | 0.51 | -0.33 | 0.92 | 0.80 |
| 3 | 0.66 | -0.05 | 0.89 | 0.78 |
| 4 | 0.56 | 0.02 | 0.88 | 0.76 |
| 5 | 0.66 | 0.16 | 0.88 | 0.76 |

Solution 18 exhibited improved performance with an MSE of 0.45±0.11, correlation coefficient of 0.91±0.01, ME of -0.01±0.08 m/s, MAE of 0.52±0.06 m/s, and SDE of 0.67 m/s. Table. 8 shows its detailed fold-wise results.

Table 8 AS prediction results of the solution 18

| Fold | MAE | ME | R | R² |
| --- | --- | --- | --- | --- |
| 1 | 0.42 | -0.14 | 0.93 | 0.87 |
| 2 | 0.52 | -0.05 | 0.90 | 0.81 |
| 3 | 0.49 | 0.08 | 0.90 | 0.82 |
| 4 | 0.58 | 0.01 | 0.90 | 0.80 |
| 5 | 0.59 | 0.04 | 0.90 | 0.81 |