Supplementary Table S1. Material properties used in finite element models

| Material | Young’s modulus (MPa) | Poisson’s ratio | Reference |
| --- | --- | --- | --- |
| Teeth | 19,600 | 0.30 | Yokoi et al. (2019) |
| Clear aligner | 528 | 0.36 | Yokoi et al. (2019) |
| Composite attachment | 12,500 | 0.36 | Yokoi et al. (2019) |
| Periodontal ligament | 0.69 | 0.45 | Yokoi et al. (2019) |

Supplementary Table S2. Element and Node numbers in finite element models

| Group | Element number | Node number |
| --- | --- | --- |
| C0 | 538144 | 132458 |
| R1 | 538254 | 132451 |
| R2 | 538356 | 132479 |
| R3 | 538076 | 132432 |
| R4 | 538222 | 132456 |
| R5 | 538562 | 132425 |
| R6 | 538059 | 132418 |

Supplementary Table S3. Three-Dimensional Tooth Displacements in Each Experimental Group

A. Anterior Displacements (mm)

| Group | Central Incisor Incisal edge |  |  | Central Incisor Root apex |  |  | Lateral Incisor Incisal edge |  |  | Lateral Incisor Root apex |  |  | Canine Cusp tip |  |  | Canine Root apex |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δx | Δy | Δz | Δx | Δy | Δz | Δx | Δy | Δz | Δx | Δy | Δz | Δx | Δy | Δz | Δx | Δy | Δz |
| C0 | 0.0610 | -0.0788 | -0.0583 | -0.0349 | 0.0936 | 0.0820 | 0.0360 | -0.1536 | -0.0598 | -0.0576 | -0.0823 | -0.0181 | 0.0455 | -0.2537 | -0.0645 | -0.0190 | 0.0707 | 0.0507 |
| R1 | 0.0949 | -0.0604 | -0.0475 | -0.0428 | 0.0780 | 0.0713 | 0.0778 | -0.1239 | -0.0389 | -0.0868 | -0.0693 | -0.0154 | 0.0709 | -0.2885 | -0.0188 | -0.0454 | 0.0769 | 0.1088 |
| R2 | 0.0961 | -0.0379 | -0.0236 | -0.0382 | 0.0555 | 0.0558 | 0.0728 | -0.1033 | -0.0111 | -0.0845 | -0.0755 | -0.0110 | 0.0624 | -0.1918 | 0.0093 | -0.0325 | 0.0465 | 0.0849 |
| R3 | 0.1159 | 0.0066 | -0.0100 | -0.0570 | 0.0525 | 0.0282 | 0.0842 | -0.0850 | 0.0006 | -0.0924 | -0.0746 | -0.0168 | 0.0466 | -0.2032 | 0.0287 | -0.0289 | 0.0527 | 0.1111 |
| R4 | 0.1002 | -0.0101 | -0.0100 | -0.0328 | 0.0400 | 0.0325 | 0.0684 | -0.0917 | -0.0035 | -0.0949 | -0.0592 | -0.0027 | 0.0713 | -0.1669 | 0.0431 | -0.0482 | 0.0309 | 0.1042 |
| R5 | 0.0699 | 0.0270 | 0.0531 | -0.0025 | -0.0253 | 0.0055 | 0.0662 | -0.0586 | 0.0082 | -0.1171 | -0.0446 | -0.0039 | 0.0617 | -0.1422 | 0.0547 | -0.0411 | 0.0347 | 0.1075 |
| R6 | 0.1982 | 0.2453 | 0.1571 | -0.1113 | -0.0529 | -0.0896 | 0.1275 | 0.0808 | 0.1228 | -0.2251 | -0.1798 | -0.1032 | -0.0012 | -0.1488 | 0.0182 | 0.1234 | 0.1801 | 0.1476 |

X-axis: buccal (+) / palatal (-); Y-axis: mesial(anterior) (+) / distal(posterior) (-); Z-axis: intrusion (+) / extrusion (-)
MB: mesiobuccal, Group C0 (distal retraction only), Group R1 (intrusion:retraction = 0.5:1), Group R2 (1:1), Group R3 (1.5:1), Group R4 (2:1), Group R5 (3:1), Group R6 (intrusion only)

B. Posterior Displacements (mm)

| Group | 2nd Premolar Buccal cusp |  |  | 2nd Premolar Root apex |  |  | 1st Molar MB Cusp tip |  |  | 1st Molar MB Root apex |  |  | 2nd Molar MB Cusp tip |  |  | 2nd Molar MB Root apex |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Δx | Δy | Δz | Δx | Δy | Δz | Δx | Δy | Δz | Δx | Δy | Δz | Δx | Δy | Δz | Δx | Δy | Δz |
| C0 | -0.004 | 0.044 | 0.034 | 0.011 | 0.005 | 0.028 | 0.012 | 0.043 | 0.008 | 0.002 | -0.007 | 0.007 | 0.044 | 0.036 | -0.013 | -0.027 | -0.002 | -0.018 |
| R1 | -0.003 | 0.002 | -0.008 | 0.036 | 0.028 | 0.001 | 0.021 | 0.009 | -0.012 | 0.003 | 0.009 | -0.012 | 0.049 | 0.000 | 0.003 | -0.034 | 0.014 | -0.004 |
| R2 | 0.014 | -0.008 | -0.020 | 0.030 | 0.024 | -0.014 | 0.030 | -0.008 | -0.012 | -0.003 | 0.019 | -0.012 | 0.056 | -0.014 | 0.011 | -0.038 | 0.019 | 0.002 |
| R3 | 0.018 | -0.006 | -0.038 | 0.031 | 0.027 | -0.025 | 0.040 | -0.022 | -0.018 | -0.006 | 0.026 | -0.018 | 0.070 | -0.028 | 0.019 | -0.042 | 0.024 | 0.007 |
| R4 | 0.005 | -0.035 | -0.053 | 0.045 | 0.036 | -0.030 | 0.024 | -0.035 | -0.026 | -0.004 | 0.032 | -0.021 | 0.051 | -0.041 | 0.015 | -0.037 | 0.028 | 0.012 |
| R5 | -0.001 | -0.054 | -0.066 | 0.059 | 0.054 | -0.036 | 0.021 | -0.049 | -0.037 | 0.004 | 0.035 | -0.030 | 0.039 | -0.056 | 0.017 | -0.040 | 0.032 | 0.014 |
| R6 | 0.041 | -0.088 | -0.071 | 0.029 | 0.037 | -0.045 | 0.065 | -0.078 | -0.042 | -0.006 | 0.029 | -0.031 | 0.092 | -0.087 | 0.027 | -0.054 | 0.034 | 0.024 |

X-axis: buccal (+) / palatal (-); Y-axis: mesial(anterior) (+) / distal(posterior) (-); Z-axis: intrusion (+) / extrusion (-) MB: mesiobuccal, Group C0 (distal retraction only), Group R1 (intrusion:retraction = 0.5:1), Group R2 (1:1), Group R3 (1.5:1), Group R4 (2:1), Group R5 (3:1), Group R6 (intrusion only)