# SUPPORTING INFORMATION FOR:

Ljunge, J., Simoni, M. U., Tofte Husøy, L. J. & Müller, D. B. 
Crushing it…? A first national mapping of the Norwegian construction aggregates system using Material Flow Analysis. Journal of Industrial Ecology.

This supporting information provides a full record of datasets, parameters, and assumptions used to model the Norwegian construction aggregates system for the year 2021. It describes the quantification process for, and discusses the uncertainty of, each modelled flow and stock change. In particular, we expand on our approach to quantify flows of Construction Excavation Materials (CEMs).

Table of contents

S1 Datasets	3

S2 Parameters	6

S3 Assumptions in model	9

S4 System quantification and uncertainty assessment	12

S5 Quantification of Construction Excavation Materials (CEMs)	19

S5.1 Excavation for road infrastructure	19

S5.2 Excavation for railway infrastructure	20

S5.3 Excavation for utility network infrastructure	20

S5.4 Excavation for other infrastructure	21

S5.5 Excavation for building foundations	21

References	26

# S1 Datasets

Datasets used to quantify the system are described and referenced in Table S1 and available in the paper repository (https://doi.org/10.5281/zenodo.15642469). An uncertainty level has been assigned for each dataset based on available metadata and/or discussion with relevant stakeholders.

Table S1 Record of datasets used to quantify the system, including location in paper repository and assigned uncertainty level.

|  | Dataset and reference | Location in repository | Uncertainty | Comment |
| --- | --- | --- | --- | --- |
| D1.1 | Mineral production statistics – mined and non-saleable sand and gravel (Norwegian Directorate of Mining with the Commissioner of Mines at Svalbard, 2022) | data/02_raw_compiled/quantities_compiled.xlsx | Medium-low | Based on mandatory yearly reports from aggregate producers. Sold and received quantities are typically measured at facility exit and entry, indicating better data quality. Mined and non-saleable quantities may be calculated or assumed, indicating poorer data quality |
| D1.2 | Mineral production statistics – sold and received quantities (Norwegian Directorate of Mining with the Commissioner of Mines at Svalbard, 2022) |  | Low |  |
| D1.3 | Received concrete, asphalt, and other materials for recirculation (Norwegian Directorate of Mining with the Commissioner of Mines at Svalbard, 2022) |  | Low |  |
| D2 | External trade statistics (Statistics Norway, 2024c) | data/01_raw/Statistics Norway/11009_20250107.xlsx | Medium-low | Varied data quality, which particularly affects small trade flows and reported units of quantity, compared to units of monetary value (Statistics Norway, 2024e). Trade codes for different materials were carefully selected, however, some codes contain a mix of products (e.g., “Building blocks and bricks of cement, concrete or artificial stone, whether or not reinforced”) where the shares of individual types are unknown |
| D3 | Tunneling statistics (Norwegian Tunnelling Society, 2021) | data/02_raw_compiled/quantities_compiled.xlsx | Medium-low | Based on surveys to companies involved in tunnel blasting |
| D4 | National road database (Norwegian Public Roads Administration, 2025) | data/01_raw/The Norwegian Public Roads Administration/ | Low | Monitored by The Norwegian Public Roads Administration |
| D5 | Municipal water supply statistics (Statistics Norway, 2025a) | data/01_raw/Statistics Norway/13143_20250520.xlsx | Low | Water pipeline length is not mentioned as a key source of error in the dataset (Statistics Norway, 2024d) |
| D6 | Municipal wastewater statistics (Statistics Norway, 2025b) | data/01_raw/Statistics Norway/13144_20250520.xlsx | Low | Sewage pipeline length is not mentioned as a key source of error in the dataset (Statistics Norway, 2024f) |
| D7 | Transmission system operator statistics (Norwegian Energy Regulatory Authority, n.d.) | data/02_raw_compiled/parameters_compiled.xlsx | Medium-low | Monitored by the Norwegian Energy Regulatory Authority, however, no metadata available |
| D8 | Norwegian building cadaster (Norwegian Mapping Authority, n.d.) | data/01_raw/Cadaster/bygninger_bebygdAreal_BygningsstatusKodeId=1_2021.xlsx | Medium-high | Dataset coverage is highly varied between municipalities. Approximately 8% of properties in the dataset had a registered footprint area equal to zero |
| D9 | Concrete production market report (Betong Norge, 2024) | data/02_raw_compiled/quantities_compiled.xlsx | Medium-low | Source is an industry association which does not disclose market coverage and thus representativeness of the data |
| D10 | Asphalt production statistics (Entreprenørforeningen Bygg og Anlegg, n.d.) | data/02_compiled/quantities_compiled.xlsx | Medium-low | Source is an industry association which does not disclose market coverage and thus representativeness of the data |
| D11 | CR&DW generation statistics (Statistics Norway, 2024a) | data/01_raw/Statistics Norway/09247_20250314.xlsx | Medium-high | Based on mandatory forms submitted to the municipality to obtain a certificate of completion for a building. However, submitting the form is only mandatory for: - Construction and extension projects exceeding or affecting 300 m2 BRA (useable area) - Rehabilitation or renovation projects affecting more than 100 m2 BRA - Demolition projects exceeding 100 m2 - Construction, rehabilitation, and demolition projects generating more than 10 tonnes of waste The statistics thus do not include waste quantities from smaller projects. In addition, it is not possible to verify whether the waste was generated in the reported year or before |
| D12 | CR&DW treatment statistics (Statistics Norway, 2024b) | data/01_raw/Statistics Norway/ 09781_20250130.xlsx |  |  |
| D13 | Asphalt recycling statistics (Kontrollordningen For Asfaltgjenvinning, 2022) | data/02_compiled/quantities_compiled.xlsx | Medium-high | Based on voluntary surveys to recycling facilities. The response rate for 2021 was 81.1 % |
| D14 | Railway ballast maintenance records (Bane NOR SF, 2024a, 2024b) | Redistribution of raw data not permitted | Medium-high | Data points are manually recorded by track or project managers. KO-BLL entries include the locations and extents of ballast placed in tracks or turnouts, specifying the type: gravel, crushed rock, or (rarely) slab track. The specific gradations used are also recorded. Ballast cleaning records are noted by year if applicable. KO-MVS describes track maintenance and rehabilitation activities, indicating the work's nature, timing, location, and the machinery used. Due to manual entry, uncertainties such as unknown ballast fractions, extent, missing data, or wrongfully logged work can be found. Bane NOR SF acknowledges potential inaccuracies, though no intentional errors are presumed |

# S2 Parameters

Parameters used to quantify the system are described and referenced in Table S2 and available in the paper repository (https://doi.org/10.5281/zenodo.15642469). An uncertainty level has been assigned for each parameter based on available metadata, own domain knowledge, and/or discussion with relevant stakeholders.

Table S2 Record of parameters used to quantify the system, including location in paper repository and assigned uncertainty level.

|  | Parameter and reference | Location in repository | Uncertainty | Comment |
| --- | --- | --- | --- | --- |
| P1 | Densities of in-situ rock, gravel, sand (Norges Vassdrags- og Energidirektorat, 2023) | data/02_raw_compiled/parameters_compiled.xlsx | Medium -low | Compilation in source is based on data from the Norwegian Public Roads Administration and a handbook on filling of excavation materials. Actual density depends on the local geology. Average values were assumed here |
| P2 | Dimensions of pipeline trenches (Norsk Vann, 2016) | data/02_raw_compiled/parameters_compiled.xlsx | Low | Several cases described for various pipeline installations |
| P3 | Dimensions of cable trenches (Elvia, 2025) | data/02_raw_compiled/parameters_compiled.xlsx | Medium-low | Deviation from standard depths requires special permission |
| P4 | Dimensions of basements (Byggteknisk Forskrift, 2017) | data/02_raw_compiled/parameters_compiled.xlsx | Low | Minimum requirements according to national building standards |
| P5 | Dimensions of concrete slabs (SINTEF, 2003) | data/02_raw_compiled/parameters_compiled.xlsx | Low | Based on minimum requirements to comply with national building standards |
| P6 | Concentration of aggregates in concrete (Kosmatka & Wilson, 2011) | data/02_raw_compiled/parameters_compiled.xlsx | Medium-low | Concrete composition varies depending on structural requirements but aggregates normally constitute 70-85% of the concrete mass – the mean was used here |
| P7 | Concentration of aggregates in bricks (Punmia et al., 2004) | data/02_raw_compiled/parameters_compiled.xlsx | Medium-low | Brick composition varies depending on material availability and manufacturing process but a suitable soil for brick production consists of 50-60% silica sand – the mean was used here |
| P8 | Share of non-saleable aggregates in production (Norwegian Directorate of Mining with the Commissioner of Mines at Svalbard, 2022) | data/02_raw_compiled/parameters_compiled.xlsx | Medium-high | In contrast to natural aggregate (sand and gravel) production, there are no data on non-saleable crushed rock – only a non-saleable share from all solid rock mining. According to the mineral production statistics, construction aggregate production has 3.4% non-saleable quantities on average. The calculated rate for natural aggregate production (where there are reported records of quantities) is 6.2% non-saleable quantities, suggesting a much more efficient production line for crushed rock |
| P9 | Transfer coefficients for sales from aggregates production to main sectors/applications (Norwegian Directorate of Mining with the Commissioner of Mines at Svalbard, 2022) | data/02_raw_compiled/parameters_compiled.xlsx | Medium-low | Taken from the mineral production statistics, which are based on mandatory yearly reports from aggregate producers. Reported sales shares to different sectors are typically not measured and may be based on the producer’s own approximation. One possible uncertainty pertains to quantities delivered to a contractor involved in multiple construction activities (e.g., pavements and unbound road layers) – these quantities may be registered only under the contractor’s main activity (e.g., paving) |
| P10 | Density of ready-mixed concrete (Interbeton Building Materials, 2021) | data/02_raw_compiled/parameters_compiled.xlsx | Low | Taken from a generic Environmental Product Declaration for ready-mixed concrete |
| P11 | Concentration of sand in mortar (Steinhardt AS, 2022) | data/02_raw_compiled/parameters_compiled.xlsx | Low | Taken from generic safety data sheet for mortar produced in Norway |
| P12 | Concentration of aggregates in asphalt (Norwegian Public Roads Administration, n.d.) | data/02_raw_compiled/parameters_compiled.xlsx | Low | Slight variations occur due to project specifications and requirements but 94-95% is stated by the Norwegian Road Administration |
| P13 | Transfer coefficients for CEMs from roads, railways, and other infrastructures to primary applications, secondary applications, and landfill (Norwegian Environment Agency, 2021) | data/02_raw_compiled/infra_CEMs_TCs_calculated.xlsx | High | Calculated based on results from a project coordinated by the Norwegian Environment Agency (2021). The project investigated management of excavation materials from a selection of infrastructure projects in various stages over 5 years and mapped or estimated volumes going to 1) recycling as aggregate, 2) depositing or filling within the project, or 3) landfill. The source report states that the published data are highly uncertain. Calculating coefficients also necessitated conversion since different units were used (theoretical solid volume or placed volume). The representativeness of the investigated projects for general infrastructure construction represents an additional uncertainty |
| P14 | Cross-section geometries of ballast beds (Bane NOR SF, 2024d) and dimensions of sleepers (Bane NOR SF, 2024c) | data/02_raw_compiled/parameters_compiled.xlsx | Low | Taken from technical regulations and specifications by Bane NOR, the Norwegian railway authority |
| P15 | Density of railway ballast (Nålsund, 2014) | data/02_raw_compiled/parameters_compiled.xlsx | Low | Norwegian ballast samples range between approximately 1.55-1.65 kg/dm3 – the mean was used here |

# S3 Assumptions in model

Assumptions made in the system quantification are described in Table S3. An uncertainty level has been assigned for each assumed parameter based on own domain knowledge and/or discussion with relevant stakeholders.

Table S3 Record of assumptions used to quantify the system where suitable parameters could not be found in the literature, including assigned uncertainty levels.

|  | Assumption | Uncertainty | Comment |
| --- | --- | --- | --- |
| A1.1 | Tunnel CEMs have an average density corresponding to that of Norwegian granite rock | Medium-low | Densities of excavated materials depend on the local geology, which was not considered in our model. However, the assumed parameters were carefully selected based on knowledge of geological conditions in Norway (Norges Vassdrags- og Energidirektorat, 2023) |
| A1.2 | 50% of road CEMs have an average density corresponding to that of Norwegian granite rock; the remaining 50% have an average density corresponding to that of sand and gravel (medium graded and firmly layered, moist) | Medium-high |  |
| A1.3 | Non-tunnel utility network CEMs have an average density corresponding to that of sand and gravel (medium graded and firmly layered, moist) | Medium-low |  |
| A1.4 | 50 % of building foundation CEMs have an average density corresponding to that of Norwegian granite rock; the remaining 50% have an average density corresponding to that of sand and gravel (medium graded and firmly layered, moist) | Medium-high |  |
| A2 | Longitudinal road network expansion corresponds to the following dimension classes: - 33% national highway H3, 33% national highway H2, and 33% national highway H1 for E-roads - 100% other highway Hø1/Hø2 for National roads - 100% other highway Hø1/Hø2 for County roads - 100% main residential street for Municipal roads | Medium-low | New road segments are constructed based on current road standards. The dimension classes described in N100 Veg- og gateutforming (2023) are defined based on Annual Average Daily Traffic (AADT) and speed limit, which were implicitly considered in our model |
| A3 | Average excavation rates for road dimension classes are: - 50 m3 CEMs/m H3 - 20 m3 CEMs/m H2 - 12 m3 CEMs/m H1 - 7 m3 CEMs/m Hø1/Hø2 - 2.5 m3 CEMs/m main residential street | Medium-high | A project coordinated by the Norwegian Environment Agency (2021) reported between 18-502 m3/m for E-roads and national roads, however, 6 out of 8 of their investigated projects included road tunnel construction. Based on road widths described in Norwegian road construction standards (N100 Veg- og gateutforming, 2023), we calculated that our assumed rates would correspond to average excavation depths of: - 2.4m for H3 - 1.7 m for H2 - 1.3 m for H1 - 0.9m for Hø1/Hø2 - 0.5m for residential streets In reality, the excavation depths (rates) vary significantly based on the in-situ terrain. However, it can be assumed with high confidence that construction of E-roads generates considerable amounts of CEMs compared to smaller, residential streets |
| A4 | 50% of added water supply pipeline is installed in joint trench with sewage pipeline; the remaining additions are installed separately | Medium-low | Joint pipeline installation is common practice; the assumed share needs to be refined in future work |
| A5 | Average width of cable trench is 1 m | Medium-low | Standard depth is 0.6m (Elvia, 2025) |
| A6 | Foundation category depends on building type (see Table S6); 2/3 (1/3) of the total footprint area of building types with a high (low) likelihood of basement consists of sub-surface basement | High | While some building types can be assumed to be more likely to have a basement (e.g., multi-family buildings) than others (e.g., factories), the share of allocated footprint area to each foundation category needs to be refined in future work. Our model also did not consider slab thickness variations due to building size/number of floors |
| A7 | 20% of utility network CEMs are utilized as backfill (i.e., an unbound structure) and the remaining 80% are sent to landfill | Medium-low | Trenches are typically backfilled with primary aggregates (Rise, 2022) |
| A8 | 20% of building foundation CEMs from properties with basements are deposited/filled on site and the remaining 80% are sent to landfill; 100% of building foundation CEMs from properties without basements are deposited/filled on site | High | Transport and disposal of excavation materials is expensive in Norway and filling on site is often preferred. However, due to the typically limited available space within a project, it can be assumed that buildings with basements (which are responsible for the majority of CEMs) generate a surplus of CEMs that needs to be removed. More research is needed on a variety of building projects to increase knowledge of the systematic management of CEMs |
| A9 | CR&DW from concrete and brick structures consists of 99% concrete and 1% bricks | Low | D11 and D12 include waste fractions “Bricks and concrete and other heavy building materials” and “Polluted bricks and concrete” but do not further distinguish concentrations of concrete and brick. Assuming a concrete concentration of 99% results in an outflow from the concrete stock that amounts to 4% of the inflows. This aligns with previous studies modelling concrete flows both in Norway (Bergsdal et al., 2007) and Ireland (Woodward & Duffy, 2011). Differences in CR&DW composition can be assumed to influence waste treatment, which was not considered in out model. However, since the non-concrete fraction of CR&DW is likely very small, the impact of this assumption is probably minor |
| A10 | Railway ballast cleaning replaces 50% of the ballast volume down to 200 mm below the sleepers. Track rehabilitation (“sporombygningstoget”) replaces 100% of the ballast volume down to 50 mm below the sleepers | Medium-low | Ballast replacement rates due to maintenance depend on: - Track placement in landscape (cut vs. fill – the former typically results in higher replacement due to wider track design and poorer ballast quality) - Track geometry variations, e.g., due to the age of the structure (historical variations in construction standards) and proximity to other railway infrastructures such as stations - Variations in ballast quality (e.g., amount of fines) Such variations were not considered in our model. However, our estimate for ballast cleaning would correspond to a replacement rate of approximately 1.7 m3 ballast/m track, which falls well within the maximum capacity of a typical ballast bed cleaning machine at 450 m3/h and 195 m/h (STRABAG Rail, n.d.) |

# S4 System quantification and uncertainty assessment

Table S4 details the quantification process by listing, for each flow or stock change: quantification approach, dependence on datasets or other flows/stock changes, and used parameters and assumptions. Calculations were made in Jupyter Notebooks using Python 3.8; all code is available in the paper repository (https://doi.org/10.5281/zenodo.15642469). The uncertainty levels assigned to each dataset (Table S1), parameter (Table S2), and assumption (Table S3) are combined in Table S4 into a final uncertainty level per flow/stock change, which can never be better than that of the original dataset. Uncertainties of flows and stock changes quantified through mass balance is determined by assessing the quantitative contributions of each balancing variable. The resulting uncertainty levels for all quantified flows in the 2021 Norwegian construction aggregates system is visualized in Figure S1. The largest uncertainties are observed for flows related to excavation in construction and management of end-of-life materials.

Figure S1 Visualization of uncertainties in the Norwegian construction aggregates system quantified for 2021.

Table S4 System quantification: approach, datasets, parameters, and assumptions for each flow (F) or stock change (dS) as well as final assigned uncertainty levels. Estimated uncertainty ranges: Low: <5%; Medium-low: 5-10%; Medium-high: 10-20%; High: >20%.

| Index Name | Quantification approach | Datasets/other flows | Parameters/assumptions | Assigned uncertainty |
| --- | --- | --- | --- | --- |
| F0-1.1 Mined sand and gravel | Observed data | D1.1 |  | Medium-low |
| F0-1.2 Mined rock | Calculation (mass balance) | F1-2.1 |  | Low |
|  |  | F1-3.2 |  |  |
|  |  | F6-1 |  |  |
|  |  | F12-1 |  |  |
|  |  | F13-1 |  |  |
| F0-2 Imported sand and gravel | Observed data | D2 |  | Medium-low |
| F0-6.1 CEMs from longitudinal road network expansion | Estimation | D3 | P1 | Medium-high |
|  |  |  | A1.1 |  |
|  |  | D4 | A1.2 |  |
|  |  |  | A2 |  |
|  |  |  | A3 |  |
| F0-6.2 CEMs from utility network expansion | Estimation | D3 | P1 | Medium-low |
|  |  | D5 | P2 |  |
|  |  |  | P3 |  |
|  |  | D6 | A1.1 |  |
|  |  |  | A1.3 |  |
|  |  | D7 | A4 |  |
|  |  |  | A5 |  |
| F0-6.3 CEMs from building foundations | Estimation | D8 | P1 | Medium-high |
|  |  |  | P4 |  |
|  |  |  | P5 |  |
|  |  |  | A1.4 |  |
|  |  |  | A6 |  |
| F0-6.4 CEMs from railway tunnels | Observed data | D3 | P1 | Medium-low |
|  |  |  | A1.1 |  |
| F0-6.5 CEMs from other infrastructure | Observed data | D3 | P1 | Medium-low |
|  |  |  | A1.1 |  |
| F0-7.1 Imported concrete products | Observed data | D2 | P6 | Medium-low |
| F0-7.2 Imported brick products | Observed data | D2 | P7 | Medium-low |
| F1-2.1 Sold crushed rock | Observed data | D1.2 |  | Low |
| F1-2.2 Sold sand and gravel | Observed data | D1.2 |  | Low |
| F1-3.1 Non-saleable sand and gravel | Observed data | D1.1 |  | Medium-low |
| F1-3.2 Non-saleable crushed rock | Calculation (transfer coefficient) | F1-2.1 | P8 | Medium-low |
| F2-0.1 Exported crushed rock | Observed data | D1.2 |  | Low |
| F2-0.2 Exported sand and gravel | Observed data | D1.2 |  | Low |
| F2-4.1 Crushed rock to production of concrete and mortar | Calculation (transfer coefficient) | F1-2.1 | P9 | Medium-low |
| F2-4.2 Sand and gravel to production of concrete and mortar | Calculation (transfer coefficient) | F0-2 | P9 | Medium-low |
|  |  | F1-2.2 |  |  |
| F2-5.1 Crushed rock to production of road pavements | Calculation (transfer coefficient) | F1-2.1 | P9 | Medium-low |
| F2-5.2 Sand and gravel to production of road pavements | Calculation (transfer coefficient) | F0-2 | P9 | Medium-low |
|  |  | F1-2.2 |  |  |
| F2-9.1 Crushed rock to construction and maintenance of unbound road structures | Calculation (transfer coefficient) | F1-2.1 | P9 | Medium-low |
| F2-9.2 Sand and gravel to construction and maintenance of unbound road structures | Calculation (transfer coefficient) | F0-2 | P9 | Medium-low |
|  |  | F1-2.2 |  |  |
| F2-10.1 Crushed rock to construction and maintenance of other unbound structures | Calculation (transfer coefficient) | F1-2.1 | P9 | Medium-low |
| F2-10.2 Sand and gravel to construction and maintenance of other unbound structures | Calculation (transfer coefficient) | F0-2 | P9 | Medium-low |
|  |  | F1-2.2 |  |  |
| dS3-3 Stored processing waste | Calculation (mass balance) | F1-3.1 |  | Medium-low |
|  |  | F1-3.2 |  |  |
| F4-0 Exported concrete products | Observed data | D2 | P6 | Medium-low |
| F4-7.1 Ready-mixed concrete | Observed data | D9 | P10 | Medium-low |
|  |  |  | P6 |  |
| F4-7.2 Mortar | Observed data | D9 | P11 | Medium-low |
| F4-7.3 Prefabricated concrete elements | Calculation (mass balance) | F2-4.1 |  | Medium-low |
|  |  | F2-4.2 |  |  |
|  |  | F4-0 |  |  |
|  |  | F4-7.1 |  |  |
|  |  | F4-7.2 |  |  |
| F5-8.1 Asphalt | Observed data | D10 | P12 | Medium-low |
| F5-8.2 Other paving materials | Calculation (mass balance) | F2-5.1 |  | Medium-low |
|  |  | F2-5.2 |  |  |
|  |  | F5-8.1 |  |  |
|  |  | F13-5 |  |  |
| F6-1 CEMs received by aggregate producers | Observed data | D1.2 |  | Low |
| F6-9 Road CEMs utilized in construction of unbound road structures | Calculation (transfer coefficient) | F0-6.1 | P13 | High |
| F6-10.1 Railway CEMs utilized in construction of other unbound structures | Calculation (transfer coefficient) | F0-6.4 | P13 | Medium-high |
| F6-10.2 Utility network CEMs utilized in construction of other unbound structures | Estimation | F0-6.2 | A7 | Medium-low |
| F6-11.1 Road CEMs filled or deposited on-site | Calculation (transfer coefficient) | F0-6.1 | P13 | High |
| F6-11.2 Railway CEMs filled or deposited on-site | Calculation (transfer coefficient) | F0-6.4 | P13 | Medium-high |
| F6-11.3 Other infrastructure CEMs filled or deposited on-site | Calculation (transfer coefficient) | F0-6.5 | P13 | Medium-high |
| F6-11.4 Building CEMs filled or deposited on-site | Estimation | F0-6.3 | A8 | High |
| F6-14.1 Road CEMs to landfill | Calculation (transfer coefficient) | F0-6.1 | P13 | High |
| F6-14.2 Railway CEMs to landfill | Calculation (transfer coefficient) | F0-6.4 | P13 | Medium-high |
| F6-14.3 Utility network CEMs to landfill | Estimation | F0-6.2 | A7 | Medium-low |
| F6-14.4 Other infrastructure CEMs to landfill | Calculation (transfer coefficient) | F0-6.5 | P13 | Medium-high |
| F6-14.5 Building CEMs to landfill | Estimation | F0-6.3 | A8 | High |
| dS7-7 Concrete and brick structures | Calculation (mass balance) | F0-7.1 |  | Medium-low |
|  |  | F0-7.2 |  |  |
|  |  | F4-7.1 |  |  |
|  |  | F4-7.2 |  |  |
|  |  | F4-7.3 |  |  |
| F7-12.1 Concrete CR&DW | Observed data | D11 | P6 | Medium-high |
|  |  |  | A9 |  |
| F7-12.2 Brick CR&DW | Observed data | D11 | P7 | Medium-high |
|  |  |  | A9 |  |
| dS8-8 Road pavement structures | Calculation (mass balance) | F5-8.1 |  | Medium-low |
|  |  | F5-8.2 |  |  |
|  |  | F8-13.1 |  |  |
|  |  | F8-13.2 |  |  |
| F8-13.1 End-of-life asphalt to recycling facilities | Observed data | D13 | P12 | Medium-high |
| F8-13.2 End-of-life asphalt to direct (not via facilities) recycling | Calculation (mass balance) | F8-13.1 |  | Medium-high |
|  |  | F13-5 |  |  |
|  |  | F13-9 |  |  |
|  |  | dS13-13 |  |  |
|  |  | F13-1 |  |  |
| dS9-9 Unbound road structures | Calculation (mass balance) | F2-9.1 |  | Medium-low |
|  |  | F2-9.2 |  |  |
|  |  | F6-9 |  |  |
|  |  | F13-9 |  |  |
| dS10-10 Other unbound structures | Calculation (mass balance) | F2-10.1 |  | Medium-low |
|  |  | F2-10.2 |  |  |
|  |  | F6-10.1 |  |  |
|  |  | F6-10.2 |  |  |
|  |  | F12-10 |  |  |
|  |  | F10-14 |  |  |
| F10-14 Railway ballast maintenance waste | Estimation | D14 | P14 | Medium-high |
|  |  |  | P15 |  |
|  |  |  | A10 |  |
| dS11-11 CEMs deposited or filled on-site | Calculation (mass balance) | F6-11.1 |  | High |
|  |  | F6-11.2 |  |  |
|  |  | F6-11.3 |  |  |
|  |  | F6-11.4 |  |  |
| F12-1 Concrete CR&DW received by aggregates producers | Observed data | D1.2 | P6 | Low |
| F12-10 CR&DW recycled as unbound aggregate | Observed data | D12 | P6 | Medium-high |
|  |  |  | P7 |  |
|  |  |  | A9 |  |
| F12-14 CR&DW sent to landfill and other waste treatment | Observed data | D12 | P6 | Medium-high |
|  |  |  | P7 |  |
|  |  |  | A9 |  |
| F13-1 End-of-life asphalt received by aggregate producers | Observed data | D1.2 | P12 | Low |
| F13-5 End-of-life asphalt recycled in asphalt production | Observed data | D13 | P12 | Medium-high |
| F13-9 End-of-life asphalt recycled in construction of unbound road stock | Observed data | D13 | P12 | Medium-high |
| dS13-13 End-of-life asphalt in intermediate storage at recycling facilities | Observed data | D13 | P12 | Medium-high |
| dS14-14 Landfilled aggregates | Calculation (mass balance) | F6-14.1 |  | Medium-high |
|  |  | F6-14.2 |  |  |
|  |  | F6-14.3 |  |  |
|  |  | F6-14.4 |  |  |
|  |  | F6-14.5 |  |  |
|  |  | F10-14 |  |  |
|  |  | F12-14 |  |  |

# S5 Quantification of Construction Excavation Materials (CEMs)

## S5.1 Excavation for road infrastructure

We quantified road CEMs based on data of highway tunnel excavation from the Norwegian Tunnelling Society (2021) in addition to road network expansion calculated from the Norwegian Public Roads Administration’s national road database NVDB (Norwegian Public Roads Administration, 2025). From NVDB, we extracted road network lengths for E-roads, national roads, county roads, and municipal roads at the beginning and end of 2021 to derive the longitudinal road network expansion per road type. Lateral expansion, for example, due to lane addition or -widening, was not included in our estimate since the public version of NVDB does not distinguish between construction of new roads and maintenance or upgrades of existing roads.

As shown in Table S5, we assigned dimension classes based on Norwegian road construction standards (N100 Veg- og gateutforming, 2023) to the added road lengths and assumed average excavation rates for each class ranging from 2.5 m3 CEMs/m road for residential streets to 50 m3 CEMs/m for national highways of dimension class H3. These excavation rates were validated against the results of a project coordinated by the Norwegian Environment Agency (2021), which investigated the management of excavation materials from 23 infrastructure projects (road, railway, energy, port) by documenting or estimating volumes recycled as aggregates, deposited or filled within the project, and sent to landfill. We calculated that their studied E-roads and national roads generated between 18-502 m3/m road; however, six out of eight of these roads had also involved tunnel construction.

Table S5 Assigned dimension classes and assumed excavation rates per road category in longitudinal road network expansion. AADT=Annual Average Daily Traffic.

| Road category | Dimension class (N100) | Share assigned to class | Characteristics of dimension class | Assumed excavation rate (m3CEMs/m) | Corresponding average excavation depth (m) |
| --- | --- | --- | --- | --- | --- |
| E-road | H3 | 33% | AADT > 12 000 90-110 km/h 20.5m min. width | 50 | 2.4 |
|  | H2 | 33% | AADT 6000- 15 000 90 km/h 12m min. width | 20 | 1.7 |
|  | H1 | 33% | AADT < 6000 80 km/h 9m min. width | 12 | 1.3 |
| National road | H1 | 100% | AADT < 6000 80 km/h 9m min. width | 12 | 1.3 |
| County road | Hø1 | 100% | AADT < 4000 80 km/h 7.5m min. width | 7 | 0.9 |
| Municipal road | Main residential street | 100% | 30-40 km/h 5.5m min. width | 2.5 | 0.5 |

The Norwegian Environment Agency’s results were also used to calculate transfer coefficients of road CEM flows towards primary applications (utilization in road construction, 28%), secondary applications (depositing or filling on-site, 68%), and landfill (4%). For simplicity, we assumed that the excavated volume consisted of 50% rock with an in-situ density of 2.65 t/m3, corresponding to that of Norwegian granite (Norges Vassdrags- og Energidirektorat, 2023), and 50% unbound CEMs with an in-situ density of 1.83 t/m3, corresponding to an average density for medium graded, firmly layered, and moist sand and gravel (Norges Vassdrags- og Energidirektorat, 2023).

## S5.2 Excavation for railway infrastructure

Railway CEMs were quantified using data of excavation for railway tunnels (Norwegian Tunnelling Society, 2021). Other excavation related to railway infrastructure could not be quantified due to data limitations. Transfer coefficients for flows towards primary applications (utilization in construction of other unbound structures, 36%), secondary applications (depositing or filling on-site, 60%), and landfill (4%) were calculated from the results reported by the Norwegian Environment Agency (2021).

## S5.3 Excavation for utility network infrastructure

Our quantification approach for CEMs from utility networks consisted of two parts. First, we applied data of excavated volumes from water supply and sewage tunnelling (Norwegian Tunnelling Society, 2021). Second, we estimated excavated volumes generated by trenching for water pipelines, sewage pipelines, and distribution cables for electricity. Data for added water and sewage pipeline lengths were collected from Statistics Norway (2025b, 2025a). We assumed that half of the total water pipeline length was installed jointly with the corresponding length of sewage pipeline, which is common practice, and that the remaining lengths were installed in separate trenches. For cables, we calculated the length expansion by comparing statistics from the Norwegian Energy Regulatory Authority (n.d.) of the sub-ground local distribution network in 2020 and 2021. Trench cross-section dimensions were then collected from technical guides and handbooks. Only sand and gravel CEMs (with same assumed in-situ density as in 5.1) were assumed to be generated from utility network trenches since rock blasting would most often not be cost-efficient for these installations. Utilization of CEMs as trench backfill is often hindered by quality requirements (Rise, 2021), hence, we assumed that 20% of generated quantities would be of sufficient quality (primary application) and that the remaining 80% would be sent to landfill due to limited filling and depositing space surrounding the trench.

## S5.4 Excavation for other infrastructure

To quantify CEMs from other types of infrastructures, we applied data of excavation related to tunnels for metro, hydropower, and other purposes (Norwegian Tunnelling Society, 2021). Other forms of infrastructure excavation could not be quantified due to data limitations. Flows towards primary (0%) and secondary (71%) applications as well as landfill (28%) were then determined using transfer coefficients calculated from non-road, non-railway projects mapped in the Norwegian Environment Agency's (2021) project.

## S5.5 Excavation for building foundations

Building foundation CEMs were estimated based on property information recorded in the Norwegian cadaster (Norwegian Mapping Authority, n.d.). Excavation can be assumed to occur in the first phases of a project; hence, we extracted the type and footprint area of all buildings registered with the status “permission to start construction” in 2021 (Figure S2). Buildings with footprint areas below 50 m2 were assumed to use lighter foundation types, such as piles, that generate negligible amounts of CEMs and were therefore excluded. We assigned an assumed foundation category per building type to the remaining properties, distinguishing between insulated and uninsulated concrete slabs as well as high and low likelihood of basement (Table S6). Residential-type buildings were assigned insulated slabs, while non-residential type buildings were assigned uninsulated slabs. For building types where we assumed a high (low) likelihood of basement, we estimated that 2/3 (1/3) of the total footprint area would be sub-surface at a depth corresponding to a minimum ceiling height (2.2 m) plus slab foundation thickness (0.35-0.45 m). The corresponding excavation depth for building types without basement was set to only the slab thickness. In reality, foundation requirements differ considerably depending on the in-situ geology and project specifications (e.g., building size/number of floors). Such variations were not considered in our model and would likely add to our calculated volume.

The composition of building foundation CEMs was assumed to be 50% crushed rock and 50% sand and gravel, using the same density conversions as stated in 5.1. Furthermore, we assumed that all CEMs from buildings sites without basement excavation were deposited or used for on-site landscaping. On the other hand, building sites with basement excavation were assumed to generate an 80% surplus of CEMs which would be transported to landfill.

Figure S2 Total footprint areas issued per building type in 2021 based on data from the Norwegian cadaster. English translations of building types are provided in Table S6.

Table S6 Foundation categories assigned to building types in the Norwegian cadaster.

| Insulated concrete slab, high likelihood of basement |  |
| --- | --- |
| Norwegian | English |
| 2 Enebolig m/hybel/sokkelleil. | Single-family house with basement apartment |
| 3 Våningshus | Multistorey building |
| 12 Stort frittliggende boligbygg på 2 etg. | Large detached residential building of 2 stories |
| 13 Stort frittliggende boligbygg på 3 og 4 etg. | Large detached residential building of 3 or 4 stories |
| 14 Stort frittliggende boligbygg på 5 etg. el. mer" | Large detached residential building of 5 stories or more |
| 16 Store sammenb. boligbygg på 3 og 4 etg. | Large combined residential building of 3 or 4 stories |
| 17 Store sammenb. boligbygg på 5 etg. el. mer | Large combined residential building of 5 stories or more |
| Insulated concrete slab, low likelihood of basement |  |
| Norwegian | English |
| 1 Enebolig | Single-family house |
| 4 Tomannsbolig, vertikaldelt | Semi-detached house, vertically divided |
| 5 Tomannsbolig, horisontaldelt | Semi-detached house, horizontally divided |
| 8 Rekkehus | Rowhouse |
| 9 Kjede/atriumhus | Chain/atrium house |
| 11 Andre småhus m/3 boliger el fl | Other small house with 3 or more units |
| 18 Bo- og servicesenter | Housing and service center |
| 20 Annen bygning for bofellesskap | Other building for residential community |
| 82 Barneskole | Primary school |
| 83 Ungdomsskole | Secondary school |
| 84 Komb. barne- og ungdomsskole | Combined primary and secondary school |
| 111 Sykehjem | Nursing home |
| 112 Bo- og behandlingssenter | Residential treatment center |
| Insulated concrete slab, no basement |  |
| Norwegian | English |
| 26 Garasjeuthus anneks til bolig | Garage or annex |
| 81 Barnehage | Kindergarten |
| 86 Annen skolebygning | Other school building |
| Uninsulated concrete slab, low likelihood of basement |  |
| Norwegian | English |
| 49 Kontor- og adm.bygning rådhus | Office and administration building town hall |
| 52 Annen kontorbygning | Other office building |
| 53 Kjøpesenter varehus | Mall |
| 64 Parkeringshus | Parking garage |
| 65 Annen garasje-/hangarbygning | Other garage/hangar building |
| 94 Idrettshall | Sports hall |
| 110 Sykehus | Hospital |
| Uninsulated concrete slab, no basement |  |
| Norwegian | English |
| 31 Fabrikkbygning | Factory building |
| 32 Verkstedbygning | Workshop building |
| 33 Bygning for renseanlegg | Building for sewage treatment plant |
| 34 Bygn.for vannfors.bla. pumpest | Building for water supply, e.g. pump station |
| 35 Annen industribygning | Other industrial building |
| 39 Lagerhall | Warehouse |
| 40 Kjøle- og fryselager | Cold and freezer storage |
| 42 Annen lagerbygning | Other warehouse building |
| 48 Annen landbruksbygning | Other agricultural building |
| 54 Butikk/forretningsbygning | Commercial or business building |
| 56 Annen forretningsbygning | Other commercial building |
| 62 Annen eksp. og terminalbygning | Other exp. and terminal building |
| 99 Annen idrettsbygning | Other sports building |
| 120 Brannstasjon ambulansestasjon | Fire station ambulance station |

# References

Bane NOR SF. (2024a). BaneData—KO-BLL - Komponentoverbygning ballast (Version 27.06.2024) [Dataset].

Bane NOR SF. (2024b). BaneData—KO-MVS - Komponentoverbygning maskinellt vedlikehold av spor (Version 27.06.2024) [Dataset].

Bane NOR SF. (2024c, November 26). Tekniske Spesifikasjoner—Overbygning/Betongsviller. https://trv.banenor.no/t/index.php?title=Overbygning/Betongsviller&oldid=5222

Bane NOR SF. (2024d, December 20). Teknisk Regelverk—Overbygning/Prosjektering/Ballast. https://trv.banenor.no/w/index.php?title=Overbygning/Prosjektering/Ballast&oldid=130270

Bergsdal, H., Brattebø, H., Bohne, R. A., & Müller, D. B. (2007). Dynamic material flow analysis for Norway’s dwelling stock. Building Research & Information, 35(5), 557–570. https://doi.org/10.1080/09613210701287588

Betong Norge. (2024). Markedsrapport Betong Norge. https://www.betong.no/siteassets/markedsrapport-juni-2024-betong-norge.pdf

Elvia. (2025, June 6). Teknisk beskrivelse av kabelgrøft. Teknisk beskrivelse av kabelgrøft. https://www.elvia.no/proff/planlegge-prosjekter/graveprosjekter/teknisk-beskrivelse-av-kabelgroft/

Entreprenørforeningen Bygg og Anlegg. (n.d.). Produsert asfalt i Norge 2021. https://www.eba.no/siteassets/dokumenter/asfalt/produsert-asfalt-pr-31-12-2021-presentasjon.pdf

Forskrift Om Tekniske Krav Til Byggverk, FOR-2017-06-19-840 (2017). https://lovdata.no/dokument/SF/forskrift/2017-06-19-840/KAPITTEL_12#KAPITTEL_12

Interbeton Building Materials. (2021). Environmental Product Declaration for Ready Mixed Concrete C12/15. https://api.environdec.com/api/v1/EPDLibrary/Files/fb8e2b00-a09d-4b38-1566-08daab5a6b31/Data

Kontrollordningen For Asfaltgjenvinning. (2022). Årsrapport 2021. https://asfaltgjenvinning.no/resources/files/arsrapporter/Arsrapport-KFA-2021.pdf

Kosmatka, S. H., & Wilson, M. L. (2011). Design and control of concrete mixtures: The guide to applications, methods, and materials (15. Aufl). Portland Cement Association.

N100 Veg- og gateutforming (No. N100:2023). (2023). Norwegian Public Roads Administration. https://viewers.vegnorm.vegvesen.no/product/859984/nb

Nålsund, R. (2014). Railway Ballast Characteristics, Selection Criteria and Performance [Doctoral thesis, Norges teknisk-naturvitenskapelige universitet]. http://hdl.handle.net/11250/233318

Norges Vassdrags- og Energidirektorat. (2023). Sikringshåndboka—Modul G2.001: Omregning av volum av masser. https://veiledere.nve.no/sikringshandboka/

Norsk Vann. (2016). Grøfteutførelse stive rør. https://www.va-blad.no/wp-content/uploads/2016/09/Blad-6_20.09.16.pdf

Norwegian Directorate of Mining with the Commissioner of Mines at Svalbard. (2022). Mineralstatistikk 2021—Harde fakta om mineralnæringen 2021. https://dirmin.no/sites/default/files/harde_fakta_-_oppslag.pdf

Norwegian Energy Regulatory Authority. (n.d.). Nøkkeltall for nettselskapene. https://www.nve.no/reguleringsmyndigheten/publikasjoner-og-data/data-og-noekkeltall/noekkeltall-for-nettselskapene/

Norwegian Environment Agency. (2021). Cross-sectoral project on management of uncontaminated soil and stone (No. M–2074). https://www.miljodirektoratet.no/publikasjoner/2021/september-2021/tverrsektorielt-prosjekt-om--disponering-av-jord-og-stein-som--ikke-er-forurenset/

Norwegian Mapping Authority. (n.d.). Norwegian building cadaster (Version 20.11.2024) [Dataset].

Norwegian Public Roads Administration. (n.d.). Vegbyggingsmaterialer. https://www.vegvesen.no/fag/teknologi/vegteknologi/vegbyggingsmaterialer/

Norwegian Public Roads Administration. (2025). National Road Database (NVDB) (Version 25.02.2025) [Dataset]. https://nvdb.atlas.vegvesen.no/

Norwegian Tunnelling Society. (2021). The Tunnel Statistics 2021. https://tunnel.no/wp-content/uploads/sites/3/2021/12/The-Tunnel-Statistics-2021.pdf

Punmia, B. C., Kumar Jain, A., & Kumar Jain, A. (2004). Basic Civil Engineering. Laxmi Publications. https://books.google.no/books?id=sWZxu_muxyIC&pg=PA33&redir_esc=y#v=onepage&q&f=false

Rise, T. (2021). Omfyllingsmasser (No. 261/2021; Norsk Vann Rapport). Norsk Vann. https://va-kompetanse.no/butikk/a-261-omfyllingsmasser/

Rise, T. (2022, May 18). Vi sløser med masser til omfylling og nedgraving av vann- og avløpsrør. Rørbloggen. https://blogg.hallingplast.no/misbruk-masser-til-omfylling-nedgraving-ror

SINTEF. (2003). Byggforskserien 522.111—Betonggolv på grunnen. https://www.byggforsk.no/dokument/332/522111_betonggolv_paa_grunnen

Statistics Norway. (2024a). 09247: Generated waste amounts from construction, rehabilitation and demolition of buildings (tonnes), by material 2004—2023 (Version 12.12.2024) [Dataset]. https://www.ssb.no/en/statbank/table/09247/

Statistics Norway. (2024b). 09781: Treatment of waste from construction, rehabilitation and demolition of buildings, by material and treatment (tonnes) 2013—2023 (Version 12.12.2024) [Dataset]. https://www.ssb.no/en/statbank/table/09781/

Statistics Norway. (2024c). 11009: External trade in goods, by commodity number (HS) and trade region/continent 1988—2023 (Version 15.05.2024) [Dataset]. https://www.ssb.no/en/statbank/table/11009/

Statistics Norway. (2024d, April 23). Statistics on Municipal water supply—About the statistics. https://www.ssb.no/en/natur-og-miljo/vann-og-avlop/statistikk/kommunal-vannforsyning#om-statistikken

Statistics Norway. (2024e, June 28). Statistics on External trade in goods—About the statistics. https://www.ssb.no/en/utenriksokonomi/utenrikshandel/statistikk/utenrikshandel-med-varer#om-statistikken

Statistics Norway. (2024f, November 5). Statistics on Municipal wastewater—About the statistics. https://www.ssb.no/en/natur-og-miljo/vann-og-avlop/statistikk/utslipp-og-rensing-av-kommunalt-avlop#om-statistikken

Statistics Norway. (2025a). 13143: Water pipelines and inhabitants connected. Municipal water supply, by region, contents and year (Version 17.03.2025) [Dataset]. https://www.ssb.no/en/statbank/table/13143

Statistics Norway. (2025b). 13144: Wastewater pipelines and inhabitants connected. Municipal wastewater, by region, contents and year (Version 17.03.2025) [Dataset]. https://www.ssb.no/en/statbank/table/13144

Steinhardt AS. (2022, November 15). Sikkerhetsdatablad Robust Murmørtel M5. https://robustnorge.no/wp-content/uploads/2023/04/SDS_Robust_Murmortel_M5.pdf

STRABAG Rail. (n.d.). RM 80 ⁄ 99 UHR – Ballast Bed Cleaning Machine. https://www.strabag-rail.com/databases/internet/_public/content30.nsf/web30?Openagent&id=EC9C9C19C7CFB062C12584010037BF05

Woodward, R., & Duffy, N. (2011). Cement and concrete flow analysis in a rapidly expanding economy: Ireland as a case study. Resources, Conservation and Recycling, 55(4), 448–455. https://doi.org/10.1016/j.resconrec.2010.12.006