# SUPPORTING INFORMATION FOR:

Tokimatsu, K., Yasuoka, R. & Kanae, S. Proposal for an interlinked global model of LCIA and IAM for the water-energy-mineral-land nexus: Global case study until 2100 for meeting the 2-degree Celsius scenario. Journal of Industrial Ecology.

This supporting information provides the original "five-node nexus" concept (Section 1), details of each model used in this study (Sections 2-6), and a list of abbreviations (Section 7).

A list of Supporting Information

Section 1: The original “five-node nexus” concept

Section 2: The water resource model interlinked with other resource models

Supporting Information SI2-1: Figure SI1-2 The water resource model interlinked with other resource models

Supporting Information SI2-2: Figure SI1-3 Detailed representation of the agriculture demand of the water model interlinked with the land-use model

Supporting Information SI2-3: Figure SI1-4 Detailed representation of the industrial demand of the water model interlinked with the models of energy, mineral, and macro-economy

Section 3: The main framework of the overall our IAM and supplemental explanations on the water model

Section 4: The three resource balance models

Section 5: LIME – Lifecycle Impact assessment based on Endpoint modeling

Section 6: A list of countries assigned to the world 10 regions

Supporting Information SI6-1: Figure SI1-7 The world 10 regions

Supporting Information SI6-2: Table SI1-2 The list of countries in the world 10 regions

Section 7: A list of abbreviations

Section 1: The original “five-node nexus” concept

Figure SI1-1 The original “five-node nexus” concept

Adopted from Bleischwitz et al. 2018 Routledge Handbook of the Resource Nexus, Routledge, New York and Andrews-Speed P et al. 2012 The global resource nexus: the struggles for land, energy, food, water, and minerals. Transatlantic Academy, Washington, DC

Section 2: The water resource model interlinked with other resource models

Figure SI1-2 The water resource model interlinked with other resource models

Figure SI1-3 Detailed representation of the agriculture demand of the water model interlinked with the land-use model

Figure SI1-4 Detailed representation of the industrial demand of the water model interlinked with the models of energy, mineral, and macro-economy

Section 3 The main framework of the overall our IAM and supplemental explanations on the water model

- The objective function under the investigated two scenarios

Economic programs are designed to maximize the social welfare function (W). Our IAM is a global model that includes 10 regional economic blocks (j) over time (t). W is regionally weighted sum by the Negishi weight (Negishi, 1960), discounted sum of the individual utility (ucj,t) multiplied the regional population number Nj,t. Discounting is made by the discount factor with the pure rate of time preference . ucj,t is expressed consumption by per capita (cj,t= Cj,t/Nj,t) with a parameter  (risk aversion factor).

ucj,t=Cj,t1-η1-ηη≠1 andlogcj,t(η=1). (1)

The generalized consumption Cj,t is balanced against net output Yj,t and investment Ij,t to maximize W. Yj,t is obtained from the gross output from the production function (Fj,t) minus total cost (TCj,t) of supplying the three resources. The environmental cost (ENVj,t) is further subtracted as (2’) from the equation (2).

Yj,t= Fj,t-TCj,t.  2…BAUYj,t= Fj,t-TCj,t-ENVj,t.  2'…2DC

ENVj,t has two scientific elements referred from LIME, expressed in Equations (3)-(5’):

ENVj,t=eMWTPe,j,t∙iEPe,i,j,t.  (3)

in which

MWTPe,j,t=MWTPe,j,t0∙yj,tyj,t0σe. (4)

EPe,i,j,t=DFe,i,j,tyj,t,TRj,t,SRj,t.  i∈climate (5)EPe,i,j,t=DRe,i,j,tInvi,j,t.  i∈nonclimate (5')

where

e (endpoints) denotes human health, natural resources, biodiversity, and photosynthetic NPP;

i (impact categories) denotes climate change (interchangeable with global warming, a frequently used technical term in LCIA models), local air pollution, ozone depletion, soil acidification, land use and its change, mineral resource extraction, and waste disposal.

The first element (MWTPe,j,t) of Equation (3) is the aforementioned economic damage assessment based on the orthodox choice experiment technique and recently estimated value is extrapolated by value transfer technique to the remote future using per capita income (i.e., GDP, yj,t= Yj,t/Nj,t) and income elasticities for respective endpoint (σe). The second element is the non-monetary endpoint damages (EPe,i,j,t). Regarding climate damage, the aforementioned damage functions (DFe,i,j,t) to build the damage coefficients of climate damage in LIME are used. The temperature rise (TRj,t) and sea-level rise (SRj,t) were obtained from the simplified climate model. As for the non-climate damages, the dose–response functions (DRe,i,j,t) are also called the cause–impact relationships that relate “causes” (Invi,j,t) and “impact” (EPe,i,j,t).

DRe,i,j,t is based on intensive peer-review works in the related literature and modeling works in sectoral, science, and engineering-based studies. Invi,j,t denotes the inventories released as consequences of natural resource transformation. The inventories include sulfur oxide (SOx) emissions and nitrogen oxide (NOx) from fossil fuel combustion, 14 ozone-depleting substances (ODS), extracting mineral resources, disposing waste, land use and its changes. All inventories except ODS are endogenous outputs from the resource balance models of land, minerals, and energy. The Special Report on Emissions Scenarios is used for the exogenous ODS scenarios (Nakicenovic and Swart 2000).

- Balances and constraints for water demand and supply

Balance equations that express Figure SI1-2 consist of three parts: (1) feedwater, including portable water, (2) wastewater, and (3) energy. (1) Raw water resources and the demands, the left most and the right most within the figure’s water resource model, are balanced via the feedwater system including portable water. The raw water resources are rainwater, agricultural drained water, surface water, groundwater, saline and brackish water, and recycled wastewater. The demands include rainfed and irrigated agriculture, livestock, and municipal and industrial waters. (2) Wastewater (denoted as “W”) discharged from municipal, industrial, treated, and portable water treatment processes is balanced with recycled wastewater. Wastewater from irrigated agriculture water (denoted as “Wa”) is balanced with agricultural drained water. (3) The amount of feedwater and its power requirements are balanced with the power consumption of the feedwater. For example, the amount of feedwater from groundwater, surface water, saline and brackish water, and recycled wastewater are multiplied by each power requirement to calculate the power consumption for pumping, H2 production in addition to desalination of saline and brackish water, wastewater treatment, and H2 production in addition to freshwater purification. Similarly, the amount of potable water from both freshwater (groundwater, surface water) and desalinized water multiplied by their power requirements calculates the power consumption for feeding these water.

The constraint equations are relevant to agriculture, industry, and drainage. Rainfall in the respective agriculture sectors (e.g., rainfed, irrigated, and livestock) equals or exceeds the feedwater in each sector. Similarly, water withdrawal in the industry sector equals or exceeds that of feedwater. In the same way, wastewater equals or exceeds that of feed wastewater in each sector. That is, agricultural drained water and irrigated agriculture, and recycled wastewater and wastewater from portable, municipal, and industrial, respectively.

- Demand projections

The demand sectors for water are agriculture including livestock, municipal, and industry. In the agriculture sector, water consumption is calculated by production from the simplified land-use model multiplied by the respective water requirements. The water consumption for grains, soybeans, and other crops is split into rainfed and irrigated water by their respective rates. Water withdrawal is calculated by multiplying the ratios of feedwater (input) to water consumption (output), considering some losses or dissipations in irrigated agriculture. In contrast, the ratios are set in unity in other agricultural water consumption and withdrawal. Municipal water consumption per capita level is estimated by per capita GDP.

The withdrawal of municipal water is obtained from its consumption level divided by the ratio of feedwater and water consumption (0.9). Water consumption in the industry sector consists of three components: (1) power, hydrogen, fossil fuels, and uranium from the energy model, (2) mineral and automobile productions from the mineral resource model, and (3) the rest of the industry. Water consumption and withdrawal of these demands are calculated using either of the water requirements for respective demands. In the case where either the data on requirements of consumption or withdrawal is unavailable, the ratios of feedwater and water consumption are employed to calculate water consumption and withdrawal.

- Water resources

Malnutrition due to irrigated crop production loss was linked with irrigation water in agriculture and with changes in food production and trade globally (Motoshita et al. 2018). Agricultural water demand (consumption) and water withdrawal are calculated by multiplying the crop (grain, soybean, other crops) and livestock production (beef, lamb, chicken, pork) using the land-use model by water intensity. The production of another crop, which is not explicitly handled in the model, is calculated by multiplying the grain production by the constant ratio of grain to other crops. Additionally, crop water consumption is multiplied by a ratio to separate rainfed from irrigated agriculture. Agricultural water withdrawal and consumption were calculated as a ratio of water withdrawal = water consumption divided by water output, using the rainfed water output rate of 1.0 and the irrigation water output rate. For livestock, water withdrawal is set as the equivalent to water consumption due to the lack of literature (Figure SI1-3).

Industrial water demand and water withdrawal in power, hydrogen, and automobile production are calculated by multiplying the water intensities of both water demand and water withdrawal with the amount of power generated and hydrogen produced in the energy model and the number of automobiles produced in the mineral resource model. When only water intensities of water withdrawal are given to fuel and mineral production, water consumption is calculated by multiplying 0.7 by the volume of water withdrawal, as in the TIAM-FR water model. An original regression model estimates water consumption in the remaining industry sector as a function of GDP/cap. The water withdrawal in the sector is obtained by dividing water consumption by 0.7 (Figure SI1-4).

- Data

The data relevant to the water resource balance model are 1) water requirements (or water use intensity), 2) energy requirements (energy use intensity), 3) efficiency, 4) footprint, and 5) ratio, fraction, or percentage for allocation percentage. Supporting Information Table SI1-1 provides a summary of the data. The water supply side (raw water sources and feedwater) and demand sides use these data, except for the municipal and the other industrial water demand. Municipal and other industrial water demand use the water requirement as a function of GDP/cap and the efficiency to calculate water withdrawal from water consumption.

The water requirement indicates the required water input for unit of output. On the supply side, the required water input is given for the unit amount of hydrogen production (seawater desalinization, freshwater purification, and gasification of fossil fuels and biomass). On the demand side, the required water input is given for the unit amount of all the agriculture products and industry (power, fuel, hydrogen, mineral resources, and automobiles), except the other demand.

The energy requirement is the required energy input for unit of output. The following are examples on the supply side: hydrogen production, wastewater treatment, and feedwater whose input are freshwater, saline and brackish water, and recycled wastewater. The output are pumped freshwater, desalinated water, treated wastewater, seawater desalinization plus hydrogen production, and freshwater purification plus hydrogen production. Note that the required energy in the water resource model is only accounted for water treatment; the energy model accounts for energy for hydrogen production from seawater desalination and hydrogen production from freshwater purification. On the feedwater demand side, the energy requirement is used for the industrial sector (hydrogen production).

The efficiency is used for connections with input to and output losses from the processes. On the supply side, raw water is the input, while feedwater, water withdrawals, and wastewater are the output. On the demand side, feedwater (withdrawal) is used as the input and demand (consumption) is used as the output. For example, with coal gasification, wastewater is 0.38 with regard to the unit volume of raw water input. On the demand side, the efficiency is used for the required water input (feedwater) to fulfill the unit amount of demand. For irrigated agriculture, the output is 0.5 to the unit volume of feedwater input (irrigated water, desalinated water, and agricultural drained water).

The footprint is used to allocate water withdrawal and consumption based on the actual values of rainwater (green water, denoted as Gn) and irrigated water (Gn, B as blue, and Gy as gray). The value of the footprint is the annual withdrawal and consumption for production (of crops, grain, soybeans, and other agricultural products).

Finally, the allocation percentage is used to allocate one source to several. For example, on the supply side, the feedwater allocation percentage is used for feedwater (rainwater, agricultural drained water, saline and brackish water, recycled wastewater, and potable water) and demand (rainfed crop products, irrigated crop products, municipal, and industrial). On the demand side, the percentage of water withdrawal of freshwater, saline and brackish water are used for industry (e.g., power generation).

AQUASTAT 2022 is the basis of water impact assessments, such as water withdrawal by sources and sectors, and wastewater. Mekonnen and Hoekstra 2011 was used for footprinting such as grazing, animal water, and total renewable water resources. The damage coefficients of infectious diseases and malnutrition were sourced from LIME3 (Motoshita et al., 2011; 2018).

References for Section 3

AQUASTAT (2022) AQUASTAT - FAO's Global Information System on Water and Agriculture, Database, Country Statistics, Database. Retrieved Feb 1st, 2025, from https://www.fao.org/aquastat/en/

Motoshita, M., Itsubo, N., Inaba, A., (2011), “Development of impact factors on damage to health by infectious diseases caused by domestic water scarcity”, Int J Life Cycle Assess 16: 65–73. https://doi.org/10.1007/s11367-010-0236-8

Motoshita, M., Ono, Y., Pfister, S., Boulay, A.M., Berger, M., Nansai, K., Tahara, K., Itsubo, N., Inaba, A., (2018) “Consistent characterisation factors at midpoint and endpoint relevant to agricultural water scarcity arising from freshwater consumption”, Int J Life Cycle Assess 23(12)2276-2287. https://doi.org/10.1007/s11367-014-0811-5

Nakicenovic, N., and Swart, R.(Eds.), (2000) Special Report on Emissions Scenarios, Cambridge University Press, Cambridge, UK.

Negishi, T., (1960), “Welfare Economics and the Existence of an Equilibrium for a competitive economy”, Metroeconomica, 12, 92-97. https://doi.org/10.1111/j.1467-999X.1960.tb00275.x

Mekonnen, M.M., Hoekstra, A.Y. (May 2011). National water footprint accounts: The green, blue and grey water footprint of production and consumption, Volume 2: Appendices, , Value of Water Research Report Series No. 50. UNESCO-IHE Institute for Water Education. Retrieved March 29, 2026, from https://ayhoekstra.nl/pubs/Report50-WaterFootprintNations-Vol2.pdf

Table SI1-1 Data given in the water model

| data | description | supply side (raw water sources and feedwater) | demand side (agriculture, livestock) | demand side (all industrial sectors excluding the other) | demand side (the other industry, municipal water) | data sources |
| --- | --- | --- | --- | --- | --- | --- |
| water use intensity | water requirement to input per unit output quantity | hydrogen production | grains, soybeans, other crops, and meat | productions of power, fuel, hydrogen, resources, and automobile | demand projections of water consumption (the other industry, municipal water) are expressed by per capita GDP. | [W2-8,13] [H1-H12] |
| energy use intensity | energy requirement to input per unit output quantity | raw water sources to feedwater; treatment of feedwater and wastewater |  |  |  | [W1] [H1-H12] |
| efficiency | connections with losses inputting to and outputting from the processes | input (raw water) → output (feedwater, water withdrawal, waste water) --- freshwater pumping, desalinization, wastewater treatment, and portable water treatment; rainfed agriculture, irrigated agriculture, municipal water, and industrial waste water; water treatment for hydrogen production | input (feedwater) → output (water demand) --- (in raifed agriculture) rainfed water (in irrigated agriculture) fresh water, desalinated water, treated agricultural waste water | input (feedwater) → output (water demand) --- (for industrial water) freshwater, desalinated water, and treated waste water; (for municipal water) raifed water, desalinated water, and treated waste water; (for hydrogen production) withdrawal and consumption | conversion to withdrawal from water consumption | [W1,2,13] [H1-H12] |
| footprint | footprint is used for allocations of water withdrawal and consumption based on the actual values of rainwater and irrigated water | annual withdrawal and consumption for production (of crops, grain, soybeans, and other agricultural products) by raifed and irrigated agriculture (green; Gn, blue; B, gray; Gy) | raifed and irrigated agriculture (Gn, B, Gy) (for crops, grain, soybeans, and other agricultural products); livestock (Gn, B, Gy) | industrial water (B,G) | municipal water (B,G) | [W3,4] |
| ratio, fraction, or percentage for allocation | they are used for allocating one source to several | feedwater allocation (rainwater, saline and brackish water, agricultural wastewater, industrial wastewater, and potable water) → consumption (rainfed crop products, irrigated crop products, municipal, and industrial); desalinated water → municipal water, industrial water |  | (for power generation) percentage of water withdrawal over freshwater, saline and brackish water (for hydrogen production ) percentage consumption over withdrawal |  | [W1,2,3,6,13] [H1-H12] |

[W1] Dubreuil, A., Assoumou, E., Selosse, S., Maïzi (2013). Water modeling in an energy optimization framework – The water-scarce middle east context, Appl. Ener. 101: 268–279. https://doi.org/10.1016/j.apenergy.2012.06.032

[W2] AQUASTAT (2022). AQUASTAT - FAO's Global Information System on Water and Agriculture, Database, Country Statistics, Database. Retrieved March 29, 2026, from https://www.fao.org/aquastat/en/

[W3] Mekonnen, M.M., Hoekstra, A.Y., (2010). The green, blue and grey water footprint of crops and derived crop products, Volume 1: Main Report. Value of Water Research Report Series No. 47. UNESCO-IHE Institute for Water Education. Retrieved March 29, 2026, from https://www.waterfootprint.org/resources/Report47-WaterFootprintCrops-Vol1.pdf

[W4] Mekonnen, M.M., Hoekstra, A.Y., (2011). National water footprint accounts: The green, blue and grey water footprint production and consumption, Volume 2: Appendices. Value of Water Research Report Series No. 50. UNESCO-IHE Institute for Water Education. Retrieved March 29, 2026, from https://ayhoekstra.nl/pubs/Report50-WaterFootprintNations-Vol2.pdf

[W6] Bouckaert, S., Assoumou, E., Selosse, S., Maïzi (2010), “A prospective analysis of waste heat management at power plants and water conservation issues using a global TIMES model”, Energy 68 (2014) 80-91. https://doi.org/10.1016/j.energy.2014.02.008

[W7] Bouckaert, S. (2010), “Implementation des usages de l'eau dans le modele energetique TIAM-FR, Rapport de mission industrielle” Sophia-Antipolis - Septembre 2010

[W8] Norgate, T. E. (2009), “Deteriorating Ore Resources, Energy and Water Impacts”, in Linkages of Sustainability, Thomas E. Graedel and Ester van der Voet (eds) ISBN-13: 9780262013581

[W13] IRENA (2023), Water for hydrogen production. Retrieved March 29, 2026, from

https://www.irena.org/-/media/Files/IRENA/Agency/Publication/2023/Dec/IRENA_Bluerisk_Water_for_hydrogen_production_2023.pdf

[H1] Guo X, Wang Z, and Li Y (2023), Freshwater supply for hydrogen production: An underestimated challenge, International Journal of Hydrogen Energy 48:10245–10257. https://doi.org/10.1016/j.ijhydene.2024.06.257

[H2] Kim, J., Park, K., Yang, D.R., Hong, S. (2019) A comprehensive review of energy consumption of seawater reverse osmosis desalination plants. Appl. Energy 254, 113652. https://doi.org/10.1016/j.apenergy.2019.113652

[H3] Jones, E.; Qadir, M., Van Vliet, M.T.H., Smakhtin, V., Kang, S.-M. (2018) The state of desalination and brine production: A global outlook. Sci. Total Environ. 657, 1343–1356. https://doi.org/10.1016/j.scitotenv.2018.12.076

[H4] International Energy Agency (IEA). 2019. The Future of Hydrogen: Seizing Today’s Opportunities; IEA Publications: Paris, France. Retrieved 10 March, 2026, from https://www.iea.org/reports/the-future-of-hydrogen

[H5] Stillwell AS. and Webber ME. (2016), Predicting the specific energy consumption of reverse osmosis desalination, Water 8:601. https://doi.org/10.3390/w8120601

[H6] Nnabuife, S.G., Darko, C.K., Obiako, P.C., Kuang, B., Sun, X., Jenkins, K. (2023). A comparative analysis of different hydrogen production methods and their environmental impact. Clean Technol. 5, 1344–1380. https://doi.org/10.3390/cleantechnol5040067

[H7] Martínez-Medina, M.À., Pérez-Martín, M.Á., Estrela, T. (2024) Desalination in Spain and the role of solar photovoltaic energy. J. Mar. Sci. Eng. 12, 859. https://doi.org/10.3390/jmse12060859

[H8] Navarro, R., Lizaso, J.L.S., Sola, I. (2023) Assessment of Energy Consumption of Brine Discharge from SWRO Plants. Water 15, 786. https://doi.org/10.3390/w15040786

[H9] Elimelech, M., Phillip, W.A. (2011) The future of seawater desalination: Energy, technology, and the environment. Science, 333, 712–717. https://doi.org/10.1126/science.1200488

[H10] Lattemann, S., Höpner, T. (2008) Environmental impact and impact assessment of seawater desalination. Desalination, 220, 1–15. https://doi.org/10.1016/j.desal.2007.03.009

[H11] Tong, T., Elimelech, M. (2016) The global rise of zero liquid discharge for wastewater management: Drivers, technologies, and future directions. Environ. Sci. Technol. 50, 6846–6855. https://doi.org/10.1021/acs.est.6b01000

[H12] Ghaffour N, Missimer T M and Amy G L (2013), Technical review and evaluation of the economics of water desalination: Current and future challenges for better water supply sustainability, Desalination 309:197–207. https://doi.org/10.1016/j.desal.2012.10.015

Section 4 The three resource balance models

The approach taken in our model combined three aspects. The first aspect represents the following target resources. These are the main causes of inventories (Inv).

- Energy: fossils (oil, gas, and coal), uranium, and various renewables (e.g., biomass, solar, hydro, and wind) to generate energy supply (power, heat, and fuel for transportation)

- Minerals: ferrous (iron ore), non-ferrous (zinc, lead, copper, and bauxite), and non-metal (limestone) for physical capital (i.e., infrastructure, automobiles, and electro-machinery)

- Land: cropland, grassland, forestry, urban land (human settlements), and other lands (e.g., desert), for production of food (e.g., rice, corn, and wheat) and biomass (e.g., paper)

The second aspect employs bottom-up and systems-engineering models of resource demand and supply together with the required transformation or conversion technologies. The three resource balance models were developed and interlinked, corresponding to the three aforementioned categories.

The third aspect encompasses components from resource inputs to the production function. The resources balance models for energy, mineral, biomass, and food are formulated as dynamic linear programming globally. Hence these models can separately be solved as the least cost minimization problem. The least cost consists entire lifecycle of the products. The three models are interlinked (Figure SI1-5) based on a detailed set of technology options. Details of each resource model and interlinking with LIME can be found in past publications (Tokimatsu et al., 2016; 2017a; 2017b; 2020).

< Insert Figure SI1-5 here>

References for Section 4

Tokimatsu, K., L. Tang, R. Yasuoka, R. Ii, N. Itsubo, M. Nishio, 2020, Toward more comprehensive environmental impact assessments: Interlinked global models of LCIA and IAM applicable to this century, The Int J Life Cycle Assess 25, 1710-1736. https://doi.org/10.1007/s11367-020-01750-8

Tokimatsu, K., S. Murakami, T. Adachi, R. Ii, R. Yasuoka, M. Nishio, 2017a Long-term Demand and Supply of Non-ferrous Mineral Resources by a Mineral Balance Model, Miner. Econ. 30(3)193-206. https://doi.org/10.1007/s13563-017-0109-8

Tokimatsu, K., R. Yasuoka, M. Nishio, 2017b Global zero emissions scenarios: the role of biomass energy with carbon capture and storage by forested land use, Appl. Energy 185; 1899-1906. https://doi.org/10.1016/j.apenergy.2015.11.077

Tokimatsu, K., S. Konishi, K. Ishihara, T. Tezuka, R. Yasuoka, M. Nishio, 2016 Role of innovative technologies under the global zero emissions scenarios, Appl. Energy 162; 1483-1493. https://doi.org/10.1016/j.apenergy.2015.02.051

Figure SI1-5 The interlinkages of the three resource models

Section 5 LIME - Lifecycle Impact Assessment Based on Endpoint Modeling

- Outline of LIME

LIME (Itsubo et al., 2015) is widely recognized as a leading modeling. LIME1 (Itsubo & Inaba, 2005) and LIME2 (Itsubo & Inaba, 2010, 2012, 2013, 2014) focused on Japan, whereas LIME3 was developed on a global scale (Itsubo & Inaba, 2018; LCA Society of Japan, 2025). LIME3 enables environmental impacts to be compared in almost all countries (Figure SI1-6). Detailed descriptions in this Supporting Information are found in Tokimatsu et al., 2020.

- Damage factors of LIME3 in our IAM

  - Global warming

Damage assessment of global warming in this study for human health and biodiversity (Tang et al., 2018d; Tang et al. (2023)). Calculated damages induced by CO2 emissions are as follows: malnutrition, diarrhea, cardiovascular disease, malaria, and coastal and inland flooding. The damage to one of the endpoints, human health, is explained by population number, the death number, the mortality rate in the baseline, relative risk due to temperature rise, and the fraction of the population affected by diseases. Both damage per death and baseline mortality depend on per capita GDP. The human health damage factors are derived from Tang et al. (2018a, 2019), with further revisions incorporated in LIME3.

  - Land use and Land-use change

Damages are focused on an increased risk of extinction of endangered vascular plant species and the degradation of potential NPP. The incremental risks of extinction of vascular plants from land-use change were used for damage assessment of biodiversity in LIME3. The risks across 2,000 species in Japan’s land use and their changes (Yamaguchi et al., 2018). Data from the International Union for Conservation of Nature and Natural Resources (IUCN) was adopted to the methodology of Yamaguchi et al. (2018) for all countries. Based on the land use pattern, the differential between current NPP and potential NPP at the country level is used to measure damages on primary production via land use and land-use change.

  - Mineral resource development and waste

Mineral resources damages are assessed for primary production and biodiversity. The transformed land areas are as follows in mining: the mine site area, the deposit site by discharged accumulated overburden, and the disposed tailing site by ore concentration and coal pretreatment. The transformed area in the produced region enables to estimation of the amount of primary production and the number of endangered species. Site-specific data are aggregated from the country and the region. The aggregated data is used for the above modeling. Primary production and biodiversity at landfilling sites of metallic and cement waste are considered for waste damage factors.

  - Water resources

Damage factors of malnutrition are regressed from agricultural water footprint. Both logarithms of the damage factors of LIME3 and the agricultural water footprint per capita, both in the ten world regions, show a linear relationship. The regional damage factors of LIME3 are weighted averages by the agricultural water footprint per capita from the country-based damage factors of LIME3 data. Data sources of the LIME3, the water footprint and the population are the first version of LIME3 (Itsubo and Inaba 2018), Mekonnen and Hoekstra 2011 (Appendix I & VII), respectively.

Damage factors of diarrhea as an example, one of the infectious diseases in this study, are expressed as a power function of per capita domestic use of fresh water across the ten world regions in our model. The power function is regressed from country-based damage factors of diarrhea in the first version of LIME3 (Itsubo and Inaba 2018) as an explained variable, while per capita domestic use of fresh water multiplied by 0.9 (considered 10% loss) is an explanatory variable. Damage factors of other infectious diseases are studied in a similar manner. The data of domestic use of fresh water is sourced from AQUASTAT 2022 (water use, municipal water withdrawal, total population).

  - Local air pollution (on human health) and ozone layer depletion

The following local air pollutants on human health, primary and secondary, were considered: SOx, NOx, black carbon, and organic carbon (Tang et al., 2018c). Respiratory disease due to increased concentrations of SOx and NOx is considered for local air pollution. Local air pollution is assessed by using global chemical transport models. Ozone layer depletion damages are included at the human health endpoint (Itsubo and Inaba, 2018).

- Damage factors of LIME2 in our IAM

Damages based on LIME2 included in our model are as follows: ozone layer depletion (on primary production, natural resources whereas human health by LIME3) and local air pollution (on primary production and natural resources by SOx and NOx) (Itsubo & Inaba, 2012). Mineral resource damages are assessed at the natural resource endpoint using the discounted flow of net rental price instead of the user cost method.

- Weighting factors of LIME3 in our IAM

Conjoint analysis enabled us to value the various attributes, conditioning that the partial effect of the monetary attributes (e.g., tax) through assessing the utilities of the pre-defined attributes. A methodology for applying conjoint analysis was developed by Itsubo et al. (2012) for weighting the endpoints as LCIA attributes.

The following procedures were used in our survey (Itsubo et al., 2015; Murakami et al., 2017; Tokimatsu et al., 2016). (1) Damages caused by environmental problems on four endpoints are explained before the eight questions for conjoint analysis begins. (2) The following explanations are provided: i) implementing political measures to mitigate negative impacts requires annual financial resources, and ii) reliable international organizations implement fundraising initiatives for the financial resources. (3) Respondents reply to the questions for conjoint analysis.

In conjoint analysis, questions are given randomly from a set of sixteen questions developed through experimental design. The choice set consists of four options, including "leave as it is". Damages are represented by dots in a 10-by-10 square (totaling 100). Those colored for left and without color mitigated are "damage XX” and “yy % reduction", respectively. Annual payment per household is provided. "damage 100 (leave as it is)" and "No cost (0 dollars)" are provided for the choice "leave as it is."

References for Section 5

AQUASTAT (2022) AQUASTAT - FAO's Global Information System on Water and Agriculture, Database, Country Statistics, Database. Retrieved Feb 1st, 2025, from https://www.fao.org/aquastat/en/

Itsubo N, Inaba A, editors. 2005. LIME – methodology and database for LCA, environmental accounting, eco-efficiency, Tokyo: Maruzen. (in Japanese)

Itsubo N, Inaba A, editors. 2010. LIME2 – an environmental impact assessment method supporting decision making. Tokyo: Maruzen. (in Japanese). Retrieved 18th July 2025, from https://lca-forum.org/english/news/index.html#lime2

Itsubo and Inaba, 2012, LIME2 – chapter 2: Characterization and damage evaluation methods (2.1 Ozone layer depletion, 2.2 Global warming, 2.3 Acidification), JLCA (Life-cycle Assessment Society of Japan) news letter, JLCA news English edition No.15, October 2012. Retrieved August 3rd 2025, from http://lca-forum.org/english/pdf/No15_Chapter2.1-2.3.pdf

Itsubo and Inaba, 2013, LIME2 – chapter 2: Characterization and damage evaluation methods (2.4 Urban air pollution), JLCA (Life-cycle Assessment Society of Japan) news letter, JLCA news English edition No.16, September 2013. Retrieved August 3rd 2025, from http://lca-forum.org/english/pdf/No16_Chapter2.4-2.6.pdf

Itsubo and Inaba, 2014, LIME2 – chapter 2: Characterization and damage evaluation methods (2.10 land use, 2.11 resource consumption, 2.12 waste), JLCA (Life-cycle Assessment Society of Japan) news letter, JLCA news English edition No.18, November 2014. Retrieved August 3rd 2025, from http://lca-forum.org/english/pdf/No18_Chapter2.10-2.13.pdf

Itsubo N, Inaba A, editors. 2018. LIME3 – an environmental impact assessment method for global scale. Tokyo: Maruzen (in Japanese).

Itsubo N, Murakami K, Kuriyama K., Yoshida K, Tokimatsu K, Inaba A. 2018. Development of weighting factors for G20 countries – Explore the difference in environmental awareness between developed and emerging countries –. Int J Life Cycle Assess 23: 2311-2326. https://doi.org/10.1007/s11367-015-0881-z

LCA Society of Japan 2025, LIME3, Retrieved August 3rd 2025, from https://lca-forum.org/english/lime/

Motoshita, M., Ono, Y., Pfister, S., Boulay, AM, Berger, M., Nansai, K., Tahara, K., Itsubo, N., Inaba, A., (2018). Consistent characterisation factors at midpoint and endpoint relevant to agricultural water scarcity arising from freshwater consumption. Int J Life Cycle Assess 23(12)2276-2287. https://doi.org/10.1007/s11367-014-0811-5

Mekonnen, M.M., Hoekstra, A.Y. (May 2011). National water footprint accounts: The green, blue and grey water footprint of production and consumption, Volume 2: Appendices, Value of Water Research Report Series No. 50. UNESCO-IHE Institute for Water Education. Retrieved March 29, 2026, from https://ayhoekstra.nl/pubs/Report50-WaterFootprintNations-Vol2.pdf

Murakami, K., N. Itsubo, K. Kuriyama, K. Yoshida, K. Tokimatsu, (2017) “Development of weighting factors for G20 countries Part 2: Estimation of willingness to pay and annual global damage cost”, The Int J Life Cycle Assess, 1-16. https://doi.org/10.1007/s11367-017-1372-1

Tang, L., Ii, R., Tokimatsu, K., Itsubo, N. 2018a. Development of human health damage factors related to CO₂ emissions by considering future socioeconomic scenarios. International Journal of Life Cycle Assessment 23(12)2288-2299. https://doi.org/10.1007/s11367-015-0965-9

Tang, L., Nagashima, T., Hasegawa, K., Ohara, T., Sudo, K., Itsubo, N. 2018b. Development of human health damage factors for tropospheric ozone considering transboundary transport on a global scale. International Journal of Life Cycle Assessment 23(12)2339-2348. https://doi.org/10.1007/s11367-015-1001-9

Tang, L., Nagashima, T., Hasegawa, K., Ohara, T., Sudo, K., Itsubo, N. 2018c. Development of human health damage factors for PM2.5 based on a global chemical transport model. International Journal of Life Cycle Assessment 23(12)2300-2310. https://doi.org/10.1007/s11367-014-0837-8

Tang L, Higa M, Tanaka N, Itsubo N, 2018d. Assessment of global warming impact on biodiversity using the extinction risk index in LCIA: A case study of Japanese plant species. International Journal of Life Cycle Assessment 23(2)314-323. https://doi.org/10.1007/s11367-017-1319-6

Tang L., Furushima, Y., Honda, Y., Hasegawa, T., Itsubo, N., 2019. Estimating human health damage factors related to CO2 emissions by considering updated climate-related relative risks. International Journal of Life Cycle Assessment, 24(6)1118-1128. https://doi.org/10.1007/s11367-018-1561-6

Tang L., Ohashi, H., Matsui, T., Hirata, A., Tanaka, N., Itsubo, N., (2023) Characterization factor estimation based on a species-specific extinction risk approach for determining CO2 emission impact on terrestrial biodiversity. Ecol. Indic., 154: 110556. https://doi.org/10.1016/j.ecolind.2023.110556

Tokimatsu, K., M. Aicha, K. Yoshida, M. Nishio, E. Endo, M. Sakagami, K. Murakami, N. Itsubo, 2016. Measuring marginal willingness to pay using conjoint analysis and developing benefit transfer functions in various Asian cities. Int. J. Sustain. Dev. World Ecol. 23(6) 541-552. http://dx.doi.org/10.1080/13504509.2016.1168326

Tokimatsu, K., L. Tang, R. Yasuoka, R. Ii, N. Itsubo, M. Nishio, 2020. Toward more comprehensive environmental impact assessments: Interlinked global models of LCIA and IAM applicable to this century. The Int J Life Cycle Assess 25, 1710-1736. https://doi.org/10.1007/s11367-020-01750-8

Yamaguchi, K., Ii, R., Itsubo, N. 2018. Ecosystem damage assessment of land transformation using species loss. International Journal of Life Cycle Assessment 23(12)2327–2338. https://doi.org/10.1007/s11367-016-1072-2

Figure SI1-6 Outline of LIME

Section 6 A list of countries assigned to the world 10 regions

Figure SI1-7 The world 10 regions

Table SI1-2 The list of countries in the world 10 regions

| North America | West Europe | Japan | Oceania | Centrally planned Asia |
| --- | --- | --- | --- | --- |
| American Samoa | Andorra | Japan | Australia | China |
| Canada | Anguilla |  | Cocos Keeling Islands | Hong Kong, China |
| Guam | Aruba |  | Cook Islands | Macao, China |
| Hawaii | Ascension |  | Fiji | Taiwan |
| Midway Islands | Austria |  | Kiribati |  |
| Northern Mariana Islands | Azores Islands |  | Marshall Islands |  |
| Saipan | Belgium |  | Micronesia, Fed. States of |  |
| United States | Bonaire, Sint Eustatius and Saba |  | Nauru |  |
| Virgin Islands（U.S.) | British Virgin Islands |  | New Zealand |  |
| Wake Island | Canary Islands |  | Niue |  |
|  | Cayman Islands |  | Norfolk Island |  |
|  | Channel Islands |  | Palau |  |
|  | Christmas Island |  | Pacific Islands |  |
|  | Curacao |  | Samoa |  |
|  | Denmark |  | Solomon Islands |  |
|  | Diego Garcia |  | Tokelau Island |  |
|  | Faeroe Islands |  | Tonga |  |
|  | Falkland Islands |  | Tuvalu |  |
|  | Finland |  | Vanuatu |  |
|  | France |  |  |  |
|  | French Guiana |  |  |  |
|  | French Polynesia |  |  |  |
|  | Germany |  |  |  |
|  | Gibraltar |  |  |  |
|  | Greece |  |  |  |
|  | Greenland |  |  |  |
|  | Guadeloupe |  |  |  |
|  | Guernsey |  |  |  |
|  | Iceland |  |  |  |
|  | Ireland |  |  |  |
|  | Isle of Man |  |  |  |
|  | Italy |  |  |  |
|  | Jersey |  |  |  |
|  | Liechtenstein |  |  |  |
|  | Luxembourg |  |  |  |
|  | Madeira |  |  |  |
|  | Malta |  |  |  |
|  | Martinique |  |  |  |
|  | Mayotte |  |  |  |
|  | Monaco |  |  |  |
|  | Montserrat |  |  |  |
|  | Netherlands |  |  |  |
|  | Netherlands Antilles |  |  |  |
|  | New Caledonia |  |  |  |
|  | Norway |  |  |  |
|  | Pitcairn |  |  |  |
|  | Portugal |  |  |  |
|  | Saint Barthélemy |  |  |  |
|  | Saint-Martin (French part) |  |  |  |
|  | San Marino |  |  |  |
|  | Sint Maarten (Dutch part) |  |  |  |
|  | Spain |  |  |  |
|  | Spanish North Africa |  |  |  |
|  | St. Helena |  |  |  |
|  | St. Pierre & Miquelon |  |  |  |
|  | Svalbard and Jan Mayen Islands |  |  |  |
|  | Sweden |  |  |  |
|  | Switzerland |  |  |  |
|  | Turks & Caicos Islands |  |  |  |
|  | United Kingdom |  |  |  |
|  | Vatican |  |  |  |
|  | Wallis and Futuna Is |  |  |  |
| Other Asia | Middle East and North Africa | Sub-Sahara Africa | Latin America | Former Soviet Union and Eastern Europe |
| Afghanistan | Algeria | Angola | Antigua and Barbuda | Albania |
| Bangladesh | Bahrain | Benin | Argentina | Armenia |
| Bhutan | Cyprus | Botswana | Bahamas, The | Azerbaijan |
| Brunei | Egypt, Arab Rep. | Burkina Faso | Barbados | Belarus |
| Cambodia | Iran, Islamic Rep. | Burundi | Belize | Bosnia and Herzegovina |
| East Timor | Iraq | Cameroon | Bermuda | Bulgaria |
| India | Israel | Cape Verde | Bolivia | Croatia |
| Indonesia | Jordan | Central African Republic | Brazil | Czech Republic |
| Korea, Dem. Rep. | Kuwait | Chad | Chile | Estonia |
| Korea, Rep. | Lebanon | Comoros | Colombia | Georgia |
| Lao PDR | Libya | Congo, Dem. Rep. | Costa Rica | Hungary |
| Malaysia | Morocco | Congo, Rep. | Cuba | Kazakhstan |
| Maldives | Occupied Palestinian Territory | Cote d'Ivoire | Dominica | Kosovo |
| Mongolia | Oman | Djibouti | Ecuador | Kyrgyz Republic |
| Myanmar | Qatar | Equatorial Guinea | El Salvador | Latvia |
| Nepal | Saudi Arabia | Eritrea | Grenada | Lithuania |
| Pakistan | Syrian Arab Republic | Ethiopia | Guatemala | Malta |
| Papua New Guinea | Tunisia | Gabon | Guyana | North Macedonia |
| Philippines | Turkey | Gambia, The | Haiti | Poland |
| Singapore | United Arab Emirates | Ghana | Honduras | Republic of Moldova |
| Sri Lanka | West Bank and Gaza | Guinea | Jamaica | Romania |
| Thailand | Yemen, Rep. | Guinea-Bissau | Mexico | Russian Federation |
| Vietnam |  | Kenya | Nicaragua | Serbia and Montenegro |
|  |  | Lesotho | Panama | Slovak Republic |
|  |  | Liberia | Paraguay | Slovenia |
|  |  | Madagascar | Peru | Tajikistan |
|  |  | Malawi | Puerto Rico | Turkmenistan |
|  |  | Mali | St. Kitts and Nevis | Ukraine |
|  |  | Mauritania | St. Lucia | Uzbekistan |
|  |  | Mauritius | St. Vincent and the Grenadines |  |
|  |  | Mozambique | Suriname |  |
|  |  | Namibia | Trinidad and Tobago |  |
|  |  | Niger | Uruguay |  |
|  |  | Nigeria | Venezuela, RB |  |
|  |  | Reunion |  |  |
|  |  | Rwanda |  |  |
|  |  | Sao Tome and Principe |  |  |
|  |  | Senegal |  |  |
|  |  | Seychelles |  |  |
|  |  | Sierra Leone |  |  |
|  |  | Somalia |  |  |
|  |  | South Africa |  |  |
|  |  | Sudan |  |  |
|  |  | Eswatini |  |  |
|  |  | Tanzania |  |  |
|  |  | Togo |  |  |
|  |  | Uganda |  |  |
|  |  | Western Sahara |  |  |
|  |  | Zambia |  |  |
|  |  | Zimbabwe |  |  |

Section 7 A list of abbreviations

AIM; Asia-Pacific Integrated Model

AQUASTAT; FAO’s information system on water and agriculture

BAU; business-as-usual

BECCS; bioenergy with carbon capture and storage

CCS; carbon capture and storage

CGE; computable general equilibrium

CO2; Carbon dioxide

CSP; concentrated solar power

DC; degree Celsius

DICE; Dynamic Integrated Climate-Economy

DRF; dose-response function

EV; electric vehicles

FCV; fuel cell vehicle

FECCS; fossil fuel energy with carbon capture and storage

FUND; Climate Framework for Uncertainty, Negotiation and Distribution

G20; Group of Twenty

GCAM; Global Change Analysis Model

GCM; Global Climate Model

GDP; gross domestic product

GHG; greenhouse gas

H2; hydrogen

I/O; input-output

IAM; integrated assessment models

IDA5; An integrated dynamic assessment methodology for the five-node nexus

IE; industrial ecology

IPBES; Intergovernmental Science-Policy Platform on Biodiversity and Ecosystem Services

ISO; International Standards Organization

LCA; lifecycle assessment

LCIA; lifecycle impact assessment

LIME3; The lifecycle impact assessment method based on endpoint modeling, version 3

MAgPIE; Model for Agricultural Production and its Impact on the Environment

MFA; material flow analysis

MR-EE-IOA; multi-regional environmentally extended input-output analysis

MWTP; marginal willingness to pay

NG; natural gas

NOx; nitrogen oxides

NPP; net primary production of photosynthesis

ODS; ozone-depleting substances

RCP; Representative Concentration Pathways

ReMIND; REgional Model of INvestments and Development

SOx; sulfide oxides

SSP; Shared Socioeconomic Pathways

TFP; total factor productivity

TIAM-FR; The Integrated Assessment Model-France

UNECE; The United Nations Economic Commission for Europe

UNESCO; The United Nations Educational, Scientific and Cultural Organization

UNU; United Nations University

VRE; variable renewable energy

WEF; Water–Energy–Food

WEFWI; The Water–Energy–Food–Climate Nexus

WEHAB; Water, Energy, Health, Agriculture, and Biodiversity

WITCH; World Induced Technical Change Hybrid

WSSD; World Summit on Sustainable Development

China, the former Soviet Union and Eastern Europe, Japan, Latin America, the Middle East and North Africa, North America, Oceania, Southeast Asia (including India), Sub-Saharan Africa, and Western Europe. See Supporting Information 4.

2020, 2030, … , 2150.