Table S2. Operating conditions of the ICP-MS instruments

| Instrument | Agilent 7700 series |  |
| --- | --- | --- |
| ICP | RF power | 1.55 kW |
|  | Plasma gas | Argon |
|  | Plasma gas flow rate | 15.0 l min−1 |
|  | Carrier gas flow rate | 1.09 l min−1 |
|  | S/C Temperature | 2°C |
|  | Reaction gas | H2, He |
|  | Reaction gas flow rate | 6 ml min−1 for H2 |
|  |  | 4.3 ml min−1 for He |
| MS | Scan mass range | 55Mn, 77Se, 78Se, 89Y, 95Mo, 111Cd, 114Cd, 115In, 206Pb, 207Pb, 208Pb, 200Hg, 201Hg, 202Hg, 205Tl |
|  | Ion count acquisition (points) | 3 |
|  | Number of replicates | 3 |

Abbreviations: ICP-MS, inductively coupled plasma-mass spectrometry.