Supplemental materials and methods

Behavioral testing

Dizocilpine-disrupted prepulse inhibition (PPI) of the startle reflex

In rodents, exposure to a strong acoustic stimulus provokes a startle response. If this strong stimulus is preceded by a milder acoustic stimulus, then the response of the animal to the startle-eliciting stimulus is attenuated and this phenomenon is termed prepulse inhibition (PPI) of the startle. It is disrupted by a dizocilpine (MK801) pretreatment1-3. Antipsychotic agents reverse this disruption and this property is considered as a proxy of their efficacy against psychotic symptoms1-3.

Between 16 and 19 days following implantation of the osmotic minipumps, startle reactivity was measured in sound-attenuating startle chambers (TSE Systems, Bad Homburg, Germany), which include enclosures (22.5x8x8.5cm) equipped with loudspeakers and a piezoelectric accelerometer that allow to deliver tone pulses and to measure animal startle responses, respectively. The first two days consist of habituation sessions (10 min and 30 min of 70dB background noise on day 1 and day 2, respectively). On day 3, PPI was measured (immediately after an i.p. injection of saline) as follows: after a 10 min acclimation period (70dB), the rat received, in a random fashion, 24 trials with a pulse-alone stimulus (120 dB, 40 ms), 12 trials with no stimulus (70-dB 200 ms), two types (3 × 12) of prepulse-and-pulse trials which include a 20-ms prepulse (80 and 85-dB) followed 100 ms later by a 120-dB pulse stimulus, as described previously4. On day 4, PPI was measured as on day 3, using dizocilpine (0.15mg/kg) instead of saline as pretreatment. The amplitude of startle responses was recorded in all trials. The magnitude of PPI was calculated as a percent inhibition of the startle amplitude in the pulse-alone trial3, 5.

Catalepsy

Catalepsy is indicative of the potential of a pharmacological agent to induce extrapyramidal symptoms6, 7. At 25 days following minipump implantation, catalepsy was assessed using a steel grid floor that was inclined at 60°. Rats were placed on the grid and the time elapsed without any front paw movements was recorded for a maximum of 3 min and used to estimate catalepsy8.

Ex vivo receptor binding measurements and in vivo imaging

Radiotracer preparation

# Preparation of [123I]IBZM  and [125I]R91150  was performed as described previously by our group9-11. All chemicals for radiotracer preparation were purchased from Sigma-Aldrich (Buchs, Switzerland) unless otherwise specified. 123I and 125I radioiodine were purchased from Perkin Elmer (Basel, Switzerland). [123I]IBZM was obtained by incubation, for 15 min at 68°C, of a mixture containing 5 μl of BZM precursor (ABX, Germany, 24 nmol/ μl  in ethanol), 2 μl of glacial acetic acid, 1 μl of 30% H2O2 and 10 mCi of carrier-free 123I sodium iodide in 0.05 M NaOH. The radiotracer was isolated by a linear gradient HPLC run (from 5% acetonitrile, ACN, to 95% ACN, 10 mM H3PO4, in 10 min). HPLC was equipped with a reverse-phase column (Phenomenex Bonclone C18, Phenomenex, Schlieren, Switzerland) and radiotracer was eluted at a flow of 3 ml/min. Fractions containing [123I]IBZM were diluted in water and loaded on a Sep-Pak cartridge (Sep-Pak C18, Waters, Switzerland). [123I]IBZM was eluted with 0.5 ml of 95% ACN, 10 mM H3PO4 and concentrated using a rotary evaporator, and the final product was diluted in saline prior to animal administration.

R91150 precursor preparation was described elsewhere12. For radiolabelling, 300 μg of R91150 precursor in 3 μL ethanol was mixed with 3 μL of glacial acetic acid, 15 μL of carrier-free 125I sodium iodide (10 mCi) in 0.05 M NaOH, and 3 μL of 30% H2O2. [125I]R91150 was isolated by an isocratic HPLC run  (ACN/water 50/50, 10 mM acetic acid buffer pH 5) with a reversed-phase column (Bondclone C18 10 μm 300 X 7.8 mm, Phenomenex, Schlieren, Switzerland) at a flow rate of 3 mL/min.

Ex vivo estimation of receptor occupancy by haloperidol and MDL-100,907

In the ex vivo dose-occupancy curve estimations, rats were administered with [123I]IBZM and [125I]R91150 to concurrently measure D2/3 and 5-HT2A receptor occupancy, respectively. At 28 days of treatment, rats were anesthetized using isoflurane anaesthesia (4% for induction, 2.5% for maintenance) and injected with 6.48±0.34 MBq of [123I]IBZM or 6.98±0.98 MBq of [123I]R91150 (depending on the administered antagonist, haloperidol or MDL-100,907, respectively). At 120 min post-injection, rats were euthanized by decapitation, their brain removed, and their striatum, frontal cortex and cerebellum dissected and weighed. Radioactivity in the dissected brain regions was immediately measured in an automated gamma counting system (expressed in KBq/g of tissue weight) for the radiotracer labelled with 123I. Radioactivity was decay-corrected to the time of the brain dissection.

For the ex vivo study, the standardized uptake ratio (SUR) for each radiotracer in the striatum and the frontal cortex was measured using the radioactivity measured in the gamma counting system as follows: SUR=(radioactivity in the target-region)/(radioactivity in the cerebellum) – 1. The % occupancy (O) of the D2/3 and the 5-HT2A receptors from their respective antagonists was estimated using the following formula: O (%)=(1-SUR/SURCON)*100, where SUR corresponds to the value obtained from an individual study in which a dose of antagonist was employed, while SURCON corresponds to the average value obtained from the control animals in which no antagonist was administered.

In vivo imaging experiments

Dual-radiotracer SPECT imaging11 was performed in the context of the main in vivo study described in this paper to assess the level of D2/3 and 5-HT2A occupancy by haloperidol and MDL-100,907 and the binding of D2/3 and 5-HT2A receptors after chronic treatment with these agents. In vivo dual radiotracer SPECT was performed as described previously11. At the end of the 28-day treatment period, the first dual-radiotracer SPECT scan was performed, to measure the occupancy of the D2/3 and the 5-HT2A receptor by their respective antagonists. One week later, an exactly similar dual-radiotracer SPECT scan was performed to index the density of the D2/3 and the 5-HT2A receptors. A polyethylene catheter (22G) was inserted in the tail vein for radiotracer injection, at a volume of 0.6 ml. Rats were simultaneously injected with a mixture of [123I]IBZM (32.7±8.2 MBq) and [125I]R91150 (26.9±6 MBq) over 30 sec. At 80 minutes post-radiotracer administration, rats were anesthetized using isoflurane (4% for induction and 2.5% for maintenance) and the scan was initiated in a U-SCAN-II SPECT camera (MiLabs, Utrecht, Netherlands)  (using 4 frames of 10-min each). The choice of this timing for the SPECT scans relies on previous work from our group and allows the quantification of both radiotracers’ binding: indeed, the SUR of [123I]IBZM is estimated over the static images corresponding to 80-110 min after the injection of the radiotracer10, while the SUR of [125I]R91150  at 100-120 min9. Body temperature was maintained at 37±1 °C by means of a thermostatically controlled heating blanket.

SPECT image reconstruction was performed using a pixel ordered subsets expectation maximization (P-OSEM, 0.4 mm voxels, 4 iterations, 6 subsets) algorithm using the MiLabs image reconstruction software. Reconstruction of dynamic SPECT images was performed using the radioactivity measured at each radioisotope’s principal energy spectrum, that is at 143,1-179,9 keV for 123I and at 15-45 keV for 125I. Radioactive decay correction was performed while correction for attenuation or scatter was not.

Separation of the two distinct images from the dual-radiotracer SPECT scan

The co-injection of [123I]IBZM  with [125I]R91150  induces a contamination of the [125I]R91150 image. 123I emits radioactivity principally at the 143,1-179,9 keV energy spectrum but also at a secondary energy spectrum, which is exactly the spectrum of 125I (15-45 keV). To correct for this contamination, we employed a method described previously by our group11. Briefly, the secondary emission of 123I is directly related to the principal one (34%). Contaminated [125I]R91150  images were thus corrected by subtracting 34% of the radioactivity measured in the [123I]IBZM images at the 143,1-179,9 keV energy spectrum.

Standardized uptake ratio (SUR) estimation in the in vivo imaging experiments

For the in vivo study, SPECT images were processed using PMOD software (version 3.9, PMOD Technologies Ltd, Zurich, Switzerland). For each rat, static [125I]R91150 SPECT images were spatially normalized on a [125I]R91150  template-image, as previously described11, and filtered with a Gaussian filter of 0.6m3 FWHM. The resulting transformation matrix was applied to the corresponding [123I]IBZM  images for each rat. A volume-of-interest (VOI) template (including 57 VOIs), incorporated in PMOD13 was used to extract the radioactivity from each brain VOI and the cerebellum (CER), which was used as reference region. SUR values were estimated as follows: (Radioactivity in the target VOI)/(Radioactivity in CER)-1. The SUR values of the first dual-radiotracer SPECT scan, which was performed on the 28th day of the treatment period, were used to estimate the occupancy of the receptor by their respective antagonists. For estimation of D2/3 receptor occupancies using in vivo imaging with [123I]IBZM, a 0.55 value was subtracted from the SUR and SURCON values. This value corresponds to the difference in the non-displaceable binding between the striatum (target region) and the cerebellum (reference region) for this radiotracer, as described in a previous paper from our group10.. The SUR values of the second SPECT scan for each rat (which is performed 7 days after the end of the treatment period) are direct indexes of the density of the respective receptor populations. For the [125I]R91150 images of the second SPECT scans, SUR was also estimated at the voxel level using the same formula.

References

1.	Swerdlow NR, Weber M, Qu Y, Light GA, Braff DL. Realistic expectations of prepulse inhibition in translational models for schizophrenia research. Psychopharmacology 2008; 199(3): 331-388.

2.	Wadenberg MG, Sills TL, Fletcher PJ, Kapur S. Antipsychoticlike effects of amoxapine, without catalepsy, using the prepulse inhibition of the acoustic startle reflex test in rats. Biological psychiatry 2000; 47(7): 670-676.

3.	Varty GB, Bakshi VP, Geyer MA. M100907, a serotonin 5-HT2A receptor antagonist and putative antipsychotic, blocks dizocilpine-induced prepulse inhibition deficits in Sprague-Dawley and Wistar rats. Neuropsychopharmacology : official publication of the American College of Neuropsychopharmacology 1999; 20(4): 311-321.

4.	Tournier BB, Ginovart N. Repeated but not acute treatment with (9)-tetrahydrocannabinol disrupts prepulse inhibition of the acoustic startle: reversal by the dopamine D(2)/(3) receptor antagonist haloperidol. European neuropsychopharmacology : the journal of the European College of Neuropsychopharmacology 2014; 24(8): 1415-1423.

5.	Martinez ZA, Oostwegel J, Geyer MA, Ellison GD, Swerdlow NR. "Early" and "late" effects of sustained haloperidol on apomorphine- and phencyclidine-induced sensorimotor gating deficits. Neuropsychopharmacology : official publication of the American College of Neuropsychopharmacology 2000; 23(5): 517-527.

6.	Kapur S, Wadenberg ML, Remington G. Are animal studies of antipsychotics appropriately dosed? Lessons from the bedside to the bench. Canadian journal of psychiatry Revue canadienne de psychiatrie 2000; 45(3): 241-246.

7.	Ginovart N, Kapur S. Role of dopamine D(2) receptors for antipsychotic activity. Handbook of experimental pharmacology 2012; (212): 27-52.

8.	Gobira PH, Ropke J, Aguiar DC, Crippa JA, Moreira FA. Animal models for predicting the efficacy and side effects of antipsychotic drugs. Rev Bras Psiquiatr 2013; 35 Suppl 2: S132-139.

9.	Dumas N et al. In Vivo Quantification of 5-HT2A Brain Receptors in Mdr1a KO Rats with 123I-R91150 Single-Photon Emission Computed Tomography. Molecular imaging 2015; 14.

10.	Tsartsalis S et al. A single-scan protocol for absolute D2/3 receptor quantification with [123I]IBZM SPECT. NeuroImage 2017; 147: 461-472.

11.	Tsartsalis S et al. Dual-radiotracer translational SPECT neuroimaging. Comparison of three methods for the simultaneous brain imaging of D2/3 and 5-HT2A receptors. Neuroimage 2018; 176: 528-540.

12.	Dumas N et al. Small-animal single-photon emission computed tomographic imaging of the brain serotoninergic systems in wild-type and mdr1a knockout rats. Molecular imaging 2014; 13(1).

13.	Schiffer WK et al. Serial microPET measures of the metabolic reaction to a microdialysis probe implant. J Neurosci Methods 2006; 155(2): 272-284.