Elisa protocol

Specimen requirement

Serum: Collect plasma using EDTA or citrate as an anticoagulant, centrifuge for 20-min at the speed of 2000-3000 r.p.m. Remove supernatant. If precipitation appeared, centrifugal again.

Assay procedure

1. Add standard: Set standard wells, testing sample wells. Add standard 50μl to standard well.

2. Add sample: Set blank wells separately (don’t add sample and HRP-Conjugate reagent; other each step operation is same). Add sample dilution 40μl to testing sample well, then add testing sample 10μl (sample final dilution is 5-fold). Don’t touch the well wall as far as possible, and gently mix.

3. Add enzyme: Add HRP-Conjugate reagent 100μl to each well, except blank well.

4. Incubate: After closing plate with closure plate membrane, incubate for 60 min at 37°C.

5. Configurate liquid: 20×wash solution diluted 20-fold with distilled water and reserve.

6. Washing: Uncover closure plate membrane, discard liquid, dry by swing, add washing buffer to every well, still for 30s then drain, repeat 5 times, dry by pat.

7. Color: Add chromogen solution A 50ul and chromogen solution B to each well, evade the light preservation for 15 min at 37°C

8. Stop the reaction: Add stop solution 50μl to each well. Stop the reaction (the blue color change to yellow color).

9. Assay: Take blank well as zero and read absorbance at 450nm after adding stop solution and within 15min.