# Supplement- Laboratory Screening and Diagnosis of Open Neural Tube Defects, 2019; Diagnostic Interpretation of Amniotic Fluid AFP and Acetylcholinesterase Results

## S.1. Patient and provider information

Patient information concerning prenatal diagnostic testing is commonly provided as part of counseling prior to amniocentesis. Many prenatal diagnostic centers and professional organizations (e.g., ACOG, NSGC) have produced and, in some cases, formally evaluated educational materials on amniocentesis and follow-up of positive screening test results that are in effective formats, at appropriate reading levels, and available in multiple languages. Laboratories should supply informational materials for health care providers that include the following:

- Detailed information about how samples should be collected, packaged and transported to the laboratory;

- Examples of completed test requisitions;

- General information on testing, such as when reflex acetylcholinesterase (AChE) will be needed, turn-around times, and when results will be returned;

- Information about reporting formats and expectations for amniotic fluid alpha fetoprotein (afAFP) and AChE test performance.

Information about diagnostic testing for ONTD is included as part of general informed consent for amniocentesis. Collecting informed consent is not the responsibility of the laboratory but laboratories may want or need to document that the process has occurred. The collection of complete and accurate intake information is necessary for providing the most reliable interpretation. Laboratories should have a mechanism to collect pre-test clinical information through a well-designed requisition form that includes:

- Name and other unique identifying information;

- Sample collection date;

- Information to date the pregnancy, preferably by ultrasound measurement(s) (e.g., the date of the ultrasound and assigned gestational age on that day);

- Indication for testing (e.g., elevated serum AFP, abnormal ultrasound findings, family history of NTD, maternal teratogen exposure);

- Name, address and telephone/fax number of referring health care provider or prenatal diagnostic center;

- Sample type and condition (e.g., spun or unspun amniotic fluid, visibly bloodstained).

## S.2 Specimen collection and transportation

Amniotic fluid aliquots are typically received from the cytogenetics laboratory. Tubes should be labeled with the patient's name, a second identifier and sample collection date. Amniotic fluid color, turbidity and presence of any visible blood should be noted; reddish or brown color may indicate the presence of maternal or fetal blood contamination.

Acceptable specimen handling from the laboratory should be specified, including acceptable aliquot tubes, mode of transportation (e.g., courier, U.S. mail, overnight transport), and temperature. AFP in amniotic fluid is not as stable as in maternal serum. Consequently, amniotic fluid samples should not be held at room temperature any longer than necessary. Samples should be shipped on cool packs or kept frozen. Avoid exposure to high temperatures if possible. Next-day delivery is advisable. AChE and pseudocholinesterase (PChE) are reasonably stable.

## S.3 Specimen processing and storage

Criteria for sample rejection must be specified by the laboratory. Variables that may affect the acceptability of a sample for a specific afAFP testing protocol, or for AChE testing, should be established by the laboratory (e.g., interval between collection and receipt by the lab, environmental limits for temperature during transport). AFP and AChE can be reliably assayed in amniotic fluid stored at 4-8 ºC for days and at -20 ºC for years. Each laboratory should establish their own policies regarding specimen retention. Specimen processing protocols should be designed to avoid contamination, tampering or substitution. Handling samples must be in accordance with OSHA guidelines, with the express understanding that any human fluids may harbor infectious agents.

## S.4 Assay methodologies

Because afAFP levels are approximately 100 times higher than msAFP levels in maternal serum at 16 weeks, samples must generally be diluted 1:50 to 1:200, depending on the assay methodology used.

Acetylcholinesterase is a second diagnostic test employed along with afAFP to detect ONTDs. AChE testing is indicated when the afAFP levels exceed a specific cut-off level; typically, 2.0 MoM. AChE is often handled as a referral test as only a small number of laboratories currently offer this testing. Positive controls are AChE positive or weak positive samples that have been coded to protect patient confidentiality, aliquoted, and stored frozen at -20 ºC. Negative samples or RACs from recent assays can be used as negative controls.

Standards and calibration procedures and the value of RACs for amniotic fluid are covered above in the section on 2nd trimester screening. An in-house pooled amniotic fluid control should be prepared, aliquoted, and stored frozen at -20 ºC. This material should be run with each batch of amniotic fluid samples as a control of the dilution process, whether the assay is performed manually or on-board an automated system.

## S.5 Quality control

Standard approaches used in the clinical laboratory are appropriate for internal QC/QA of afAFP, and AChE. As part of the initial method validation, the laboratory should demonstrate that intra- and inter-assay variation reported by the AFP kit manufacturer can be reproduced. Standard approaches to routine equipment calibration and preventive maintenance used in the clinical laboratory are appropriate. In many cases, calibration and maintenance protocols are set by the product/equipment manufacturer.

## S.6 Assay results

For afAFP measurements to be interpreted, they are first converted to MoM for a given gestational week. It has been established that afAFP values obtained by different kit manufacturers or different lots from the same manufacturer may demonstrate systematic bias. Therefore, it is essential that each laboratory establishes its own normative data or at least demonstrates that data obtained from another source are appropriate for its screened population. The afAFP concentrations decrease 13% to 15% per week from 15 to 22 weeks of gestation. Package insert (commercial) medians should not be used, even for a short time.

Historically, 100 samples for each gestational week from 15 through 20 would be used to calculate afAFP median values. Because afAFP is stable, it is possible to use stored frozen specimens collected over several years. However, collecting the above number of amniotic fluid samples is difficult, particularly at the later gestational weeks, and fewer samples are likely to be available. Regression analysis allows use of fewer samples (e.g., 300 over the 15- to 20-week period) to establish reasonable medians.

As with msAFP medians, smoothing the observed median afAFP values by weighted log-linear regression analysis (logs of medians regressed versus gestational age in completed weeks, weighted by the square root of the number of observations in each category) provides reliable and accurate medians, and allows median values to be extrapolated for later gestational weeks in which little data are available and for which the log-linear relationship continues to be valid. Median afAFP values from 12 to 14 weeks do not follow the same pattern as the medians begin to drop. However, methods have been published for extrapolating medians for these earlier weeks based on the smoothed medians obtained for 15 to 20 weeks.70

Abnormal afAFP results typically are defined as elevations of 2.0 MoM or greater. afAFP results can be reported in MoM levels from 11 to 25 weeks of gestation. The optimal time for detection of ONTDs is 13 to 22 weeks of gestation. The accuracy of testing before 13 weeks is not well established and results should be reported with a warning that diagnostic performance is poorly defined.71,72 For calculation of afAFP MoM levels, gestational age is most commonly expressed in completed weeks (e.g., 15 weeks and 5 days is 15 completed weeks).

AChE and PChE testing are usually performed via gel electrophoresis. A negative AChE test result is defined by the presence of a pseudo cholinesterase (PChE) band and no AChE band. A positive test is defined by the presence of a PChE band and a weak or strong positive AChE band. The test is considered uninterpretable if the PChE band is absent or very weak. This pattern may be observed in amniotic fluid samples that are diluted with excessive fetal urine (e.g., hydronephrosis), obtained very early in pregnancy, or mishandled (e.g., exposed to heat in transit). The AChE band is generally confirmed by adding a specific AChE inhibitor (e.g., BW284C51) that would cause the AChE band to disappear.

AChE results can most reliably be interpreted between 13 and 22 weeks of gestation.72,73 While testing at 11 to 12 weeks has some predictive power, the detection rate for open defects is not well defined, and the false positive rate for AChE may be significantly higher at this time.72 A limited interpretation, with appropriate cautions, can be provided for amniotic fluid samples at 23 weeks and later.

In rare cases, falsely positive AChE result may result from contamination of amniotic fluid with fetal calf serum (FCS).74,75 The usual source of FCS contamination of the amniotic fluid aliquot is a tube or pipette in the cytogenetics laboratory that has a residue of culture medium.

The relative densities of the AChE and PChE bands, expressed as the AChE/PChE ratio, are useful in distinguishing between ONTD and open ventral wall defects (OVWD).76-78 Nearly all ONTDs have PChE and AChE bands and AChE/PChE ratios greater than 0.13. Nearly all OVWDs have unusually heavy PChE bands and weak positive AChE bands, and ratios of 0.13 or lower. Ratios do not help to identify false positive AChE bands, found more commonly in early amniocentesis (11 to14 weeks) and in samples contaminated with fetal blood.

Because afAFP MoM values are calculated using reference data collected in the past, it is important that the kits/reagents selected are stable over a long period of time and that lot-to-lot variability is minimized. It is also important to determine that automated systems can perform the necessary dilutions of amniotic fluid samples and achieve the precision and relative accuracy needed. In-house amniotic fluid pooled dilution control and repeat assay controls are valuable for monitoring long-term assay drift and lot-to-lot variability.

Median values should be reviewed at regular intervals by the Laboratory Director and recalculated at least annually. Medians should be recalculated if there is a significant shift in afAFP values (whether due to observed assay drift or reagent lot change). Observations from the past should be used only when epidemiological monitoring shows the median MoM has been stable. Alternative methods of revising medians like those used in maternal serum AFP values may be necessary if a significant shift has been observed. Comparison data for afAFP can be used applying the same procedure as for msAFP.

As part of epidemiological monitoring, each laboratory should periodically (weekly or monthly, depending on the number of samples tested) compute the median amniotic fluid AFP MoM, determine the statistical significance of any deviation from 1.00, and document any necessary corrective action.

Failure rates for amniotic fluid AFP and AChE testing are low. Testing failures are usually related to inadequate specimens, due to sample mishandling (e.g., heating), gross contamination with fetal blood, or a specimen that is not amniotic fluid (e.g., urine).

Each laboratory must participate in an external proficiency testing program(s) that evaluates assay performance for afAFP and AChE (if performed in-house), or must utilize other recommended external proficiency testing methods, (e.g., scheduled inter-laboratory comparisons; or split sample analysis with another laboratory).

Many of the issues surrounding analytic validity (analytic sensitivity and specificity), confirmatory testing, assay robustness, clinical validity (clinical sensitivity and specificity) are addressed in the section on 2nd trimester msAFP screening.

Using a cut-off level of 2.0 MoM, detection rates for OSB are expected to be 90% at 13 to 15 weeks, 96% at 16 to 18 weeks, and 99% at 19 to 21 weeks. Associated false positive rates are expected to be about 1%, 2% and 5%, respectively. The detection rate for anencephaly at this cut-off level is even higher.

When AChE testing is performed on all amniotic fluid specimens that have an afAFP level of 2.0 MoM or greater and are free of fetal blood contamination, the overall detection rate for OSB is 96% with a false positive rate of about 0.14%. In the presence of visible blood contamination, the detection and false positive rates for OSB are about 91% and 1.2%, respectively. The detection rate for anencephaly using this algorithm is even higher. Among women with a positive msAFP result the prevalence (PPV) is about 1 in 40. Thus, the overall positive predictive power of subsequent afAFP testing that includes AChE testing in samples that are not bloodstained would be about 6% [(96% / 0.14%) x 1 in 40, or about 1 in 17]. This is somewhat lower than published estimated odds of 40:179 likely due to the reduced prevalence of ONTD resulting from current folic acid fortification and supplementation.

## S.7 Result reporting

Final reports of test results must be clear to a non-geneticist professional and must include:

- Patient's name, date of birth and other identifying information;

- Name of referring health care provider/health center to receive the report;

- Type of specimen;

- Date when sample was obtained;

- Accession number(s) to uniquely identify the sample;

- Pregnancy-related information used in the interpretation (e.g., gestational age);

- Analytic results in both mass units (e.g., KIU/mL or µg/mL) and interpretive units (i.e., MoM);

- Normative ranges for afAFP measurements should be reported in multiples of the median (MoM).  The normative range would be less than the screening laboratory’s afAFP MoM cut-off level that usually falls in the range of <2.0 to <2.5 MoM;

- Clinical interpretation including recommendations for further studies.

Negative results are usually not associated with reflexive testing, although there are situations (e.g., family history of NTD) in which AChE is routinely run even if afAFP results are not elevated. Written reports of negative results can be transmitted to the referring health care provider by U.S. mail, courier, electronic transmission, or overnight carrier.

All positive test results (i.e., elevated afAFP and/or positive AChE) should be promptly transmitted to the referring health care provider, by some method that ensures prompt receipt by the referring provider, usually by phone and/or fax. Whenever possible, direct electronic transmission to the patient’s electronic medical record is preferred. Optimally, elevated afAFP results are not reported until all reflexive testing is completed. If preliminary afAFP results are reported, there should be a clear indication on the report that full interpretation is not possible until the results of reflexive AChE testing are complete.

Conditions other than ONTD that are associated with elevated afAFP and/or positive AChE include:

- Normal variation;

- Open ventral wall defects – Gastroschisis is associated with very elevated afAFP; AChE is positive in over 95% of cases with A/P ratio less than 0.13. Omphalocele can have normal to elevated afAFP; AChE is negative in over 95% of cases but A/P ratio is less than 0.13 when present;

- Congenital nephrosis – This is a rare condition associated with very elevated afAFP and negative AChE;

- Fetal demise – AChE may be present, usually presenting as a smeared and streaked pattern with some prominent anodal bands;

- Cystic hygroma fluid – Inadvertent aspiration of hygroma fluid will result in very elevated afAFP level and positive AChE (similar to anencephaly);

- Fetal blood contamination;

- Maternal blood contamination – Can occasionally result in a cholinesterase band in the AChE region, but this band will not be inhibited by BW284C51;

- Fetal calf serum contamination.

Rarely, very low values of afAFP are identified. Aspiration of urine from the maternal bladder may result in extremely low values. Application of the fern test will help to discriminate between urine and amniotic fluid. The primary issue when urine is identified is that no meaningful clinical testing of the amniotic fluid has been performed. In addition, congenital absence of AFP has been reported in a handful of cases, with no evidence of harm to the fetus.

S.8. References

70.	Palomaki GE, Knight GJ, Haddow JE. Calculating amniotic fluid alpha-fetoprotein median values in the first trimester. Prenat Diagn. 1993;13(9):887-889.

71.	Burton BK, Nelson LH, Pettenati MJ. False-positive acetylcholinesterase with early amniocentesis. Obstet Gynecol. 1989;74(4):607-610.

72.	Crandall BF, Chua C. Detecting neural tube defects by amniocentesis between 11 and 15 weeks' gestation. Prenat Diagn. 1995;15(4):339-343.

73.	Drugan A, Syner FN, Greb A, Evans MI. Amniotic fluid alpha-fetoprotein and acetylcholinesterase in early genetic amniocentesis. Obstet Gynecol. 1988;72(1):35-38.

74.	Brock DJ, Barlow RD, Wald NJ, et al. Fetal calf serum as a cause of false positive amniotic fluid acetylcholinesterase gel tests. Lancet. 1982;2(8306):1044.

75.	Barlow RD, Cuckle HS, Wald NJ. The identification of false-positive amniotic fluid acetylcholinesterase results due to fetal calf serum contamination. Br J Obstet Gynaecol. 1984;91(10):986-988.

76.	Goldfine C, Miller WA, Haddow JE. Amniotic fluid gel cholinesterase density ratios in fetal open defects of the neural tube and ventral wall. Br J Obstet Gynaecol. 1983;90(3):238-240.

77.	Goldfine C, Haddow JE, Hudson GA, Miller WA. Densitometry as an aid in amniotic fluid gel acetylcholinesterase analysis. Am J Obstet Gynecol. 1983;145(3):317-318.

78.	Wald NJ, Barlow RD, Cuckle HS, Turnbull AC, Goldfine C, Haddow JE. Ratio of amniotic fluid acetylcholinesterase to pseudocholinesterase as an antenatal diagnostic test for exomphalos and gastroschisis. Br J Obstet Gynaecol. 1984;91(9):882-884.

79.        Wald N, Cuckle H, Nanchahal K. Amniotic fluid acetylcholinesterase measurement in the prenatal diagnosis of open neural tube defects. Second report of the Collaborative Acetylcholinesterase Study. Prenat Diagn. 1989;9(12):813-829.