# Origami-based self-folding

# of co-cultured NIH/3T3 and HepG2 cells

# into 3D microstructures

# Qian He1, Takaharu Okajima2, Hiroaki Onoe3, Agus Subagyo4,

# Kazuhisa Sueoka2 and Kaori Kuribayashi-Shigetomi1*

# 1 Institute for the Advancement of Higher Education, Hokkaido University, Sapporo, Japan

# 2 Graduate School of Information Science and Technology, Hokkaido University, Sapporo, Japan

# 3 Department of Mechanical Engineering, Faculty of Science and Technology, Keio University, Tokyo, Japan

# 4 Creative Research Institution Sousei, Hokkaido University, Sapporo, Japan

# * Corresponding author: kaorik@ist.hokudai.ac.jp

Supplementary Figure 1. The process of producing the microplates with the alginate layer. a) The alginate was coated on the glass substrate followed by parylene and aluminium deposition. b-c) After photoresist coating, the shape of the microplates can be patterned by UV exposure and aluminium etching. d) After the exposed parylene was removed by O2 plasma, the shape of the microplates was revealed. e) MPC polymer was coated on the entire surface of the glass substrate. f) Finally, the aluminium with the MPC polymer on the microplates was removed to reveal the final parylene microplate.

Supplementary Figure 2. Co-culture in folding and unfolding groups. a) Pentagon-shaped microplates were produced by the microfabrication technique. b-c) NIH/3T3 cells were seeded and cultured on the microplates for 24 h followed by HepG2 seeding. d-e) Alginate lyase removes the superficial layer, allowing the co-culture cells on the microplates to form a 3D microstructure.

Optimization of initial cell seeding concentration of HepG2 cells

We performed experiments to detect the secreted albumin from both folding and unfolding groups, using 1×105 cells/ml, 5×105 cells/ml and 2.5×106 cells/ml as CH. The HepG2 cells could not be fully wrapped inside the 3D microstructures when using 2.5×106 cells/ml; therefore, CH should be less than 2.5×106 cells/ml.

Supplementary Figure 3. Secreted albumin from co-culture cells with different CH. The secreted albumin in both groups was much lower when using 1×105 cells/ml than when using 5×105 cells/ml as CH, and the significant difference in folding and unfolding groups was found on both the 5th and 7th days.