SUPPLEMENTARY INFORMATION

Prediction of Steam Burns Severity using Raman Spectroscopy on ex vivo Porcine Skin

Lina Zhai1,2, Christian Adlhart3, Fabrizio Spano2, Riccardo Innocenti Malini2, Agnieszka K. Piątek2, Jun Li1,4 and René M. Rossi2

1Protective Clothing Research Center, College of Fashion and Design, Donghua University, 200051, Shanghai, China

2Empa, Swiss Federal Laboratories for Materials Science and Technology, Laboratory for Biomimetic Membranes and Textiles, CH-9014, St. Gallen, Switzerland

3Institute of Chemistry and Biotechnology, Zurich University of Applied Sciences, ZHAW, CH-8820, Wädenswil, Switzerland

4Key Laboratory of Clothing Design & Technology, Ministry of Education, Shanghai 200051, China

Correspondence should be addressed to R. M. Rossi (rene.rossi@empa.ch)

（a）                                  （b）

Figure S1 | Typical baseline corrected Raman spectra of human skin and porcine ear skin before steam exposure. (a) Surface (0 μm) (b) Viable epidermis (40 μm)

The typical baseline corrected Raman spectra profiles of human skin and porcine ear skin before exposure are shown in Fig.S1. The CH-vibration signals for protein (2910 - 2966 cm-1) and OH-vibration signals (3350 - 3550 cm-1) for water are similar with that of human skin1. This further supported the observation of Tfaili et al2. They found that Raman vibrations at 1047 cm-1 and 1526 cm-1 were the only vibrations that presented significant differences. As the water content is calculated3 based on the range of 2910 - 3550 cm-1, the porcine skin is viewed as a good substitute for human skin.

Figure S2 | Change in porcine skin SC thickness after different steam exposure times

There is an approximately linear increase of SC thickness during steam exposure. SC thickness increased from (20.7 ± 2.1) μm to (24.6 ± 1.8) μm (increased by 3.9 μm) after 3 min;

Reference

1.	Dąbrowska, A. K. et al. In vivo confirmation of hydration-induced changes in human-skin thickness, roughness and interaction with the environment. Biointerphases 11, 31015 (2016).

2.	Tfaili, S. et al. Confocal Raman microspectroscopy for skin characterization: a comparative study between human skin and pig skin. Analyst 137, 3673–3682 (2012).

3.	Egawa, M., Hirao, T. & Takahashi, M. In vivo estimation of stratum corneum thickness from water concentration profiles obtained with Raman spectroscopy. Acta Derm. Venereol. 87, 4–8 (2007).