An integrated life cycle and water footprint assessment of nonfood crops based bioenergy production

Jun Li a, e *#,Fengyin Xiong b,d#  Zhuo Chen c *

a School of International Relations, Sun Yat-sen University, Guangzhou, China.

b  Key Laboratory of Energy Plants Resource and Utilization, Ministry of Agriculture, South China Agricultural University, Guangzhou, 510642, China.

c Institute of Communication and Global Public Opinion, Xi’an International Studies University, Xi’an, 710061, China.

d State Key Laboratory of Urban and Regional Ecology, Research Center of Eco-Environmental Science, Chinese Academy of Sciences, Beijing, 100085, China.

e School of Management, Curtin University, Australia.

*Corresponding authors

Jun Li  lijun267@sysu.edu.cn ; Zhuo Chen  jintak1988@hotmail.com

#JL and FYX contributed equally to the manuscript.

A1

Life cycle inventory of three crops cultivation systems. Values are presented as per functional unit.

|  | Categories | unit | Maize | Sorghum | Hybrid Pennisetum |
| --- | --- | --- | --- | --- | --- |
|  | Seeds | kg | 7.92 | 0.67 | - |
|  | Fertilizer a | kg | 55.44 | 19.71 | 97 |
| Input | Pesticide | kg | 1.32 | 0.05 | 2.03 |
|  | Diesel b | kg | 36.74 | 12.65 | 21 |
|  | Electricity c | kwh | 680.62 | 141.6 | 1462.9 |
|  | yield | ton | 13.3 | 4.05 | 14.98 |
| Emission to air |  |  |  |  |  |
|  | Carbon monoxide | g | 23.25 | 7.99 | 13 |
|  | Nitrogen oxides | kg | 1.89 | 0.73 | 2.379 |
| Output | Sulfur oxides | g | 147.74 | 50.62 | 84 |
|  | Carbon dioxide | kg | 117.78 | 40.29 | 67 |
|  | Methane | g | 0.25 | 0.09 | 0.15 |
|  | N2O | kg | 1.57 | 0.62 | 2.11 |
|  | NMVOC | g | 3.70 | 1.27 | 2.1 |
|  | Ammonia | g | 180.34 | 71.26 | 241 |
| Emission to water |  |  |  |  |  |
|  | Nitrate | kg | 2.10 | 0.82 | 2.795 |
|  | Phosphorus, total | g | 107.57 | 37.96 | 330 |

a N/P2O5/K2O kg

b machinery utilization including harrow, harvest, irrigation

c machinery utilization for irrigation

Data Source: (Huo et al., 2016); (Gao et al., 2012);(Mochizuki et al., 2014);(Brentrup et al., 2004);(Gaynor and Findlay, 1995); (Xiao et al., 2004); (Liang et al., 2009)

A2

Life cycle inventory of anaerobic digestion. Values are presented as per functional unit.

|  | Categories | unit | maize | sorghum | Hybrid Pennisetum |
| --- | --- | --- | --- | --- | --- |
|  | Lubricant | g | 277.7 | 245 | 277.2 |
|  | Heat | MJ | 1569.38 | 478.15 | 1765.88 |
| Input | Diesel | kg | 5.36 | 1.63 | 6.03 |
|  | Electricity | kwh | 61.7 | 18.8 | 69.42 |
|  | biogas | m3 | 526 | 556 | 500 |
| Emission to air |  |  |  |  |  |
|  | Carbon monoxide | g | 3.37 | 1.03 | 3.79 |
|  | Nitrogen oxides | g | 25.7 | 7.83 | 28.92 |
| Output | Sulfur oxides | g | 21.42 | 6.53 | 24.10 |
|  | Carbon dioxide | kg | 17.08 | 5.2 | 19.22 |
|  | Methane | kg | 13.57 | 11.95 | 13.55 |
|  | Nitrous Oxide | g | 315.69 | 278.03 | 278.1 |
|  | NMVOC | g | 0.54 | 0.16 | 0.6 |
|  | Ammonia | kg | 42.92 | 37.81 | 42.84 |

Data Source: (Jury et al., 2010); (Meyer-Aurich et al., 2012); (Lukehurst et al., 2010); (Yi et al., 2015)

A3

Life cycle inventory of digestate. Values are presented as per functional unit

|  | Categories | unit | maize | sorghum | Hybrid Pennisetum |
| --- | --- | --- | --- | --- | --- |
|  | Phosphate Fertilizer | kg | 199.04 | 270.5 | 517 |
|  | Nitrogen Fertilizer | kg | 486.3 | 661 | 1263 |
|  | Potash Fertilizer | kg | 170 | 231 | 441 |
| `Emission to air |  |  |  |  |  |
|  | Methane | mg | 828 | 1125 | 2150 |
|  | Ammonia | g | 6531 | 8875 | 16960 |
| Output | Nitrous Oxide | g | 510 | 692 | 1323 |
|  | Nitrogen oxides | g | 589 | 800 | 1529 |
|  | Carbon monoxide | g | 77 | 105 | 200 |
|  | NMVOC | mg | 12325 | 16750 | 32008 |
|  | Sulfur oxides | mg | 49117 | 66750 | 127556 |
|  | Carbon dioxide | kg | 395.5 | 537.5 | 1027 |
| Emission to water |  |  |  |  |  |
|  | Nitrate compounds | g | 6530 | 8875 | 16960 |
|  | Nickel | g | 653 | 887.5 | 1696 |
|  | Copper | g | 618 | 840 | 1605 |
|  | Cadmium | g | 26 | 35 | 67.6 |
|  | Chromium | g | 65.3 | 88.75 | 170 |
|  | Plumbum | g | 260.3 | 354 | 676 |

Data Source:(Lukehurst et al., 2010); (Wang et al., 2002); (Liu et al., 2003); (Zeng et al., 1994); (Xu et al., 2012); (Su, 2005); (Yi et al., 2015)

A4

Life cycle inventory emissions

|  | Unit | S1 | S2 | S3 |
| --- | --- | --- | --- | --- |
| NOX | g | 113.4 | 110.16 | 124.2 |
| methane | g | 67.83 | 65.892 | 74.29 |
| NMVOC | g | 2.94 | 2.856 | 3.22 |
| Carbon monoxide | g | 57.33 | 55.692 | 62.79 |
| Nitrous oxide | g | 0.8316 | 0.80784 | 0.9108 |
| Formaldehyde | g | 4.4415 | 4.3146 | 4.8645 |
| sulfur dioxide | g | 3.99 | 3.876 | 4.37 |

Data source：(Kristensen et al., 2004)；(Eggleston et al., 2006)

A5

Weather data for water footprint calculation

| City：Yantai Altitude：20m Latitude：37。N Longitud:120。E |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Min Temperature | Max Temperature | Humidity | wind | Sun | Rainfall | Rainfall |
|  | ℃ | ℃ | % | km/day | hours | mm | Days |
| January | -12.8 | 14.6 | 68 | 363 | 6.0 | 11.5 | 6.4 |
| February | -12.6 | 19.8 | 66 | 346 | 6.4 | 9.7 | 5 |
| March | -8.1 | 25.0 | 41 | 337 | 7.1 | 16.5 | 4.7 |
| April | -2.6 | 31.9 | 43 | 363 | 7.4 | 35.7 | 6.2 |
| May | 6.6 | 35.3 | 56 | 346 | 7.9 | 49.9 | 6.2 |
| June | 11.5 | 38.0 | 61 | 294 | 7.3 | 70.0 | 8.5 |
| July | 14.7 | 36.9 | 77 | 251 | 5.9 | 150.0 | 11.7 |
| August | 15.0 | 36.2 | 65 | 251 | 7.2 | 161.6 | 10.3 |
| September | 10.7 | 32.4 | 60 | 277 | 7.3 | 83.7 | 7.1 |
| October | 0.8 | 30.4 | 49 | 337 | 7.1 | 39.0 | 6.2 |
| November | -4.9 | 25.1 | 49 | 372 | 6.3 | 25.1 | 6.2 |
| December | -10.8 | 18.8 | 46 | 380 | 5.9 | 6.2 | 7.5 |
| Average | 0.6 | 28.7 | 57 | 326 | 6.8 | 55.1 | 7.2 |

A6

List of Acronyms and Abbreviations

AD		Anaerobic digestion

ALO	Agricultural land occupation

CC		Climate change

CHP	Combined heat and power

DM		Dry materials

FAO	Food and Agriculture Organization

FD		Fossil depletion

FE		Freshwater eutrophication

FET	Freshwater ecotoxicity

FU		Functional unit

GHG	Greenhouse gas

GWP	Global warming potential

HT		Human toxicity

IPCC	Intergovernmental Panel on Climate Change

IR		Ionizing radiation

LCA	Life cycle assessment

MD		Metal depletion

ME		Marine eutrophication

MET	Marine ecotoxicity

NLT	Natural land transformation

OD		Ozone depletion

PMF	Particulate matter formation

POF	Photochemical oxidant formation

TA		Terrestrial acidification

TET	Terrestrial ecotoxicity

TS		Total solid

ULO	Urban land occupation

VS		Volatile solid

WD		Water depletion

WF		Water footprint

WFb	Blue water footprint

WFg	Green water footprint

WFgr	Grey water footprint

References

- Brentrup, F., Küsters, J., Lammel, J., Barraclough, P., Kuhlmann, H., 2004. Environmental impact assessment of agricultural production systems using the life cycle assessment (LCA) methodology II. The application to N fertilizer use in winter wheat production systems. European Journal of Agronomy 20(3), 265-279.

- Eggleston, S., Buendia, L., Miwa, K., Ngara, T., Tanabe, K., 2006. 2006 IPCC guidelines for national greenhouse gas inventories. Institute for Global Environmental Strategies Hayama, Japan.

- Gao, H., Hu, S., Li, Y., Chen, D., Zhu, B., 2012. Greenhouse gas emission of sweet sorghum ethanol in life-cycle. Transactions of the Chinese Society of Agricultural Engineering 28(1), 178-183.

- Gaynor, J., Findlay, W., 1995. Soil and phosphorus loss from conservation and conventional tillage in corn production. Journal of environmental quality 24(4), 734-741.

- Huo, L., Zhao, L., Meng, H., Yao, Z., Cong, H., Wang, G., 2016. life cycle assessmnet analysis for cogeration of fuel gas and biochar. Transactions of the Chinese Society of Agricultural Engineering 26(1), 261-266.

- Jury, C., Benetto, E., Koster, D., Schmitt, B., Welfring, J., 2010. Life Cycle Assessment of biogas production by monofermentation of energy crops and injection into the natural gas grid. Biomass and Bioenergy 34(1), 54-66.

- Kristensen, P.G., Jensen, J.K., Nielsen, M., Illerup, J.B., 2004. Emission factors for gas fired CHP units< 25 MW. IGRC, November.

- Liang, L., Chen, Y., Gao, W., Sui, P., Chen, D., Zhang, W., 2009. Life cycel assessment impact assessment in winter wheat-summer maize system in north China plain. Journal of Agro-Environemnt Science 28(8), 1773-1776.

- Liu, X., Ju, X., Zhang, F., Pan, J., Christie, P., 2003. Nitrogen dynamics and budgets in a winter wheat–maize cropping system in the North China Plain. Field Crops Research 83(2), 111-124.

- Lukehurst, C.T., Frost, P., Al Seadi, T., 2010. Utilisation of digestate from biogas plants as biofertiliser. IEA bioenergy 2010, 1-36.

- Meyer-Aurich, A., Schattauer, A., Hellebrand, H.J., Klauss, H., Plöchl, M., Berg, W., 2012. Impact of uncertainties on greenhouse gas mitigation potential of biogas production from agricultural resources. Renewable Energy 37(1), 277-284.

- Mochizuki, J., Yanagida, J.F., Kumar, D., Takara, D., Murthy, G.S., 2014. Life cycle assessment of ethanol production from tropical banagrass (Pennisetum purpureum) using green and dry processing technologies in Hawaii. Journal of Renewable and Sustainable Energy 6(4), 043128.

- Su, J., 2005. Life cycle assessment on biomass-based ethanol fuel in China. Shanghai Jiaotong University, Shanghai.

- Wang, C., Liu, X., Ju, X., Zhang, F., 2002. Determination of Ammonia Volatilization from Wheat-mazie rotation system field in North China. Acta Ecologica Sinica 22(3), 359-365.

- Xiao, X., Feng, Q., Liu, Z., Wang, H., 2004. Enrichment characteristics of heavy metals(Cu、Pb、Zn、Cd) in wheat. Energy Environmental production 18(3), 28-31.

- Xu, Y., Tian, X., Li, D., 2012. The nutrient content compare of different raw biogas fermentation residues. Bulletion of Agricultural Science and Technology 5, 100-102.

- Yi, R., Zhang, W., Zhou, J., Li, W., Wu, S., 2015. Environmental impact analysis on the production and utilization of digestate based on LCA method. Renewable Energy Resources 33(2), 301-307.

- Zeng, J., Wang, Z., Zhang, Y., 1994. Flux of N2O emission from the fields in a wheat and maize rotation system. Chinese Journal of Environmental Science 16(1), 32-67.