Supplementary Materials

Simulator Acceleration and Inverse Design of Fin Field-Effect Transistor using Machine Learning

Insoo Kim1, So Jeong Park1, Changwook Jeong2, Munbo Shim2, Dae Sin Kim2, Gyu-Tae Kim1, Junhee Seok1,*

1School of Electrical Engineering, Korea University, Seoul, Korea

2Computational Science and Engineering Team,

Data and Information Technology Center, Samsung Electronics, Samsungjeonja-ro, Hwaseong-si,Gyeonggi-do 18448, South Korea

* Correspondence should be addressed to jseok14@korea.ac.kr

Table S1. Detailed description of design parameters, primary electromagnetic properties, and secondary electromagnetic properties of a FinFET device used in the experiments.

| Variable | Description | Value type |
| --- | --- | --- |
| WT | FinFET channel top width | Design parameter |
| WB | FinFET channel bottom width | Design parameter |
| TSi | FinFET channel thickness | Design parameter |
| VBg | Back gate Voltage | Design parameter |
| ID | Drain Current | Primary property |
| μ | Effective mobility | Primary property |
| QN | Electron charge density | Primary property |
| SSw | Minimum value of gradient of log ID versus VG | Secondary property |
| VTh | Voltage where ID and VG starts to show linear relation. | Secondary property |
| μDeg | The ratio of effective mobility due to the maximum effective mobility at specific voltage | Secondary property |

Table S2. Detailed description of the conditions of the FinFET simulator that generated the sample used throughout the experiments.

| Type | Equation | description |
| --- | --- | --- |
| Variable | V | effective voltage |
| Design | W_top = 1~W_bot | Channel top width (WT), nm |
| Design | W_bot = 10 ~ 250 | Channel bottom width (WB), nm |
| Design | tsi = 10~50 | Channel thickness (TSi), nm |
| Constant | tox = 1.2 | EOT-Gate oxide thickness, nm |
| Constant | tbox=145 | Real box thickness, nm |
| Constant | ratio = 10 | Ratio of the effective box |
| Constant | tboxeff = tbox/ratio | Unit, nm |
| Constant | L = 1000 | Device Length, nm |
| Constant | k = 86e-6 | k/q, re-Boltzman constant, 8.617e-5 V/K |
| Constant | T = 300 | Temperature, K |
| Constant | q = 1.6e-19 | Elementary charge, C |
| Constant | Nd = 1.4e-11 | Si doping, /nm3 |
| Constant | ni = 1.4e-11 | Intrinsic doping, /nm3 |
| Constant | Ng = ni | TiN gate doping, midgap material |
| Constant | Vfb=-k*T*ln(ni2/Ng/Nd) | Flat band voltage |
| Constant | epssi = 12*8.85e-21 | Si permittivity, F/nm |
| Constant | epsox = 4*8.85e-21 | SiO2 permittivity, F/nm |
| Constant | epsbox = epsox/ratio | Thickness compensation |
| Constant | eps0 = 8.85e-21 | Vacuum permittivity, F/nm |
| Equation | Cox=epsox/tox*(W_top+2*((((W_bot-W_top)*0.5)2+tsi2)0.5)) | 2D-GateOxide capacitance |
| Constant | mu0 = 400 | Max mobility |
| Constant | n0 = Nd | Electron concentration |
| Constant | p0 = ni2/Nd | Hole concentration |
| Equation | E = -∇V | Electric field in x axis |
| Equation | Emag = ¦E¦ | Magnitude of electric field |
| Constant | Ec = 0.1 | Critical electric field in high-doped Si, V/nm |
| Constant | alpha = 1.5 | Power law for mobility attenuation |
| Variable | Vg = 2.5*(stage-1)/50 -1 | Gate Voltage, -1~1.45V |
| Constant | Vd = 0.01 | Effective drain Voltage, V |
| Design | Vbox = 0~40 | Back gate Voltage, V |
| Equation | mu=mu0/(1+(Emag/Ec)alpha) | Mobility with degradation factor |
| Constant | n = n0*exp(V/kT) | Electron concentration in a field |
| Constant | p = p0*exp(V/kT) | Hole concentration in a field |
| Output | Qn = q(n,2) | Electron charge density (QN) |
| Equation | Gd = (mu*q*n,2) | Conductance |
| Output | Id = Gd * Vd | Drain current (Id), V |
| Output | Mueff = Gd/(Qn*L) | Effective mobility (μ) |
| Equation | V = Vg – Vfb | Line voltage, V |
| Equation | ∇ ° ε∇VG = 0 | Gauss Law |

Figure S1. Scatter plots of the semiconductor device designs (a) WT, (b) WB, (c) TSi, and (d) VBg of 1,000 test samples and predicted designs which are the output values of the inverse design model.