PEEK chamber 3D printing protocol

M106 P2 S1

M106 P3 S0.6 F5

M572 D0:1 S0.06 ;Pressure advance for PEEK CF

; Set extruder temperatures

G10 P0 S[extruder0_temperature] R[extruder0_temperature]

G10 P1 S[extruder1_temperature] R[extruder1_temperature]

; Set Chamber temperature

M141 S[extruder5_temperature]

T99 ; Disable all tools

T0 ; Choose the right extruder

G92 E0 ; zero extruder

G1 Z5 F5000 ; Raise bed into position

; M42 P5 S1 ; Enable Vacuum (disabled for now)

; END OF STARTING SCRIPT

; ENDING SCRIPT

G92 E0	; Set active tool's extruder position to 0

M83	; Set extruder to relative mode

G91	; Set axis to relative mode

G1 E-3 F600	; Retract 3mm

T99	; Disable tools

G28 Z0	; Home Z

G90	; Absolute mode

G10 P0 S0 R0	; Turn Extruder OFF

G10 P1 S0 R0	; Turn Extruder OFF

G28 X0

G1 Y180 F4800  ; Part to Front

M42 P4 S0	;Turn off VACUUM

M42 P5 S1	;Open VACUUM release valve

G4 S1	;Wait 1s

M42 P5 S0	;Close VACUUM release valve

M106 P0 S0 	; Turn off hot blower

M140 S0	; Turn off bed

M141 S0	; Turn off chamber