SUPPLEMENTARY

Imaging features

Lattice-based texture estimation adopted for lung 3D CT scan was used for feature extraction. Three sets of features were derived: Gray-level histogram, Co-occurrence, Run-length features.

Gray-level histogram features are first-order histogram statistics calculated from the gray-level intensity histogram of each image.

| Feature | Mathematical Notation | Qualitative description |
| --- | --- | --- |
| Mean | kk*g(k)K | Mean gray-level value |
| Min | min(k) | Minimum gray-level value |
| Max | max(k) | Maximum gray-level value |
| 5th Percentile | k: 5% of values ≤ k | The histogram bin that 5% of gray-level values are less than or equal to. |
| 5th Mean | kk*g(k)kg(k) for k ≤ 5th Percentile | Mean value of the gray-level values which less than or equal to the 5th Percentile. |
| 95th Percentile | k: 95% of values ≥ k | The histogram bin that 95% of gray-level values are less than or equal to. |
| 95th Mean | kk*g(k)kg(k) for k ≥ 95th Percentile | Mean value of the gray-level values which larger than or equal to the 95th Percentile. |
| Sum | kk*g(k) | Sum of gray-level values |
| Sigma | kk-Mean2*g(k) | Variation of gray-level values around the Mean |
| Entropy | -kgk*log⁡(g(k)) | Measure of histogram uniformity |
| Kurtosis | Sigma-4kk-Mean4*g(k)-3 | Measure of histogram flatness |
| Skewness | Sigma-3kk-Mean3*g(k) | Measure of histogram symmetry |
| where k is the histogram bin, g is the frequency of the histogram bin and K=kg(k). |  |  |

Co-occurrence features capture the spatial relationship between pixels and are based on the gray-level co-occurrence matrix (GLCM). In a GLCM, each element, f(i,j), corresponds to the frequency with which two neighboring pixels, one with gray level i and the other with  j, occur within a specified distance (d). In our study, the GLCM matrices were estimated using  different window sizes=4, 6, 8, 10, 12, 14, 16, 18, 20 pixels.

| Feature | Mathematical Notation | Qualitative description |
| --- | --- | --- |
| Cluster Shade | iji-µi+j-µj3*f(i,j) | Asymmetry in gray-level values |
| Correlation | iji-µi*j-µj*f(i,j)σi*σj | Linear gray-level dependence |
| Haralick Correlation | ijij*fi,j-µi*µjσi*σj |  |
| Energy | ijfi,j2 | Certainty of gray-level co-occurrence |
| Entropy | -ijfi,j*log⁡(fi,j) | Uncertainty of gray-level co-occurrence |
| Inertia | iji-j2*f(i,j) | Local variation of gray-level intensity |
| Inverse Difference Moment | ijf(i,j)1+i-j2 | Local homogeneity in gray-level values |
| where μi=ji*f(i,j), μj=ij*f(i,j), σi2=ji-μi2*f(i,j), and σj2=ij-μj2*f(i,j). |  |  |

Run-length features capture the coarseness of a texture in specified directions, where a run is defined as a string of consecutive pixels with similar gray-level intensity along specific linear orientation. Similar to GLCMs, a run-length matrix R is defined, with each element, Ri,j, representing the number of runs with pixels of gray-level intensity equal to i and length of run equal to j along the specific orientation. The size of the matrix R is M×N, where N is equal to the maximum run length. The following run-length statistics were estimated using  different window sizes=4, 6, 8, 10, 12, 14, 16, 18, 20 pixels.

| Feature | Mathematical Notation | Qualitative description |
| --- | --- | --- |
| Gray Level Non-uniformity | 1nri=1Mj=1NRi,j2 | Dissimilarity in runs across gray-level values |
| High Gray Level Run Emphasis | 1nri=1Mj=1NRi,j*i2 | Runs of high-gray-level values |
| Long Run Emphasis | 1nri=1Mj=1NRi,j*j2 | Emphasis on the long runs |
| Low Gray Level Run Emphasis | 1nri=1Mj=1NRi,ji2 | Runs of low-gray-level values |
| Run Length Non-uniformity | 1nrj=1Ni=1MRi,j2 | Dissimilarity in runs across lengths |
| Run Percentage | nr#pixels | Homogeneity and distribution of runs |
| Short Run Emphasis | 1nri=1Mj=1NRi,jj2 | Emphasis on the short runs |
| where nr=i=1Mj=1NRi,j is the total number of runs. |  |  |

Clustering Analysis

| W = 4mm |  |  |  |
| --- | --- | --- | --- |
| W = 8mm |  |  |  |
| W = 20mm |  |  |  |
| Fig. S1: Consensus clustering for different number of clusters (K = 2, 3, 4) for finding the best optimal cluster for patients with I130f kernel. |  |  |  |

Entanglement

| W = 4mm | W = 8mm | W = 20mm |
| --- | --- | --- |
| Fig. S2: Entanglement after applying feature harmonization comparing the two clusters between reconstruction kernels. |  |  |

Feature Harmonization

Outlier detection in the harmonization process resulted in 8 patients (images) being classified as outliers from the W = 4mm, 8mm, and 20mm window sizes in the two-reconstruction kernel dataset. Similarly, 12, 15, and 36 patients were detected as outliers from the W = 4mm, 8mm, and 20mm in the PFT dataset. The effects of dropping scans with residuals classified as outliers are shown for a sample representative feature (gray entropy) and covariate (BMI) from the two-reconstruction kernel dataset in Figure S3. The outlier removal resulted in a more even distribution of imaging features.

Figure S3. Residual plots for gray entropy versus BMI for the raw features (left) and the ComBat-harmonized features with the outliers dropped (right) for the two-reconstruction kernel dataset with window size 4mm.

Table S1. Number of features with statistically significant differences in distribution due to reconstruction kernel as detected with the KS test (p < 0.05) for the two-reconstruction kernel dataset.

Table S2. Number of features with statistically significant differences in distribution due to the reconstruction kernel as detected with the KS test (p < 0.05) for the PFT dataset.

CT Acquisition Parameters

Images used in study have voxel size of 0.5mm x 0.5mm x 0.5mm. Some CT parameters are shown in Figure S4.

| Fig. S4. Some CT acquisition parameters used for image analysis |  |
| --- | --- |

## Availability of Data and Material

The datasets generated and/or analyzed during the current study are not publicly available due to privacy and its proprietary nature but are available from the corresponding author on reasonable request.