Supplementary

Mechanics variables definition

End-inspiratory transpulmonary pressure (PL,EI) was computed according to the formula

PL,EI = Pplat – Pes,EI

where Pplat is end-inspiratory plateau pressure and Pes, EI is end-inspiratory esophageal pressure.

Lung stress was defined as PL,EI.

The end expiratory lung volume (EELV) measure was based on the nitrogen washout/washin technique through dedicated software (FRC Inview, GE Healthcare). The principle of this technique is as follows: The volume of lung gas includes a volume of nitrogen (V (1) N2) that is determined by alveolar fraction of nitrogen (FAN2(1)) and by the EELV:

V(1)N2 = FAN2(1) X EELV

The alveolar fraction of nitrogen can change by changing the administered FiO2 (FAN2(2)), resulting in a new volume of nitrogen (V(2)N2) in the lung after a period of balance.

V(2)N2 = FAN2(2) X EELV

Assuming that after the change of FiO2 the EELV does not change, until a new balance of the alveolar gas is reached, the following equation can be written:

V(1)N2 – V(2)N2 = (FAN2(2) –FAN2(1)) X EELV

As the changes in FAN2 are specular to the changes in FiO2 the EELV can be calculated as:

EELV = ΔN2 (ml)/ΔFiO2

where ΔN2 is equal to the exhaled nitrogen after the change in FiO2 once equilibrium has been reached (20 breaths). The algorithm used by the Engstrom Carestation (FRC Inview, GE Healthcare) employs tidal concentration of oxygen and carbon dioxide to obtain an estimate of nitrogen concentration in expired and inspired air. For more details, see Chiumello et al.10 and Olegård et al. 20.

Global lung strain was defined according to the formula

Global strain = V/ functional residual capacity (FRC)

where V was computed as follows: EELV – FRC + Vt5,10 and FRC is functional residual capacity. FRC was considered as EELV at ZEEP.

Specific elastance was computed at each PEEP level according to the formula

Specific elastance = PL,EI /V/FRC)

Equilibrium of a pressurized composite shell

Consider a spherical shell made of n layers composed of incompressible isotropic hyperelastic materials. Let X=XR,Θ,Φ denote the position of a material particle in the undeformed conﬁguration, where the geometry is defined as

Ri≤R≤Re,  0≤Θ≤π, 0≤Φ≤2π,				(1)

with Ri and Re the inner and outer radii of the sphere, respectively. The sphere is divided into n layers, being the j-th layer comprised between the radii Rj and Rj+1, j=1, 2,…n, with R1=Ri and Rn+1=Re. Let hj:=Rj+1-Rj  denote the thickness of the j-th layer and εn+1≔hRi to denote the (small) ratio of total thickness h to internal radius Ri.

The multilayered shell is subject to a uniform internal pressure P. Let x=xr,θ,φ denote the position of a material particle in the deformed conﬁguration and let spherical symmetry be retained during inflation, so that a finite deformation of the shell is given by

r=fR,   θ=Θ,    φ=Φ.						(2)

The deformed geometry is described by

r1≤r≤rn+1,  0≤θ≤π, 0≤φ≤2π.				(3)

with r1 and rn+1 the inner and outer radii of the deformed conﬁguration, respectively.

The principal stretches are λr=drdR in the radial direction and λθ=λφ=rR=:λ  in the circumferential directions, respectively. Incompressibility requires that λr=λ-2 and radial deformation can be calculated as

fR=R3+r13-R1313.						(4)

By continuity, the latter equation holds for R1≤R≤Rn+1. We also introduce the following notation: rj≔fRj, and λj≔rjRj, j=1, 2,…n, being λj the stretches at the inner surfaces between the layers. Using Equation (4), these stretches can be related to the relative volume change ΔVV inside the sphere as follows:

λj=1+1(1+εj)3ΔVV1/3,				(5)

with εj≔(Rj-R1)/R1, j=1, 2, …n+1. Substituting the relation Rj=R1+k=1j-1hk into the expression of εj, we find

εj≔k=1j-1hkR1=εn+1k=1j-1χk,       j=2,…n+1,    			(6)

with χk≔hk/h the fraction of wall thickness of the k-th layer. For a very thin shell, the thickness fraction can be approximated by the volume fraction of the k-th layer up to a term O(εn+1). By definition ε1=0, thus the stretch at the inner surface is simply given by

λ1=1+ΔVV1/3.					(7)

Note that, in view of Equation. (7), we have λj>λj+1, j =1, 2…n+1.

The behavior of the material composing the j-th layer can be described by the strain energy density Wjλr,λθ,λφ, function of λr,λθ,λφ  via the strain invariants. The Cauchy principal stress components in the j-th layer are given by

σrj=λr∂Wj∂λr-pj,          σθj=σφj=λθ∂Wj∂λθ-pj,    j =1, 2…n+1,		(8)

where pj,  j=1, 2…n+1, are arbitrary hydrostatic pressures. Axial symmetry implies that the only nontrivial equilibrium equations are

dσrjdr+2(σrj-σθj)r=0,  j =1, 2, …n+1.     			(9)

After introducing the auxiliary functions wjλ≔Wjλ-2,λ,λ,  j=1, 2, …n+1, it can be shown that Equations (9) are equivalent to

dσrjdλ=-wjλ3-1,         j=1, 2, …n+1,				(10)

where the dot indicates differentiation with respect to λ21. After integrating and imposing the boundary conditions σr1λ1=-P and σrNλN+1=0, we obtain that

σr1λ=λλ1w1ll3-1 dl-P,    λ2≤λ≤λ1,     			(11)

σrjλ=λλjwjll3-1 dl+k=1j-1λk+1λkwkll3-1 dl-P,    λj+1≤λ≤λj,     j=2, …n+1,	(12)

P=j=1nλj+1λjwjll3-1 dl.					(13)

Equation (13) gives the internal pressure as a function of the stretches λj. Finally, given σrjλ from Equations (8,9), the circumferential stress can be calculated using the second of Equation. (8) as follows:

σθjλ=12λwjλ+σrjλ,   λj+1≤λ≤λj,   j=1, 2, …n+1.	(14)

As discussed in22, in normal test conditions alveoli can be considered thin shells with a thickness to radius ratio εn+1 close to 0.05. Thus, each εj in Equation (11) can be assumed to be a small parameter. Substituting Equations (6) and (5) into Equation (13) and expanding to first order with respect to εn+1 leads to the following approximation

P=  εn+1j=1nχjwj(λ1)λ12,					(15)

with λ1 given by Equation (7). This equation is the formula for inflation pressure for a multilayered balloon. The case of homogenous balloon is considered for example in 23 .

Constitutive modelling of collagen and elastin

Collagen fibers display a high nonlinear behavior and stiffening at large strains, while elastin and ground substance are softer. A simple choice for their strain energy densities is

W1λr,λθ,λφ=c1λr2+λθ2+λφ2-3,      				(16)

W2λr,λθ,λφ=c2λr2+λθ2+λφ2-33,      				(17)

with c1, c2 positive material constants. Following the previously introduced notation, in Equations (16, 17), the index 1 stands for material of the first layer, that is composed for elastin, while the index 2 stands for material of the second layer, composed of collagen. In the remaining of the paper, we will assume that the layers with odd index are composed of the same material of layer 1 (elastin) and layers with even index are composed of the same material of layer 2 (collagen), in such a way that Wi=W1 for i=3, 5, 7,…, and Wi=W2 for i=2, 4, 6,….

The choice provided by Equation (16,17) agrees with the constitutive model of lung parenchyma experimentally determined via uniaxial tension tests on living precision-cut rat lung slices 24, 25. The main difference between Equations (16), (17) and the constitutive model proposed by Rausch and coworkers 24 is that here we assume the materials to be incompressible, while they account for an energy term to control the volumetric change. We also note that the stress-strain response in traction of the constitutive model (17) can be interpreted as a phenomenological description of the toe region followed by gradual, nonlinear stiffening of curly collagen fibers, due to progressive recruitment of fibers characterized by different amounts of slackness26. For multiscale modeling of collagen and proteins with unfolding domains the reader is referred to the literature22, 26, 27, 28, 29. It is worth mentioning that while the material constant c1 is proportional to the initial elasticity modulus of elastin in traction test, the material constant c2 can not be thought to represent such a quantity for collagen. Indeed, the stress-strain relation in traction corresponding to the energy given by Equation (17) presents a vanishing slope at the origin, corresponding to zero initial elasticity modulus. So, an increase of c2 can be more appropriately interpreted as an increase of collagen stiffening occurring at larger strains.

Pressure-volume relation

Even though the present model allows for the description of multiple layers of collagen alternated to layers composed of elastin and ground material, for computational testing we assume the alveolus made of only three layers, the two external ones composed of elastin and ground material, and one in between composed of collagen.

We take χ2 to denote the thickness fraction of collagen, and 1-χ2 the remaining fraction of elastin and ground substance. With this notation, the pressure-volume for the shell model can be obtained by substituting Equations (16,17) into Equation (13). We find that

P=-g1λ2+g1λ1-g2λ3+g2λ2-g1λ4+g1λ3,		(18)

with

g1z:= w1ll3-1 dl=-c1z4-4c1z,						(19)

g2z≔ w2ll3-1 dl=3c2(-13z12-49z9+3z8-83z6+245z5-9z4-163z3+24z2-36z-48z+16z33+16lnz).

(20)

In Equation (18), the stretches λj,  j=1,2,3,4, defined by Equation (5), can be more conveniently rewritten in terms of χ2 and ΔVV  as follows:

λ2=1+11+  εn+1(1-χ2)/23ΔVV13,				(21)

λ3=1+11+  εn+1(1+χ2)/23ΔVV13, 				(22)

λ4=1+11+  εn+13ΔVV13. 					(23)

Notably, the isolated contributions of elastin (and ground substance) and collagen in Equation (18) are given by the terms containing the functions g1 and g2, respectively.

Radial and hoop stress distributions

For the three layers configuration constituted of a layer of collagen interposed between two outermost layers of elastin and ground material, the piecewise distribution of radial stress along the thickness of the deformed shell can be obtained by substituting Equations (16,17) into (11,12) to get the following relation:

σr1rr1=-P+g1lrr1-g1λ1,          ζ1≤rr1≤ζ2,σr2rr1=-P+g1λ2-g1λ1+g2lrr1-g2λ2,         ζ2<rr1≤ζ3,σr3rr1=-P+g1λ2-g1λ1+g2λ3-g2λ2+g1lrr1-g1λ3,     ζ3<rr1≤ζ4,	(24)

with

lz=z3z3-ΔVV1+ΔVV13,						(25)

and ζj≔rjr1, j=1,2,3,4, the ratios between the deformed radius rj at the surfaces between the layers and the inner deformed radius r1. Thus ζ1=1, while ζj, j=2,3,4 take the following form in terms of χ2 and ΔVV:

ζ1=ΔVV1+ΔVV+1+εn+1(1-χ2)/231+ΔVV13,					(26)

ζ2=ΔVV1+ΔVV+1+εn+1(1+χ2)/231+ΔVV13, 					(27)

ζ3=ΔVV1+ΔVV+1+εn+131+ΔVV13. 						(28)

From Equation (24), using Equation (14), the piecewise distribution of hoop stress along the thickness of the deformed shell can be shown to take the following expression:

σθ1rr1=12 lrr1 w1lrr1+σr1rr1,         ζ1≤rr1≤ζ2,σθ2rr1=12 lrr1 w2lrr1+σr2rr1,          ζ2<rr1≤ζ3,σθ3rr1=12 lrr1 w1lrr1+σr3rr1,         ζ3<rr1≤ζ4.	(29)

eFigure 1

eFigure 1. Study flow diagram.

EUIP, acute exacerbation of interstitial lung disease with usual interstitial pneumonia pattern; ARDS, acute respiratory distress syndrome; MV, mechanical ventilation; PEEP, positive end-expiratory pressure; EELV end-expiratory lung volume; BMI, body mass index

eTable 1

| Parameter | EUIP (n=5) | ARDS (n=5) | p value |
| --- | --- | --- | --- |
| Age, years | 62 (56 – 68) | 64 (53 – 70) | 0.9 |
| Male, n | 4 (80) | 4 (80) | 0.9 |
| BMI, kg/m2 | 22.3 (20.8 – 25) | 22.4 (20.8 – 23.8) | 0.8 |
| Height, m | 1.77 (1.71 – 1.84) | 1.83 (1.79 – 1.85) | 0.2 |
| Charlson index, score | 4 (3 – 5) | 4 (3 – 5) | 0.9 |
| APACHE, score | 13 (11.5 – 14) | 14 (12.5 – 14) | 0.7 |
| SAPS II, score | 27 (24.5 – 32.5) | 31 (24 – 32) | 0.6 |
| †PaO2/FiO2, mmHg | 90 (77 – 115) | 85 (73 – 111) | 0.8 |
| RICU/ICU survival, n | 0 (0) | 3 (60) | 0.01 |

eTable 1. General and clinical characteristics in the study groups on admission. Data are presented as number (n) and percentage for dichotomous values or median and IQR for continuous values.

† The values of PaO2/FiO2 ratio used for matching these groups were those measured at the time of RICU or ICU admission

EUIP, acute exacerbation of interstitial lung disease with usual interstitial pneumonia pattern; ARDS, acute respiratory distress syndrome; BMI, body mass index; APACHE II, Acute Physiology and Chronic Health Evaluation II; SAPS II; Simplified Acute Physiology Score; RICU, respiratory intensive care unit; ICU, intensive care unit; IQR, interquartile range

eFigure 2

eFigure 2. Measured individual values of strain in the matched study groups at ZEEP, PEEPLOW and PEEPTITRATED phase. When testing as an interaction for whether the change in strain at incremental PEEP value was different between EUIP and ARDS, statistical difference was found (p=0.003).

ZEEP, zero positive end-expiratory pressure; PEEP, positive end-expiratory pressure; EUIP, acute exacerbation of interstitial lung disease with usual interstitial pneumonia pattern; ARDS, acute respiratory distress syndrome

Mechanical considerations on stress distribution

eFigure 3 and eFigure 4 show the piecewise distributions of the radial and hoop stress components, σr and σθ respectively, in three deformed (inflated) configurations of the alveolus, corresponding to three different values of the relative volume change ΔVV, with V=FRC.  The plots have been obtained by inserting the values of the material parameters listed in Table 2 into Equations (24-29). For normal lungs (DA and L data) and ARDS lungs, the following values of the relative volume change have been considered: ΔVV=0.25, 0.75 and  1.5. For AE-ILD-UIP lungs, characterized by a steeper pressure-volume response, the distributions have been plotted for ΔVV=0.25, 0.75 and  0.85.  In eFigure 3 and eFigure 4, the stress components σr and σθ have been represented versus the normalized radial position in the deformed configuration, rr1, with r the radial position and r1 the (deformed) radius of the internal shell surface. The normalized radial position ranges from 1 to ζ4, with r4=ζ4 r1 the (deformed) radius of the external shell surface. As ΔVV increases the shell gets thinner, and correspondingly ζ4 and the domain on the horizontal axis shrink.

Equilibrium at the interfaces between the layers implies the stress component σr to be a continuous function of rr1. Equilibrium at the boundary implies σr to equal -P at the inner surface and to vanish at the outer surface.

The discontinuity of the stress component σθ along the sphere thickness, i.e., with respect to rr1, is due to the inhomogeneity of the material. The plots on the bottom of eFigure 3 and eFigure 4 show that σθ takes values up to two orders of magnitude greater than σr. For a homogeneous alveolus, this effect would be due to equilibrium, as schematically represented on the right-hand side of Figure 1, where the case of a homogeneous sphere is shown. Considering for simplicity the case of a very thin shell (a balloon) of radius R and thickness h, force equilibrium requires 2πRhσθ=πR2P, implying σθ=RP2h. For a ratio h/R of the order of 0.05 (like in normal lungs, see Jawde et al. 22), σθ turns out to be one order of magnitude greater than the pressure P. The internal collagen layer, undergoing increasing stiffening for large strains, withstands an even larger stress, that can reach over two orders of magnitude the value of the internal pressure, as indicated by the distribution of hoop stress for the largest values ΔVV in the plots on the bottom of eFigure 3 and eFigure 4.

eFigure 3

eFigure 3. Piecewise distributions of radial and hoop stress in the deformed configuration of a three-layer alveolus, composed of two external layers of elastin and ground material and an inner layer of collagen. Black thick lines correspond to the collagen layer while the other lines correspond to the elastin and ground substance layers. Plots on the left column have been obtained using the material parameters estimated from L data (healthy lungs)15, plots on the right column using the material parameters estimated from EUIP data (Table 2).

EUIP, acute exacerbation of interstitial lung disease with usual interstitial pneumonia pattern; V=FRC, functional residual capacity.

eFigure 4

eFigure 4. Piecewise distributions of radial and hoop stress in the deformed configuration of a three-layer alveolus, composed of two external layers of elastin and ground material and an inner layer of collagen. Black thick lines correspond to the collagen layer while the other lines correspond to the elastin and ground substance layers. Plots on the left column have been obtained using the material parameters estimated from DA data (healthy lungs)14, plots on the right column using the material parameters estimated from ARDS data, cf. Table 2.

ARDS, acute respiratory distress syndrome; V=FRC, functional residual capacity.

References

5. Gattinoni, L., Tonetti, T. & Quintel, M. Regional physiology of ARDS. Crit. Care 21, 312 (2017).

10. Chiumello, D. et al. Lung Stress and Strain during Mechanical Ventilation for Acute Respiratory Distress Syndrome. Am. J. Respir. Crit. Care Med. 178, 346–355 (2008).

20. Olegård, C., Söndergaard, S., Houltz, E., Lundin, S. & Stenqvist, O. Estimation of Functional Residual Capacity at the Bedside Using Standard Monitoring Equipment: A Modified Nitrogen Washout/Washin Technique Requiring a Small Change of the Inspired Oxygen Fraction: Anesth. Analg. 101, 206–212 (2005).

14. D’Angelo, E. et al. Respiratory mechanics in anesthetized paralyzed humans: effects of flow, volume, and time. J. Appl. Physiol. 67, 2556–2564 (1989).

15.	 Levy, P. et al. A method for studying the static volume-pressure curves of the respiratory system during mechanical ventilation. J. Crit. Care 4, 83–89 (1989).

21. deBotton, G., Bustamante, R. & Dorfmann, A. Axisymmetric bifurcations of thick spherical shells under inflation and compression. Int. J. Solids Struct. 50, 403–413 (2013).

22. Jawde, S. B. et al. Inflation instability in the lung: an analytical model of a thick-walled alveolus with wavy fibres under large deformations. J. R. Soc. Interface 18, 20210594 (2021).

23. Beatty, M. F. Topics in Finite Elasticity: Hyperelasticity of Rubber, Elastomers, and Biological Tissues—With Examples. Appl. Mech. Rev. 40, 1699–1734 (1987).

24. Rausch, S. M. K., Martin, C., Bornemann, P. B., Uhlig, S. & Wall, W. A. Material model of lung parenchyma based on living precision-cut lung slice testing. J. Mech. Behav. Biomed. Mater. 4, 583–592 (2011).

25. Birzle A.M., Hobrack S.M.K., Martin C., Uhlig S., Wall A.W. Constituent-specific material behavior of soft biological tissue: experimental quantification and numerical identification for lung parenchyma. Biomech Model Mechanobiol 2019; 18: 1383-1400.

26. Fratzl, P. et al. Fibrillar Structure and Mechanical Properties of Collagen. J. Struct. Biol. 122, 119–122 (1998).

27. Marino, M. & Vairo, G. Multiscale Elastic Models of Collagen Bio-structures: From Cross-Linked Molecules to Soft Tissues. in Multiscale Computer Modeling in Biomechanics and Biomedical Engineering (ed. Gefen, A.) vol. 14 73–102 (Springer Berlin Heidelberg, Berlin, Heidelberg, 2013).

28. De Tommasi, D., Puglisi, G. & Saccomandi, G. Multiscale mechanics of macromolecular materials with unfolding domains. J. Mech. Phys. Solids 78, 154–172 (2015).

29. Puglisi, G., De Tommasi, D., Pantano, M. F., Pugno, N. M. & Saccomandi, G. Micromechanical model for protein materials: From macromolecules to macroscopic fibers. Phys. Rev. E 96, 042407 (2017).