Scan analysis conditions

LC Conditions:

- Mobile phase:

| Solvent C | ACN + 0.1 % Formic Acid |  |
| --- | --- | --- |
| Solvent B | H2O + 0.1 % Formic Acid |  |

- Column: C18 3µ, 4.6×150 mm

- Column temperature: 35 ̊C

- Flow rate: 0.3 ml/min

- Injection volume:  5  µL

MASS conditions:

Mode: ESI -/+

Cone Volt: 25 V

Capillary Volt: 4 kV

Extractor: 3 V

RF Lens: 0.1 V

Collision Energy: 30 eV

Gas nebulizer: N2 (grade 5)

Flow gas: 300 L/h

Source temperature: 120 ̊C

Desolvation temperature: 300 ̊C

Device model:

HPLC: Waters Alliance 2695

Mass Spectrometer: Micromass Quattro micro API