Extraction Method

1- Sample Storage: In the refrigerator (2-6 C°)

2- Sample preparation (Filtration):

| Step 1 | Membrane Filter 1 µm pore size 47 mm diameter |
| --- | --- |
| Step 2 | Membrane Filter 0.45 µm pore size 47 mm diameter |

Figure 1: Schematic representation of the sample preparation process for liquid chromatography-mass spectrometry (LC-MS) analysis.

These images illustrate the processes involved in preparing leachate samples for the analysis of pharmaceutical compounds using LC-MS

3- Solid Phase Extraction:

Extraction volume: 500 ml

Final Volume: 1 ml

Degree of enrichment: 500

ENV Cartridge:

BOND ELUT (6 mL, 500 mg Sorbent per Cartridge)

Figure 2: Packaging of Bond Elut-ENV (500 mg, 6 mL), manufactured by Varian for solid-phase extraction (SPE) processes.

The image shows a box of VARIAN Bond Elut-ENV solid phase extraction (SPE) cartridges. The specifications are:

- Part Number: 12255011

- Cartridge Specification: 500 mg, 6 mL

- Quantity: 30 cartridges per pack

- Lot Number: 9525507

- Manufacturing Code: 11508T

These SPE cartridges are used in sample preparation for isolating and concentrating analytes, particularly in the context of preparing samples for chromatographic analysis such as LC-MS.

Table 1: Sample Preparation Steps and Cartridge Conditions for LC-MS/MS Injection

| Cartridge conditions | 1) 4 mL Methanol 2) 4 mL Water |
| --- | --- |
| Sample loading | 5-10 mL/min |
| Washing | 1) 5 mL H2O |
| Elution | 6 mL methanol |
| Drying | N2 gas (grade 5) 40 Cº |
| Final volume | 1 mL MeOH |
| Filtration | Syringe filter PTFE, 0.22 µm pore size |
| Injection to LC-MS/MS | 5 µL |

Figure 3: Common steps for a solid-phase extraction (SPE) method: Conditioning, Loading, Washing, and Elution.

Figure 4: Vacuum manifold system used for solid-phase extraction (SPE) method in sample preparation.

Figure 5: Microplate Evaporator used for sample concentration under controlled temperature conditions.

Images Overview: Solid Phase Extraction (SPE) Method for Sample Preparation

- Common Steps for an SPE Method:

  - Condition: The cartridge is conditioned with solvents to activate the stationary phase.

  - Load: The sample is loaded onto the cartridge, allowing target analytes to bind to the stationary phase.

  - Wash: Unwanted components are washed off the cartridge, leaving only the bound analytes.

  - Elute: The analytes are eluted from the cartridge using a suitable solvent for further analysis.

- SPE Setup:

  - This image shows a multi-syringe SPE manifold setup, where multiple samples can be processed simultaneously. The manifold system applies vacuum to draw samples and solvents through the cartridges.

- Evaporation of Eluates:

  - This image depicts a microplate evaporator used to concentrate the eluates after SPE. The evaporation process removes solvents, leaving behind concentrated analytes ready for subsequent analysis, such as by LC-MS.

These steps and equipment are essential for preparing clean and concentrated samples, crucial for accurate and sensitive detection of target compounds in analytical methods

Extraction Method

Extraction volume: 500 ml

Final Volume: 1 ml

Degree of Preconcentration: 500